Source:	Meta Ireland (Rapporteur)

Title:	Applications of Microphone Description

Agenda Item:	4.3

Document for:	Discussion and Agreement

- Introduction

In this document we outline the applications of microphone descriptions discussed and agreed [2].

- Microphone Array Description

A microphone array description is provided in [1] and further extended below as an example. The sending or receiving iRTC client, or audio infra may identify the direction of each microphone from a set of angles and a type (e.g., pick up pattern). Alternatively they may identify the position of each microphone from a set of cartesian coordinates.

In many spatial audio formats, a microphone processing block converts raw signals from microphones into a device-independent representation.

The angles are expressed in units of 1/10000 radians. For example, 3.1416 () radians is expressed as 31416.

| Parameter | Unit | Definition | Note |
| --- | --- | --- | --- |
| Yaw (𝛹, psi) | int | Direction angle | -31416 < 𝛹 ≦ 31416 |
| Pitch (𝛳, theta) | int | Elevation angle | -31416 < 𝛳 ≦ 31416 |
| Roll (𝛷, phi) | int | Rotation angle | -31416 < 𝛷 ≦ 31416 |
| MicrophoneType | int | A number that uniquely identifies microphone type | May be used for indicating vendor-defined types |

These parameters can describe the orientation of each microphone relative to a camera, and its type. The rotation angle, not defined in [1], can be used to indicate the rotation of a directional microphone whose pick up pattern is not axisymmetric. It is assumed that the RGB or D cameras for capturing 2/3D video are located on a plane parallel to the ZY plane, pointing objects in the direction of X axis. This information on the direction of microphones (with respect to cameras) can be used for improved spatial capture of the sound field and subsequent post processing in terms of noise reduction or sound source decomposition.

- Differences in UE Form Factor

Although typical UEs (at the time of writing this contribution) have 2-3 microphones, they are rarely located on the same plane. Usually a microphone is located on top of a display while another is located at the bottom of UE. Sometimes a microphone is located on the other side of display (with main cameras). Therefore the Linear or Planar mode [1], designated with the wMicArrayType parameter, would not be applicable.

In some contemporary UEs, e.g., foldable devices, the direction or elevation angle may be different in each session or even vary during a session, and the sending or receiving iRTC client, or audio infra may need to be informed of such changes for proper rendering. In addition to the geometric description, if applicable, other or additional information may be beneficial to the microphone processing, e.g.:

- acoustic overload point (AOP) for estimating the loudness (pressure level) of sounds in recorded audio signals

- total harmonic distortion (THD) for understanding nonlinearity in captured audio signals

- microphone sensitivity response for potential post equalisation.

Editor’s Note: Need for transmitting this information will be discussed and studied further.

- Proposal

It is requested to include the texts explaining the application of microphone description in the permanent document and take them into account in the discussion of spatial audio capture and related topics.

- References

[1] Windows microphone array geometry descriptor format, Microsoft

[2] S4aR220022 Microphone description for iRTC client in the terminal (rev)