Title: 	[IBACS] Fixes to call flows in permanent document

Source: 	Nokia Corporation1

Document For: 	Agreement

Agenda item: 	4.4

# Introduction

The document proposes changes to permanent document to align the call flows to the agree basic AR call flow.

-----------Changes ---------

    - 4.1.1 Basic Call Flow

The figure shows the call flow for a basic AR call. Enhanced MRF consists of both the Data channel media function and the AR media function. The figure may be updated later based on SA2 work.

NOTE: The terminology in the figure should be aligned with SA2 work.

Figure 4.1.1.1 Basic AR Call Flow

The steps are as follows:

A. Call Setup

- The calling UE, in this case UE1, sends a SIP invite to the P-CSCF to initiate the call

- The P-CSCF identifies this as an AR call and forward the invite request to the MMTel AS

- The MMTel AS sends a request to the DCSF to request the data channel resources for the AR call data channel application.

- The DCSF selects a Enhanced MRF and requests the allocation of data channel resources and the execution of the AR call data channel application

- The Enhanced MRF confirms the successful allocation of resources and the DCSF forwards the confirmation to the MMTel AS

- The MMTel AS then forwards the invite to UE2 via the P/S/I-CSCF

- UE2 accepts the invitation and informs UE1. Both UE1 and UE2 establish a data channel connection and channels for media flows to the Enhanced MRF.

NOTE: Need to check with SA2 if the SDP answer from UE2 would go to MMTel AS so it can assign appropriate resources in the EMRF based on the response.

B. Scene description distribution

- Enhanced MRF prepares the scene description based on media descriptions and assets for the call.

- Enhanced MRF delivers the scene description to the UEs.

C. Scene description update

- A UE may trigger a scene update e.g., when a new object is added/removed in the scene, or a spatial information update is sent. The figure shows the update is triggered by UE1, but this can be either UE.

- The Enhanced MRF will process the new information and creates a scene description update. It is also possible for the Enhanced MRF to initiate an update without an update from the UEs.

- Enhanced MRF distributes scene description update to all UEs.

NOTE: Spatial data related updates may be required for collaborative AR calls, e.g., when multiple users are physically collocated and also part of the same AR experience. The type of spatial description updates is FFS.

D. AR Media and Metadata Exchange

- Both UEs will send and receive the AR media required (and negotiated) for the call as part of the call setup and scene description information received. Prestored media can be fetched from the Enhanced MRF. Real-time conversational media can flow via the Enhanced MRF. The Enhanced MRF may process the AR media before delivery.

- Related metadata (e.g. User Pose) may be needed during the AR session. The metadata may be delivered as RTP header extension, RTCP feedback or over data channel; this aspect is FFS.

NOTE: Delivery of real-time media over data channel is FFS.

NOTE: split rendering for UE1 and/or UE2 needs to adhere to the procedures and formats that are defined by the SR_MSE work item. The Enhanced MRF may be the entity that performs the split rendering for the UE.

    - 4.1.2 3D video call flow

Figure 4.1.2.1 Call flow for bidirectional 3D video call

Steps 1-12 under A. Call Setup, B. Scene description retrieval and C. scene description update will be the same as for the basic AR call flow. The remaining steps are defined below.

D. AR Media and Metadata Exchange

- UE1 will send 3D video (e.g., RGB-D streams, or volumetric media represented as mesh or point cloud) to the MRF.

- UE2 also sends 3D video to MRF.

- The MRF processes AR media, e.g., for viewport-dependent processing (VDP), combining AR objects into a single frame, transcoding from one 3D representation to another, etc. Note that VDP requires a feedback channel from UEs to MRF not shown in the call flow.

- MRF delivers 3D video to UE1.

- MRF delivers 3D video to UE2.

NOTE: Delivery of real-time media over data channel is FFS.

  - 4.2 AR call with real-time Animated Avatar

3D modelling of real-time objects can be done from a series of images of the object captured from different angles. For an AR call, these images may be provided by the user by capturing while moving the camera or by capturing with multiple cameras.

A 3D model of a humanoid can be represented using a 3D representation format, e.g., glTF. The representation contains vertices and associated mesh.

Skinning, or rigging, is a technique used for animating a 3D object.  A skin is created when a 3D model is bound to a skeleton.  The skeleton is a hierarchical representation of joints.  Each joint is bound to a portion of the mesh/point cloud, and a skin weight is assigned to the corresponding vertices.  The weight determines the influence of the skeletal joint transformation on each vertex (in case of mesh) or point (in case of point cloud) when the joints move.

To animate the 3D model, motion signals based on the user’s movement are generated. The method for generating motions signals depends on the implementation and is out of scope for this work. However, two possible methods are provided only as examples here. In the first method, the sender or MRF may compare the views of the input cameras to the views rendered from the 3D model using the same camera configuration.  The rendered 2D views overlaid with the actual views may display a transformation that may be encoded as motion signals.

In a second method, the sender may perform real-time human segmentation on one or more input cameras.  For example, the KINECT® Azure supports this out of the box.  The segmentation may provide joint transformation in 2D space, which may be converted into a 3D signal, when the camera configuration is known.  If more than one camera performs the joint tracking, the 3D joint transformation may be combined between different views to achieve better 3D motion tracking.  As an example, averaging of motion vectors per joint could be performed to improve the quality of the joint motion signal.

    - 4.2.1 MRF-assisted 3D modelling and skinning

Figure 4.2.1.1 shows a call flow where an MRF assists in creating a 3D model and motion signals from images received from UE1 and delivering it to UE2. The call flow is shown as unidirectional for clarity but can work also as bi-directional. MRF assistance alleviates the need for UE1 to process the images itself.

Figure 4.2.1.1: Call flow for avatar based AR call with MRF assistance for model and motion signal generation

Steps 1-12 under A. Call Setup, B. Scene description retrieval and C. scene description update will be the same as for the basic AR call flow. The remaining steps are defined below.

- UE1 will send source images (e.g., RGB or RGB-D streams) to the Enhanced MRF as an image or video stream. The media description of the streams contains the camera configuration as well.

- The Enhanced MRF processes the received images to create a rigged 3D model.

- The Enhanced MRF delivers the 3D model to UE2.

- UE1 continues sending source images (the stream in Step 16 and Step 19 are the same).

- The Enhanced MRF uses the 3D model and the new images to create motion signals i.e., transformation information.

- The Enhanced MRF delivers the motions signals to UE2.

- UE2 renders the 3D model with animation based on motion signals.

Figure 4.2.1.2 shows a call flow where an MRF assists in creating a 3D model from images received from UE1 and delivering it to both UEs. The call flow is shown as unidirectional for clarity but can work also as bi-directional. UE1 creates joint transformations to capture user motion locally on the device and sends motion signals to UE2.

Figure 4.2.1.2: Call flow for avatar based AR call with MRF assistance for model generation

- UE1 will send source images (e.g., RGB or RGB-D streams) to the Enhanced MRF as an image or video stream. The media description of the streams contains the camera configuration as well.

- The Enhanced MRF processes the received images to create a skinned 3D model.

- The Enhanced MRF delivers the 3D model to UE2.

- The Enhanced MRF delivers the 3D model to UE1. UE1 can stop sending images once the model is generated. The model is rigged already to a human skeleton.

- UE1 creates motion signals based on the motion of the user for the 3D model.

- UE1 delivers the motions signals to UE2 via Enhanced MRF.

- UE2 renders the 3D model with animation based on motion signals.

Requirements:

Based on the above call flows, an MRF-assisted 3D avatar conversational AR call has the following requirements:

- An SDP indication for generation of a 3D model by the MRF, including an indication that the MRF can further create motion signals. The SDP negotiation would include the media properties of the image stream provided by the source (sender UE).

- A media format for the source images. Any existing encoded video/image streams can be used for this purpose. Furthermore, a volumetric video stream may be used as well.

- Transport and format for the motion signals.

# Proposal

The proposal is to make the necessary changes to the permanent document of IBACS.