3GPP TSG-S4  ad hoc Meeting #Post 121-e	S4aR230014

Online, 30th November 2022 –10th February 2023

# Title:	(draft) Reply LS on PDU Set Handling

# Response to:	(R2-2213351) LS on PDU Set Handling

# Release:	Release 17

# Work Item:	5G_RTP

Source:	SA4

To:	RAN2

Cc:	SA2

Contact Person:

Name:	Thorsten Lohmar

E-mail Address:	Thorsten.lohmar@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA4 would like to thank RAN2 for the LS on PDU Set Handling. SA4 has discussed the Liaison Statement and would like to give some immediate feedback on the following topic:

Feedback: An RTP/UDP receiver contains a chain of functions for packet and frame processing. One of the first functions is a de-jitter buffer function, which collects and keeps all received RTP/UDP packets. This buffer is typically providing the packets in-sequence (according to the sequence numbers within the RTP headers) to the next function in the chain. In general, in-sequence reception from the media decoder side is an ideal scenario for low-latency XR applications since the RTP receiver can reduce the de-jitter buffer to a minimum. However, PDU set re-ordering in the network may cause unnecessary packet delay, and the effect of user-perceived quality of experience is unknown to SA4.

When the 5G System starts assigning different priorities or different importance’s to PDU Sets in the stream, SA4 wonders whether the 5G System will maintain in-order delivery of the PDUs.

2. Actions:

To RAN2 group.

ACTION: SA4 kindly asks RAN2 to take above information into account. Maintaining “in-sequence delivery” is clearly preferred. Further, SA4 kindly asks RAN2 to clarify the expected impact on in-sequence delivery, when RAN handles PDU Sets with different importance.

3. Date of Next SA4 Meetings:

3GPP TSG SA4#122		20th to 24th February 2023	Athens, Gr