3GPP TSG-SA4 Meeting #3-e (AH) RTC SWG post 122	S4aR230048

Online, 15th Mar 2023 - 29th Mar 2023

Source:	China Mobile Com. Corporation

Title:	AR Call Solution For Smartphones or Tablets

Agenda item:	4.3

Document for:	Discussion and Agreement

- Introduction

In this contribution, we proposed an AR Call solution for smartphones or tablets (defined as XR5G-P1 device type in TR 26.928). In this case, the 5G modem and AR media processing are integrated in the device. The rendered AR video frames are converted into a standard YUV video format before sending out to the remote UE via WebRTC media channel. The user’s input (e.g., keyboard, mouse, touch events, and other custom events) can be exchanged via WebRTC data Channel.

- Proposed Solution

- Figure X. Overall architecture of the proposed AR video calling system.

- 2.1 AR Video Frames Acquisition

The UE uses commercially available frameworks such as the Google ARCoreTM and the Apple ARKitTM to acquire the rendered AR video frames. For example, according to the documentation of the Google ARCoreTM SDK, the rendered AR video frames can be acquired by calling the specific APIs as follows:

- Figure X. The rendered AR video frames acquisition.

- The UE gets each ARFrame through the ARSession callback, and reads the corresponding AR data from the ARFrame.

- The AR Scene Manger uses Graphic Engine (Vulkan, OpenGL, Metal, DirectX, etc) and the corresponding AR data to render the AR content on the camera feed.

- The AR Scene Manger passes the rendered AR video frames to the Video Format Conversion module.

- 2.2 Video Format Conversion

The rendered AR video frames include textures information, which can not be accepted by the encoder. Therefore, they need to be converted into a standard YUV video format before sending to the remote UE. Depending on different planar and storage formats, YUV can be further subdivided into YUV I420, YUV420sp (NV12), YUV420sp (NV21) and so on.

The detailed descriptions in each step are as follows:

- It first copies a rendered AR video frame into a provided Bitmap.

- Then, it gets all pixel data of the Bitmap and use the the RGBA to YUV fast conversion formula (1) to convert the R, G, B value of the Bitmap into the Y, U, V value. The A value is set to be 0 by default.

- (1)

- Finally, it follows the YUV storage format to store the Y, U, V values. For example, YUV420sp (NV21) has one luminance plane Y and one plane with V and U values interleaved. Every four Y pixels there is one V and one U. The sampling is every other pixel and every other scan line.

- Figure X. The data blocks of an w×h image in YUV420sp (NV21) format.

- The converted video frames can be sent to the remote UE via WebRTC media channel. WebRTC data channel can be used for exchanging user’s input such as keyboard, mouse, touch events, and other custom events (Optional).

- Proposal

We propose to include section 2 of this document into clause 5.2.3 of the permanent document as a AR Call solution for discussion.

- References

- 3GPP TR 26.928: "Extended Reality (XR) in 5G".

- 3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

- Google ARCoreTM: "Overview of ARCore and supported development environments",

- https://developers.google.com/ar/develop