Source:	China Mobile, Huawei, HiSilicon

Title:	Why It Need to be Supported That Multiple DC Apps Multiplex One "m=" line

Document for:	Discussion and Agreement

Agenda Item:	4.9

1	Introduction

This contribution discusses why the requirement that multiple DC Apps multiplex one "m=" line need to be supported.

2	Discussion

It’s general that multiple data channel applications are used in an IMS session. For example, subscriber A shows to subscriber B how to change configuration on the UE through a DC App called "Sharing Screen" after the IMS session is established between them, and then subscriber A selects another DC App called "Chess" in the IMS session to play chess with subscriber B.

If these two DC Apps only use one application data channel without specific QoS requirement respectively, then in the SDP offer of re-INVITE request generated by UE A to establish the application data channel of “Chess” App, there are a total of six "m=" lines, which describe the media streams below:

- audio stream between UE A and UE B;

- video stream between UE A and UE B;

- bootstrap data channels between UE A and network A;

- bootstrap data channels between UE A and network B;

- application data channel of “Sharing Screen” App between UE A and UE B;

- application data channel of “Chess” App between UE A and UE B.

If the two DC Apps use more application data channels with specific QoS requirement, and/or other DC Apps are selected during the IMS session, the total number of "m=" lines will increase continuously.

Then why reducing the total number of "m=" lines in the SDP is required? It will be discussed from the following three aspects:

- To shorten TTM of IMS data channel related services.

- To reduce the network capacity consumed by dedicated bearers processing

- To reduce the bandwidth reserved for multiple applications.

- To ease NAT traversal and simplify firewall administration.

- It has no impact on developing IMS DC Apps based on WebRTC API.

2.1	Shorten TTM of IMS data channel related services

Although IETF RFC 3261 [1] and RFC 3264 [2] does not limit the size of a SIP message and the total number of "m=" lines in an SDP, the UEs and network devices in the existing networks always have constraints on them during implementation.

After IMS data channel is introduced into an IMS session, the total number of "m=" lines in the SDP of the session is much larger than that of an IMS audio/video session. In the example mentioned above, there is at least six "m=" lines in the SDP, which is three times of total number of "m=" lines in the SDP of an audio/video session. If the allowed maximum number of IMS DC Apps used in an IMS session is five, then there is at least nine "m=" lines in the SDP. This most likely exceeds the constraints of the UEs and the network devices.

Then the operators have to push the vendors of UEs and network devices in the existing networks to upgrade for supporting more "m=" lines in the SDP and larger SIP message size before they begin to deploy IMS data channel related services, including those network devices which don’t involve IMS data channel related handling and need not to be upgraded originally.

Therefore, it’s helpful for the operators to shorten the TTM of IMS data channel related services if the total number of "m=" lines in an SDP is reduced.

2.2	To reduce the network capacity consumed by dedicated bearers processing

For each "m=" line in the SDP, the AF needs to signal "media component" respectively via Rx/N5 interface. PCRF/PCF need to generate corresponding PCC rules accordingly and signal them via Gx/N7 interface, afterwards PGW-C/SMF need to handle those PCC rules and perform PGW-U/UPF controlling via PFCP interfaces. All these procedures consume network capacity. Reduction of media lines in the SDP is to reduce network consumption.

It may be argued that PCC rules can be aggregated even if they belong to different media components. However the aggregation is not mandated to be performed, operators have to rely on vendor’s implementation.

2.3	To reduce the bandwidth reserved for multiple applications

Statistical multiplexing is very common in CT world, it can significantly reduce the resources need to be reserved for the services. For instance there are 3 applications, say A, B and C, running on the UE, if each application has one dedicated "m=" line, the network has to create dedicated bearer for each application and reserve bandwidth respectively. Since the user may switch among applications thus the total amount of bandwidth actually consumed will be less than the sum of reserved bandwidth. In that case, to put all these 3 applications into one media line can achieve statistical multiplexing of bandwidth, i.e. to save network resources.

2.4	Ease NAT traversal and simplify firewall administration

In the SDP of an IMS session, different "m=" lines mean separate UDP ports. The increase of "m=" lines means increased use of Network Address Port Translation (NAPT) and becomes problematic, since maintaining multiple NAT bindings can be costly. It also complicates firewall administration, since multiple ports must be opened to allow media traffic. In IETF RFC 8843 [3], it states that,

"When the SDP offer/answer mechanism is used to negotiate the establishment of multimedia communication sessions, if separate transports (5-tuples) are negotiated for each individual media stream, each transport consumes additional resources (especially when Interactive Connectivity Establishment (ICE) is used). For this reason, it is attractive to use a single transport for multiple media streams."

IETF has been studying on reducing the number of concurrent transport flows used by applications and has released several specifications on this aspect, such as IETF RFC 5761 [4], RFC 8843 [3], RFC 8860 [5] and so on.

Similarly, reducing the number of concurrent transport connections required by an IMS session is also important in operators' networks for it is helpful to ease NAT traversal, simplify firewall administration and ultimately reduce OPEX and CAPEX. So reducing the total number of "m=" lines in an SDP is required.

2.5	Not impact on developing IMS DC Apps based on WebRTC API

Figure 1 below briefly illustrates the SDP process procedure on UE A and UE B when UE A initiates an re-INVITE request to negotiate IMS data channel media with UE B.

Figure 1: SDP Process Procedure on UE A and UE B

- The DC App-1 and App-2 on UE A invoke WebRTC API to generate an independent "m=" line of the required application data channel respectively, and then interact with the DCMTSI client.

- The DCMTSI client on UE A will merge these data channel media descriptions with the audio/video media descriptions it generated and forms a complete SDP, and then sends an re-INVITE message with this SDP to UE B.

- After receiving the re-INVITE message, the DCMTSI client on UE B parses the SDP, interacts with the corresponding IMS DC App-1 and App-2.

The process of "m=" line multiplexing on UE A and UE B is handled by the DCMTSI clients, not the DC Apps. As a result, it has no impact on WebRTC API based DC App.

3	Solution

To sum up, it’s necessary to support multiple DC Apps multiplexing one "m=" line to reduce the total number of "m=" lines in the SDP. The multiplexing includes the following scenarios:

- When a UE initiates a SIP request to negotiate the bootstrap data channels with local network and remote network, these bootstrap data channels may multiplex one "m=" line. See Scenario 1 in Figure 2 below.

- When a UE initiates a SIP request to negotiate the application data channel with local network and the application data channel has no specific QoS requirement, it may multiplex the "m=" line used by other application data channels between the UE and local network, which also have no specific QoS requirement. It’s the same case when the application data channel is between the UE and remote network.  See Scenario 2 in Figure 2 below.

- When a UE initiates a SIP request to negotiate the application data channel with its peer UE and the application data channel has no specific QoS requirement, it may multiplex the "m=" line used by other application data channels between the two UEs, which also have no specific QoS requirement. See Scenario 3 in Figure 2 below.

Figure 2: Scenarios of Multiplexing one "m=" line

4	Proposal

It is proposed to support that multiple DC Apps multiplex one "m=" line.

5	References

[1] 	IETF RFC 3261, "SIP: Session Initiation Protocol"

[2]	IETF RFC 3264, "An Offer/Answer Model with the Session Description Protocol (SDP) "

[3]	IETF RFC 8843, "Negotiating Media Multiplexing Using the Session Description Protocol (SDP)"

[4]	IETF RFC 5761, "Multiplexing RTP Data and Control Packets on a Single Port"

[5]	IETF RFC 8860, "Guidelines for Using the Multiplexing Features of RTP to Support Multiple Media Streams"