3GPP TSG-SA4 Meeting #3-e (AH) RTC SWG post 122	S4aR230060

Online, 15th Mar 2023 - 29th Mar 2023

Source:	China Mobile Com. Corporation

Title:	AR Call Solution For Smartphones or Tablets

Agenda item:	4.3

Document for:	Discussion and Agreement

- Introduction

In this contribution, we proposed an AR Call solution for smartphones or tablets (defined as Device design type 4 in MeCAR).

- Proposed Solution

The use case in the above figure establishes a bidirectional AR call. In this case, the 5G modem and AR media processing are integrated in the device. The following requirements need to be considered:

- Signalling for establishing a bidirectional call. It can be done in a similar way as for traditional MTSI/WebRTC calls.

- 2D video frames acquisition. The local UE can acquire the rendered video frames by using commercially available AR frameworks.

- Real-time video format conversion. The rendered AR video frames are converted into a standard YUV video format in real time before sending out to the remote UE.

- The converted video frames can be sent to the remote UE via WebRTC media channel. WebRTC/IMS data channel can be used for exchanging user’s input such as keyboard, mouse, touch events, and other custom events (Optional).

- 2.1 AR Video Frames Acquisition

The rendered AR video frames can be acquired from AR Runtime by the following steps:

- The AR Application is launched.

- The AR application creates AR Session in the AR Runtime.

- The AR application communicates with the scene manager to provide a scene description either through an application interface or through a well-defined scene description document, possibly retrieved over the network.

- Based on the scene requirements, the capabilities of the AR Runtime, as well as the capabilities of the media access function, a set of media pipelines in the uplink and downlink are established.

- The media access function accesses the network resources or sends data to the network using the established media pipelines.

- The AR Runtime captures the relevant AR data to match the device capabilities, and uses Graphic Engine (OpenGL ES, Vulkan, Metal, DirectX, etc) to render the AR objects on the camera feed.

- The AR Runtime presents the rendered AR video frame locally, and simultaneously sends the rendered AR Video Frames to the Video Format Conversion module.

- 2.2 Video Format Conversion

The rendered AR video frames include textures information, which may not be directly accepted by the encoder. Therefore, they need to be converted into a standard YUV video format before sending to the remote UE. Depending on different planar and storage formats, YUV can be further subdivided into YUV I420, YUV420sp (NV12), YUV420sp (NV21) and so on.

The detailed descriptions in each step are as follows:

- It first copies a rendered AR video frame into a provided Bitmap.

- Then, it gets all pixel data of the Bitmap and use the RGBA to YUV fast conversion formula (1) to convert the R, G, B value of the Bitmap into the Y, U, V value. The A value is set to be 0 by default.

- (1)

- Finally, it follows the YUV storage format to store the Y, U, V values. For example, YUV420sp (NV21) has one luminance plane Y and one plane with V and U values interleaved. Every four Y pixels there is one V and one U. The sampling is every other pixel and every other scan line.

- Figure X. The data blocks of an w×h image in YUV420sp (NV21) format.

- Proposal

We propose to include section 2 of this document into clause 5.2.3 of the permanent document as an AR Call solution for discussion.

- References

- 3GPP TR 26.928: "Extended Reality (XR) in 5G".

- 3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

- 3GPP TSG SA WG4 S4-230307, “MeCAR Permanent Document v5.0.0”, February 2023.