Source:	Huawei Technologies Co., Ltd.

Title:	IMS-based AR communication split rendering call flow

Agenda Item:	4.4

Document for:	Discussion and Agreement

- Introduction

At the 3GPP-SA4#122, S4-230320 was agreed that adds split rendering function to the objective of IBACS.

In this contribution, we propose the signalling that is required to invoke split rendering in an AR call over IMS.

- Procedures

When the UE has poor rendering capability which can’t satisfy the requirements of AR communication, the split rendering is involved between the UE and IMS.

TR 22.988 defines a use case UC#17 in clause A.2. That is, UE-A using a phone and UE-B wearing AR glass enter a communication, the UE-B shares a 3D model of house with the UE-A, UE-A can put some 3D objects into the house. The UE-A can only render the 3D objects due to its status, and the virtual 3D model of house needs to be rendered in the network.

The procedure for the split rendering is described in the following call flow:

The steps are as follows:

1. The UE-A initiates an AR communication session and establishes audio and video session connections with the UE-B. Then the bootstrap and application data channels are established for the UE-A and UE-B.

2. When the UE-A determines that its status (such as power status, signal status, computing power status, internal storage status, etc) cannot meet the AR communication related media rendering requirements, the UE-A decides to start split rendering call flow.

3. The UE-A initiates the application data channels between the UE-A and the DCMF, for the split rendering confirmation and metadata transmission between the UE and the IMS network.

4-5. The UE-A calculates which AR objects that can be rendered by itself based on its status, and then decides some part of the AR objects to be rendered in the UE-A and the others to be rendered in the IMS network. The UE-A sends a Split Rendering Confirmation Request to the DCMF through the established application data channel, the request includes the information of the AR objects to be rendered in IMS network, the DCMF transfers the request to the AR AS.

6. The AR AS decides whether to provide AR media rendering function based on the request message received from the UE-A and the media rendering resource available on the ARMF.

7. The AR AS sends a request to the DCSF to allocate media rendering resource on the ARMF for the AR objects that should be rendered in IMS network, the DCSF transfers the request to the MMTel AS.

8. The MMTel AS forwards the media resource allocation request to the ARMF, to reserve AR media rendering resource for the UE-A.

9. When the ARMF resource applied successfully, the MMTel AS return a successful response to the DCSF, the DCSF transfers the response to the AR AS.

10-11. The AR AS returns a successful Split Rendering Confirmation Response carrying the confirmation result to the DCMF. The DCMF then transfers the response message to the UE-A through the application data channel.

12. The MMTel AS then initiates a media re-negotiation procedure, to anchor the UE-A and UE-B’s audio/video connection to the ARMF for media rendering.

13. The UE-A starts AR media rendering procedure based on the response received in step11.

14-15. The UE-A sends metadata to the DCMF through the metadata dedicated application data channel, the metadata includes the information of the AR objects that should be rendered in the IMS network, then the DCMF transfer the metadata to the ARMF.

16. The ARMF renders the part of AR objects based on the metadata received.

17. (Optional) The UE-A renders the part of AR objects that should be done in the UE-A according to the confirmation result in step11. This step is not needed if all the AR objects are rendered in the IMS network.

18. The UE-A sends audio/video media stream including rendered AR objects to the ARMF, if all the AR objects are rendered in the IMS network, the UE-A should send original audio/video media stream to the ARMF.

19-20. The ARMF decodes the audio/video media stream received from the UE-A, and combines with the AR media rendered by the ARMF, then sends the combined audio/video media stream to the UE-B.

21.The ARMF sends the rendered audio/video overlay RTP media stream to the UE-A when it needs to send video back to the UE-A through the video back channel.

- Proposal

# We propose to agree to include the proposed split rendering call flow into the IBACS PD.