Source:	China Mobile Com. Corporation

Title:	[IBACS] Updates on AR communication architecture and Network Function

Agenda item:	4.5

Document for:	Discussion and Agreement

- Introduction

At SA2#157 meeting, two CRs (S2-2307527 and S2-2307534) on TS 23.228 to support AR communication were agreed. The changes have been incorporated into the latest version of TS 23.228(V18.2.0). This contribution proposes updates on “AR communication architecture” and “Network function for AR rendering” in IBACS PD considering relevant works in SA2.

- Proposed Updates

    - 2.3.2 Network function for AR rendering

IMS AR communication architecture is enhanced for supporting split rendering within IMS network, including the following network nodes and their functions:

-  AR Application Server, responsible for AR service control related to AR communication, including AR session media controlling, AR media rendering negotiation and AR service handling.

-  ARMF, responsible for AR communication media transmission and media rendering function, including AR Rendering Logic control and 3D Rendering Engine for rendering the scenes, virtual human models and 3D object models according to the field of view, posture, position.

-	=

NOTE 1:	The ARMF is a logic network function element, and can be deployed together with other media functions such as MF/MRF.

Media interface for IMS AR communication and AR rendering is as follow:

-  DC4: Service based reference point between the AR Application Server and the DCSF for AR service handling and AR session media control .

-	DC2: Service based reference point between the IMS AS and the MF for AR media resource management.

-	Mr'/Cr: Reference point between the IMS AS and the MRF for AR media resource management.

-	MDC2: Reference point of data channel media between the AR Application Server and the MF for AR media rendering negotiation.

-	Mb: The media interface for audio/video media stream transmission and/or data media stream transmission, including AR media and AR specific data.

# 3 AR IMS Communication Architecture

  - 3.1 Potential Reference Architecture (SA2)

[Editor’s Note: this section needs further updates aligned with the work in SA2]

[The following diagram refers to TS 23.228 AC.9.2-1, which describes the enhancements to the IMS architecture to support AR communication. For the UE centric case, AR traffic is transparent to the IMS network and is exchanged between the two peer UEs via the deployed media functions (e.g., IMS AGW/TrGW).]

Figure 3.1.1: Architecture to support AR communication

- 3	Proposal

We propose to include section 2 of this document into the IBACS PD.

- 4	References

- 3GPP TS 23.228 V18.2.0: "IP Multimedia Subsystem (IMS);Stage 2", 2023.06.