3GPP SA4-e (AH) RTC SWG post 126, telco #17	S4aR230139

# Online, 10 January 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR |  | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [iRTCW] Update of procedures for RTC-7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | iRTCW |  |  |  |  |  | Date: |  |  | 2024-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedures for RTC7 interface need to be described. RTC-7 interface is an UE internal interface between a WebRTC framework and a native WebRTC Application to directory communicate media-specific information. The functionalities of WebRTC framework (e.g., API) provided over RTC-7 interface are equivalent to the WebRTC API defined in W3C and IETF. There is nothing to be specified for procedures on RTC-7 in this release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding description for RTC-7 procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Technical Specification wiil not not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x2]	W3C Recommendation: "WebRTC 1.0: Real-Time Communication Between Browsers".

[x3]	IETF RFC 7478 (2015): "Web Real-Time Communication Use Cases and Requirements".

[x4]	3GPP TS 26.119: "Media Capabilities for Augmented Reality"

[x5]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)"

[x6]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[x7]	3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[x8]	IETF RFC 8825 (2021): "Overview: Real-Time Protocols for Browser-Based Applications".

[x9]	IETF RFC 8835 (2021): "Transports for WebRTC".

[x10]	ITU-R Recommendation BT.601-7 (03/2011): "Studio encoding parameters of digital television for standard 4:3 and wide screen 16:9 aspect ratios".

[x11]	Microsoft: "Microphone Array Geometry Descriptor Format".

[x12]	Apple: "Getting Raw Accelerometer Events".

[x13]	Google: "Sensor Coordinate System".

[x14]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[x15]	3GPP TS 37.324: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Service Data Adaptation Protocol (SDAP) specification".

[x16]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[x17]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[x18]	3GPP TS 23.003: "Numbering, addressing and identification".

[x19]	IETF RFC 8829 (2021): "JavaScript Session Establishment Protocol (JSEP)".

[xxx]	W3C Candidate Recommendation Draft, Media Capture and Streams", <https://www.w3.org/TR/mediacapture-streams/>.

* * * Next Change * * * *

4.3.2	UE media delivery (RTC-7) procedures

RTC-7 interface is an interface between a WebRTC framework and a native WebRTC Application to directly communicate media-specific information. The native WebRTC Application uses WebRTC framework for media handling (e.g., gathering media capability information of the UE, controlling media transport). The functionalities provided on this interface are equivalent to WebRTC API defined in W3C and IETF specifications such as W3C WebRTC [x2], W3C Media Capture and Streams [xxx], IETF RFC 8825 [x8], IETF RFC 8829 [x19] and IETF RFC 8835 [x9]. There is nothing to be specified for procedures on RTC-7 in this release.

* * * End of Changes * * * *