Post SA4#126 RTC Telco	S4aR230161

9 January 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | pseudo | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP] Harmonizing the ABNF syntax for the RTP header extensions and updating the message format for the RTCP extended report block |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 09/01/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. The ABNF sytax for the SDP for the RTP header extension fo the PDU Set marking is presented differently from those for the other two RTP header extensions in TS26.522, and this may cause confusions. - To represent this line: extmap-attr="a=extmap:" 1*5DIGIT ["/" direction] SP "urn:3gpp:pdu-set-marking:rel-18" SP extensionattributes we only need to define extensionname and extensionattributes. - In the line: extensionattributes = *3(format / "pdu-set-size"), the maximum repetition should be 2. 2. The time_info field in the RTCP XR block for QoE timing information is 4 bits, but Figure 5.2.2.1-1 shows 5 bits. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description of the ABNF syntax to be consistent with those for the other two RTP header extensions, and correct the RTCP XR block message format. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion in the spec, hinderance of the IANA registration process, error in the RTCP XR block format. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

.

# Proposed changes

In the Clause 4.4.2.5:

# * * * * 1st change * * * *

#### 4.4.2.5	SDP Signaling

An AS or sender UE capable of sending PDU set marking HE shall use the SDP attribute extmap for PDU set marking HE in the media description of the RTP stream(s) carrying the PDU set HE. A receiver that does not support PDU set marking HE can ignore the RTP header when included. The signaling of the PDU Set and End-of-Burst marking RTP header extension shall follow the SDP signaling design and the syntax and semantics of the "extmap" attribute as outlined in RFC8285.The URN for the PDU Set marking shall be set to "urn:3gpp:pdu-set-marking:rel-18".

The header extension identifier shall be registered with IANA as maintained in Real-Time Transport Protocol (RTP) Parameters (iana.org).

The ABNF syntax for the extmap attribute for the signaling of PDU Set Information and End of Burst marking is defined as follows:

extensionname = "urn:3gpp:pdu-set-marking:rel-18"

extensionattributes =  [format SP]"pdu-set-size"

format = "short" / "long"

The extension attributes have the following semantics:

-	format: indicates if the RTP header extension for PDU Set and End-of-Burst marking uses the 1-byte (short) or the 2-byte (long) format.

-	pdu-set-size: if present, this attribute indicates that the application server will provide the PDU Set size in bytes in the RTP header extension with every RTP packet. This results in an additional 3 bytes of length for the RTP header extension.

# * * * * End of 1st  change * * * *

In clause 5.2.2.1:

# * * * * 2nd change * * * *

The QoE timing information Report Block has the following format:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     BT=TBD    | resv  |time_info |         block length          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                         SSRC of source                        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                         RTP time stamp                        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                    estimated-at-time (T1)                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                    start-to-render-at-time (T3)               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                    server-output-time (T5)                    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                    scene-update-time (T6)                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 5.2.2.1-1: RTCP XR block format for QoE timing information data

The semantics of the fields in QoE time information Extended Report (XR) block are as follows:

-	block type (BT) [8 bits]: A QoE time information Report Block is identified by a constant value.

-	block length [16 bits]: The length of this report block, including the header, in 32-bit words minus one.

-	resv [4 bits]: This field is reserved for future definition. In the absence of such definition, the bits in this field MUST be set to zero by the sender and MUST be ignored by the receiver.

-	time_info [4 bits]: This field bits represent the time stamps that are present in the XR report block. When T1 is present in the XR report, 1st bit (least significant bit) is set to 1. When the LSB is set to 0, T1 information shall not be present. When T3, T5 and T6 are present in the RTCP XR block data, bits 2, 3 and 4 are set to 1 respectively. When T1, T3, T5 and T6 are present in an RTCP XR block data, the time_info field value shall be b1111. The timing information when present shall follow the order T1, T3, T5 followed by T6. For example, when the time_info field value is b0101, the XR block carries the T1 information followed by T5. T3 and T6 timing information will not be present in that XR block content.

# * * * * End of 2nd  change * * * *