3GPPSA4-SA4-e (AH) RTC SWG post 127-bis-e	S4aR240026

Online, May 6 – , 2024	(revision of S4-240xxx)

Source:	Huawei, HiSilicon

Title:	[FS_5G_RTP_Ph2] Key Issue #12: Data Burst marking enhancement

Document for:	Approval

Agenda Item:	4.7

Work Item / Release:	FS_5G_RTP_Ph2 / Rel-19

- Discussion

TS 26.522 includes an End of Data Burst field in the PDU Set marking header extension in Release 18.

As agreed in SP-240482,  guidelines for setting and potential enhancement of this field, considering the developments in the FS_XRM_Ph2 work in SA2 will be studied in FS_5G_RTP_Ph2 Key issue #12.

As discussed in SA2 TS 23.501, the data burst indicates a set of multiple PDUs generated and sent by the application in a short period of time. The traffic characteristics regarding the data burst, e.g., the End of Data Burst indication, could be helpful to the network, e.g., for power saving, efficient radio resource management. The study of this key issue may therefore both benefit 5GS operation and media application operation.

As discussed, and agreed in S4-240666, paced sending is one of the implementations in WebRTC, where the RTP sender puts the packets into a queue within the pacer, awaiting opportune moments to send them. In this case, the packets sent at a specific time slot/window can be regarded as a data burst and the RTP sender can know well on the traffic characteristics before sending out the data bursts.

Currently, TR 23.700-70 in FS_XRM_Ph2 also documents key issues and solutions considering burst related information for the 5G System.

2. Text Proposal

It is proposed to capture the following changes vs. TR 26.822.

# * * * * First change * * * *

5.12	Key Issue #12: Enhancements of Data Burst Marking

5.12.1	Description

Editor’s note:	This clause provides a description of the key issue.

A data burst indicates a set of multiple PDUs generated and sent in a short period of time as defined in clause 3.1 of TS 23.501 [3]. Data burst is a common transmission characteristic in communication networks.

The traffic characteristics regarding the data burst transmission could be beneficial for the 5GS network, e.g., power saving and efficient radio resource management. In Release 18, the End of Data Burst indication has been introduced to enable the UE power saving in the 5GS, i.e., the NG-RAN node can configure to move UE into CDRX for power saving after transmitting the end PDU of the data burst.

Similarly, as stated in the draft TR 23.700-70 from SA2 Rel-19 FS_XRM_Ph2, it also aims to study the 5GS network enhancements to support the burst related traffic characteristics.

Therefore, the following enhancements to data bust marking are studied in this key issue.

-	Identify additional burst related traffic characteristics beneficial to the 5GS network, for example,

Time to next burst, burst size and other potentially relevant characteristics

Editor’s Note:	This is to be collaborated with SA2 and RAN WG.

-    Identify and document the way RTP senders can generate data bursts e.g.

- WebRTC paced sending implementation

- Other common RTP implementations or libraries that are commonly used

-	Develop additional signalling in the 5G RTP Header Extension to include additional burst related traffic characteristics.

-	Develop guidelines and recommendations for the setting of data burst related parameters in the RTP Header Extension (if needed).

# * * * * End of changes * * * *