3GPP TSG SA WG4 RTC SWG Telco                                                                	S4aR240038

# 26th June 2024, online							revision of S4-241281

Source:	Nokia Corporation

Title:	[FS_5G_RTP_Ph2] Sol KI#2: Application-specific PSI mapping for lone PDUs

Agenda item:	4.5

Document for:	Agreement

# Introduction

In the SA4#127-bis-e, the key issue #2 on QoS handling requirements for lone PDU was incorporated into TR 26.822. The key issue proposes to study two aspects:

- whether there is any issue when applying PDU Set QoS parameters to the lone PDUs from the application layer perspective?

- how to handle the issue of missing PDU Set Information in case of lone PDUs

This contribution proposes a solution regarding the missing PDU Set Importance for lone PDUs. The aim is to provide a session-specific PSI configuration by the sender and use this mapping to set the PSI values rather than leaving the setting to a default configuration by the UPF.

# Proposal

A solution to KI#2 is proposed for incorporation into TR 26.822.

=== CHANGE 1 (all new) ===

## 6.X	Solution #X: Application-specific PSI mapping for lone PDUs

### 6.X.1	Key Issue mapping

This solution addresses the key issue #2.

### 6.X.2	Description

### 6.X.2.1	Background

As of Rel-18, there is no mechanism to mark PDUs carrying protocol data other than RTP. Thus, PDUs belonging to protocols such as RTCP, STUN, etc. cannot be marked.

In Rel-18, SA2 has agreed that the PSA UPF marks, in the downlink, each N6-unmarked PDU (“lone PDU”) with PDU Set Information into a PDU Set. If the UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, the UPF still maps it to a PDU Set and determines the PDU Set Information by implementation-specific means.

Observation 1: PDUs of non-RTP protocols (e.g. RTCP) are mapped by the UPF into PDU Sets. The associated PDU Set Information is determined by the UPF.

When RTP is used, the RTP sender may prefer not to mark RTP PDUs that carry certain media types due to different reasons. For example, considering the typically low bitrates of audio and haptics streams, the sender may consider the overhead of adding the RTP HE for PDU Set marking prohibitive for such streams. For instance, for audio data with a bitrate of 64kbps and frame size of 20ms (50 packets per second), the overhead of the HE (assuming two-byte HE format with optional fields) would be 50*16=800 bytes=6.4bps which is already 10% of the original bitrate. Consequently, those PDUs lack PDU Set information and are assigned to a PDU Set by the UPF based on the UPF pre-configuration.

Observation 2: RTP senders may opt against adding the RTP HE for PDU Set marking to low bitrate streams due to the overhead of the required bandwidth.

When the PDU Set Information is not provided by the sender, the UPF may be able to reliably obtain most of its elements based on the UPF implementation. However, that is not the case for PDU Set Importance (PSI) which needs to be obtained from the media payload (cf. TS 26.522, clause 4.2.6.2). If SRTP is used, the media is encrypted, and the payload is not visible to the UPF. Even if the stream is unencrypted, the UPF would have to parse the RTP payload and derive the PSI from the NAL unit headers (for video). This is not feasible considering that the UPF processes data from several endpoints simultaneously under tight latency constraints.

Any default PSI setting at the UPF may not be accurate since the importance of different PDU Sets is application- and codec-dependent. For example, some RTCP message types may be considered more important for low latency applications. Another example: the application may determine the importance of reference pictures based on their sending frequency (random access interval).

Observation 3: The UPF cannot reliably determine the PSI for lone PDUs based on pre-configuration. Sender applications are in the best position to determine the PSI.

### 6.X.2.1	Solution description

In this solution, the sender creates a mapping between a set of PSI values and PDU types. The sender can define session-specific PSI values for all or some of the lone PDU types transported in the session. These can be PDUs of non-RTP protocols and PDUs carrying non-video payload types such as audio or haptics data. The sender can create such mapping based on the application requirements. This mapping can then be sent to the network which can use it to determine the missing PSI values for the lone PDUs.

PDU type may refer to the payload type for RTP or packet type for RTCP.

- In RTP, the payload type is carried in the RTP header. Payload identifiers 96–127 are used for payloads defined dynamically during a session. For example, the sender may be using the dynamic payload types 96 for H.264 and 97 for AMR-WB, respectively.

- In RTCP, packet type is carried in the RTCP header and defined for different RTCP messages by the related specifications (e.g. 200 for sender reports, 206 for payload-specific messages). For RTCP feedback messages, further differentiation is provided by the feedback message type (FMT).

- There may be PDUs carrying other protocol data such as STUN.

The sender may consider the relative importance of different media types for the receiver application while establishing the PSI mapping. For example, the sender may map audio PDUs to PSI=1 and haptics PDUs to PSI=15, if the sender considers audio data critical and haptics data optional.

The sender may differentiate the PSI values assigned to different RTCP message types based on their importance for the application. For example, the sender may choose to map RTCP sender/receiver reports to PSI=1 while mapping RTCP XR messages to PSI=15. In another example, RTCP SR/RR may be transmitted at a higher frequency and the sender may prefer to map RTCP SR/RR to PSI=5 since loss of one report would have less impact. The sender may also consider the RTCP mode of operation while determining the PSI values for different RTCP message types. E.g. RTCP messages sent in the Immediate Feedback mode (as defined in the RTP/AVPF profile [RFC 4585]) may be mapped to a lower PSI than those sent in the regular RTCP mode.

Below is an example mapping. The type field corresponds to the payload type for RTP and packet type for RTCP.

psi-mapping:

- rtp: # See Observation 2 above

amr-wb:

name: "AMR-WB audio"

type: 97

psi: 2

some-haptic-codec:

name: "Haptics data"

type: 98

psi: 12

- rtcp: # See Observation 1 above

sr:

name: "RTCP sender reports"

type: 200

psi: 5

rr:

name: "RTCP receiver reports"

type: 201

psi: 5

xr:

name: "RTCP XR messages"

type: 207

psi: 15

- stun:

name: "STUN messages"

psi: 1

The sender can send the PSI-to-PDU type mapping to the network via control plane signaling. For example, the AF may signal this information to the network in the Protocol Description, as part of the PDU Set assistance information [TS 23.501]. Whenever a lone PDU is received, the network can look up the PSI value and assign it to the lone PDU.

NOTE:	The solution #29 in TR 23.700-70 (agreed for interim conclusions of KI#4 at SA2#163) enables the network to differentiate multiplexed streams sent in the same media transport such that they can mapped to distinct QoS flows according to their QoS requirements. However, the PDU Set Information would still be missing for the lone PDUs such that it would have to assigned by the UPF using a preconfiguration.

The benefit of the proposed solution is that the network does not have to rely on UPF configuration to determine the PSI values and can make a more reliable decision based on the mapping provided by the sender.

Editor’s Note: This solution requires coordination with SA2.

=== END OF CHANGE 1 (all new) ===

# References

[RFC 4585] “Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)”

[TS 23.501] “System architecture for the 5G System (5GS),” v18.5.0

[TR 23.700-70] “Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2” v1.0.0