SA4-e (AH) RTC SWG post 129-e	S4aR240094

# 25th Sept 2024, online 															      Revision of S4aR240051

Source:	InterDigital Europe

Title:	Haptics source and input formats

Spec:	3GPP TR 26.854

Agenda item: 	6.4.2

Document for:	Agreement

1. Introduction

As per the minutes of the sept. 25th call, this revision of S4aR240051 proposes a first text and information on Haptics source and input formats in more details.

A JSON file is attached which is proposed to be attached to or included in an annex of the TR.

2. Reason for Change

Provide initial text for Haptics source and input formats to populate section 6, some references for section 2, acronyms for section 3 of TR 26.854.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[SMPTE]	SMPTE ST 2100-1:2017 “Definition and Representation of Haptic-Tactile Essence for Broadcast Production Applications”.

[AHAP]	Apple Haptic and Audio Pattern JSON format for Haptics https://developer.apple.com/documentation/corehaptics/representing_haptic_patterns_in_ahap_files, retrieved Sept. 2024.

[IEEE]	xxx

[23090-31]	ISO/IEC 23090-31 Information technology — Coded representation of immersive media Part 31: Haptics coding

[HAPT]	xxx

[IVS]	https://www.immersion.com/technology/ retrieved Sept. 15th. 2024.

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

AI	Artificial Intelligence

AHAP	Apple Haptic and Audio Pattern

HJIF	Haptic JSON Interchanged Format

HMD	Head Mounted Display

JSON	JavaScript Object Notation

MECS	Mobile Edge Computing Server

XR	Extended Reality

* * * Next Change * * * *

## 6.2	Haptics source formats

### 6.2.1	Introduction

Haptic source formats are essential in capturing and transmitting tactile sensations in a variety of applications, ranging from simple vibrations in mobile devices to complex force feedback in virtual reality systems. Two types of source formats exist, time-sampled signals captured by sensors, and synthetic data, or parametric description of haptic effects, created from authoring tools.

## 6.2.2		Time-sampled signals

Haptics time sampled signals are similar to audio Pulse Code Modulation (PCM) signals but with a lower bandwidth. In Figure 1, two examples of signals are provided, on the left is the samples of 3D angular orientations and linear accelerations of a horse captured with an accelerometer positioned on the horseman. On the right is the haptic signal of a texture (here a carpet) captured from the sound of a pen moving on the surface of the carpet in one direction.

Those PCM signals are generally used for real signals captured from sensors.

Figure 1 - Example of time sampled signals (left: Horse riding 3D angular orientations and linear accelerations captured with an accelerometer, right: texture captured from sound on one direction).

Generally, the sampling frequency is set between 2kHz and 16kHz (8kHz in Figure 1) with the haptic perceptual bandwidth being around [5Hz-1kHz], with some variations depending on the haptic modality, rendering device and human haptic receptor. 5Hz and below is particularly significant for force feedback. This leads to a raw data-rate average of 128 kbps per channel.

## 6.2.3		Parametric description of haptic effects

Parametric signals are mostly generated with design tools and thus correspond to synthetic data. In Figure 2, two examples of signals are provided, on the left a heartbeat synthetic signal (thus the regularity of the signal), and on the right a synthetic rain effect (quasi random design) with 4 channels corresponding to 4 different target locations on the user.

Figure 2 - Example of parametric rendered haptic (left: heart beat synthetic signal, right: synthetic rain effect with 4 channels for 4 different locations on the user).

The synthetic aspect of those signals compared to real signals is obvious, as well as the temporal sparsity of the signals, even if Figure 2 only shows a short duration. In general, a parametric representation is used to synthesize a short duration effect and is used only at some events or at dedicated point in time during a gaming or movie experience.

Parametric signals need a synthesizer to convert the parametric representation to a timed signal compatible with the actuators used. The raw bit-rate dependents on the number of effects and their distribution in time.

## 6.2.4		Comparison and applications

The choice between time-sampled signals and parametric descriptions depends on the application and the desired level of control. Time-sampled signals are more suitable for applications requiring high fidelity and real-time capture of physical interactions, such as in medical simulations, playback of recorded signals (motion) or fine-grained tactile feedback systems. Parametric descriptions, on the other hand, are ideal for applications where flexibility and ease of use are paramount, such as in mobile notifications, communication, movies or gaming, where the haptic feedback needs to be easily adjustable and consistent across a variety of devices. Parametric is also well suited to XR to attach haptic effect to events or objects.

* * * Next Change * * * * All new

# 6.3 	Haptic input formats

## 6.3.1 introduction

In this section input formats are the formats representing the raw data (or source formats) and used at the production side (e.g. for encoding, packaging and delivery). Most current input formats for haptics are proprietary. However standardised interchangeable formats (or mezzanine formats) have now been defined.

## 6.3.2		PCM raw format

PCM or Pulse Code Modulation is a digital format that represents analog signals in digital format as a series of samples at a regular sampling frequency. It has been defined for audio but is very often used to store haptic signals from sensors.

Various file format support PCM such as the audio WAV format. ISO/IEC MPEG Haptic [23090-31] is using WAV as one of the input file for time-sampled haptic signals. Headers or metadata are often used to specify ranges or other information related to the signals, but since no real standard has been defined for raw signals it is often application dependent.

This format is suitable for storing or distributing sensors data and is very easy to implement. The drawback is that PCM signals are continuous as opposed to parametric signals that are sparse.

## 6.3.4		Proprietary parametric formats

Descriptive haptic is represented through parametric primitives. It can be described by an amplitude, a frequency, a duration, and a waveform shape such as sinusoid, or square. Two types of parametric effects are usually defined: transient and continuous. A transient signal is a short pulse with an amplitude, a position in time or space, and a reference frequency. A continuous signal is represented by a frequency and an amplitude information modulated on the timeline.

Existing proprietary descriptive schemes use JSON or XML to describe haptic effects. Two of those proprietary formats are commercially deployed: AHAP from Apple [AHAP] and IVS [IVS] from immersion corp.

AHAP have been mainly developed for vibrotactile signals and implementation into iOS Core Haptic framework.

IVS was targeting media applications (movies, gaming) and communication (smartphone) using mainly vibrotactile signals described in XML format.  Aa binary version is available for networks transmission.

HAPT is a vendor-specific haptic coding format based on the RIFF format. It is designed to enable efficient coding of a device-specific haptic effect.

These two formats can further be encoded and compressed using haptic codecs. The ISO/IEC MPEG Haptic [23090-31] codec supports both formats, while the IEEE codec [IEEE] only supports PCM input signals.

## 6.3.5	Standardized and Interchangeable parametric formats

SMPTE defined the Haptic-Tactile Essence for Broadcast Production Applications [SMPTE ST 2100-1:2017]. However, it was mainly dedicated to position, orientation, velocity and acceleration.

The Guitammer Company developed and implemented this format for broadcasting sports physical events. Proof of concepts have been done with NHRA and NHL sports.

MPEG HJIF: MPEG defined in section 6 of specification [23090-31] a standardized and interchanged parametric JSON based format (or mezzanine format) for describing haptics effects called HJIF

This format is compatible with the proprietary AHAP format. Transcoding from and to the other is possible. Similarly, the IVS format can be transcoded from XML to JSON, for most of the functionalities, since similar parametric patterns/functions are used. Transcoding (for the primitives used in the formats) is lossless.

In addition, it can integrate a binary encoded version of a PCM signal.

The main advantage of this format is its flexibility to support various haptic modalities, various input formats and flexibility for heterogeneous end-user devices and experiences. Thanks to the JSON format, it is also easy to edit and modify the file.

This format can further be encoded and compressed using MPEG haptic codecs, in a binary format for networks transmission.

* * * End of Changes * * * *