3GPP SA4 RTC SWG Telco Post #130	S4aR250032

# Online, 5 February 2025

Source:	InterDigital France

Title:	Pseudo-CR on OpenXR API for Haptic Media

Spec:	3GPP TS 26.854 v1.0.0

Agenda item:	4.9

Document for:	Agreement

1. Introduction

During the previous RTC call, the section 7.3.4 was empty with an editor note: TBD and a text was proposed to fill the section. It was agreed with square bracket.

This contribution removes the square bracket around the provided and unchanged text for the OpenXR API.

2. Proposal

Agree to remove the “[]”

* * * First Change  * * * *

### 7.3.4	OpenXR APIs

The OpenXR API [6] standardizes the use of a range of cross-platform XR device capabilities, including haptic devices. Current support mainly addresses XR and game controllers handled by the user hands (most commercial controllers are supported). Functions such as trigger, click, touch, squeeze, grip are input actions with a potential haptic output specified by an hapticsAction and hapticPath. The hapticsAction specifies the type of haptic feedback (e.g. vibrations) and hapticPath specifies where the effect is applied (e.g. left hand). Haptic feedback is sent to a device using the ApplyHapticFeedback and StopHapticFeedback functions.

The only haptics type supported by the OpenXR API v 1.1.43 is XrHapticVibration for haptic vibrations (vibration amplitude, duration, frequency). Some proprietary extensions also provide support for vibrations described by a haptic amplitude envelope or PCM signal (XrHapticAmplitudeEnvelopeVibrationFB, XrHapticPcmVibrationFB).

In 2022, the HIF (Haptic Industry Forum) submitted a high-level proposal for advanced haptics APIs to the OpenXR consortium. The advanced haptics APIs is designed to extend the OpenXR API to other haptic modalities and process several haptics standards among which the MPEG HJIF, IEEE P2861.3 standards as well as other proprietary formats (AHAP, IVS). No public information is available so far on the status of this collaboration.

* * * End of Changes * * * *