3GPP SA4 RTC SWG Telco Post #130	S4aR250035

# Online, 5 February 2025                                                               revision of S4aR250015

Source:	InterDigital France

Title:	Pseudo-CR on codec performances

Spec:	3GPP TS 26.854 v1.0.0

Agenda item:	4.9

Document for:	Agreement

1. Introduction

This contribution proposes additional details to finalize the characterization of the haptics media codecs and to remove the square brackets.

The revision addresses the comments received during the previous call:

- - add reference to MUSHRA

- - revise the figure to include the axis label

- - clarify that the results are MPEG results and provide reference.

2. Proposal

Agree to the additional details, clean-ups to finalize the section 7.1

* * * First Change * * * *

### 7.1.1	MPEG Haptics Coding

#### 7.1.1.1	Overview

JTC 1/SC 29/WG 7 (MPEG-3DGH) has completed the development of the MPEG Haptics Coding standard at the April 2023 meeting and is pending publication as ISO/IEC 23090-31 [7].

In addition to a codec and bitstream format, this MPEG Haptic coding standard supports multiple haptic modalities and specifies a human readable JSON coded representation format (HJIF) for descriptive and quantized media formats. Haptic primitive from AHAP and IVS parametric signals can be losslessly converted to and from HJIF. The HJIF data model is a flexible hierarchical structure that can describe one or more channels, allowing for mono, stereo or multi-channel haptics media experiences, targeting one or more actuators or devices. Application areas especially targeted for the use of MPEG Haptics coding include media streaming and broadcast, immersive applications and XR services, real time communications, as well as tactile communication, the latter being out of scope of this study.

During the standardization process, MPEG also worked on a reference and conformance software [ISO/IEC 23090-33] that was publicly released.

An in-depth evaluation of the performances of this reference codec was conducted with both objective metrics (PSNR) and subjective tests MUlti Stimulus test with Hidden Reference and Anchor (MUSHRA) as defined in ITU-R recommendation [BS.1534-3] for different target bitrates and different input test streams. Three representative sets of test streams were provided by different companies and corresponded to market needs: two sets for vibrotactile signals (short effects and long effects) and one set for kinesthetic signals (including force signals, acceleration, or movement). For a total of 43 test streams.

The objectives performances reported by MPEG in [N1165] are given in Figure 1. On the left the PSNR is given for three configurations of the MPEG encoder CRM3.2 considering PCM input signals (C2V: vectorial encoding, C2W: wavelet encoding, C2VWR: hybrid encoding). On the right the histogram of the bit-rate is depicted for parametric transcoding of the .ivs and .ahap parametric input streams.

Encoded PCM signals

Figure 1 - Objective performances (left: PCM signals, right: parametric signals).

The transcoding of parametric input content is usually around 1-7 kbps (with lossless coding since it consists in transcoding the input parametric file to the MPEG parametric format). The results on PCM input data show that signals encoded with the 2 kbps target bitrate present perceptible distortions with an average PSNR of 24db. Signals encoded with a 8 kbps target bitrates show some distortions, but not annoying, with an average PSNR of 39.17db. Finally signals encoded with a 16kpbs bitrate have no perceptible distortion with an average PSNR of 45.62db. Thus 8kbps is considered as a reasonable average bit-rate for MPEG-Haptic encoding. Considering an average input bit-rate being 128kbp (8kHz sampling for 16bits samples), it leads to a compression ratio of 16 per channel.

Subjective performances, performed by three independent laboratories and two reference haptic devices, confirmed the above results. The MUSHRA score was higher than 94 (maximum is 100) for bit-rates of 8kbps and higher.

* * * Second Change * * * *

### 7.1.2	IEEE

#### 7.1.2.1	Overview

The IEEE Standard for Haptic Codecs for the Tactile Internet was developed in 2016 and approved in 2024 [30]. This standard specifies 3 different codecs (no-delay kinesthetic codec, delay-robust kinesthetic codec, and a tactile codec) supporting only a time-sampled media format (PCM) and no parametric media formats.

Some performances of the IEEE Haptic Codecs were documented in [2] however it is not clear if these data relate to one or to the 3 codecs contained in the IEEE standard. It appears it may mostly relate to the closed loop tactile codec (with a reliability of 99,999%), which is out of scope of this study.

Further, with no support for parametric media formats, these codecs could not be used as mezzanine format nor interoperable formats with parametric haptic source content widely used in use cases of section 5.

* * * Third Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

….

[BS.1534-3]	BS.1534 : Method for the subjective assessment of intermediate quality level of audio systems

[N1165] 	S/N 24923 Report on the MPEG Haptics Phase 1 Performances, Jan’25, Geneva