3GPP TSG-SA4 Meeting #0-e (AH) RTC SWG post 130	S4aR250039

in revision of

S4aR250027

# Online, , 26th Nov 2024 - 5th Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | 0009 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Guideline for PDU Handling marked and unmarked PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 26.822 conclusions`in 7.2 state the following: Consider guidelines for handling lone PDU in TS 26.522 [2]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a guideline on how to deal with lone PDU (i.e. marked and unmarked PDUs) in a network element such as UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Goals for study item not complete, confusion on the “lone PDU” case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A (new sub clause) , clause 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - Address comments and editorial improvements - Clarify the situation around RTCP PT vs RTP PT and the absence of RTP HE |  |  |  |  |  |  |  |  |

*** CHANGE***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-T Rec H.264 (08/2021): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2022: "Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding".

[3]	ITU-T Rec H.265 (08/2021): "High efficiency video coding" | ISO/IEC 23008-2:2023: "High Efficiency Coding and Media Delivery in Heterogeneous Environments – Part 2: High Efficiency Video Coding".

[4]	IETF RFC 3550 (2003): "RTP: A Transport Protocol for Real-Time Applications", H. Schulzrinne, S. Casner, R. Frederick and V. Jacobson.

[5]	IETF RFC 6184 (2011): "RTP Payload Format for H.264 Video", Y.-K. Wang, R. Even, T. Kristensen, R. Jesup.

[6]	IETF RFC 7798 (2016): "RTP Payload Format for High Efficiency Video Coding (HEVC)", Y.-K. Wang, Y. Sanchez, T. Schierl, S. Wenger, M. M. Hannuksela.

[7]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[8]	3GPP TR 26.998: "Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[9]	IETF RFC 768 (1980): "User Datagram Protocol", J. Postel.

[10]	IETF RFC 5761 (2010): "Multiplexing RTP Data and Control Packets on a Single Port", C. Perkins, M. Westerlund.

[11]	IETF RFC 8285 (2017): "A General Mechanism for RTP Header Extensions", D. Singer, H. Desineni, R. Even.

[12]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[13]	IETF RFC 5905 (2010): "Network Time Protocol Version 4: Protocol and Algorithms Specification”, D. Mills, J. Martin, J. Burbank, W. Kasch.

[14]	IEEE 1588-2019 – IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems, June 2020.

[15]	IETF RFC 4574 (2006): "The Session Description Protocol (SDP) Label Attribute", O. Levin, G. Camarillo.

[16]	IETF RFC 3611 (2003): "RTP Control Protocol Extended Reports (RTCP XR)", T. Friedman, R. Caceres, A. Clark.

[17]	3GPP TS 26.119: "Media Capabilities for Augmented Reality".

[18]	IETF RFC 7656 (2015): "A Taxonomy of Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources ", J. Lennox, K. Gross, S. Nandakumar, G. Salgueiro, B. Burman.

[19]	IETF RFC 5888 “The Session Description Protocol (SDP) Grouping Framework”, G. Camarillo et al.

[20]	ISO/IEC 60559:2020: “Floating-point arithmetic”.

[XX]	3GPP TS 38.415 "NG-RAN; PDU Session User Plane Protocol".

*** CHANGE (new clause all new text)***

# A.X	Obtaining PDU Set information from Marked and Unmarked (lone) PDU’s

Aguideline is provided to support this case both marked and unmarked packets exist in a stream to which RTP Header Extension for PDU Set marking is applied. Packets from some streams contain an RTP Header Extension while some packets do not contain an RTP Header extension for PDU Set marking. An example could be a stream of multiplexed audio and video packets with only video packets marked. In this case the video stream RTP packets include RTP Header Extension for PDU Set marking for each RTP Packet but on the other hand the audio stream RTP Packets do not contain the RTP Header Extension. Another example could be RTCP packets multiplexed in a stream, these packets do not contain RTP HE signalling PDU Set information.

In this case the network element in the 5G System (e.g. UPF) needs to map both marked and unmarked packets to PDU Sets including the PDU Set information, as PDU Set QoS, when enabled is applied to all packets in a flow. An example guideline for determining PDU Set information at the UPF from either RTP HE or unmarked PDU is given in Table A.X-1.

The middle column indicates how the UPF can derive PDU Set information for packets that include RTP HE for PDU Set information. The right most column indicates how UPF can derive PDU Set information for unmarked packets (lone PDUs).  The left column lists the PDU Set information parameters set in the PDU Session User Plane protocol [XX].

Table A.X-1: Determining PDU Set information at UPF from RTP HE and unmarked PDU

| PDU Set information (PDU Session User Plane Protocol) [XX] | RTP HE | Lone/unmarked PDU |
| --- | --- | --- |
| PDU Set importance | Set by interpreting PSI field RTP HE | Set by 5G System to a configured value based on the payload/packet type (RTP Payload or RTCP packet type) |
| PDU Set Size | Optionally transmitted in additional PSSize field and derived from this field, otherwise this needs to be calculated. | PDU Size |
| End of Data Burst | Can be set by EoDB flag | N/A for lone PDU |
| PDU Sequence number (within a PDU set) | From PDU sequence number in RTP HE | Set to 0 |
| PDU Set Sequence number | Separate number space, e.g. PSSN field from RTP HE with most significant bit is set to 0 (another partition is also possible) | Separate number space e.g. ,set by UPF with most significant bit set to 1 (another partition is also possible) |
| end of PDU set [XX] | End of the PDU Set in RTP HE | Always 1 |

PDU Set importance can be set based on a configured value in the 5G system for unmarked and from the RTP HE for marked PDU’s.

The PDU Set size can be derived from the RTP HE if set, otherwise it needs to be calculated, for unmarked packets it equals the PDU Size (assuming single packet per PDU Set in these cases).

The PDU Sequence number could be retrieved from the PSN in RTP HE, or when no RTP HE is present unmarked PDUit can be set to 0 as only a single PDU is present in the PDU Set.

Deriving the PDU Set Sequence Number includes some additional steps to enable using a different number space for marked and unmarked PDU’s. As an example, the UPF can only use the 9 least significant bits of the RTP HE to number marked PDU;s from RTP. In addition, for unmarked PDU’s it can use it own numbering of PSSN PDU Set Information information and set the most significant bit of PSSN in PDU Set information to 1. Other separations of the number space are also possible.

End of data burst and end of PDU set should not be set for unmarked PDU’s.

NOTE 1:	The RTP HE PSSN cannot map directly to PSSN for PDU Set information when lone PDU's are present as the UPF needs to assign numbers to both.

NOTE 2:	This is solution is to show a possible mapping of PSSN from RTP HE and non RTP HE packets can be done at UPF to enable implementability at UPF. Other solutions can be equally valid and applicable by the UPF. This example was included to illustrate this issue as in practice not all packets are marked and both marked and unmarked packets can exist.