SA4-e (AH) RTC SWG post 130	S4aR250040

# 5 February 2025, Online

Source:	Huawei, Nokia

Title:	[5G_RTP_Ph2] Additional Evaluation of TTNB

Agenda item:	4.6

Document for:	Agreement

# Introduction

This document provides additional evaluation on the usage and feasibility of TTNB considering the work presented in TR 26.822. The conclusion in 7.13 of TR 26.822 should be updated to take this additional evaluation into account.

# Work in SA 4 FS 5G RTP Ph2 and TR 26.822

In FS 5G RTP phase 2 experiments have been carried out to investigate the sender behavior of common RTP sender implementations. The results are shown in Table 6.13.2.5-1 to 6.13.2.5-3 in TR 26.822 (Table 1- 3). Table 1 shows the results for different statistics related to the bursty traffic. Table 2 and Table 3 show the codes for the different scenarios and statistics collected. For more details about the experiments we refer to clause 6.13 of TR 26.822 [1].

Table 1:	Statistical results of experiments in different scenarios [1]

| Scenario (direction) | ibt (mean) | Ibt (stdev) | Bd (mean) | Bs (mean) | Ppb (mean) | %lp (%) | %bp (%) | %lb (%) | %bb (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| w_h (a) | 0.0352 | 0.01437 | 0.0002 | 3463 | 3.55 | 23.2 | 76.7 | 6.6 | 93.4 |
| w_h (b) | 0.01592 | 0.019 | 0.00016 | 3756 | 3.3 | 19.2 | 80.8 | 17.4 | 82.6 |
| u_h (a) | 0.03132 | 0.00873 | 0.00017 | 13895 | 12,3 | 40.2 | 59.8 | 22.6 | 77.4 |
| u_h (b) | 0.42321 | 0.37918 | 0.00007 | 213.92 | 2 | 72.2 | 27.8 | 73.8 | 26.2 |
| g_v_a | 0.01642 | 0.01175 | 0.00052 | 2864 | 7.73 | 2.4 | 97.6 | 0.7 | 99.3 |
| g_v_b | 0.01796 | 0.00967 | 0.00029 | 3109 | 8.6 | 0.2 | 99.8 | 0.2 | 99.8 |
| g_va_a | 0.01252 | 0.0114 | 0.00071 | 4144 | 6.6 | 14.2 | 85.8 | 20.1 | 79.9 |
| g_va_b | 0.01414 | 0.00955 | 0.00033 | 4472 | 7.5 | 14.8 | 85.2 | 24.7 | 75.3 |
| w_s (a) | 0.08732 | 0.03483 | 0.00079 | 24822 | 42.3 | 0.2 | 99.8 | 0.0 | 100.0 |
| w_r (a) | 0.03778 | 0.04627 | 0.00054 | 10671 | 18.24 | 0.7 | 99.3 | 0.1 | 99.9 |
| m_rv | 0.02749 | 0.0404 | 0.00084 | 6630 | 6.38 | 2.5 | 97.5 | 0.5 | 99.5 |
| m_sav (a) | 0.04777 | 0.03838 | 0.00056 | 11906 | 12.18 | 12.6 | 87.4 | 3.3 | 96.7 |
| m_rav (a) | 0.02573 | 0.03412 | 0.00086 | 6692 | 6.8 | 12.2 | 87.8 | 3.2 | 96.5 |

Table 2: Experiment codes carried out in [1] for reading vertical axis of Table 1

| Scenario code |  |
| --- | --- |
| w_h | Windows hangout without network throttling duplex case (Figure 6.13.2.2.2-1) [1] |
| u_h | Ubuntu Hangout with network throttling simplex case (Figure 6.13.2.2.2-2)[1] |
| g_v_a | Gstreamer video only measured on machine A (Figure 6.13.2.3.2-2) [1] |
| g_v_b | Gstreamer video only measured on machine B (Figure 6.13.2.3.2-3) [1] |
| g_va_a | Gstreamer pcm audio + video at receiver (Figure 6.13.2.3.3-1) [1] |
| g_va_b | Gstreamer pcm audio + video at sender (Figure 6.13.2.3.3-2)[1] |
| w_s | Wowza at server measured at server (Figure 6.13.2.4.2-2) [1] |
| w_r | Wowza stream measured at player receiver (Figure 6.13.2.4.2-3) [1] |
| m_rv | MTX server measured vlc player receiver (Figure 6.13.2.4.2-5) [1] |
| m_sav | MTX server video + PCM (Figure 6.13.2.4.2-7) [1] monitored at server |
| m_rav | MTX server video + PCM (Figure 6.13.2.4.2-7) [1] monitored at vlc client |

Table 3 Codified statistics for reading horizontal axis of Table 1

| Code | Metric | Meaning | Additional notes | Statistics |
| --- | --- | --- | --- | --- |
| Bt | Burst Time (Bt) | The time of the burst (corresponding to the first packet), measured in seconds | Bursts are measured as instantaneous transmission of 2 or more packets | Deterministic |
| Ibt | Inter Burst Time (Ibt) | The average duration between 2 consecutive bursts (burst), measured in seconds | This does not consider single ‘lone’ packets transmitted between the bursts | Average and Standard Deviation |
| Bd | Burst Duration (Bd) | The duration between start and end of a burst, measured in seconds | Measures the duration of the burst (in a maximum timeslot of 2 milliseconds ) | Average and Standard Deviation |
| Bs | Burst Size (Bs) | The Size of the Data Burst measured in bytes | Measures the size of the entire burst (in a maximum timeslot of 2 milliseconds) | Average and Standard Deviation |
| Ppb | Packets per burst (Ppb) | Number of packets in each burst | Only bursts of 2 packets or more are considered | Average and Standard deviation |
| #bp | Total Burst packets (#bp) | Number of packets in a burst | This is the number of packets in the trace that is part of a burst | Deterministic |
| #lp | Total Lone packets (#lp) | Number of packets not in a burst | This is the number of packets in the trace that is part not part of a burst | Deterministic |
| #bb | Total Burst Bytes (#bb) | Number of bytes that are part of a burst | This is the number of bytes that is part of a burst | Deterministic |
| #lb | Total Lone Bytes (#lb) | Number of bytes that are not part of a burst | This is the number of bytes in lone packets | Deterministic |
| %lp | Percentage of lone packets (%lp) | Ratio of lone packets to all packets |  | Percentage |
| %bp | Percentage of burst packets (%bp) | Ratio of burst packets to all packets |  | Percentage |
| %lb | Percentage of lone bytes (%lb) | Ratio of lone packet bytes to all bytes |  | Percentage |
| %bb | Percentage of burst bytes (%bb) | Ratio of burst packet bytes to all bytes |  | Percentage |

Some additional observation/evaluation of these results considering the inter-burst time or time to next burst:

Observation 1:  burst durations (Bd) are under 1 millisecond.

Observation 2:  inter burst times (TTNB) (ibt) are dynamically changing (stddev of inter burst time is greater than zero)

Observation 3:  Average inter burst time observed in experiments is between 3 ms and 500 ms (423 ms rounded up)

NOTE:	Signaling could also contain optional extension for signaling 1/10th of millisecond for future applications that have more frequent and shorter burst durations.

NOTE:	Jitter on N6 may make signaling time to next burst which such low granularity unsuitable in some practical cases

NOTE:	in case of 40 ms inter frame times  (e.g. 25 fps) applications may still use multiple burst per frame transmission in case the application supports it a frame can be transmitted in separate short bursts (under 1 millisecond) depending on the sender implementation, this is in case a paced sending option is used spreading packets over a larger interval.

Observation 4:   In typical RTP cases for real time communication, up to 99 percent of bytes is carried in bursts, enabling optimization for up to 99 percent of traffic using burst based processing..

Conclusion based on SA4 work from FS 5G_RTP ph2:

Optimizing scheduling of bursty RTP traffic is a huge opportunity for real time (RTP) case affecting up to 99 percent of traffic.

TTNB can be signaled using millisecond accuracy and resolutions based on observed patterns (burst time under 1 ms, inter burst time 3- 500 ms). Anchoring to first or last packet should be specified, but should not have a huge impact in practice.

Bursts are short, in the case of RTP up to about 1 millisecond, this also follows the SA2 definition of a data burst (” Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time.

NOTE 1:	A Data Burst can be composed of one or multiple PDU Sets.”).

User plane signaling is beneficial as the inter burst times changes dynamically (stdev ibt > 0). Control plane signaling would introduce more overhead and sync issues for non-stationary cases.

User plane and control plane signaling should be complementary and cover different use cases, both are present in [2].

User plane signaling is preferable for dynamically changing traffic characteristics as to avoid sync issues and overhead present in control plane signaling.

RAN2 has already indicated that TTNB is useful when provided and accurate. From the conclusion of Key Issue 12 in TR 26.822, it is not clear what is needed to be further studied, as both SA4 and RAN2 have identified clear benefits.

# Work in SA2 and updates to TS 23.501

SA2 work following observations of traffic characteristics explicitly enables signaling of dynamically changing traffic characteristics in the user plane (again the inter burst times/periodicity is not fixed), SA2 stage 2 explicitly refers to this as dynamically changing traffic characteristics contrary to stationary dynamic traffic characteristics in clause 5.37.10 [2], including both data burst size and time to next burst. For dynamically changing traffic characteristics it is clear they benefit from signaling in the user plane. SA2 is relying on SA4 to define the exact signaling in TS 26.522.

# Proposal

- Update the TR [1] to include the additional observations and evaluation of TTNB in clause 6.13

- Update the TR [1] conclusions for key issue #12 to proceed with normative work on TTNB :

“Do normative work for signaling time to next burst, when deterministically known, and dynamically changing, from RTP senders in a HE”.

Remove NOTE 2 in clause 7.13 of [1]

- Proceed with the standardization/normative work for signaling TTNB in the user plane using RTP HE.

# References

[1] 			3GPP TR 26.822 Study on 5G Real-time Transport Protocol Configurations

[2] 			3GPP TR 23.501 System architecture for the 5G System (5GS)