SA4-e (AH) RTC SWG post 130	S4aR250041

# 5 February 2025, Online

Source:	Huawei, Nokia

Title:	[5G_RTP_Ph2] Additional considerations for definition of TTNB in SA4

Agenda item:	4.6

Document for:	Agreement

# Introduction

This document provides additional evaluation on the usage and feasibility of TTNB considering the work presented in TR 26.822. The proposal is to take this into account for the definition of Time To Next Burst (TTNB).

# Work in SA4 FS_5G_RTP_Ph2 and TR 26.822

In FS_5G_RTP_Ph2, experiments have been carried out to investigate the sender behavior of common RTP sender implementations. The results are shown in Table 6.13.2.5-1 to 6.13.2.5-3 in TR 26.822 (Table 1- 3). Table 1 shows the results for different statistics related to the bursty traffic. Table 2 and Table 3 show the codes for the different scenarios and statistics collected. For more details about the experiments we refer to clause 6.13 of TR 26.822 [1].

Table 1:	Statistical results of experiments in different scenarios [1]

| Scenario (direction) | ibt (mean) | Ibt (stdev) | Bd (mean) | Bs (mean) | Ppb (mean) | %lp (%) | %bp (%) | %lb (%) | %bb (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| w_h (a) | 0.0352 | 0.01437 | 0.0002 | 3463 | 3.55 | 23.2 | 76.7 | 6.6 | 93.4 |
| w_h (b) | 0.01592 | 0.019 | 0.00016 | 3756 | 3.3 | 19.2 | 80.8 | 17.4 | 82.6 |
| u_h (a) | 0.03132 | 0.00873 | 0.00017 | 13895 | 12,3 | 40.2 | 59.8 | 22.6 | 77.4 |
| u_h (b) | 0.42321 | 0.37918 | 0.00007 | 213.92 | 2 | 72.2 | 27.8 | 73.8 | 26.2 |
| g_v_a | 0.01642 | 0.01175 | 0.00052 | 2864 | 7.73 | 2.4 | 97.6 | 0.7 | 99.3 |
| g_v_b | 0.01796 | 0.00967 | 0.00029 | 3109 | 8.6 | 0.2 | 99.8 | 0.2 | 99.8 |
| g_va_a | 0.01252 | 0.0114 | 0.00071 | 4144 | 6.6 | 14.2 | 85.8 | 20.1 | 79.9 |
| g_va_b | 0.01414 | 0.00955 | 0.00033 | 4472 | 7.5 | 14.8 | 85.2 | 24.7 | 75.3 |
| w_s (a) | 0.08732 | 0.03483 | 0.00079 | 24822 | 42.3 | 0.2 | 99.8 | 0.0 | 100.0 |
| w_r (a) | 0.03778 | 0.04627 | 0.00054 | 10671 | 18.24 | 0.7 | 99.3 | 0.1 | 99.9 |
| m_rv | 0.02749 | 0.0404 | 0.00084 | 6630 | 6.38 | 2.5 | 97.5 | 0.5 | 99.5 |
| m_sav (a) | 0.04777 | 0.03838 | 0.00056 | 11906 | 12.18 | 12.6 | 87.4 | 3.3 | 96.7 |
| m_rav (a) | 0.02573 | 0.03412 | 0.00086 | 6692 | 6.8 | 12.2 | 87.8 | 3.2 | 96.5 |

Table 2: Experiment codes carried out in [1] for reading vertical axis of Table 1

| Scenario code |  |
| --- | --- |
| w_h | Windows hangout without network throttling duplex case (Figure 6.13.2.2.2-1) [1] |
| u_h | Ubuntu Hangout with network throttling simplex case (Figure 6.13.2.2.2-2)[1] |
| g_v_a | Gstreamer video only measured on machine A (Figure 6.13.2.3.2-2) [1] |
| g_v_b | Gstreamer video only measured on machine B (Figure 6.13.2.3.2-3) [1] |
| g_va_a | Gstreamer pcm audio + video at receiver (Figure 6.13.2.3.3-1) [1] |
| g_va_b | Gstreamer pcm audio + video at sender (Figure 6.13.2.3.3-2)[1] |
| w_s | Wowza at server measured at server (Figure 6.13.2.4.2-2) [1] |
| w_r | Wowza stream measured at player receiver (Figure 6.13.2.4.2-3) [1] |
| m_rv | MTX server measured vlc player receiver (Figure 6.13.2.4.2-5) [1] |
| m_sav | MTX server video + PCM (Figure 6.13.2.4.2-7) [1] monitored at server |
| m_rav | MTX server video + PCM (Figure 6.13.2.4.2-7) [1] monitored at vlc client |

Table 3 Codified statistics for reading horizontal axis of Table 1

| Code | Metric | Meaning | Additional notes | Statistics |
| --- | --- | --- | --- | --- |
| Bt | Burst Time (Bt) | The time of the burst (corresponding to the first packet), measured in seconds | Bursts are measured as instantaneous transmission of 2 or more packets | Deterministic |
| Ibt | Inter Burst Time (Ibt) | The average duration between 2 consecutive bursts (burst), measured in seconds | This does not consider single ‘lone’ packets transmitted between the bursts | Average and Standard Deviation |
| Bd | Burst Duration (Bd) | The duration between start and end of a burst, measured in seconds | Measures the duration of the burst (in a maximum timeslot of 2 milliseconds ) | Average and Standard Deviation |
| Bs | Burst Size (Bs) | The Size of the Data Burst measured in bytes | Measures the size of the entire burst (in a maximum timeslot of 2 milliseconds) | Average and Standard Deviation |
| Ppb | Packets per burst (Ppb) | Number of packets in each burst | Only bursts of 2 packets or more are considered | Average and Standard deviation |
| #bp | Total Burst packets (#bp) | Number of packets in a burst | This is the number of packets in the trace that is part of a burst | Deterministic |
| #lp | Total Lone packets (#lp) | Number of packets not in a burst | This is the number of packets in the trace that is part not part of a burst | Deterministic |
| #bb | Total Burst Bytes (#bb) | Number of bytes that are part of a burst | This is the number of bytes that is part of a burst | Deterministic |
| #lb | Total Lone Bytes (#lb) | Number of bytes that are not part of a burst | This is the number of bytes in lone packets | Deterministic |
| %lp | Percentage of lone packets (%lp) | Ratio of lone packets to all packets |  | Percentage |
| %bp | Percentage of burst packets (%bp) | Ratio of burst packets to all packets |  | Percentage |
| %lb | Percentage of lone bytes (%lb) | Ratio of lone packet bytes to all bytes |  | Percentage |
| %bb | Percentage of burst bytes (%bb) | Ratio of burst packet bytes to all bytes |  | Percentage |

Some additional observation/evaluation of these results considering the inter-burst time or time to next burst are given as follows:

Observation 1:  Burst durations (Bd) are under 1 millisecond.

Observation 2:  Average inter-burst time observed in experiments is between 3 ms and 500 ms (423 ms rounded up).

NOTE:  In typical RTP cases it will be hard to measure the exact packet departure time for each packet in a burst as RTP is in implemented in user space, not in kernel or lower levels of the devices. The actual transmission of packets is handled by the kernel and network interface. Even after RTP hands off packets to the OS, network interface controllers (NICs) and other hardware buffers may introduce additional transmission delays.

NOTE: Time to next burst is only to be used when N6 jitter is negligible as indicated in TS 23.501

Conclusion 1:  Time to Next Burst can be indicated in milliseconds.

NOTE:	Signaling should also contain optional extension for signaling 10th of millisecond for future applications that have more frequent and shorter burst durations

NOTE:	Jitter on N6 may make signaling time to next burst which such low granularity unsuitable in some practical cases

Conclusion 2:  In this case, the definition of time to next burst (anchored to first, middle, or last) may be less practically relevant, and perhaps this is a reason no rigorous definition was yet made.

In a practical system, it may be beneficial to use the transmission time of the first packet as an anchor point, as one may not know the exact transmission time of later packets when sending the first packet that may already include the TTNB information. However, on the millisecond scale, these differences will often be negligible.

For using TTNB, only the deterministic way should be considered where an application decides on the next sending time in a scheduler that runs independent from a media encoder or RTP packager. Prediction of TTNB is FFS.

In addition, SA2 has provided an updated definition of time to next burst in CR in S2-2501110 (agreed in SA2) that changes the definition to:

“The time to next data burst, which is the interval between the transmission of the last PDU in the current data burst and the first PDU of the next data burst”,

# Proposal

- Take this information into account when defining the time to next burst

- Add these additional observations on TTNB in clause 6.13.2.5 of the TR 26.822

# References

[1] 			3GPP TR 26.822 Study on 5G Real-time Transport Protocol Configurations

[2] 			3GPP TR 23.501 System architecture for the 5G System (5GS)

[3]			3GPP  Change Request S2-2501110 Update to data burst related