SA4-e (AH) RTC SWG post #130	S4aR250044

# 5 February 2025, Online

Source:	Nokia, Huawei

Title:	[5G_RTP_Ph2] 3GPP progress on Time to Next Burst

Agenda item:	4.6

Document for:	Agreement

# Background

This document summarizes the Rel-18 and Rel-19 progress on Time to Next Burst (TTNB) in SA4 and other WGs, discusses the required actions considering the current state and proposes a way forward for SA4.

### SA4 progress on TTNB

In FS_5G_RTP_Ph2, SA4 had extensive discussions on Time to Next Burst (TTNB) and included several related solutions in TR 26.822.

- Solution #6: Time to next burst extension for the RTP HE for PDU Set marking

- Solution #13: Dynamic traffic characteristics of RTP sender implementations

- Solution #16: RTP header extension for dynamic traffic characteristics

- Solution #24: Traffic pattern prediction for real-time video communication

- Solution #25: Definition of the time to the next data burst

#6 and #16 propose that the application signals a TTNB value to the network. The difference is that #6 proposes the TTNB field to be added to the existing RTP HE for PDU Set marking, whereas #16 proposes a new header extension for the burst characteristics including a TTNB field.

#13 explores dynamic traffic characteristics of various RTP sender implementations and presents statistics for several metrics including TTNB (a.k.a. inter-burst time), which is defined as “the average duration between 2 consecutive bursts”.

#24 identifies two factors that affect TTNB: video encoding time and rate adaptation.

#25 offers different definitions for the starting time based on which TTNB is measured.

Based on the above solutions, conclusions of KI#12 in TR 26.822 includes the following:

- Define TTNB in coordination with SA2 and RAN2.

NOTE 2:	RAN2 has indicated that TTNB may be useful if provided in time and is reliable. SA4 needs further evaluation before proceeding with normative work.

Based on this conclusion, SA4 included the following objective in the WI 5G_RTP_Ph2:

8.	Conduct normative work on burst size, time to next burst, data boosting indication and the definition of data burst.

NOTE: RAN2 has indicated that TTNB may be useful if provided in time and is reliable. SA4 needs further evaluation before proceeding with normative work.

Following up on the solution #25, an updated version was presented by Qualcomm in the RTC SWG telco on January 15. The document was revised based on the online comments and noted (S4aR250013).

### Related LS exchanges

SA4 was the first WG to identify the potential benefit of signaling TTNB / inter- burst time to the 5G network. After discussions within the WG, SA4 contacted SA2 and RAN2 for feedback [S4-230739]. An excerpt from the LS is below:

In addition to marking the last PDU of the data burst, SA4 sees the benefit of using additional bits to indicate inter-burst time, which may change dynamically due to various reasons, including application-layer rate control. SA4 believes that this can enable the RAN to switch to the most appropriate power state. SA4 kindly requests feedback from SA2 and RAN2 on the value and feasibility of such solution and if that can be supported within Rel. 18 timeframe.

The discussion continued with some questions from SA2 to SA4 and RAN2. SA4 sent the following reply to SA2 [S4-241776]:

Question2 [for SA4 and RAN2]: There is some discussion about time to next burst.

To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?

Reply 2: Yes, it may be possible depending on the sender implementation, connectivity between the sender and 5G System and predictability of the traffic source. If the N6 link is covered by transport level agreements and N6 jitter and bandwidth variation is known or can be predicted, senders can estimate how much time it will take to send all packets in a data burst and determine the start time of the next burst within an implementation-dependent error margin. Therefore, application servers may be able to provide an estimate of TTNB in the first packet of the burst for such cases and conditions.

SA4 would like to note that TTNB estimation is only applicable for bursty traffic, and the same UE may receive other traffic which may be continuous. A UE may also receive multiple traffic flows originating from different traffic sources with respective TTNBs and the resulting data burst traffic pattern seen at the RAN may not be well represented by any of the TTNBs.

SA4 is studying the topic in Key Issue #12: “Enhancements of Data Burst Marking” of FS_5G_RTP_Ph2 as documented in clause 5.12 of draft TR 26.822.

SA4 also asked RAN2 whether TTNB is still useful if received in the last packet of the burst. RAN2 referred to their previous reply to SA2 [R2-2407733] below and confirmed that “the reply provided earlier still holds for the case where TTNB is provided at the end of the burst.”

Q: Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

Regarding Question 2 on time to next burst, RAN2 thinks that time to next burst may be useful for the network scheduling for downlink, if it is provided in advance and is reliable and accurate at RAN.

### Ongoing Rel-19 work in SA2 on TTNB

SA2 has been working on integration of data burst handling into the 5GC specifications, including the definition of TTNB signaling. Excerpts from two CRs agreed in SA2#166 (Orlando) are copied below for convenience.

S2-2412943 – CR5785 to TS 23.501

#### 5.37.x.y	Time to Next Burst marking

The time to next data burst may be provided to the NG-RAN by the UPF to assist NG-RAN’s behavior in downlink. As described in TS 23.503 [45] clause 6.1.3.27.5 the PCF may include a Time to Next Burst Marking Indication within a PCC Rule to request the UPF to identify and mark the time to next data burst for the corresponding QoS Flow. The PCF also includes the DL Protocol Description received from AF.

Based on the Time to Next Burst indication in a PCC Rule, the SMF instructs the UPF to identify and mark the time to next data burst in downlink. The SMF may provide the UPF the Protocol Description used by the service data flow received in the PCC Rule.

According to the Time to Next Burst indication and DL Protocol Description from the SMF, the UPF identifies the time to next data burst carried in an N6 RTP Header Extension/transport protocol header as defined in TS 26.522 [179] and sends the identified time to next data burst to NG-RAN in a downlink GTP-U header.

Time to next data burst marking is only used when the deployment is such that the time to next data burst is sufficiently accurate i.e., the jitter (e.g. the N6 jitter) is sufficiently limited without impact to the accuracy of the time to next data burst.

S2-2412952 – CR1324 to TS 23.503

##### 6.1.3.27.5	Data Burst Based Handling

The Data Burst Based Handling includes the detection and marking of End of Data Burst, the identification and marking of time to next Data Burst and the identification and marking of Data Burst Size.

The detection and marking of End of Data Burst is defined in clause 5.37.8.3 of TS 23.501 [2]. Based on the AF provided DL Protocol Description (see clause 5.37.5 of TS 23.501 [2] and TS 26.522 [40]) and/or local configuration, the PCF generates the End of Data Burst Marking Indication (see clause 5.37.8.3 of TS 23.501 [2]) and provides it and the DL Protocol Description in the PCC rule to the SMF.

The identification and marking of Data Burst size is defined in clause 5.37.x.1 of TS 23.501 [2]. Based on the AF provided a Data Burst Size Marking Support Indication (which indicates that the Data Burst size is supported and marked in the header of the protocol identified by the DL Protocol Description) and DL Protocol Description (see clause 5.37.5 of TS 23.501 [2] and TS 26.522 [40]) and operator policy, the PCF provides the Data Burst Size Marking Indication (see clause 5.37.x.1 of TS 23.501 [2]) and the DL Protocol Description in the PCC rule to the SMF.

The identification and marking of time to next Data Burst is defined in clause 5.37.x.y of TS 23.501 [2]. Based on the AF provided a Time to Next Burst Marking Support Indication (which indicates that the time to next Data Burst is supported and marked in the header of the protocol identified by the DL Protocol Description) and DL Protocol Description (see clause 5.37.5 of TS 23.501 [2] and TS 26.522 [40]) and operator policy, the PCF provides the Time to Next Burst Marking Indication (see clause 5.37.x.y of TS 23.501 [2]) and the DL Protocol Description in the PCC rule to the SMF.

The identification and marking of time to next Data Burst is only used when the deployment is such that the time to next Burst is sufficiently accurate i.e., the jitter (e.g. the N6 jitter) is sufficiently limited without impact to the accuracy of time to next Data Burst.

# Discussion

In the study phases of Rel-18 and Rel-19, SA4 had extensive discussions on TTNB and identified its potential benefit for the network, as documented in TR 26.822.

SA4 liaised with SA2 and RAN2 and received confirmation on the feasibility and potential value of TTNB for the 5G System.

As a result of this exchange, SA2 has been working on integration of TTNB into the core network specifications within their Rel-19 WI XRM_Ph2. In their agreed CRs, it is acknowledged that N6 jitter needs to be considered while assessing the accuracy of TTNB, aligned with the previous communication among SA2, SA4 and RAN2.

Based on the above progress, SA4 needs to enable the application signaling for TTNB within Rel-19 and clarify the definition of TTNB. It is important that SA4 clarifies the TTNB definition as soon as possible, so that the same definition could be reused by SA2 in their specifications.

TTNB signaling could be included in a new RTP HE designed specifically for signalling of dynamically changing traffic characteristics, assuming that SA4 reaches consensus on the introduction of such HE.

Regarding the further evaluation on TTNB mentioned in the conclusion of KI#12 in TR 26.822, we believe there is sufficient evaluation documented in the FS_5G_RTP_Ph2 study as detailed in tdoc S4aR250042.

# Proposal

- Take into account the evaluations given in TR 26.822 on TTNB and burst durations.

- Enable signaling of TTNB in an appropriate RTP header extension considering the solutions #6 and #16 in TR 26.822.

- Further discuss the TTNB definition and agree on a definition to be added to TS 26.522 by the Geneva meeting in February.