# Scenario 1: 1-DOF, Pre-rendered format=SBA

## Capabilities

UE-1:					IVAS – cf-send=SBA,Stereo, cf-recv=all immersive formats

UE-2a (pre-rendering device or MGW):	IVAS – cf-send=SR (sr-dof=1; sr-tc=LCLD,LC3+), 
IVAS-pass-through, cf-recv=all immersive formats

UE-2b (lightweight device):		IVAS – cf-send=Stereo,  cf-recv=SR (sr-dof=3; sr-tc=LCLD,LC3+)

Besides (limited) IVAS support, all devices support EVS, AMR-WB, AMR-NB

## Call established by UE-1

UE-1 offer to UE-2a

m=audio 49152 RTP/AVP 96 97 98 99 100 101 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-send=SBA,Stereo; max-red=220 a=rtpmap:97 EVS/16000/1 a=fmtp:97 max-red=220 a=rtpmap:98 AMR-WB/16000/1 a=fmtp:98 mode-change-capability=2; max-red=220 a=rtpmap:99 AMR-WB/16000/1 a=fmtp:99 mode-change-capability=2; max-red=220; octet-align=1 a=rtpmap:100 AMR/8000/1 a=fmtp:100 mode-change-capability=2; max-red=220 a=rtpmap:101 AMR/8000/1 a=fmtp:101 mode-change-capability=2; max-red=220; octet-align=1 a=ptime:20 a=maxptime:240

UE-1 offers SBA and stereo on send and open immersive on receive.

UE-2a offer to UE-2b after reception of UE-1 offer

m=audio 49152 RTP/AVP 96 97 98 99 100 101 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-send=SR,SBA,Stereo; sr-dof=1; sr-tc=LCLD,LC3+; pi-types-recv=rhor; pi-br=20; max-red=220 a=rtpmap:97 EVS/16000/1 a=fmtp:97 max-red=220 a=rtpmap:98 AMR-WB/16000/1 a=fmtp:98 mode-change-capability=2; max-red=220 a=rtpmap:99 AMR-WB/16000/1 a=fmtp:99 mode-change-capability=2; max-red=220; octet-align=1 a=rtpmap:100 AMR/8000/1 a=fmtp:100 mode-change-capability=2; max-red=220 a=rtpmap:101 AMR/8000/1 a=fmtp:101 mode-change-capability=2; max-red=220; octet-align=1 a=ptime:20 a=maxptime:240

As UE-2a supports SR and IVAS pass-through, it offers to UE-2b SR and the offered send format from UE-1.

UE-2a offers LCLD first because the offer from UE-1 indicates SBA which coincides with UE2a receive preference. For SBA pre-rendered format, LCLD coding is default. If ISM had been offered and answered and should be used in the receive session from UE-1, LC3+ would be the first codec choice indicated to UE-2b.

UE-2b answer to UE-2a

m=audio 49152 RTP/AVP 96 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-recv=SR; cf-send=Stereo; sr-dof=1; sr-tc=LCLD; pi-types-send=rhor; pi-br=20; max-red=220 a=ptime:20 a=maxptime:240

UE-2b confirms SR receive support with LCLD. On send, it offers IVAS stereo.

UE-2a answer to UE-1

m=audio 49152 RTP/AVP 96 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-recv=SBA,Stereo; cf-send=Stereo; max-red=220 a=ptime:20 a=maxptime:240

UE-2a confirms the offered send formats from UE-1 and prepares SR pre-renderer.

## Call established by UE-2b

UE-2b offer to UE-2a

m=audio 49152 RTP/AVP 96 97 98 99 100 101 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-send=Stereo; cf-recv=SR; sr-dof=3; pi-types-send=rhor; pi-br=20; max-red=220 a=rtpmap:97 EVS/16000/1 a=fmtp:97 max-red=220 a=rtpmap:98 AMR-WB/16000/1 a=fmtp:98 mode-change-capability=2; max-red=220 a=rtpmap:99 AMR-WB/16000/1 a=fmtp:99 mode-change-capability=2; max-red=220; octet-align=1 a=rtpmap:100 AMR/8000/1 a=fmtp:100 mode-change-capability=2; max-red=220 a=rtpmap:101 AMR/8000/1 a=fmtp:101 mode-change-capability=2; max-red=220; octet-align=1 a=ptime:20 a=maxptime:240

For SR receive clients, we need to allow a limited cf-recv offer (cf-recv=SR) as MONO may be the only alternative receive format for IVAS.

Offer from lightweight device on SR codec should be open or always list the two codec options (LCLD or LC3plus).

UE-2a offer to UE-1 after reception of UE-2b offer

m=audio 49152 RTP/AVP 96 97 98 99 100 101 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-send=Stereo; cf-recv=OSBA,SBA,OMASA,MASA,ISM,MC,Stereo; max-red=220 a=rtpmap:97 EVS/16000/1 a=fmtp:97 max-red=220 a=rtpmap:98 AMR-WB/16000/1 a=fmtp:98 mode-change-capability=2; max-red=220 a=rtpmap:99 AMR-WB/16000/1 a=fmtp:99 mode-change-capability=2; max-red=220; octet-align=1 a=rtpmap:100 AMR/8000/1 a=fmtp:100 mode-change-capability=2; max-red=220 a=rtpmap:101 AMR/8000/1 a=fmtp:101 mode-change-capability=2; max-red=220; octet-align=1 a=ptime:20 a=maxptime:240

The SR pre-renderer supports 1-DOF pre-rendering of any IVAS-Immersive format. Accordingly, it offers to UE-1 all immersive formats in receive. The cf-send=Stereo offer from UE-2b is forwarded as UE-2a will just pass through such payloads without transcoding.

Note: For MGWs instead of UE-2a, it should be allowed to offer SR as additional send format towards UE-1.

UE-1 answer to UE-2a

m=audio 49152 RTP/AVP 96 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-recv=Stereo; cf-send=SBA; max-red=220 a=ptime:20 a=maxptime:240

UE-1 supports SBA in send. It confirms Stereo in receive.

UE-2a answer to UE-2b after reception of UE-1 answer

m=audio 49152 RTP/AVP 96 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-recv=Stereo; cf-send=SR; sr-dof=1; sr-tc=LCLD; pi-types-recv=rhor; pi-br=20; max-red=220 a=ptime:20 a=maxptime:240

UE-2a confirms the SR session to UE-2b but indicates that it merely does DOF1. The confirmed Stereo receive from UE-1 is forwarded.

UE-2a selects LCLD because the answer from UE-1 indicated SBA for which LCLD coding is default. If ISM had been listed in the answer and should be used in the receive session from UE-1, LC3plus would be the codec choice indicated to UE-2b.

## Call established by UE-2a

UE-2a offer to UE-2b

m=audio 49152 RTP/AVP 96 97 98 99 100 101 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-send=OSBA,SBA,OMASA,MASA,ISM,MC,Stereo,SR; sr-dof=1; sr-tc=LCLD,LC3plus; pi-types-recv=rhor; pi-br=20; max-red=220 a=rtpmap:97 EVS/16000/1 a=fmtp:97 max-red=220 a=rtpmap:98 AMR-WB/16000/1 a=fmtp:98 mode-change-capability=2; max-red=220 a=rtpmap:99 AMR-WB/16000/1 a=fmtp:99 mode-change-capability=2; max-red=220; octet-align=1 a=rtpmap:100 AMR/8000/1 a=fmtp:100 mode-change-capability=2; max-red=220 a=rtpmap:101 AMR/8000/1 a=fmtp:101 mode-change-capability=2; max-red=220; octet-align=1 a=ptime:20 a=maxptime:240

UE-2a offers all formats in send to UE-2b including SR. Since the immersive format it would like to receive from UE-1 is OSBA, the SR codec listed first in the offer is LCLD. In case, the intended receive format was ISM or MC, the SR codec listed first in the offer would be LC3plus.

UE-2b answer to UE-2a

m=audio 49152 RTP/AVP 96 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-send=Stereo; cf-recv=SR; sr-dof=1; sr-tc=LCLD; pi-types-send=rhor; pi-br=20; max-red=220 a=ptime:20 a=maxptime:240

UE-2b selects SR as receive format and echoes the related send parameters while confirming support of LCLD.

UE-2a offer to UE-1 after reception of UE-2b answer

m=audio 49152 RTP/AVP 96 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-send=Stereo; cf-recv=OSBA,SBA,OMASA,MASA,ISM,MC,Stereo; max-red=220 a=ptime:20 a=maxptime:240

UE-2a forwards the send format from UE-2b and indicates a preference for receive direction to use a format that operates in CLDFB domain like the LCLD codec.

Note: For MGWs instead of UE-2a, it should be allowed to offer SR as additional send format towards UE-1.

UE-1 answer to UE-2a

m=audio 49152 RTP/AVP 96 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-recv=Stereo; cf-send=SBA; max-red=220 a=ptime:20 a=maxptime:240

UE-2a prepares SBA decoding and SBA SR pre-rendering instance.

It is possible that UE-1 responds with cf-send=ISM (or any other format with rendering in time domain) in which case the default split rendering codec will change to LC3plus. Given that IVAS split rendering supports both codecs with all formats, the existing negotiation will work. Another approach would be to re-negotiate the sr-tc between UE-2a and UE-2b and change the codec to LC3plus.

# Scenario 2: 0-DOF, Pre-rendered format=ISM

## Capabilities

UE-1:					IVAS – cf-send=ISM,Stereo, cf-recv=all immersive formats

UE-2a (pre-rendering device or MGW):	IVAS – cf-send=SR (sr-dof=0; sr-tc=LCLD,LC3plus), 
IVAS-pass-through,  cf-recv=all immersive formats

UE-2b (lightweight device):		IVAS – cf-send=Stereo, cf-recv=SR (sr-dof=3; sr-tc=LCLD,LC3plus)

Besides (limited) IVAS support, all devices support EVS, AMR-WB, AMR-NB

## Call established by UE-1

UE-1 offer to UE-2a

m=audio 49152 RTP/AVP 96 97 98 99 100 101 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-send=ISM,Stereo; max-red=220 a=rtpmap:97 EVS/16000/1 a=fmtp:97 max-red=220 a=rtpmap:98 AMR-WB/16000/1 a=fmtp:98 mode-change-capability=2; max-red=220 a=rtpmap:99 AMR-WB/16000/1 a=fmtp:99 mode-change-capability=2; max-red=220; octet-align=1 a=rtpmap:100 AMR/8000/1 a=fmtp:100 mode-change-capability=2; max-red=220 a=rtpmap:101 AMR/8000/1 a=fmtp:101 mode-change-capability=2; max-red=220; octet-align=1 a=ptime:20 a=maxptime:240

UE-1 offers ISM and stereo on send and open immersive on receive.

UE-2a offer to UE-2b after reception of UE-1 offer

m=audio 49152 RTP/AVP 96 97 98 99 100 101 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-send=SR,ISM,Stereo; sr-dof=0; sr-tc=LC3plus,LCLD; pi-types-recv=rhor; pi-br=20; max-red=220 a=rtpmap:97 EVS/16000/1 a=fmtp:97 max-red=220 a=rtpmap:98 AMR-WB/16000/1 a=fmtp:98 mode-change-capability=2; max-red=220 a=rtpmap:99 AMR-WB/16000/1 a=fmtp:99 mode-change-capability=2; max-red=220; octet-align=1 a=rtpmap:100 AMR/8000/1 a=fmtp:100 mode-change-capability=2; max-red=220 a=rtpmap:101 AMR/8000/1 a=fmtp:101 mode-change-capability=2; max-red=220; octet-align=1 a=ptime:20 a=maxptime:240

As UE-2a offers SR and IVAS pass-through, it offers to UE-2b SR and the offered send format from UE-1.

UE-2a offers LC3plus because the offer from UE-1 indicates ISM which coincides with UE-2a receive preference. For ISM pre-rendered format, LC3plus coding is default.

UE-2b answer to UE-2a

m=audio 49152 RTP/AVP 96 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-recv=SR; cf-send=Stereo; sr-dof=0; sr-tc=LC3plus; pi-types-send=rhor; pi-br=20; max-red=220 a=ptime:20 a=maxptime:240

UE-2b confirms SR receive support. On send, it offers IVAS stereo.

UE-2a answer to UE-1

m=audio 49152 RTP/AVP 96 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:556 b=RS:0 b=RR:2000 a=rtpmap:96 IVAS/16000/1 a=fmtp:96 cf-recv=ISM,Stereo; cf-send=Stereo; max-red=220 a=ptime:20 a=maxptime:240

UE-2a confirms the offered send formats from UE-1 and prepares SR pre-renderer.

Note: Once the session has been established, as this is 0-DOF, the LC3plus payload formatting within the IVAS payload type could be reused on the link between pre-rendering device and lightweight format.