3GPP TSG-SA4 RTC SWG Telco 	S4aR250056

# 5th Feb 2025, 12-2pm CET

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.567 | CR |  | rev |  | Current version: | 0.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_IMS] Functions of SR-DCMTSI Client |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_IMS |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding detailed functions of SR-DCMTSI client to address the editor’s note in clause 4.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding detailed functions of SR-DCMTSI client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Technical Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 4.4   Split Rendering DCMTSI Client (SR-DCMTSI)

An SR-DCMTSI is a DCMTSI client in terminal defined as per clause 3.1 of TS 26.114 which supports split rendering. It is responsible for acquiring media capabilities and interacting with IMS functions during a split rendering session. It is responsible for the signalling with DCSF via IMS AS to set-up a split rendering session and negotiating with the MF and DCAS about the parameters of the split rendering session, for example, the scene to be rendered and the split of rendering operations.

The SR-DCMTSI Client performs the following functions:

-	Initiate a request of split rendering session establishment,

-	Establish split rendering session, including signalling with DCSF via IMS AS to set up the data channel, and MF and DC AS to set up the split rendering,

-	Operate the rendering loop on the UE,

-	Update the rendering loop according to the network parameters and media capabilities with IMS functionalities,

-	Stop the split rendering session or switch from a split rendering session to a local rendering session.

_____________________ End of 1st Change________________________________