3GPP TSG-SA4 RTC SWG Telco 	S4aR250057

# 5th Feb 2025, 12-2pm CET

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.567 | CR |  | rev |  | Current version: | 0.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_IMS] Functions of MF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_IMS |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding detailed functions of MF for split rendering to address the editor’s note in clause 4.5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding detailed functions of MF for split rendering. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Technical Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 4.5	Media Function (MF)

MF is a Media Function which supports split rendering. It is responsible for interacting with SR-DCMTSI client and the DC AS during split-rendering process, monitoring resource usage, managing and running the split rendering process, etc.

The MF performs the following functions:

- Negotiate and interact with SR-DCMTSI Client (via IMS AS) and DC AS to establish the split rendering session,

- Operate the rendering loop, including, receiving meta-data from the SR-DCMTSI client, rendering frames, and forwarding requests to DC AS based on the split rendering logic,

- Expose its capabilities, capacity and media processing and rendering capabilities to the DC AS,

- Update the rendering loop according to the split rendering logic and requests from UE. If supported, the update can be based on the network parameters and interactions with other IMS functionalities,

- Stop/Pause/Resume the split rendering session according to the requests from UE.

_____________________ End of 1st Change________________________________