Source:		Dolby Germany GmbH

Title:	Permanent Document for FS_FGS (Feasibility Study for Film Grain Synthesis)

Version:		1.0.1.

Agenda Item:		9.9

Document for:		Agreement

Work Item information:

| Work Item Name | FS_FGS (Feasibility Study for Film Grain Synthesis) |
| --- | --- |
| Acronym | FS_FGS |
| Unique ID | 1000016 |
| Target release | Rel-18 |

Source details:

| Rapporteur | Name | Brian Lee |
| --- | --- | --- |
|  | Company | Dolby Labs |
|  | Email | Brian.lee@dolby.com |
| Editor | Name |  |
|  | Company |  |
|  | Email |  |

Revision history:

| Version | Date | Meeting | Subject/Comment |
| --- | --- | --- | --- |
| 1.0.1. | 2023-10-23 | SA4 post 125e | Updated contents |
| 1.0.0 | 2023-08-24 | SA4#125 | Initial Submission |

# Contents

Contents	2

1	Introduction	3

2	Proposed Scenario and evaluation framework	3

3	Considerations on UE requirements	5

4	Communication with JVET	5

# 1	Introduction

It was decided to run permanent document on the side of the above referred study item to keep track of the ongoing tasks required to complete the study item objectives as define here:

- Define motivating use cases and scenarios for the use of Film Grain synthesis in 5G video video services.

- Document relevant existing Film Grain Synthesis technologies that are not included in 3GPP today.

- Provide evaluation using HEVC of the benefits/drawbacks of corresponding solutions, including film grain characteristics SEI message (ITU-T H.274) including performance results, complexity and implementation aspects, interoperability, system integration, etc. following the example in TR26.955 based on selected scenarios.

- Use the characterization framework in TR26.955 when possible and extend it when necessary, i.e. with subjective tests results.

- Study and identify relevant UE requirements for consistent usability of the technology.

- Collaborate with MPEG/JVET and other organizations to ensure broad interoperability across different ecosystems.

- Identify relevant interoperability and system level aspects to potentially support Film Grain Synthesis.

- Identify if any new normative work would be justified and if so, provide relevant conclusions.

This study will be done in collaboration with other organizations as needed, e.g. JVET, MPEG, CTA WAVE, 5G-MAG, DASH-IF etc.

The approved WID can be found here: SP-230539 New SID on Feasibility Study on Film Grain Synthesis

# 2	Proposed Scenario and evaluation framework

In order to evaluate the performance and justify potential benefits a qualitative and quantitative analysis on what would be the costs and benefits would be needed.

| # | Parameter | Definition |
| --- | --- | --- |
| 1 | Scenario name | Film Grain Synthesis Scenario |
| 2 | Motivation for the scenario | Study, test and analyze effect of film grain synthesis applied to varied test scenarios encoded with H.265/HEVC, in order to ascertain whether there is benefit in (1) artistic intent and/or (2) masking artifacts / saving bitrate while maintaining visual acuity, etc. |
| 3 | Description of the scenario | Film grain can be unavoidably lost by conventional video compression workflow shown in Figure 1. The high frequency grain in the source content (x) is either completely or partially lost due to quantization in adaptive streaming or broadcast applications, the resulting decoded output x is void of the artistic effect of grain intented in source. Even if grain is partially preserved at high bitrates, overall coding efficiency is poor as grain is not spatially or temporally predictable and significant percentage of bits are used in coding high frequency components of the noisy grain. In case of Film Grain Preservation Workflow as per Figure 2, the source video (x) is input to a denoising process that outputs a video sequence from which noise or film grain is attenuated or removed. A film grain parameterization process then compares the source and denoised videos to determine film grain model parameter values, which relate to the variance, spatial frequency characteristics, colour correlation, and other statistical characteristics of the film grain. The process of denoising followed by the film grain model parameter estimation is commonly referred to as the film grain analysis process. After these processes are performed, the denoised video is then encoded and the film grain model parameter values are either signalled in the coded bitstream (ydn) or provided to the decoder by some external means. On the decoder side, the film grain model parameter values are parsed and input to a film grain synthesis process that generates simulated film grain and blends the grain with the decoded video (xdn) to output decoded video with simulated film grain (xg). |
| 4 | Supporting companies and 3GPP members |  |
| 5 | Source format properties - Spatial resolutions - Chroma Format - Chroma Subsampling - Aspect ratios - Frame rates - Colour space formats - Transfer Characteristics - Bit depth - Other signal properties | 1920x1080, 3840,2160 YUV 420P 16: 9 24, 29.97, 30 ,59.94 ,60 BT709, BT2020 PQ 8, 10 |
| 6 | Encoding and decoding constraints and settings: Typical encoding constraints and settings such as - Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. - Random access frequency - Error resiliency requirements - Bitrates and quality requirements - Bitrate parameters (CBR, VBR, CAE, HRD parameters) - ABR encoding requirements (switching frequency, etc.) - Latency requirements and specific encoding settings - Encoding context: real-time encoding, on device encoding, cloud-based encoding, offline encoding, etc. - Required decoding capabilities | HEVC/H.265 Main Profile, Level 5.3 Detailed config parameters embedded in cfg file below |
| 7 | Performance Metrics and Requirements - A clear definition on how the performance needs to be evaluated including metrics, etc addressing the main KPIs of the scenario. - Objective measures such as PSNR, VMAF, etc, may be used. - Subjective evaluation is not excluded and may be done, but needs commitment | Objective metrics - PNSR - VMAF - BRISQUE - NIQE - PIQE Subjective Metris - MOS (pair wise method with hidden reference) - Relative Grading (A v/s B method) Based on the tests conducted following metrics are suitable for assessment of FGS - BRISQUE - MOS (Subjective) - VMAF |
| 8 | Interoperability Considerations for the application - Streaming with DASH/HLS/CMAF - RTP based delivery | Following media formats have been tested and validated for FGS interoperability. - MP4 - MPEG-DASH - HLS |
| 9 | Test Sequences - A set of selected test sequences that are provided by the proponents in order to do the evaluation. They should cover a set of source format properties | Preserving Artistic Intent Scenario FHD - BQTerrace - OldTownCross - InToTree Scenario 4K-TV - Crowdrun - Meridian - DinnerScene |
| 10 | Detailed test conditions: - Provides a proposal for detailed test conditions, for example based on a reference software together with the sequences and configuration parameters. | Following denoiser, HEVC software codecs with film grain analysis and synthesis support are used to assess grain characterization: - Denoiser: ffmpeg ‘nlmeans’ filter is used to denoise source content as pre-processing step before encoding. Sample Commnad - ./ffmpeg -f rawvideo -pix_fmt yuv420p -s:v 1920x1080 -i ../InToTree_1080p50.yuv -vf nlmeans=3.5:5:3:9:7 InToTree_nlmeans_3.5_5_3_9_7.yuv; - Film grain analysis algorithm: Frequency filtering analysis algorithm in HM-18.0 reference encoder is used to model the grain parameters to be inserted in FGC SEI - Film grain synthesis algorithm: SMPTE RDD5 based Frequency filtering grain synhthesis algorithm in HM-18.0 reference decoder is used for blending grain with decoded video to output grain sytnhesized video xg - Browser player with FGS support: For Subjective evaluation, Ittiam HEVC FGS web browser player is used as it supports SMPTE RDD5 based frequency film grain synthesis using the FGC SEI messages embedded in HEVC bitstream. https://demo.ittiam.com/demo/i265_fgs/ |
| 11 | External Performance data |  |
| 12 | Additional Information |  |

An independent scenario for FGS may be defined.

In order to kick off evaluation, initial focus should be on

- 5 Source format properties

- 7 Performance Metrics and Requirements

- 9 Test Sequences

- 10 Detailed test conditions

Note that the evaluation does not necessarily have to be as detailed as in TR 26.955, relying on external performance data is good. However, it should be possible to understand if and how the information fits into the 3GPP relevant use cases. Repeatability should also be considered. If JVET completes such an evaluation, this would be preferable.

# 3	Considerations on UE requirements

In a possible specification for requirements, what could be done is the following:

- 3GPP Film grain synthesis reference process (possibly by reference to JVET)

- Process to verify "not having perceptually significant differences" to 3GPP Film grain synthesis reference process (possibly by reference to JVET)

- Receiver Requirements:

  - Either

    - 3GPP Film grain synthesis reference process implemented, or

    - not having perceptually significant differences to 3GPP Film grain synthesis reference process

  - making sure that available receiver implementations meet the receiver requirements

- In 3GPP, this could be a new specification or an Annex to an existing specification TS 26.116

# 4	Communication with JVET

It is agreed to send an LS to JVET informing them of our intent to evaluate the impact of FGS on our 3GPP scenarios listed in this study.

This LS is intended to request information on their ongoing activity with AG5 on similar evaluation in order to see if they can meet our timeline.

This clause is a placeholder for information we want to share with JVET and our requests.

- Inform JVET on the study item on Film Grain Synthesis (list the objectives or point to/attach the SID)

- Inform JVET on our timeline justified by the intent to meet release18 for normative work.

- Explain to JVET the type of evaluations that we will conduct in SA4

- Ask JVET if they have similar activities planned/ongoing/done and see if they can share results (or by when).

An LS is expected to be sent before the next JVET meeting (11th October 2023).

Update: The LS has been sent (S4-231587) to MPEG/JVET (m65645) and a response statement is generated from the October MPEG/JVET meeting, to be received and handled at the 3GPP SA4 #126 (13-17 Nov. in Chicago).

ANNEX: FGS Testing Methodology and Results

FGS workflow for preserving artistic intent.

Film grain can be unavoidably lost by conventional video compression workflow shown in Figure 1. The high frequency grain in the source content (x) is either completely or partially lost due to quantization in adaptive streaming or broadcast applications, the resulting decoded output x is void of the artistic effect of grain intented in source. Even if grain is partially preserved at high bitrates, overall coding efficiency is poor as grain is not spatially or temporally predictable and significant percentage of bits are used in coding high frequency components of the noisy grain.

Figure 1 HEVC workflow without film grain

HEVC Encoder

Video Stream

Film Grain Analysis Disabled

HEVC Decoder

x

y

x

Following notations are used for the film grain preservation workflow depicted in Figure 2:

- x is source video (uncompressed)

- xdn is denoised source video.

- ydn is compressed bitstream of denoised video.

- xdn is the decoded denoised video.

xg is the grain synthesized video, using the FGC SEI parameters.

Figure 2 HEVC film grain modeling and synthesis workflow

De-Noising

HEVC Encoder

x

-

Film Grain Analysis

VideoStream

Film Grain Analysis Enabled

Difference Noise

Film Grain SEI

HEVC Decoder

+

xdn

ydn

xdn

xg

Conventional codec

Film grain framework modules

Encoder side

Decoder side

Film Grain Synthesis

Quality Evaluation Methods for FGS

The benefits of grain characterization and synthesis to preserve artefact are assessed using populare reference based video quality metrics such as VMAF and PSNR, non-reference metrics such as BRIQSUE as well as subjective visual verifications test scores of  XX viewers (TODO: Added number of viewers after final tests.)

Objective metrics evaluation software:

The workflows of Figure 1 without  explicit grain characterization and Figure 2 with film grain characterization and synthesis are evaluated for

Full reference video quality metrics:

Full reference video quality metrics such as VMAF and PSNR are used to compare benefits of denoising as pre-processing step.

- Reference: PSNR and VMAF scores of decoded video x of vanilla workflow in Figure 1 are computed using source video (x) as reference.

- Test: PSNR and VMAF scores of denoised decoded video (xdn) of film grain workflow in  Figure 2 are computed w.r.t source video, (x)

- BD-Rate of boh Reference and Test are computed based on respective bitrates and VMAF/PSNR scores.

Non-refrence video quality metrics:

BRISQUE Score based comparison:

BRISQUE stands for Blind/Reference less Image Spatial Quality Evaluator. It is a non-reference metric to evaluate image quality in the spatial domain. It extracts features from the image being measured and runs them through a support vector regression (SVR) model pre-trained on existing image databases with corresponding differential mean opinion score (DMOS) values. The values of the BRISQUE metric for a certain image can range from 0 to 100, wherein a lower BRISQUE score indicates a higher quality and vice versa.

BRISQUE scores are sed to compare benefits of film grain as follows:

- Reference: BRISQUE scores of decoded video x of vanilla workflow in Figure 1 are computed as reference.

- Test: BRISQUE scores of denoised decoded video (xdn) of film grain workflow in  Figure 2 are computed.

- BD-Rate of boh Reference and Test are computed using respective bitrates and (100 – BRISQUE) scores.

Subjective Performance Evaluation Methods

- ITU-T_REC-P190 based subjective performance evaluation method (pair-wise testing with hidden reference). This  method is a category judgement where the test sequences are presented one at a time and are rated independently on a category scale (1 to 10). The present test procedure must include a reference version of each test sequence shown as any other test stimulus.

| Test Site | On-site |
| --- | --- |
| Display, size, connection (resolution setting) | Samsung 65” S95B, HDMI (3840×2160) |
| Viewing distance | 2 viewers sitting at 1.5H, 1 views standing at 1.6H |
| Viewing angle | ±75°, 90° (at screen center) |
| Total number of viewers | 14 (5 female, 9 male) |

Scenario FHD Test Results (Objective)

VMAF Comparision Results – 1080p (FHD) Denoised

Table below shows the VMAF BD-Rate gains by denoising the source using ffmpeg nlmeans filter as pre-processing step or film grain characterization:

|  | FGA Disabled (x) |  | Denoised nlmeans (xdn) |  | BDRate |
| --- | --- | --- | --- | --- | --- |
| 1080p | Bitrate (kbps) | VMAF | Bitrate (kbps) | VMAF |  |
| BQTerrace | 5122.25 | 97.42 | 4920.86 | 97.38 | -0.3% |
|  | 2403.07 | 95.83 | 2363.37 | 95.78 |  |
|  | 1250.40 | 90.95 | 1240.97 | 90.88 |  |
|  | 855.89 | 86.29 | 849.79 | 86.26 |  |
| IntoTree | 5262.74 | 93.26 | 5712.38 | 94.01 | -3.5% |
|  | 2283.50 | 87.71 | 2162.57 | 87.65 |  |
|  | 968.34 | 78.68 | 939.71 | 78.63 |  |
|  | 570.18 | 71.69 | 555.50 | 71.69 |  |
| OldTown | 2624.74 | 95.50 | 2785.76 | 95.75 | -1.0% |
|  | 1425.31 | 92.59 | 1391.85 | 92.53 |  |
|  | 793.19 | 86.15 | 783.15 | 86.09 |  |
|  | 553.65 | 79.93 | 547.54 | 79.83 |  |

BRISQUE Comparision Results – 1080p (FHD) Grain Synthesized

Table below shows the improvement in BRISQUE scores (lower score indicates better quality) of the rain syhtnesized content (xg)  over  FGA disabled content  (x):

|  | FGA Disabled (x) |  |  |  | FGA Enabled (xg) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test cases | Bitrate (kbps) | BRISQUE | 100-BRISQUE |  | Bitrate (kbps) | BRISQUE | 100-BRISQUE |
| 1080p - FHD |  |  |  |  | 1080p- FHD |  |  |
| BQTerrace | 23025.30 | 28.32 | 71.68 |  | 20413.93 | 22.53 | 77.47 |
| (w 1920; h 1080) | 5122.25 | 28.46 | 71.54 |  | 4920.86 | 23.65 | 76.35 |
|  | 2403.07 | 33.39 | 66.61 |  | 2363.37 | 23.80 | 76.20 |
|  | 1250.40 | 35.56 | 64.44 |  | 1240.97 | 24.24 | 75.76 |
| IntoTree | 855.89 | 36.12 | 63.88 |  | 849.79 | 25.33 | 74.67 |
| (w 1920; h 1080) | 16921.17 | 26.85 | 73.15 |  | 16464.98 | 29.03 | 70.97 |
|  | 5262.74 | 30.82 | 69.18 |  | 5712.38 | 31.78 | 68.22 |
|  | 2283.50 | 35.65 | 64.35 |  | 2162.57 | 25.30 | 74.70 |
| OldTown | 968.34 | 37.79 | 62.21 |  | 939.71 | 15.52 | 84.48 |
| (w 1920; h 1080) | 570.18 | 39.36 | 60.64 |  | 555.50 | 18.91 | 81.09 |
|  | 7926.75 | 24.53 | 75.47 |  | 8529.97 | 13.29 | 86.71 |
|  | 2624.74 | 28.67 | 71.33 |  | 2785.76 | 13.84 | 86.16 |

Scenario FHD Test Results (Subjective)

Scenario 4K-TV Test Results (Objective)

VMAF Comparision Results – 2160p (4K) Denoised

Table below shows the VMAF BD-Rate gains by denoising the source using ffmpeg nlmeans filter as pre-processing step or film grain characterization:

|  | FGA Disabled (x) |  | Denoised nlmeans (xdn) |  | BDRate |
| --- | --- | --- | --- | --- | --- |
| 2160p | Bitrate (kbps) | VMAF | Bitrate (kbps) | VMAF |  |
| CrowdRun | 38169.366 | 96.239195 | 36580.109 | 96.110827 | -1.1% |
|  | 19303.381 | 89.335938 | 18844.347 | 89.153498 |  |
|  | 9905.625 | 79.235657 | 9763.171 | 79.147126 |  |
|  | 6654.954 | 72.566822 | 6572.866 | 72.505673 |  |
| DinnerScene1 | 39638.991 | 91.27438 | 42483.148 | 91.788863 | -22.6% |
|  | 21557.398 | 90.415263 | 21190.75 | 90.801083 |  |
|  | 9692.059 | 89.394119 | 8956.27 | 89.598488 |  |
|  | 4948.825 | 88.805454 | 3822.583 | 88.608568 |  |
| Meridian | 41183.95 | 95.707443 | 43093.712 | 95.483009 | 9.6% |
|  | 18528.067 | 93.497765 | 17889.124 | 93.032104 |  |
|  | 9664.451 | 90.912864 | 10281.852 | 90.82131 |  |
|  | 5292.271 | 87.607008 | 6078.219 | 88.198284 |  |

BRISQUE Comparision Results – 2160P (4K) Grain Synthesized

Table below shows the improvement in BRISQUE scores (lower score indicates better quality) of the rain syhtnesized content (xg)  over  FGA disabled content  (x):

|  | FGA Disabled (x) |  |  |  | FGA Enabled (xg) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test cases | Bitrate (kbps) | BRISQUE | 100-BRISQUE |  | Bitrate (kbps) | BRISQUE | 100-BRISQUE |
| CrowdRun | 38169.366 | 40.086 | 59.91 |  | 36580.109 | 11.759 | 88.24 |
| (w 3840; h 2160) | 19303.381 | 39.27 | 60.73 |  | 18844.347 | 12.638 | 87.36 |
|  | 9905.625 | 38.963 | 61.04 |  | 9763.171 | 16.916 | 83.08 |
|  | 6654.954 | 40.937 | 59.06 |  | 6572.866 | 20.448 | 79.55 |
| DinnerScene1 | 39638.991 | 43.941 | 56.06 |  | 42483.148 | 37.97 | 62.03 |
| (w 3840; h 2160) | 21557.398 | 43.772 | 56.23 |  | 21190.75 | 39.682 | 60.32 |
|  | 9692.059 | 44.517 | 55.48 |  | 8956.27 | 41.328 | 58.67 |
|  | 4948.825 | 45.47 | 54.53 |  | 3822.583 | 43.054 | 56.95 |
| Meridian | 41183.95 | 30.751 | 69.25 |  | 43093.712 | 25.41 | 74.59 |
| (w 3840; h 2160) | 18528.067 | 35.288 | 64.71 |  | 17889.124 | 24.64 | 75.36 |
|  | 9664.451 | 40.16 | 59.84 |  | 10281.852 | 24.576 | 75.42 |
|  | 5292.271 | 42.725 | 57.28 |  | 6078.219 | 25.205 | 74.80 |

Scenario 4K-TV Test Results (Subjective)

FGS workflow for masking coding artifacts

The classical use case of film grain technology is to be used in an encoder to remove the original film grain from the source video to reduce bitrate for distribution. The film grain model parameters of the original film grain are coded as parameters in the bitstream. The receiver can use the film grain parameters to re-synthesize the film grain to help preserve the original look of the video. Film grain technology can also be used with video sources that do not have original film grain. In this case, it can be used as a compression aid to improve the subjective quality by subtly masking artifacts such as blockiness, banding and blurred textures at low bitrates.

Figure 3 HEVC workflow for film grain estimation for masking coding artifacts

HEVC Encoder

Film Grain Estimation

VideoStream

Film Grain Estimation Workflow

Film Grain SEI

HEVC Decoder

+

xd

y

x

xg

Conventional codec

Film grain framework modules

Encoder side

Decoder side

Film Grain Synthesis

y

FGC SEI Inserter

yfg

Following notations are used for the film grain estimation for masking coding artifacts workflow depicted in 3:

- x is source video (uncompressed)

- yfg is compressed bitstream with FGC SEI message inserted.

- xd is the decoded video.

xg is the grain synthesized video, using the FGC SEI parameters.

BRISQUE Comparision Results

Subjective Test Results A|B Method with DMOS

Note: more subjective test results are expected – utilizing the JND (Just Noticeable Difference/Distortion) methodology.