3GPP TS 26.247 V1.2.0 (2010-11)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects

Transparent end-to-end Packet-switched

Streaming Service (PSS);

Progressive Download and 
Dynamic Adaptive Streaming over HTTP (3GP-DASH)

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP, 3GP-DASH

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2010, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	Overview	8

5	System Description	8

5.1	Overview	8

5.2	Service Access	9

6	Progressive Download over HTTP	10

6.1	General	10

6.2	Progressive Download	10

6.3	Profiles	10

7	Dynamic Adaptive Streaming over HTTP Framework	11

7.1	Introduction	11

7.2	Media Presentation	11

7.2.1	Introduction	11

7.2.2	Period	12

7.2.3	Representation	12

7.2.4	Segments	13

7.2.4.1	Definition	13

7.2.4.2	Segment URLs and Media Segment Start Times	13

7.2.4.2.1	Overview	13

7.2.4.2.2	Template-based Media Segment URLs	14

7.3	Media Presentation Description	15

7.3.1	Introduction	15

7.3.2	Media Presentation Description Attributes and Elements	15

7.3.3	Media Presentation Description Schema	21

7.3.4	Media Presentation Description Updates	24

7.4	Client Behaviour	24

7.4.1	Introduction	24

7.4.2	Overview	25

7.4.3	Segment List Generation	25

7.4.3.1 	General	25

7.4.3.2 	Template-based Generation of Media Segment List	26

7.4.3.3 	Playlist-based Generation of Media Segment List	27

7.4.3.4. 	Media Segment List Restrictions	28

7.4.4	Seeking	28

7.4.5	Support for Trick Modes	29

7.4.6	Switching Representations	29

7.4.7	Reaction to Error Codes	29

8	3GPP Adaptive HTTP Streaming	30

8.1	Instantiation of 3GPP Adaptive HTTP Streaming	30

8.2	Usage of the 3GP File Format	30

8.2.1	Introduction	30

8.2.2	Segment Types and Formats	30

8.2.2.1	Segment Types	30

8.2.2.2	Initialisation Segment Format	30

8.2.2.3	Media Segment Format	31

8.2.2.4	Self-Initialising Media Segment Format	31

8.2.3	Usage on Server and Client	32

8.2.4	Media Presentation Authoring Rules for specific MPD flags	32

8.2.4.1	General	32

8.2.4.2	Segment Alignment	32

8.2.4.3	Bitstream Switching	32

8.3	Media Codecs	33

8.4	Security	33

8.4.1	Content Protection	33

8.4.2	Transport Security	33

Annex A (informative): Guidelines for Adaptive HTTP Streaming	34

A.1	Content-Preparation Modes	34

A.1.1	Introduction	34

A.1.2	Static Mode	34

A.1.3	Dynamic Mode	34

A.2	Mapping MPD structure and semantics to SMIL	35

A.2.1	General	35

A.2.2	Examples	37

A.2.2.1	Example 1: MPD for on-demand content with multiple Periods and alternate Representations	37

A.2.2.2	Example 2: MPD for live content	38

Annex B (informative): Change history	39

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

The 3GPP transparent end-to-end packet-switched streaming service (PSS) specification consists of seven 3GPP TSs: 3GPP TS 22.233 [1], 3GPP TS 26.233 [2], 3GPP TS 26.234 [3], 3GPP TS 26.244 [4], 3GPP TS 26.245 [5], 3GPP TS 26.246 [6], and the present document.

The TS 22.233 contains the service requirements for the PSS. The TS 26.233 provides an overview of the PSS. The TS 26.234 provides the details of the protocols and codecs used by the PSS. The TS 26.244 defines the 3GPP file format (3GP) used by the PSS and MMS services. The TS 26.245 defines the Timed text format used by the PSS and MMS services. The TS 26.246 defines the 3GPP SMIL language profile. The present document defines Progressive Download and Dynamic Adaptive Streaming over HTTP.

The TS 26.244, TS 26.245 and TS 26.246 start with Release 6. Earlier releases of the 3GPP file format, the Timed text format and the 3GPP SMIL language profile can be found in TS 26.234.

The TS 26.cde started with Release 10. Earlier releases of Progressive Download and Dynamic Adaptive Streaming over HTTP can be found in TS 26.234.

# Introduction

Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH) collects a set of technologies how progressive download and adaptive streaming of continuous may be exclusively carried out over HTTP.

# 1	Scope

The present document specifies Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH). This specification is part of Packet-switched Streaming Service (PSS). HTTP-based progressive download and adaptive streaming are separated from TS 26.234 to differentiate from RTP-based streaming that is maintained TS 26.234. HTTP-based progressive download and adaptive streaming may be deployed independently from RTP-based PSS, for example by using standard HTTP/1.1 servers for hosting the services.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 22.233: "Transparent End-to-End Packet-switched Streaming Service; Stage 1".

[2]	3GPP TS 26.233: "End-to-end transparent streaming service; General description".

[3]	3GPP TS 26.234: "Transparent end-to-end packet switched streaming service (PSS); Protocols and codecs".

[4]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[5]	3GPP TS 26.245: "Transparent end-to-end packet switched streaming service (PSS); Timed text format".

[6]	3GPP TS 26.246: "Transparent end-to-end packet switched streaming service (PSS); 3GPP SMIL Language Profile".

[7]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[8]	IETF STD 0007: "Transmission Control Protocol", Postel J., September 1981.

[9]	IETF RFC 2616: "Hypertext Transfer Protocol – HTTP/1.1", Fielding R. et al., June 1999.

[10]	Open Mobile Alliance, Service and Content Protection for Mobile Broadcast Services, Approved Version 1.0, February 2009.

[11]	ISO/IEC 14496-12:2005 | 15444-12:2005: "Information technology – Coding of audio-visual objects – Part 12: ISO base media file format" | "Information technology – JPEG 2000 image coding system – Part 12: ISO base media file format".

[12]	IETF RFC 2818: "HTTP Over TLS", E. Rescorla, May 2000.

[13]	IETF RFC 5646: "Tags for Identifying Languages", A. Phillips, M. Davis, September 2009.

[14]	IETF RFC 4281: "The Codecs Parameter for "Bucket" Media Types", R. Gellens, D. Singer, P. Frojdh, November 2005.

[15]	Open Mobile Alliance: "DRM Content Format V 2.0".

[16]	IETF RFC 3986: "Uniform Resource Identifiers (URI): Generic Syntax", Berners-Lee T., Fielding R. and Masinter L., January 2005.

[17]	IETF RFC 1952: GZIP file format specification version 4.3", P. Deutsch, May 1996.

[18]	IETF RFC 2818:" HTTP Over TLS", E. Rescorla, May 2000.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [7] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [7].

continuous media: media with an inherent notion of time. In the present document speech, audio, video, and timed text.

http-URL: a URI with a fixed scheme of “http://” or https://.

media component: A media component is an encoded version of one individual media type such as audio, video or timed text.

media content: A set of media components, (e.g. audio, video, timed text) that have a common timeline as well as relationships one how they may be presented for example individually, jointly, or mutually exclusive. An example for a media content is a program or a movie.

media presentation: a structured collection of data that is accessible to the HTTP-Streaming Client.

media presentation description (MPD): contains information required by the HTTP-Streaming Client to construct appropriate URLs to access Segments and to provide the streaming service to the user.

Period: A period is a timely subset of the media presentation. The sequence of periods constitutes the media presentation. Periods are consecutive and non-overlapping.

random access point (RAP): A point (indexed by a byte offset) in a media segment that the media format has identified as being a position for which it is possible to start playback using only the information contained in the media segment from that position onwards, preceded by initializing with the initialisation segment, if any.

Representation: A structured collection of data which contains one or more media components with specific attributes, e.g. bandwidth, language, resolution etc.

Segment: a unit of media that can be referenced by an http-URL, possibly restricted by a byte range, included in the MPD.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [x] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [7].

3GP	3GPP file format

AVC	Advanced Video Coding

DRM	Digital Rights Management

HSD	HTTP Streaming and Download

HTTP	Hypertext Transfer Protocol

HTTPS		Hypertext Transfer Protocol Secure

MPD	Media Presentation Description

MPEG-2 TS	Moving Picture Experts Group Transport Stream

MIME	Multipurpose Internet Mail Extensions

OMA	Open Mobile Alliance

PDCF	Packetized DRM Content Format

PSS	Packet-switched Streaming Service

RAP	Random Access Point

RFC	Request For Comments

RTP	Real-time Transport Protocol

SMIL	Synchronised Multimedia Integration Language

URI	Uniform Resource Identifier

URL	Uniform Resource Locator

UTC	Universal Time Coordinated

UTF-8	Unicode Transformation Format (the 8-bit form)

W3C	WWW Consortium

XML	eXtensible Markup Language

XSLT	eXtensible Stylesheet Language Transformation

# 4	Overview

The present document specifies Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH) for continuous media. The features are separated from the umbrella specification TS 26.234 [3] to differentiate from RTP-based streaming that is specified and maintained in TS 26.234. Services relying exclusively on these features may be deployed independently from RTP-based PSS servers, for example by using standard HTTP/1.1 servers for hosting the services.

The specification covers the following aspects:

- System Description: describes the relationship to the PSS architecture and refines the architecture and interfaces that are defined in this specification.

- Progressive Download over HTTP.

- Framework for Adaptive Streaming over HTTP: describes the data model of a Media Presentation. It also provides an overview on elements and attributes that may be used to describe components and properties of a media presentation in a Media Presentation Description (MPD).

- 3GPP Adaptive HTTP Streaming: Specifies the Media Presentation framework for 3GPP Adaptive HTTP Streaming including the Media Presentation Description format, the file and segment formats, codecs and other mechanisms.

- Annex A provides implementation guidelines for Adaptive Streaming Services.

# 5	System Description

## 5.1	Overview

Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH) enables to provide services to deliver continuous media content over HTTP in a sense that all resources that compose the service are accessible through http-URLs and the HTTP/1.1 protocol as specified in RFC 2616 [9] may be used to deliver the metadata and media data composing the service. This enables that standard HTTP servers and standard HTTP caches can be used for hosting and distributing continuous media content. Figure 5.1 shows the architecture for services using progressive download and Figure 5.2 shows the architecture for services using 3GP-DASH.

This specification only deals with the specification of interfaces between the Client and the Server. Specifically, it defines the formats that may be delivered exclusively over the HTTP interface to enable progressive download and streaming services.

Figure 5.1: Architecture for Progressive Download over HTTP

Services using the features defined described in this specification may be deployed within PSS as specified in TS 26.234. In this case the Download/Adaptive Streaming Server may be a sub-function of the PSS server and the Download/Adaptive Streaming Client may be a sub-function of the PSS client.

Figure 5.2: Architecture for 3GP-DASH

Services using the features defined in this specification may also be deployed independent of the PSS servers and clients. In this case the Progressive Download and Adaptive Streaming Client shall support the formats and codecs as according to this specification.

Access to services based on the features defined in this document is introduced in clause 5.2.

The protocol support for services using the features defined in this specification is provided in clause 5.3.

Clients supporting progressive download-based services shall support the features and formats as specified in clause 6 of this specification.

Clients supporting 3GP-DASH shall support the features and formats as specified in clause 8 of this specification.

## 5.2	Service Access

Service access refers to the method by which a Client initially accesses the service. Service access for services based in the specification can be achieved e.g. by a Media Presentation Description or just an URL to the media file.

The service access URL can be made available to a client in many different ways. Clients supporting services based on the features in this specification shall be able to access services that are provided through an http-URL. However, it is out of the scope of this specification to mandate any specific mechanism. A preferred way may be to embed URLs for service establishment within HTML pages.

Figure 5.3: Overview of the protocols stack

# 6	Progressive Download over HTTP

## 6.1	General

As an alternative to conventional streaming, a Client may download, typically through HTTP, a media file that encapsulates continuous media and may play the media from the local storage. A PSS client shall support progressive download and playout of 3GP files [4] as specified in the remainder of this clause.

The media file encapsulating the continuous media is accessed directly by issuing one or more HTTP GET or partial GET requests to the referenced media file. An example of a valid URL is http://example.com/morning_news.3gp.

Editor’s Note: the URL does not work.

## 6.2	Progressive Download

Progressive download uses normal HTTP download using HTTP GET or partial GET requests. The differences between regular download and Progressive Download are that 1) the content may be authored as Progressively downloadable, and 2) the terminal recognises that the content is suitable for progressive download.

A client downloading continuous media may decide to start playout of the encapsulated media data before the download of the media file is completed.

## 6.3	Profiles

The following profiles of the 3GP file format in TS 26.244 [4] shall be supported by clients supporting Progressive Download over HTTP:

-	Basic profile, and

-	Progressive-download profile.

# 7	Dynamic Adaptive Streaming over HTTP Framework

## 7.1	Introduction

The 3GPP Dynamic Adaptive Streaming over HTTP (3GP-DASH) Framework specified in this clause provides streaming services over HTTP. This enables delivering content from standard HTTP servers to an HTTP-Streaming client and enables caching content by standard HTTP caches.

Figure 7.1 shows the architecture for 3GP-DASH. This document only deals with the specification of interface 1 between the HTTP-Streaming Client and the HTTP-Streaming Server. All other interfaces are out-of-scope of this specification.

It is assumed that the HTTP-Streaming Client has access to a Media Presentation Description (MPD). An MPD provides sufficient information for the HTTP-Streaming Client to provide a streaming service to the user by sequentially downloading media data from an HTTP server and rendering the included media appropriately.

Figure 7.1: System Architecture for 3GP-DASH

To initiate the streaming service to the user, the HTTP Streaming Client establishes a Media Presentation by downloading the relevant metadata and subsequently the media data. The Media Presentation is defined in clause 7.2. The Media Presentation Description is provided in clause 7.3. Guidelines on the Client Behaviour are presented in clause 7.5.

## 7.2	Media Presentation

### 7.2.1	Introduction

A Media Presentation is a structured collection of data that is accessible to the HTTP-Streaming Client. The HTTP-Streaming client requests and downloads media data information to present the streaming service to the user.

A Media Presentation is described in a Media Presentation Description (MPD) including any possible updates of the MPD. The MPD, including all attributes and elements, is specified in clause 7.3.2.

A Media Presentation consists of:

- A sequence of one or more Periods.

- Each Period contains one or more Representations from the same media content.

- Each Representation consists of one or more Segments.

- Segments contain media data and/or metadata to decode and present the included media content.

The Media Presentation has a timeline that is defined by the concatenation of the timeline of each Period.

NOTE: The playout procedure of the media may need to be adjusted at the end of the preceding Period to match the start time of the new Period as documented as there may be small overlaps or gaps with a Representation at the end of the preceding Period.

The timeline in each Period is common to all Representations.

The Media Presentation may be of type OnDemand or Live. The MPD attribute type provides the type of the Media Presentation.

If present, the MPD attribute availabilityStartTime gives the earliest time at which the Media Presentation is available at the server.

If present, the MPD attribute availabilityEndTime gives the time after which the Media Presentation will no longer be available at the server.

If present, the MPD attribute mediaPresentationDuration describes the duration of the entire Media Presentation. If not present, the duration of the Media Presentation is unknown.

### 7.2.2	Period

A Media Presentation consists of one or more Periods. Periods are defined by Period elements in the MPD.

Each Period has an attribute start. Each Period may be assigned an identifier that is unique within a Media Presentation. This identifier is provided by the attribute id on Period level.

For live services, the sum of the start attribute of the Period and the MPD attribute availabilityStartTime specifies the availability time of the Period in UTC format, in particular the first Media Segment of each Representation in this Period.

For on-demand services the start attribute of the first Period shall be 0. For any other Period the start attribute specifies the timeoffset between the start time of the Period relative to the start time of the first Period.

Each Period extends until the start of the next Period, or until the end of the Media Presentation in the case of the last Period.

Period start times are precise. They reflect the actual timing resulting from playing the media of all prior Periods.

### 7.2.3	Representation

Each Period consists of one or more Representations. Period elements shall be physically ordered in the MPD in increasing order of the value of their start attribute.

A Representation is one of the alternative choices of the media content or a subset thereof typically differing by the encoding choice, e.g. by bitrate, resolution, language, codec, etc.

Each Representation is assigned an identifier that is unique within a Period. This identifier is provided by the attribute id on Representation level.

A Representation starts at the start of the Period and continues to the end of the Period.

A Representation consists of one or more Segments.

Each Representation either contains an Initialisation Segment or each Media Segment in the Representation is self-initialising.

Each Representation includes one or more media components, where each media component is an encoded version of one individual media type such as audio, video or timed text. Media components are time-continuous across boundaries of consecutive Media Segments within one Representation.

Representations are assigned to a group indicated by the group attribute. Representations are assigned to the same group to signal that they are alternatives to each other. The media content within one Period is represented by:

- either one Representation from group 0, if present,

- or the combination of at most one Representation from each non-zero group.

The timing within each Representation is relative to the start time of the Period that contains this Representation.

For future releases or proprietary extensions, if different semantic or new non-compatible features are defined, such that HTTP Streaming clients can not properly render a Representation ignoring those extensions, then a new Representation element shall be defined with a new associated namespace. E.g. a new element <Representation2010> and namespace: "urn:3GPP:ns:PSS:AdaptiveHTTPStreamingMPD:2010". In this example, an MPD may mix both Representation and Representation2010 elements.

### 7.2.4	Segments

#### 7.2.4.1	Definition

A Segment is defined as a unit that can be uniquely referenced by an http-URL included in the MPD, where an http-URL is defined as an <absolute-URI> according to RFC 3986 [16], clause 4.3, with a fixed scheme of “http://” or https://, possibly restricted by a byte range if the range attribute is provided. The byte range is expressed as a Ranges-specifier as defined in RFC 2616 [9], section 14.35.1, restricted to a single expression identifying a contiguous range of bytes.

The Initialisation Segment contains initialisation information for accessing the Representation. The Initialisation Segment shall not contain any media data. If the MPD contains information that is also documented in the Initialisation Segment, then the information in the MPD shall be such that no contradication between these two values occur.

A Media Segment contains media components that are either described within this Media Segment or described by the Initialisation Segment of this Representation. In addition, a Media Segment

- is assigned a Url element.

- is explicitly or implicitly assigned a start time relative to the start of the Representation provided by the MPD. The client can therefore download the appropriate Segments in regular play-out mode or after seeking. The start time shall be drift-free between the time indicated in the MPD and internal clock of the Media Segments, i.e. the accuracy of the start time documented in the MPD relative to the internal clock does not depend on the position of the Segment in the Representation.

- provides random access information, namely whether this Representation can be randomly accessed within this Segment and if yes, how to randomly access the Media Presentation within this Segment, e.g. exact timing, byte position. There is no requirement that a Media Segment starts with a random access point (RAP), but it is possible to signal in the MPD that all Segments within a Representation start with a RAP. The first Media Segment of a Representation shall always start with a RAP.

- when considered in conjunction with the information and structure of the MPD, contains sufficient information to time-accurately present each contained media component in the Representation without accessing any previous Media Segment in this Representation provided that the Media Segment contains a RAP. The time-accuracy enables seamlessly switching Representations and jointly presenting multiple Representations.

- may contain information for randomly accessing subsets of the Segment by using partial HTTP GET requests.

- if it is the first one in a Representation, is assigned presentation time 0 in the Media Presentation time line relative to the Period start time. If the first sample of any of the media components has a Media Presentation time relative to the Period start time greater than 0, then this presentation time shall be signalled in the Media Segment. No sample of any of the media components shall have Media Presentation time relative to the Period start time smaller than 0.

#### 7.2.4.2	Segment URLs and Media Segment Start Times

##### 7.2.4.2.1	Introduction

Each Representation shall contain at most one SegmentInfo Element that together with a possibly present SegmentInfoDefault Element on Period level permits to generate the Segment access information of each Segment within a Representation.

Specifically, the combination of the SegmentInfo Element and the SegmentInfoDefault Element contains sufficient information to generate a list of Media Segment URLs (possibly restricted by byte ranges) and Media Segment start times relative to the start of the Representation, for example by using the procedures described in Section 7.4.3.

The following rules apply for an MPD:

- URLs within the MPD may be relative or absolute as defined in RFC 3986 [16]. Relative URLs at each level of the MPD are resolved with respect to the baseURL attribute specified at that level of the document or the document “base URI” as defined in RFC3986 Section 5.1 in the case of absence of the the baseURL attribute at the MPD level.. If only relative URLs are specified and the document base URI cannot be established according to RFC3986 then the MPD is not valid.

- SegmentInfo elements may contain at most one InitialisationSegmentURL element. If no InitialisationSegmentURL element is present in a SegmentInfo element, then each Media Segment within the Representation shall be self-initialising.

- The elements SegmentInfoDefault and SegmentInfo as well as some other timing attributes in the MPD define the URLs (possibly restricted by byte ranges) and approximate time spans within a Period of the available Media Segments. An example segment list generation process as specified in section 7.4.3 generates a valid list of Media Segments.

- Each SegmentInfo element shall contain either at most one UrlTemplate element or one or more Url elements. If neither UrlTemplate nor Url element is present, then a UrlTemplate element is implied.

- If a UrlTemplate element is present or implied, each UrlTemplate element shall contain at most one sourceURL attribute.

  - If present, the sourceURL attribute specifies the URL construction for Media Segments in this Representation. In this case, the duration attribute shall be present either in the SegmentInfo element or in the SegmentInfoDefault element.

  - If the sourceURL attribute is not present, the id attribute on Representation level is used to generate the URLs for the Media Segments. In this case, the SegmentInfoDefault element on Period level shall contain a sourceUrlTemplatePeriod attribute with further restrictions as specified in section 7.2.4.2.2. The duration attribute shall be present either in the SegmentInfo element or in the SegmentInfoDefault element.

- Url elements provide a set of explicit URL(s), each of which may contain a range attribute, for Media Segments. If more than one Url element is provided in the SegmentInfo element, the duration attribute shall be present either in the SegmentInfo element or in the SegmentInfoDefault element, where as in case for a single Url element, the duration attribute may not be present.

- If the duration of the last Media Segment of any Representation in a Period is significantly shorter than the value of the duration attribute for this Representation (i.e it can not be justified by segmentation inaccuracy), or if the duration attribute for this Representation is not present, then MPD shall either include at least the start attribute of the next Period or, in case this is the last Period in the Media Presentation, include the mediaPresentationDuration attribute.

##### 7.2.4.2.2	Template-based Media Segment URLs

The SegmentInfo Element may contain a UrlTemplate element and the SegmentInfoDefault element may contain a sourceUrlTemplatePeriod attribute. The sourceURL attribute of the UrlTemplate element or the sourceUrlTemplatePeriod attribute represents a string that contains one or more of the identifiers as listed in Table 7.1. The sourceUrlTemplatePeriod attribute, when present, shall contain both the $RepresentationID$ identifier and the $Index$ identifier. The sourceURL attribute, when present, shall contain the $Index$ identifier and shall not contain the $RepresentationID$ identifier.

A sub-string "$<Identifier>$" shall name a substitution placeholder matching a mapping key of "<Identifier>". In the request URL, the substitution placeholder shall be replaced by the substitution parameter as defined in Table 7.1. Substitution is performed left to right and identifier matching is case-sensitive. If unrecognized identifiers are part of the UrlTemplate the URL formation to fail and the client shall ignore the Representation element and processing of the MPD shall continue as if this Representation element was not present.

Table 7.1: Identifiers for URL Templates

| $<Identifier>$ | Substitution parameter |
| --- | --- |
| $$ | Is an escape sequence, i.e. "$$" is replaced with a single "$" |
| $RepresentationID$ | This identifier is substituted by the attribute id of the requested Representation in the MPD. |
| $Index$ | This identifier is substituted by the Index, where the Media Segments within a Representation have assigned consecutive Segment indices from startIndex to endIndex. For an example client using this identifier to construct the list Media Segment URLs, refer to section 7.4.3. |

## 7.3	Media Presentation Description

### 7.3.1	Introduction

The Media Presentation Description (MPD) contains metadata required by the client to construct appropriate URLs to access Segments and to provide the streaming service to the user.

The Media Presentation may be available in different Representations (different bitrates, languages, resolutions, etc.). The service may be on-demand or live. The MPD contains information that enables the client to build the URLs to access any available Segment (or parts thereof) of the Media Presentation.

The MPD is an XML-document that is formatted according to the XML schema provided in clause 7.3.3. The MIME type of the MPD shall be “video/vnd.3gpp.mpd”. The delivery of the MPD is not in scope of this specification.

If the MPD is delivered over HTTP, then the MPD may be content encoded for transport, as described in [17] using the generic GZip algorithm RFC 1952 [17]. HTTP Streaming clients shall support GZip content decoding of the MPD when delivered over HTTP (GZIP RFC 1952 [17], clause 9).

A 3GP-DASH client shall implement correct XML behaviour insofar as it shall ignore any XML attributes or elemtns that it does not recognise. An MPD shall be authored in such a way that if a client were to encounter an XML attribute or element that it does not understand, the MPD should still be usable.

### 7.3.2	Media Presentation Description Attributes and Elements

The MPD contains attributes and elements as presented in Table 7.2.

Table 7.2: Semantics of Media Presentation Description

| Element or Attribute Name |  |  |  |  |  |  |  |  |  |  | Use |  |  |  |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPD |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | The root element that carries the Media Presentation Description for a Media Presentation. |
|  | type |  |  |  |  |  |  |  |  |  | OD default: OnDemand |  |  |  |  |  |  |  | “OnDemand” or “Live”. Indicates the type of the Media Presentation. Currently, on-demand and live types are defined. If not present, the type of the presentation shall be inferred as OnDemand. |
|  | availabilityStartTime |  |  |  |  |  |  |  |  |  | CM Must be present for type=”Live” |  |  |  |  |  |  |  | Gives the availability time (in UTC format) of the start of the first Period of the Media Presentation. |
|  | availabilityEndTime |  |  |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Gives the availability end time (in UTC format). After this time, the Media Presentation described in this MPD is no longer guaranteed to be available. When not present, the value is unknown. |
|  | mediaPresentationDuration |  |  |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Specifies the duration of the entire Media Presentation. If the attribute is not present, the duration of the Media Presentation is unknown. |
|  | minimumUpdatePeriodMPD |  |  |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Provides the minimum period the MPD is updated on the server. If not present the minimum update period is assumed to be infinite. |
|  | minBufferTime |  |  |  |  |  |  |  |  |  | M |  |  |  |  |  |  |  | Provides the minimum amount of initially buffered media that is needed to ensure smooth playout provided that each Representation is delivered at or above the value of its bandwidth attribute. |
|  | timeShiftBufferDepth |  |  |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Indicates the minimum duration of the time shifting buffer that is available for a live presentation. When not present, the value is unknown. |
|  | baseURL |  |  |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Base URL on MPD level |
|  | ProgramInformation |  |  |  |  |  |  |  |  |  | 0, 1 |  |  |  |  |  |  |  | Provides descriptive information about the program |
|  |  | moreInformationURL |  |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | If specified, this attribute contains an absolute URL which provides more information about the Media Presentation |
|  |  | Title |  |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | May be used to provide a title for the Media Presentation |
|  |  | Source |  |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | May be used to provide information about the original source (for example content provider) of the Media Presentation. |
|  |  | Copyright |  |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | May be used to provide a copyright statement for the Media Presentation. |
|  | Period |  |  |  |  |  |  |  |  |  | 1…N |  |  |  |  |  |  |  | Provides the information of a Period |
|  |  | Start |  |  |  |  |  |  |  |  | M |  |  |  |  |  |  |  | Provides the accurate start time of the Period relative to the value of the attribute availabilityStart time of the Media Presentation. |
|  |  | id |  |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Provides a unique identifier for this Period within the Media Presentation. The id shall remain the same across MPD updates. |
|  |  | segmentAlignmentFlag |  |  |  |  |  |  |  |  | OD Default: false |  |  |  |  |  |  |  | When set to ‘true’, indicates that all presentation start and end times of media components of any particular media type are temporally aligned in all Segments across all Representations with the same value of the duration attribute on Representation level in this Period. The presentation end time of a media component is defined as the sum of the presentation time of the last sample and the duration of the last sample. |
|  |  | bitstreamSwitchingFlag |  |  |  |  |  |  |  |  | OD Default: false |  |  |  |  |  |  |  | The concatenation of an Initialization Segment, if present, with all consecutive Media Segments of a single Representation within a Period, starting with the first Media Segment, results in a syntactically valid bitstream (according to the specific bitstream format) that is also semantically correct (i.e. if the concatenation is played, the media content within this Period is correctly presented). When this flag is set to ‘true’, such consecutive Segments following the same constraints may come from any Representation within the same group within this Period. This flag shall not be set to ‘true’ when segmentAlignmentFlag is set to ‘false’. See clause 8.2.4 for the implications when this flag is set to ‘true’ for the instantiation of 3GPP Adaptive HTTP Streaming. |
|  |  | SegmentInfoDefault |  |  |  |  |  |  |  |  | 0,1 |  |  |  |  |  |  |  | Provides default Segment information about Segment durations and, optionally, URL construction. |
|  |  |  | duration |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Default duration of Media Segments |
|  |  |  | startIndex |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Default start index |
|  |  |  | baseURL |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Base URL on Period level |
|  |  |  | sourceUrlTemplatePeriod |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | The source string providing the URL template on period level. |
|  |  | Representation |  |  |  |  |  |  |  |  | 1 … N |  |  |  |  |  |  |  | This element contains a description of a Representation. |
|  |  |  | bandwidth |  |  |  |  |  |  |  | M |  |  |  |  |  |  |  | The minimum bandwidth of a hypothetical constant bitrate channel in bits per second (bps) over which the representation can be delivered such that a client, after buffering for exactly minBufferTime can be assured of having enough data for continuous playout. |
|  |  |  | id |  |  |  |  |  |  |  | M |  |  |  |  |  |  |  | Provides a unique identifier for this Representation within the Period. |
|  |  |  | width |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Specifies the horizontal resolution of the video media type in an alternative Representation, counted in pixels. |
|  |  |  | height |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Specifies the vertical resolution of the video media type in an alternative Representation, counted in pixels. |
|  |  |  | lang |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Declares the language code(s) for this Representation according to RFC 5646 [13]. Note, multiple language codes may be declared when e.g. the audio and the sub-title are of different languages. For a full mapping of languages to media types, the initialization segment or a media segment may have to be accessed. |
|  |  |  | mimeType |  |  |  |  |  |  |  | M |  |  |  |  |  |  |  | Gives the MIME type of the Initialisation Segment, if present; if the Initialisation Segment is not present it provides the MIME type of the first Media Segment. Where applicable, this MIME type shall include the codec parameters for all media types. The codec parameters shall also include the profile and level information where applicable. For 3GP files, the MIME type is provided according to RFC 4281 [14]. |
|  |  |  | group |  |  |  |  |  |  |  | OD Default: 0 |  |  |  |  |  |  |  | Specifies the group to which this Representation is assigned. |
|  |  |  | startWithRAP |  |  |  |  |  |  |  | OD Default: False |  |  |  |  |  |  |  | When ‘true’, indicates that all Segments in the Representation start with a random access point |
|  |  |  | qualityRanking |  |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Provides a quality ranking of the Representation relative to other Representations in the Period. Lower values represent higher quality content. If not present then the ranking is undefined. |
|  |  |  | ContentProtection |  |  |  |  |  |  |  | 0 … N |  |  |  |  |  |  |  | This element provides information about the use of content protection for the segments of this representation. When not present the content is not encrypted or DRM protected. |
|  |  |  |  | schemeIdUri |  |  |  |  |  |  | M |  |  |  |  |  |  |  | Provides an URI to identify the content protection scheme. This URI should be an URN or an absolute URL used as an identifier. This attribute, possibly in conjunction with the SchemeInformation element, enables a client to determine compatibility for the content protection technologies required to play the protected segments of this representation, such as the DRM system(s), encryption algorithm(s), and key distribution scheme(s). |
|  |  |  |  | SchemeInformation |  |  |  |  |  |  | O |  |  |  |  |  |  |  | This element gives the information about the used content protection scheme. The element can be extended in a separate namespace to provide more scheme specific information. For more details refer to section 8.4.1. |
|  |  |  | TrickMode |  |  |  |  |  |  |  | 0, 1 |  |  |  |  |  |  |  | Provides the information for trick mode. It also indicates that the Representation may be used as a trick mode Representation. |
|  |  |  |  | alternatePlayoutRate |  |  |  |  |  |  | OD Default: 1 |  |  |  |  |  |  |  | Specifies the maximum playout rate as a multiple of the regular playout rate, which this Representation supports with the same decoder profile and level requirements as the normal playout rate. |
|  |  |  | SegmentInfo |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | Provides Segment access information. |
|  |  |  |  | duration |  |  |  |  |  |  | CM Must be present in case duration is not present on period level and the Representation contains any Media Segment with index greater than 1. |  |  |  |  |  |  |  | If present, gives the constant approximate segment duration. All Segments within this SegmentInfo element have the same duration unless it is the last Segment within the Period, which could be significantly shorter. If this attribute is not present, the value of this attribute is derived to be equal to the value of the duration attribute on Period level, if present. |
|  |  |  |  | startIndex |  |  |  |  |  |  | OD default: 1 |  |  |  |  |  |  |  | The index of the first accessible Media Segment in this Representation. In case of on-demand services or in case the first Media Segment of the Representation is accessible, then this value shall not be present or shall be set to 1. |
|  |  |  |  | baseURL |  |  |  |  |  |  | O |  |  |  |  |  |  |  | Base URL on Representation level |
|  |  |  |  | InitialisationSegmentURL |  |  |  |  |  |  | 0, 1 |  |  |  |  |  |  |  | This element references the Initialisation Segment. If not present each Media Segment shall be self-contained. |
|  |  |  |  |  | sourceURL |  |  |  |  |  | O |  |  |  |  |  |  |  | The source string providing the URL, If not present, then any baseURL is mapped to the sourceURL attribute and the range attribute shall be present. |
|  |  |  |  |  | range |  |  |  |  |  | O |  |  |  |  |  |  |  | The byte range restricting the above URL. If not present, the resources referenced in the sourceURL are unrestricted. The format of the string shall comply with the format as specified in section 7.2.4.1. |
|  |  |  |  | UrlTemplate |  |  |  |  |  |  | 0, 1 Must be present if the Url element is not present. |  |  |  |  |  |  |  | The element includes attributes to generate a Segment list for the Representation associated with this element. |
|  |  |  |  |  | sourceURL |  |  |  |  |  | O |  |  |  |  |  |  |  | The source string providing the template. If the template is not present, the id attribute on Representation level provides the necessary information to construct the template. |
|  |  |  |  |  | endIndex |  |  |  |  |  | O |  |  |  |  |  |  |  | The index of the last accessible Media Segment in this Representation. If not present the endIndex is unknown. |
|  |  |  |  | Url |  |  |  |  |  | 0 …N | 0 …N Must be present if the UrlTemplate element is not present. |  |  |  |  |  |  |  | Provides a set of explicit URL(s) for Segments. Note: The URL element may contain a byte range. |
|  |  |  |  |  | sourceURL |  |  |  |  |  | O |  |  |  |  |  |  |  | The source string providing the URL. If not present, then any baseURL is mapped to the sourceURL attribute and the range attribute shall be present. |
|  |  |  |  |  | Range |  |  |  |  |  | O |  |  |  |  |  |  |  | The byte range restricting the above URL. If not present, the resources referenced in the sourceURL are unrestricted. The format of the string shall comply with the format as specified in section 7.2.4.1 |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.3.3	Media Presentation Description Schema

The XML schema for the MPD in section 7.3.2 is as follows:

<?xml version="1.0" encoding="UTF-8"?> <xs:schema targetNamespace="urn:3GPP:ns:PSS:AdaptiveHTTPStreamingMPD:2009" attributeFormDefault="unqualified" elementFormDefault="qualified" xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns="urn:3GPP:ns:PSS:AdaptiveHTTPStreamingMPD:2009"> <xs:annotation> <xs:appinfo>Media Presentation Description</xs:appinfo> <xs:documentation xml:lang="en"> This Schema defines 3GPP Media Presentation Description! </xs:documentation> </xs:annotation> <!-- MPD: main element --> <xs:element name="MPD" type="MPDtype"/> <!-- MPD Type --> <xs:complexType name="MPDtype"> <xs:sequence> <xs:element minOccurs="0" name="ProgramInformation" type="ProgramInformationType"/> <xs:element maxOccurs="unbounded" name="Period" type="PeriodType"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute default="OnDemand" name="type" type="PresentationType"/> <xs:attribute name="availabilityStartTime" type="xs:dateTime"/> <xs:attribute name="availabilityEndTime" type="xs:dateTime"/> <xs:attribute name="mediaPresentationDuration" type="xs:duration"/> <xs:attribute name="minimumUpdatePeriodMPD" type="xs:duration"/> <xs:attribute name="minBufferTime" type="xs:duration" use="required"/> <xs:attribute name="timeShiftBufferDepth" type="xs:duration"/> <xs:attribute name="baseURL" type="xs:anyURI"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Type of presentation - live or on-demand --> <xs:simpleType name="PresentationType"> <xs:restriction base="xs:string"> <xs:enumeration value="OnDemand"/> <xs:enumeration value="Live"/> </xs:restriction> </xs:simpleType> <!-- Period of a presentation --> <xs:complexType name="PeriodType"> <xs:sequence> <xs:element minOccurs="0" name="SegmentInfoDefault" type="SegmentInfoDefaultType"/> <xs:element maxOccurs="unbounded" name="Representation" type="RepresentationType"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="start" type="xs:duration"/> <xs:attribute name="id" type="xs:string"/> <xs:attribute default="false" name="segmentAlignmentFlag" type="xs:boolean"/> <xs:attribute default="false" name="bitStreamSwitchingFlag" type="xs:boolean"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Program information for a presentation --> <xs:complexType name="ProgramInformationType"> <xs:sequence> <xs:element minOccurs="0" name="Title" type="xs:string"/> <xs:element minOccurs="0" name="Source" type="xs:string"/> <xs:element minOccurs="0" name="Copyright" type="xs:string"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="moreInformationURL" type="xs:anyURI"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Default Segment access information --> <xs:complexType name="SegmentInfoDefaultType"> <xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="baseURL" type="xs:anyURI"/> <xs:attribute name="duration" type="xs:duration"/> <xs:attribute default="1" name="startIndex" type="xs:unsignedInt"/> <xs:attribute name="sourceUrlTemplatePeriod" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- A Representation of the presentation content for a specific Period --> <xs:complexType name="RepresentationType"> <xs:sequence> <xs:element name="SegmentInfo" type="SegmentInfoType"/> <xs:element minOccurs="0" maxOccurs="unbounded" name="ContentProtection" type="ContentProtectionType"/> <xs:element minOccurs="0" name="TrickMode" type="TrickModeType"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="id" type="xs:string" use="required"/> <xs:attribute name="bandwidth" type="xs:unsignedInt" use="required"/> <xs:attribute default="0" name="group" type="xs:unsignedInt"/> <xs:attribute name="width" type="xs:unsignedInt"/> <xs:attribute name="height" type="xs:unsignedInt"/> <xs:attribute name="lang" type="xs:string"/> <xs:attribute name="mimeType" type="xs:string" use="required"/> <xs:attribute default="false" name="startWithRAP" type="xs:boolean"/> <xs:attribute name="qualityRanking" type="xs:unsignedInt"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Segment access information --> <xs:complexType name="SegmentInfoType"> <xs:sequence> <xs:element minOccurs="0" name="InitialisationSegmentURL" type="UrlType"/> <xs:choice minOccurs="0"> <xs:element minOccurs="0" name="UrlTemplate" type="UrlTemplateType"/> <xs:sequence> <xs:element maxOccurs="unbounded" name="Url" type="UrlType"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:choice> </xs:sequence> <xs:attribute name="baseURL" type="xs:anyURI"/> <xs:attribute name="duration" type="xs:duration"/> <xs:attribute default="1" name="startIndex" type="xs:unsignedInt"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- A Segment URL --> <xs:complexType name="UrlType"> <xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="sourceURL" type="xs:anyURI"/> <xs:attribute name="range" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- A URL template --> <xs:complexType name="UrlTemplateType"> <xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="sourceURL" type="xs:anyURI"/> <xs:attribute name="endIndex" type="xs:unsignedInt"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Gives information about the content protection --> <xs:complexType name="ContentProtectionType"> <xs:sequence> <xs:element minOccurs="0" name="SchemeInformation" type="xs:string"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="schemeIdUri" type="xs:anyURI" use="required"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Gives information about trick mode --> <xs:complexType name="TrickModeType"> <xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute default="1" name="alternatePlayoutRate" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> </xs:schema>

Namespaces may be used to extend functionalities. As with typical XML practice, any elements or attributes not supported by the terminal shall be ignored. Therefore, all extended elements and attributes added to a Representation in particular shall be such that they can be safely ignored by HTTP streaming clients.

An example for a valid MPD is as follows:

<?xml version="1.0" encoding="UTF-8"?> <MPD type="Live" baseUrl="http://www.example.com" minimumUpdatePeriodMPD="PT20S" minBufferTime="PT10S" mediaPresentationDuration="PT2H" availabilityStartTime="2010-04-01T09:30:47Z" availabilityEndTime="2010-04-07T09:30:47Z" timeShiftBufferDepth="PT30M" xsi:schemaLocation="urn:3GPP:ns:PSS:AdaptiveHTTPStreamingMPD:2009 3GPP-MPD-r1.xsd" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:3GPP:ns:PSS:AdaptiveHTTPStreamingMPD:2009"> <ProgramInformation moreInformationURL="http://www.example.com"> <Title>Example</Title> </ProgramInformation> <Period start="PT0S"> <Representation mimeType="video/3gpp; codecs=s263, samr" bandwidth="256000" id="256"> <SegmentInfo duration="PT10S" baseURL="rep1/"> <InitialisationSegmentURL sourceURL="seg-init.3gp"/> <Url sourceURL="seg-1.3gp"/> <Url sourceURL="seg-2.3gp"/> <Url sourceURL="seg-3.3gp"/> </SegmentInfo> </Representation> <Representation mimeType="video/3gpp; codecs=mp4v.20.9, mp4a.E1" bandwidth="128000" id="128"> <SegmentInfo duration="PT10S" baseURL="rep2/"> <InitialisationSegmentURL sourceURL="seg-init.3gp"/> <Url sourceURL="seg-1.3gp"/> <Url sourceURL="seg-2.3gp"/> <Url sourceURL="seg-3.3gp"/> </SegmentInfo> </Representation> </Period> <Period start="PT30S"> <SegmentInfoDefault duration="PT10S" sourceUrlTemplatePeriod="http://example.com/$RepresentationId$/$Index$.3gp"/> <Representation mimeType="video/3gpp; codecs=mp4v.20.9, mp4a.E1" bandwidth="256000" id="1"> <SegmentInfo> <InitialisationSegmentURL sourceURL="seg-init-1.3gp"/> </SegmentInfo> </Representation> <Representation mimeType="video/3gpp; codecs=mp4v.20.9, mp4a.E1" bandwidth="128000" id="2"> <SegmentInfo> <InitialisationSegmentURL sourceURL="seg-init-2.3gp"/> </SegmentInfo> </Representation> </Period> </MPD>

### 7.3.4	Media Presentation Description Updates

For any time instant when the MPD is available on the server, the server maintains the MPD validity in a sense that an example client using the segment list generation process as provided in section 7.4.3 would be able to access any available Segment in the Segment list for any Representation.

The server may update the MPD during the Media Presentation.

When the MPD is updated any Representation with the same id and within the same Period as a Representation appearing in the previous MPD shall be the identical to the previous one in a sense that all Representation attributes are identical and all segments with the same indexes within one Representation are identical.

Furthermore, if the MPD is updated, then the updates to the MPD shall be such that the updated MPD is compatible with the previous MPD in the following sense: An example client using the segment list generation process as provided in section 7.4.3 would generate an identically functional Segment List from the updated MPD for any time up to the CheckTime as defined in section 7.4.3.4 of the previous MPD as it would have done from the previous MPD.

The requirement ensures that

- clients may immediately begin using the new MPD without synchronisation with the old MPD, since it is compatible with the old MPD before the update time; and

- the update time needs not be synchronised with the time at which the actual change to the MPD takes place: i.e. changes to the MPD may be advertised in advance.

## 7.4	Client Behaviour

### 7.4.1	Introduction

The information on client behaviour is purely informative and does not imply any normative procedures on client implementations.

### 7.4.2	Overview

A 3GPP Adaptive HTTP Streaming client is guided by the information provided in the MPD. It is assumed that the client has access to the MPD that it fetched at time FetchTime. FetchTime is defined at the client as the time the client has succeeded in fetching an instance of an MPD. It is considered as a successful fetching either if the client obtains an updated MPD or the client verifies that the MPD has not been updated since the previous fetching. An example client behaviour is introduced. For providing a continuous streaming service to the user:

- The client parses the MPD and creates a list of accessible Segments for each Representation for the actual client-local time NOW taking into account the procedures specified in section 7.4.3.

- The client selects one or multiple Representations based on the information in the Representation attributes and other information, e.g. available bandwidth and client capabilities. Representations assigned to group 0 are presented without any other Representation. Representations assigned to a non-zero group are typically presented in combination with Representations from groups other than their own, not including the group 0. For each Representation, the client acquires the Initialisation Segment, if present, and the Media Segments of the selected Representations.

- The client accesses the content by requesting Segments or byte ranges of Segments. The client requests the Media Segment of the selected Representation by using the generated Segment list.

- The client buffers media of at least minBufferTime duration before starting the presentation.

- Once the presentation has started, the client continues consuming the media content by continuously requesting Media Segments or parts of Media Segments . The client may switch Representations taking into account updated MPD information and/or updated information from its environment, e.g. change of available bandwidth. With any request for a Media Segment containing a random access point, the client may switch to a different Representation.

- When moving forward, i.e. the NOW time advancing, the client consumes the accessible segments. With each advance in NOW the client possibly expands the list of accessible Segments for each Representation according to the procedures specified in section 7.4.3. If

  - the mediaPresentationDuration attribute is not declared, or if any media described in the MPD does not reach to the end of the Media Presentation and

  - the current playback time gets within a threshold (typically described by the sum of the value of the minBufferTime attribute and the value of the duration attribute on Representation level) of the media described in the MPD for any consuming or to be consumed Representation

then the client should fetch a new MPD, with new fetch time FetchTime. Once received the client now takes into account the possibly updated MPD and the new FetchTime in the generation of the accessible Segment Lists.

In the following a brief overview on Segment list generation, seeking, support for trick modes and switching Representations are provided.

### 7.4.3	Segment List Generation

#### 7.4.3.1 	General

Assume that the HTTP-streaming client has access to an MPD. This section describes how a client may generate a Segment list as shown in Table 7.3 from an MPD at a specific client-local time NOW. In this description, the term NOW is used to refer to “the current value of the clock at the reference client when performing the construction of an MPD Instance from an MPD”.  A client that is not synchronised with a HTTP Streaming server, which is in turn synchronised to UTC, may experience issues in accessing segments due to availability. HTTP Streaming clients should synchronize their clocks to a globally accurate time standard.

Table 7.3: Segment List

| Parameter Name | Cardinality | Description |
| --- | --- | --- |
| Segments | 1 | Provides the Segment URL list. |
| InitialisationSegment | 0, 1 | Describes the Initialisation Segment. If not present each Media Segment is self-initialising. |
| URL | 1 | The URL where to access the Initialisation Segment (the client would restrict the URL with a byte range if one is provided in the MPD). |
| MediaSegment | 1 … N | Describes the accessible Media Segments. |
| startTime | 1 | The approximate start time of the Media Segment in the Period relative to the start time of Period. To obtain the start time of the Media Segment in the Media Presentation, the start time of the Period needs to be added for On-Demand services. For live services, in addition also the value of the availabilityStartTime attribute needs to be added. |
| URL | 1 | The URL where to access the Media Segment (the client would restrict the URL with a byte range if one is provided in the MPD). |

Each SegmentInfo element is used to generate a list of accessible Segments for each Representation.

The following rules apply for SegmentInfoDefault elements or SegmentInfo elements in a MPD:

- The client uses URI reference resolution as discussed in section 7.2.4.2.1. If the MPD is known to be supplied using a URL and over a suitable protocol, that URL establishes a base URL for the segments URLs within the MPD. There may be a baseURL attribute on MPD level or in the SegmentInfoDefault element on Period level or the SegmentInfo element. If the baseURL attribute supplied at any level is absolute, it gives the base URL for the levels below it. Otherwise the base URL for levels below it is formed from the base URL of the higher level composed with the value of the baseURL attribute. Normal URL composition may be used, using relative URLs, which are composed against a base URL. The composition of a relative URL with an effective base URL is done using normal URL Reference Resolution (see RFC 3986 [16], section 5.2).

- If the SegmentInfo element contains a URLtemplate element or a URLtemplate element is implied, then the procedures in section 7.4.3.2 are used to generate a list of Media Segments.

- If the SegmentInfo element contains one or more Url elements providing a set of explicit URL(s) for Media Segments, then the procedures in section 7.4.3.3 are used to generate a list of Media Segments.

- If the type attribute of the MPD is Live, then the restrictions on Media Segment Lists as provided in section 7.4.3.4 need to be taken into account.

The client should only request Segments that are included in the Segment list at time instant NOW.

#### 7.4.3.2 	Template-based Generation of Media Segment List

If the SegmentInfo element contains a URLtemplate element or a URLtemplate element is implied, then the procedures in this section are used to generate a list of Media Segment parameters, i.e. segment URLs and start times, and no byte ranges are associated with the URLs.

The Segment information for a Representation is obtained by combining the  SegmentInfo element with the SegmentInfoDefault element on Period level. The duration attribute of the SegmentInfo element overrides the same attribute of the SegmentInfoDefault element on Period level when both are present.

Assume that the Period end time documented in the current MPD with fetch time FetchTime is defined as PeriodEndTime. For any Period in the MPD except for the last one, the PeriodEndTimeis obtained as the value of the start attribute of the next Period. For the last Period in the MPD

- if the mediaPresentationDuration attribute is present, then PeriodEndTime is defined as the end time of the Media Presentation.

- if the mediaPresentationDuration attribute is not present, then PeriodEndTime is defined as FetchTime + minimumUpdatePeriodMPD).

If the SegmentInfo Element contains a UrlTemplate element containing a sourceURL attribute, then this UrlTemplate is used as the valid UrlTemplate for this Representation. Otherwise, the sourceUrlTemplatePeriod attribute is present; in this case the $RepresentationID$ identifier in the sourceUrlTemplatePeriod attribute is replaced by the value of the id attribute on Representation level and the result string is used as the sourceURL attribute in the UrlTemplate element that is valid for the current Representation.

Assume that Media Segments within a Representation have been assigned consecutive indices i=1,2,3…., i.e. the first Media Segment has been assigned the index i=1, the second Media Segment has been assigned the index i=2, and so on.

A valid list of Media Segments with Segment indices Index[i], MediaSegment.StartTime[i] and MediaSegment.URL[i], i=startIndex, startIndex+1, …, is obtained as follows using the duration attribute for this Representation:

- Set i=startIndex.

- The start time of the first Media Segment is obtained as (startIndex-1)*duration, i.e. MediaSegment.StartTime[1] = (startIndex-1)*duration. If the duration attribute is not provided, then the MediaSegment.StartTime[1] of the only provided Segment is set to 0.

- The URL of the Media Segment i, MediaSegment.URL[i], is obtained by replacing the $Index$ identifier by Index[i] in the sourceURL string of the valid UrlTemplate. Furthermore, any relative URLs are resolved as specified in section 7.2.4.2.1.

- If ((Period.start + MediaSegment.StartTime[i] + duration) <= PeriodEndTime) and if applicable, the index i is smaller than value of the attribute endIndex

  - then

    - A new Media Segment is added to the list, i.e. i = i + 1;

    - MediaSegment.StartTime[i] = MediaSegment.StartTime[i-1] + duration.

    - Proceed with step 3.

  - else

    - The restrictions as specified in section 7.4.3.4 are applied for the creation of the accessible list of Media Segments.

#### 7.4.3.3 	Playlist-based Generation of Media Segment List

If the SegmentInfo element contains one or more Url elements, then the procedures specified in this section apply to generate a valid list of accessible Media Segment URLs and start times described in each SegmentInfo element taking into account the procedures to integrate information from SegmentInfoDefault elements.

Assume that Media Segments within a Representation have been assigned consecutive indices i=1,2,3…., i.e. the first Media Segment has been assigned i=1, the second Media Segment has been assigned i=2, and so on.

A valid list of Media Segments with segment indices i=startIndex, startIndex+1, …, MediaSegment.StartTime[i] and MediaSegment.URL[i] is obtained as follows:

- Set i=startIndex.

- The URL of the Media Segment i, MediaSegment.URL[i], is obtained as the sourceURL attribute of the (i-startIndex+1)-th Url element in the SegmentInfo element taking into account URI reference resolution, restricted to the byte range specified in the range attribute of the same Url element, if present.

- If the duration attribute is provided, then the MediaSegment.StartTime[i] of Media Segment i is obtained as (i-1)*duration. If the duration attribute is not provided, then the MediaSegment.StartTime[1] of the only provided Segment is set to 0.

- If this is not the last Url element, a new Media Segment is added to the list, i.e. i = i + 1, and proceed with step 2; Otherwise proceed with step 5.

- The restrictions as specified in section 7.4.3.4 are applied for the creation of the accessible list of Media Segments.

#### 7.4.3.4. 	Media Segment List Restrictions

The Media Segment List is restricted to a list of accessible Media Segments, which may be a subset of the  Media Segments of the complete Media Presentation. The construction is governed by the current value of the clock at the client NOW.

Generally, Segments are only available for any time NOW between availabilityStartTime and availabilityEndTime. For times NOW outside this window, no Segments are available.

In addition, for live services, assume the variable CheckTime associated to an the MPD with FetchTime is defined as:

- If the minimumUpdatePeriodMPD attribute in the client is provided, then the check time is defined as the sum of the fetch time of this operating MPD and the value of this attribute, i.e. CheckTime = FetchTime + minimumUpdatePeriodMPD.

- If the minimumUpdatePeriodMPD attribute in the client is not provided, external means are used to determine CheckTime, such as a priori knowledge, or HTTP cache headers, etc.

The CheckTime is defined on the MPD-documented media time axis; when the client’s playback time reaches CheckTime it should fetch a new MPD.

Then, the Media Segment list is further restricted by the CheckTime together with the MPD attribute timeShiftBufferDepth such that only Media Segments for which the sum of the start time of the Media Segment and the Period start time falls in the interval [NOW-timeShiftBufferDepth-duration, min(CheckTime, NOW)] are included.

### 7.4.4	Seeking

Assume that a client attempts to seek to a specific presentation time tp in a Representation with start time PeriodStart. PeriodStart defines the absolute start time of the Media Segment in the Media Presentation, i.e. for On-Demand services the start time of the Period needs to be added and for live services, in addition also the value of the availabilityStartTime attribute needs to be added. Before accessing the Media Segments of a Representation, the client needs to download the Initialisation Segment, if present.

Based on the MPD, the client has access to the Media Segment start time and Media Segment URL of each Segment in the Representation. The Segment index segment_index of the Segment most likely to contain media samples for presentation time tp is obtained as the maximum Segment index i, for which the start time MediaSegment[i].StartTime is smaller or equal to the presentation time relative to the Representation start time tp-PeriodStart, i.e.

segment_index = max { i | MediaSegment[i].StartTime <= tp - PeriodStart }.

The Segment URL is obtained as MediaSegment[segment_index].URL.

Note that timing information in the MPD may be approximate due to issues related to placement of Random Access Points, alignment of media tracks and media timing drift. As a result, the Segment identified by the procedure above may begin at a time slightly after tp and the media data for presentation time tp may be in the previous Media Segment. In case of seeking, either the seek time may be updated to equal the first sample time of the retrieved file, or the preceeding file may be retrieved instead. However, note that during continuous playout, including cases where there is a switch between alternative versions, the media data for the time between tp and the start of the retrieved Segment is always available.

For accurate seeking to a presentation time tp, the HTTP-Streaming Client needs to access a random access point (RAP). To determine the random access point in a Media Segment in case of 3GPP Adaptive HTTP Streaming, the client may, for example, use the information in the ‘sidx’ box if present to locate the random access points and the corresponding presentation time in the Media Presentation. In the case that a Segment is a 3GPP movie fragment, it is also possible for the client to use information within the ‘moof’ and ‘mdat’ boxes, for example, to locate RAPs and obtain the necessary presentation time from the information in the movie fragment and the segment start time derived from the MPD. If no RAP with presentation time before the requested presentation time tp is available, the client may either access the previous Segment or may just use the first random access point as the seek result. When Media Segments start with a RAP, these procedures are simple.

Also note that not necessarily all information of the Media Segment needs to be downloaded to access the presentation time tp. The client may for example initially request the ‘sidx’ box from the beginning of the Media Segment using byte range requests. By use of the ‘sidx’, segment timing can be mapped to byte ranges of the Segment. By continuously using partial HTTP requests, only the relevant parts of the Media Segment may be accessed for improved user experience and low start-up delays.

### 7.4.5	Support for Trick Modes

The client may pause or stop a Media Presentation. In this case client simply stops requesting Media Segments or parts thereof. To resume, the client sends requests to Media Segments, starting with the next fragment after the last requested fragment.

If the MPD for a specific Representation contains the TrickMode element, then this Representation is explicitly enabled for the use with trick modes. The client may play the Representation with any speed up to the regular speed times the specified alternatePlayoutRate attribute with the same decoder profile and level requirements as the normal playout rate.

The client may use multiple Representations to support trick mode behaviour.

### 7.4.6	Switching Representations

Based on updated information during an ongoing Media Presentation, a client may decide to switch Representations. Switching to a “new” Representation is equivalent to tuning in or seeking to the new Representation from the time point where the "old" Representation has been presented. Once switching is desired, the client should seek to a RAP in the “new” Representation at a desired presentation time tp later than and close to the current presentation time. Presenting the “old” Representation up to the RAP in the “new” Representation enables seamless switching.

Aligning RAPs across different Representations may be advantageous in locating RAPs in other Representations.

### 7.4.7	Reaction to Error Codes

The HTTP Streaming client provides a streaming service to the user by issuing HTTP requests for Segments at appropriate times. The HTTP Streaming client may also update the MPD by using HTTP requests. In regular operation mode, the server typically responds to such requests with status code 200 OK (for regular GET) or status code 206 Partial Content (for partial GET) and the entity corresponding to the requested resource. Other Successful 2xx or Redirection 3xx status codes may be returned.

HTTP requests may result in a Client Error 4xx or Server Error 5xx status code. Some guidelines are provided in this section as to how an HTTP client may react to such error codes.

If the HTTP Client receives an HTTP client or server error (i.e. messages with 4xx or 5xx error code), the client should respond appropriately to the error code.

If the HTTP Client receives a repeated HTTP error for the request of an MPD, the appropriate response may involve terminating the streaming service.

If the HTTP Client receives an HTTP client error (i.e. messages with 4xx error code) for the request of an Initialisation Segment, the Period containing the Initialisation Segment may not be available anymore or may not be available yet. In this case the client should check if the precision of the time synchronization to a globally accurate time standard is sufficiently accurate. In case of repeated errors, the client should check for an update of the MPD.

If the HTTP Client receives an HTTP client error (i.e. messages with 4xx error code) for the request of a Media Segment, the requested Media Segment may not be available anymore or may not be available yet. In this case the client should check if the precision of the time synchronization to a globally accurate time standard is sufficiently accurate. In case of repeated errors, the client should check for an update of the MPD.

Upon receiving server errors (i.e. messages with 5xx error code), the client should check for an update of the MPD. The client may also check for alternative representations that are hosted on a different server.

# 8	3GPP Adaptive HTTP Streaming

## 8.1	Instantiation of 3GPP Adaptive HTTP Streaming

The Media Presentation framework as introduced in clause 7 is instantiated in this section using the 3GP File Format as specified in [4]. This instantiation is referred to as “3GPP Adaptive HTTP Streaming”. A 3GPP Adaptive HTTP Streaming service is described by an MPD as specified in section 7.3. The MIME type of the MPD shall be “video/vnd.3gpp.mpd”. The mimeType attribute of each Representation shall be provided according to RFC 4281 [14] and Annex A of TS 26.244 [4]. Segment Types and Formats according to 8.2.2 shall be used.

## 8.2	Usage of the 3GP File Format

### 8.2.1	Introduction

3GPP Adaptive HTTP Streaming uses many elements of fragmented 3GP files to define the segment formats. This enables reuse of existing content, easy encoding and recording, etc. This section introduces how the features specified in TS 26.244 [4] are used for the segment formats.

### 8.2.2	Segment Types and Formats

#### 8.2.2.1	Segment Types

3GPP Adaptive HTTP Streaming defines a Segment format that is used in the delivery of media data over HTTP. A Segment shall contain one or more boxes in accordance with the boxed structure of the ISO-base media file format [4].

Three different Segment types are defined for 3GPP adaptive HTTP streaming.

- -	Initialisation Segment with a MIME type as defined in Annex A of TS 26.244 [4] .

- -	Media Segment with a MIME type “video/vnd.3gpp.segment” if accessed through a URL without byte ranges or as defined in Annex A of TS 26.244 [4] if accessed through a URL with byte ranges.

- -	Self-Initialising Media Segment with a MIME type as defined in Annex A of TS 26.244 [4] .

NOTE: the MIME type for Media Segments is defined in Annex A.1.4 of TS 26.244 [4].

For 3GPP adaptive HTTP streaming the following applies:

- In all cases for which a Representation contains more than one Media Segment, the following applies:

- The Initialisation Segment as defined in section 8.2.2.2 shall be present. The Initialisation Segment shall be available to the HTTP Streaming Client before any Media Segment is processed within the Representation.

- Media Segments shall not be self-initialising. The Media Segment format is defined in section 8.2.2.3.

  - In case a Representation contains only a single Media Segment, then either one of the following two options is used:

- An Initialisation Segment as defined in section 8.2.2.2 and one Media Segment as defined in section 8.2.2.3.

- One Self-Initialising Media Segment as defined in section 8.2.2.4.

#### 8.2.2.2	Initialisation Segment Format

The Initialisation Segment is conformant with the 3GPP file format, adaptive streaming profile and shall be branded as “3gh9”.

The Initialisation Segment consists of the “ftyp” box, the “moov” box, and optionally the “pdin” box. The “moov” box shall not contain any samples (i.e. the entry_count in the “stts”, “stsc”, and “stco” boxes shall be set to 0) and is then very small in size. This reduces the start-up time significantly as the Initialisation Segment needs to be downloaded before any Media Segment can be processed.

The “mvex” box shall be contained in the “moov” box to indicate that the client has to expect movie fragments. The “mvex” box also sets default values for the tracks and samples of the following movie fragments.

The Initialisation Segment provides the client with the metadata that describes the media content. The client uses the information in the “moov” box to identify the available media components and their characteristics.

The Initialisation Segment shall not contain any “moof” or “mdat” boxes.

#### 8.2.2.3	Media Segment Format

The following constraints shall apply to a Media Segment conforming to Media Segment Format for 3GPP adaptive HTTP streaming:

- Each Media Segment may contain an ‘styp’ box.

- Each Media Segment shall contain one or more whole self-contained movie fragments. A whole, self-contained movie fragment is a movie fragment (‘moof’) box and a media data (‘mdat’) box that contains all the media samples referenced by the track runs in the movie fragment box.

- Each ‘moof’ box shall contain at least one track fragment.

- The ‘moof’ boxes shall use movie-fragment relative addressing. Absolute byte-offsets shall not be used. In a movie fragment, the durations by which each track extends should be as close to equal as practical. In particular, as movie fragments are accumulated, the track durations should remain close to each other and there should be no 'drift'.

- Each ‘traf’ box may contain a ‘tfad’ box.

- Each Media Segment may contain one or more ‘sidx’ boxes. If present, the first ‘sidx’ box shall be placed before any ‘moof’ box and the subsegment documented by the first Segment Index box shall be the entire segment.

- Further rules on media segments in combination with certain MPD attributes are provided in section 8.2.4.

#### 8.2.2.4	Self-Initialising Media Segment Format

The following constraints shall apply to a Media Segment conforming to Self-Initialising Media Segment Format for 3GPP adaptive HTTP streaming:

- The Self-Initialising Media Segment shall comply to the 3GP Adaptive-Streaming profile as specified in TS 26.244 [4].

- Movie fragment (‘moof’) boxes may be present.

- Each ‘moof’ box, if present, shall contain at least one track fragment.

- The ‘moof’ boxes, if present, shall use movie-fragment relative addressing. Absolute byte-offsets shall not be used. In a movie fragment, the durations by which each track extends should be as close to equal as practical. In particular, as movie fragments are accumulated, the track durations should remain close to each other and there should be no 'drift'.

- Each ‘traf’ box, if present, may contain a ‘tfad’ box.

- A Self-Initialising Media Segment may contain one or more ‘sidx’ boxes. If present, the first ‘sidx’ box shall be placed before any ‘moof’ box and the subsegment documented by the first ‘sidx’ shall be the entire Media Segment.

### 8.2.3	Usage on Server and Client

3GPP Adaptive HTTP-Streaming uses 3GP files according to the 3GP Adaptive-Streaming profile as specified in TS 26.244 [4]. Content may be prepared as 3GP files according to the 3GP Adaptive-Streaming profile. Initialisation Segments and Media Segments may be generated by segmenting such 3GP files. Segment Index ‘sidx’ boxes may be pre-contained in 3GP files or may be generated during the segmentation process. Clients may store a concatenation of a received Initialisation Segment and a sequence of Media Segments to create a compliant 3GP file according to the Adaptive Streaming profile without accessing any media samples.

NOTE: 	As specified in TS 26.244, the MPD may be linked or embedded in the ‘meta’ box of the ‘moov’ box. This enables clients to access the MPD from a 3GP file that was made available from other means than 3GPP Adaptive HTTP Streaming (e.g. progressive download).

### 8.2.4	Media Presentation Authoring Rules for specific MPD flags

#### 8.2.4.1	General

The content, especially the segments across Representations at the same media time may have been prepared in a joint or at least coordinated manner. To expose these properties to the client, certain flags in the MPD can be set to true to indicate such coordinated content preparation. Clients consuming 3GP-DASH formatted media presentations may benefit from properly authored content when switching between or presenting Representations.

The content authoring rules for the media segments when certain flags are set to true or false are provided in the remainder of this section.

#### 8.2.4.2	Segment Alignment

The following applies

to the segmentAlignmentFlag:

  - when set to ‘true’, it indicates that all presentation start and end times of media components of any particular media type are temporally aligned in all Segments across all Representations with the same value of the duration attribute on Representation level in this Period. Such aligned Representations are referred to as “segment-aligned Representations.”

  - when set to ‘false’ or if any Representation is contained in a non-zero group and if the Period contains multiple Representations,

    - then at least one ‘sidx’ box shall be present in each Media Segment. This ‘sidx’ shall document the decoding_time for all tracks present in this Representation.

    - proper track alignment shall be ensured for each movie fragment which is indexed in the second loop of any ‘sidx’ box. If the tracks of the referenced movie fragment are not aligned and if a ‘sidx’ does not provide the track alignment for a movie fragment, then a ‘tfad’ box shall be present for the referenced movie fragment.

  - when set to ‘true’ and all Representations are contained in group zero, and if the tracks are not time-aligned at the segment start, then either the ‘sidx’ (for a Media Segment) or ‘tfad’ (for at least the first movie fragment in the Media Segment) shall be provided.

#### 8.2.4.3	Bitstream Switching

If the bitstreamSwitchingFlag in a Period is set to ‘true’, all following conditions shall be satisfied:

- The segmentAlignmentFlag for the Period is set to ‘true’ and the value of the duration attribute on Representation level is identical for all Representations in the Period.

- For any particular media type, all track fragments within any group in the Period have the same value of track_ID.

- Let X be the concatenation of an Initialization Segment with consecutive Media Segments of a single Representation within a Period, starting with the first Media Segment, and Y be the concatenation wherein the Media Segments with the same constraints come from at least two Representations within the same group. The following applies to all possible instances of X and Y: for any media sample commonly present in X and Y, the decoding time and composition time derived when playing X are identical to the decoding time and composition time, respectively, derived when playing Y, by a legacy player.

## 8.3	Media Codecs

For HTTP-Streaming clients supporting a particular continuous media type, the corresponding media decoders are specified in TS 26.234 [1], section 7.2 for speech, 7.3 for audio, 7.4 for video and 7.9 for timed text.

## 8.4	Security

### 8.4.1	Content Protection

Clients that support content protection may support OMA DRM 2.0 [75] or OMA DRM 2.1 [109]. Other content protection schemes may be supported. The ContentProtection element in the MPD should be used to convey content protection information.

The schemeIdUri attribute is used to identify the content protection schemes employed. This attribute should provide sufficient information, possibly in conjunction with the SchemeInformation element, such as the DRM system(s), encryption algorithm(s), and key distribution scheme(s) employed, to enable a client to determine whether it can possibly play the protected content. The SchemeInformation element can be extended in a separate namespace to provide information specific to the content protection scheme (e.g., particular key management systems or encryption methods). Scheme-specific information can also be provided in the Initialization Segment(s) using the appropriate file format primitives instead of, or in addition to, the SchemeInformation element. The client may have to receive and analyze the protected content (typically only the Initialization Segment, if present), before it can determine whether it has already acquired a license and/or key for accessing the protected content, or to determine from where it can acquire a missing license and/or key, in case this information is not available from the SchemeInformation element.

When using OMA DRM V2.0 or OMA DRM V2.1 schemefor content protection, the non-streamable Packetized DRM Content Format (PDCF) shall be used. An OMA-DRM encrypted Representation shall include the brands “3gh9” and “opf2”.OMA-DRM [74] defines the procedures for acquiring the Rights Object from the Rights Issuer to decrypt PDCF protected content.

### 8.4.2	Transport Security

Transport security in adaptive HTTP streaming is achieved using the HTTPS (Hypertext Transfer Protocol Secure) specified in RFC 2818 [18]. HTTPS may be used to authenticate the server and to ensure secure transport of the content from server to client.

NOTE: The use of HTTPS for delivering Media Segments may inhibit caching at proxies and add overhead at the server and the client.

Annex A (informative):
Guidelines for Adaptive HTTP Streaming

# A.1	Content-Preparation Modes

## A.1.1	Introduction

The specification on adaptive HTTP Streaming is restricted to the interface between the HTTP -Streaming Client and the HTTP-Streaming Server. The content preparation on the network-side is out of scope of this specification. In this clause, guidelines on two different modes how the network can prepare the content to serve the HTTP requests issued by the HTTP-Streaming client.

## A.1.2	Static Mode

Static content preparation mode is an approach for delivering media content over HTTP as static content. The server is not required to prepare the content in any way. Instead, the content preparation is done in advance, possibly offline, by a separate entity. The server may be a web server that serves the media file(s) as any other regular static file. Figure A.1 shows an example HTTP-Streaming architecture for the static content serving mode. Figure A.1 shows an example HTTP-Streaming architecture for the static content serving mode.

Figure A.1: HTTP-Streaming Architecture for the Static Content Serving Mode

## A.1.3	Dynamic Mode

In dynamic content serving mode, the streaming server dynamically tailors the streamed content to a client based on requests from the client. The HTTP streaming server interprets the incoming HTTP GET request and identifies the requested media samples from a given content. The server then locates the requested media samples in the content file(s) or from the live stream. It then extracts and envelopes the requested media samples in a container. Subsequently, the newly formed container with the media samples is delivered to the client in the HTTP GET response body. Figure A.2 shows an example HTTP-Streaming architecture for the dynamic content serving mode. Figure Q.2 shows an example HTTP-Streaming architecture for the dynamic content serving mode.

Figure A.2: HTTP-Streaming Architecture for the Dynamic Content Serving Mode

# A.2	Mapping MPD structure and semantics to SMIL

## A.2.1	General

The mapping presented in this Annex allows transformation of the MPD table and the XML schema defined in section 7.3 to a SMIL-based syntax. This transformation may be effected automatically, for instance, using XSLT, at the client or the server. The MPD structure and semantics will be retained in the SMIL-based syntax.

The first 3 columns of Table Q.1 below contain the elements and attributes from the MPD table 7.2. Column 1 contains an MPD element, column 2 lists its children elements and column 3 lists its attributes.

Column 4 indicates how elements/attributes from this structure can be mapped to elements/attributes in 3GPP SMIL. That is, it indicates which 3GPP SMIL attributes/elements can be used to provide equivalent functionality.

Note that “3GPP SMIL” as used in this document refers to the 3GPP SMIL Language profile defined in TS 26.246 [6].

In some cases to match the semantics from Table 7.2, new attributes/elements that are not defined in 3GPP SMIL, are required. Column 5 lists these attributes or elements. These would be added to 3GPP SMIL as extensions indicated by the “3g9” identifier defined in the same namespace as that for 3GPP HTTP streaming in section 7.3, viz., xmlns="urn:3GPP:ns:PSS:AdaptiveHTTPStreamingMPD:2009".

In many cases deployments that do not use the new elements and attributes from column 5 are feasible– these elements and attributes are either optional or existing SMIL constructs can be used as an alternative (e.g., playlists can be used instead of templates). System deployers should only use constructs in column 4 if they want compatibility with legacy 3GPP SMIL clients. If support for extension elements and attributes has been added to 3GPP SMIL clients, deployments can leverage constructs in column 4 as well as 5.

Table Q.1: Mapping MPD structure, semantics and syntax to SMIL

| Element | Children Elements | Attribute | Mapping to 3GPP SMIL | Extension to 3GPP SMIL |
| --- | --- | --- | --- | --- |
| MPD | Period, ProgramInformation |  | body |  |
|  |  | type |  | type |
|  |  | availabilityStartTime |  | availabilityStartTime |
|  |  | availabilityEndTime |  | availabilityEndTime |
|  |  | duration |  | duration |
|  |  | minimumUpdatePeriodMPD |  | minimumUpdatePeriodMPD |
|  |  | minBufferTime |  | minBufferTime |
|  |  | timeShiftBufferDepth |  | timeShiftBufferDepth |
| Period | Representation, SegmentInfoDefault |  | seq, switch |  |
|  |  | start | begin |  |
|  |  | segmentAlignmentFlag |  | segmentAlignmentFlag |
|  |  | bitstreamSwitchingFlag |  | bitStreamSwitchingFlag |
| ProgramInformation | Source, Copyright, Title |  | meta |  |
| Representation | SegmentInfo, ContentProtection, TrickMode |  | seq |  |
|  |  | id | id |  |
|  |  | bandwidth | systemBitrate |  |
|  |  | width, height | systemScreenSize |  |
|  |  | language | systemLanguage |  |
|  |  | mimeType | systemComponent |  |
|  |  | startWithRAP |  | startWithRAP |
|  |  | qualityRanking |  | qualityRanking |
| SegmentInfo | InitialisationSegmentUrl, Url, UrlTemplate |  | par, seq When track alignment across Segments cannot be guaranteed, par should be used with each children URL containing begin and dur attributes. | SegmentInfo element with playback semantics identical to those defined in section 7.2. That is, all children elements of SegmentInfo are time-continuous across boundaries of consecutive Media Segments within one Representation. |
|  |  | duration | dur specified for each Url in playlist, possibly in conjunction with begin |  |
|  |  | baseURL |  | baseURL |  |
| InitSegmentUrl |  |  | See Url below. To identify initialization segments, either dur can be set to “0” or type can be set to “init” |  |
| Url |  |  | MEDIA-ELMS (ref, video, audio, etc.) as defined in 3GPP SMIL along with all attributes defined for those elements |  |
|  |  | sourceURL | src (only allows absolute URIs). | sourceURL with URI resolution semantics defined in 7.3.2.1. This attribute is also defined for the MPD, SegmentInfo, SegmentInfoDefault and Url elements |
|  |  | range |  | range |
| UrlTemplate |  |  | Url playlists may be used as an alternative to urlTemplate | UrlTemplate (along with attributes defined for UrlTemplate) |
| SegmentInfoDefault | UrlTemplate |  | This element may be skipped and information provided directly at SegmentInfo level instead | SegmentInfoDefault |
|  |  | duration |  | See duration in Segment |
| ContentProtection | SchemeInfo |  | For 3GP files, content protection may be achieved through mechanisms defined in 3GP file format | ContentProtection |
|  |  | schemeId |  | schemeId |
| SchemeInformation |  |  |  | SchemeInfo |
| TrickMode | alternatePlayoutMode |  |  | TrickMode |

The examples below illustrate the use of a SMIL-based syntax. The examples include 3GPP SMIL constructs as well as extensions to 3GPP SMIL.

## A.2.2	Examples

### A.2.2.1	Example 1: MPD for on-demand content with multiple Periods and alternate Representations

<smil xmlns="http://www.w3.org/2001/SMIL20/Language"

xmlns:3g9="urn:3GPP:ns:PSS:AdaptiveHTTPStreamingMPD:2009">

<body>

<!-- Period 1-->

<seq begin=”0s”>

<!-- alternate set of Representations available during this Period -->

<switch>

<!-- low bitrate  Representation with 15-sec Segments -->

<seq systemBitrate=”128000”>

<video dur=”0s”  src="http://server/path/rep1/clip_init.3gp"/>

<par>

<video begin=”0s” dur=”15s” src="http://server/path/rep1/clip_1.3gp"/>

<video begin=”15s” dur=”15s” src="http://server/path/rep1/clip_1.3gp"/>

...

<video begin=”585s” dur=”15s” src="http://server/path/rep1/clip_40.3gp"/>

</par>

</seq>

<!-- mid bitrate Representation with 30-sec Segments -->

<seq systemBitrate=”256000”>

<video dur=”0s” src="http://server/path/rep2/clip_init.3gp”/>

<par>

<video begin=”0s” dur=”30s” src="http://server/path/rep2/clip_1.3gp"/>

...

<video begin=”570s” dur=”30s” src="http://server/path/rep2/clip_20.3gp"/>

</par>

</seq>

<!-- high bitrate Representation with 30-sec Segments-->

<seq systemBitrate=”512000”>

<video type=”init” dur=”0s” src="http://server/path/rep3/clip_init.3gp”/>

<par>

<video begin=”0s” dur=”30s” src="http://server/path/rep3/clip_1.3gp"/>

...

<video begin=”570s” dur=”30s” src="http://server/path/rep3/clip_20.3gp"/>

</par>

</seq>

</switch>

</seq> <!-- end of Period 1 -->

<!-- Period 2 begins 10 minutes after presentation start -->

<seq begin=”600s”>

<switch>

<!-- english ad -->

<seq systemLanguage=”en” >

<seq>

<video src=”http://adserver/getad.php?id=1“/>

</seq>

</seq>

<!-- french ad -->

<seq systemLanguage=”fr” >

<seq>

<video src=”http://adserver/getad.php?id=2“/>

</seq>

</seq>

</switch>

</seq>

<!--start  of Period 3. Note that mid bitrate Representation is missing during this Period. Also the server used to deliver Segments is   different than the one in the previous Period. The use of URL resolution as defined by sourceURL is illustrated -->

<seq begin=”630s”  3g9:sourceURL=”http://new-server/new-path/”>

<switch>

<!-- low bitrate  -->

<seq systemBitrate=”128000” 3g9:baseURL=”rep1/”>

<!-- no initialisation Segment -->

<par>

<video begin=“0s“ dur=“15s“ 3g9:sourceURL="clip41.3gp”/>

...

</par>

</seq>

<!-- high bitrate -->

<seq systemBitrate=”512000” 3g9:baseURL=”rep3/”>

<video type=”init” dur=”0s' 3g9:sourceURL=”clip2x_init.3gp”/>

<par>

<!-- URLs can include a byte range -->

<video begin=”0s” dur=”30s” 3g9:sourceURL=”clip2x.3gp” 3g9:range=“500-2000”/>

<video begin=”30s” dur=”30s” 3g9:sourceURL=”clip2x.3gp” 3g9:range=“2001-2500”/>

…

</par>

</seq>

</switch>

</seq>

<!-- more Periods can go here as required -->

…

</body>

</smil>

### A.2.2.2	Example 2: MPD for live content

MPD that includes availabilityStartTime and availabilityEndTime extensions to enforce lifetime and the minimumUpdatePeriodMPD extension to help clients choose an update period. Note that Segment format used in the example is MPEG-2 TS.

<smil xmlns="http://www.w3.org/2001/SMIL20/Language"

xmlns:3g9="urn:3GPP:ns:PSS:AdaptiveHTTPStreamingMPD:2009">

<body 3g9: minimumUpdatePeriodMPD =”120s” 3g9:availabilityStartTime=”2010-01-27T13:00Z” 3g9:availabilityEndTime=”2010-01-27T15:00Z”3g9:minBufferTime=”10s” 3g9:type=”live”>

<!-- start of a Period -->

<seq begin=”0s”>

<!-- a single Representation -->

<seq>

<par>

<video begin=“0s“ dur=“10s“ src=“http://server/live_clip_1.m2ts“/>

…

<video begin=“110s“ dur=“10s“ src=”http://server/live_clip_12.m2ts”/>

</par>

</seq> <!-- end of Representation -->

</seq> <!-- end of Period -->

</body>

</smil>

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2010-09 | 49 |  |  |  | Presented for information |  | 1.0.0 |