3GPP TSG-SA4 Ad hoc meeting on 5GMS	S4-AHI866

# The Hague, The Netherlands, November 12-14, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DASH delivery enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This dCR addresses steps in the procedures for unicast downlink media streaming: - Localization of 5GMSd AS(s) hosting the media content: In Clause 5.2 the 5GMSd AS represents a CDN edge hosting the media content. Figures 5.2.-1 and and 5.2-2 include a step for establishing the transport session with the 5GMSd AS. In actual deployments this is not just a single step, but it includes locating the correct 5GMSd AS, i.e. the AS that has the requested media content. Locating the 5GMSd AS involves DNS resolving (in the operator network), potentially followed up by HTTP redirect(s) to the 'final' AS. The location of the 5GMSd AS may be cached for when requesting future media segments, in the case of DASH. The authors believe that 5GMSd AS localization should be reflected in the stage 2 procedures. - Fetching expected media segments for increased delivery performance: With the trend of CDN edges (thus 5GMSd AS(s)) moving further towards the edges of the network (i.e. located near the end-user), the 5GMSd AS may not hold the entire video catalogue. The 5GMSd AS may effectively become a cache instead of a traditional content delivery node. To improve cache-hits, and as such the delivery performance of media content to the 5GMS player, this dCR proposes an additional step to inform the 5GMSd AS about future segment requests, allowing the 5GMSd AS to keep the segment in the cache or fetch the content in advance. The current architecture does not foresee this delivery model, which we believe should be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This dCR adds: - a description on the localization and establishment of transport sessions with 5GMSd AS(s); - steps to the procedure for informing the 5GMSd AS about expected requests for media segments and fetching the segments in advance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transport session establishment does not reflect current practices and DASH segment caching would be sub-optimal. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note, this dCR uses the Ericsson contribution S4-191236, which was agreed during the Busan meeting, as a base. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

## 5.2	Baseline procedure for Unicast Media Downlink Streaming Session establishment

### 5.2.1	General

This clause describes the baseline procedure for establishment of a unicast media downlink streaming session. This baseline procedure assumes that the 5GMSd AF and the 5GMSd AS resides in the external DN and that there are no interactions between the 5GMSd AF and the 5G System.

### 5.2.2	Progressive Download of On-Demand Content

This baseline procedure describes the establishment of a unicast downlink streaming session. A streaming session may use 3GP File Format (Progressive Download), 3GP Time Text or other (potentially non-3GPP defined) formats.

Figure 5.2-1: High Level Procedure for progressive download for on-demand media

Prerequisite:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMSd Aware Application has received the service announcement from the 5GMSd Application Provider.

Steps:

1: The 5GMSd Aware Application triggers the Service and Content Discovery procedure.

The Content Discovery procedure only involves the 5GMSd Aware Application and the 5GMSd Application Provider and hence is out of scope of the present document.

2: A Media Player Entry is selected.

3: The 5GMSd Aware Application triggers the Media Session handler to start the playback. The media player entry is provided to the Media Session handler.

4: The Media Session Handler triggers the 5GMS Player to start the session.

5: The Media Player localizes the 5GMSd AS (e.g. via DNS and/or HTTP redirects) and establishes the transport session.

6: The Media Player sends the request for the progressive download content.

7: The Media Player receives the initialization information of the progressive download content. The initialization information contains configuration parameters for reception of the media, and optionally also DRM information.

8: The Media Player configures the rendering pipeline for media playback.

9: The Media Player notifies the media session handler, providing the transport session information and some media content related information.

10: Optional: Media Player acquires a DRM License from an External AS.

11: The Media Player receives media content and puts it into the rendering pipeline.

12: The Media Player continuously receives and plays back the media content.

### 5.2.3	DASH Streaming

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.2-2: High Level Procedure for DASH content

Prerequisite:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMSd Aware Application has received the service announcement from the 5GMSd Application Provider.

Steps:

1: The 5GMSd Aware Application triggers the Service and Content Discovery procedure. The Content Discovery procedure only involves the App and the external Application Server.

2: A media content item is selected.

3: The 5GMSd Aware Application triggers the Media Session handler to start media playback. The Media Player Entry is provided to the Media Session handler.

4: The Media Session Handler triggers the 5GMS Player to start the session.

5: The Media Player establishes the transport session for acquiring the manifest referenced by the Media Player Entry.

6: The Media Player requests the manifest

7: The media access client receives the manifest

8: The Media Player processes the manifest. It determines for example the number of needed transport sessions for media acquisition. The 5GMS Player should be able to use the manifest information to initialize the media pipelines for each media stream. The manifest should also contain information to start the DRM client initialization, when DRM is used.

9: The Media Player notifies the media session handler about the manifest. The notification may contain parameters from the manifest

10: Optional: the Media Player acquires the necessary DRM information, for example a DRM License.

11: The Media Player configures the media rendering pipeline.

12: The Media Player localizes the 5GMSd AS (e.g. via DNS and/or HTTP redirects) hosting the Initiation information and media segments. The Media Player uses the location of the 5GMSd AS for subsequent requests. The Media Player establishes the necessary transport sessions for the content. For example, the media access client may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations.

13: The media session handler is notified about the established transport sessions for the content.

14: The Media Player requests initialization information. The Media Player repeats this step for each required initialization segment.

15: The Media Player receives the initialization information.

16: The Media Player informs the 5GMSd AS about expected media segment request(s).

17: When the 5GMSd AS has the expected media segments, it keeps the media segment(s) at least until the Media Player has requested the media segments. Otherwise, the 5GMSd AS fetches the media segment(s) from external Application Server.

18 The Media Player requests media segments according to the manifest.

19: the Media Player receives media segments and puts the information into the according media rendering pipeline.

20: Previous steps are repeated according to the manifest information.

*** End of first change ***