3GPP TS 26.501 V16.1.0 (2019-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

5G Media Streaming (5GMS);

General description and architecture

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<5G, Media, Streaming, Live, Uplink, Codec, Multimedia, transport>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definition of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	6

3.3	Abbreviations	6

4	Media Streaming General Service Architecture	7

4.1	Overall Media Architecture	7

4.2	5G Unicast Media Downlink Streaming Architecture	7

4.2.1	Standalone – Non-Roaming	7

4.2.2	UE Media Functions	10

4.3	5G Media Uplink Streaming Architecture	13

4.3.1	Media Architecture	13

4.3.2	UE Media Functions	14

4.4	Network Slicing	15

5	Procedures for Downlink Streaming	15

5.1	General	15

5.2	Establish a Unicast Media Downlink Streaming Session	17

5.2.1	Progressive Download of On-Demand Content	17

5.2.2	DASH Streaming	19

5.3	Provisioning Session for Media Streaming	20

5.4	Media Ingest for Downlink Streaming	20

5.5	Metrics collection and reporting	21

5.6	Consumption reporting	25

6	Procedures for Uplink Streaming	26

6.1	General	26

6.2	Preparing for Uplink Streaming	26

6.2.1	Introduction	26

6.2.2	Sink Configuration at the Media AF / AS	26

6.2.3	Source Configuration at the UE	27

6.3	Establishment of an Uplink Streaming Session	28

6.4	Termination of an Uplink Streaming Session	29

6.5	Providing Network Assistance	29

6.6	Providing Remote Control	30

6.7	RAN Signaling based Support for Uplink Network Assistance	31

7	5GMS Network Media Processing	31

7.1	General	31

7.2	Media Processing Procedures for Downlink	31

7.3	Media Processing Procedures for Uplink	33

7.4	Edge Computing	34

Annex A (informative):	Change history	35

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the 5G Media Streaming (5GMS) architecture. The 5GMS supported services include MNO and 3rd party Media Downlink Streaming Services, and MNO and 3rd party Media Uplink Streaming Services. The 5GMS supports related network and UE functions and APIs, backwards compatible functions for EUTRAN deployments (with and without MBMS) and 5G specific features.

NOTE:	Support of 5G Media streaming over MBMS with 5GC is not considered in the current version of the present document.

The 5GMS architecture is functionally divided into independent components enabling different deployments with various degrees of integration between 5G MNOs and Content Providers. It is specified as a set of extensions to TS 23.501 "System Architecture for the 5G System".

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 26.238: "Uplink streaming".

[6]	3GPP TS 26.307: "Presentation layer for 3GPP services".

[7]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[8]	3GPP TS 26.234: "Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs".

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G Media Streaming Client for downlink (5GMSd Client): UE function that includes at least a 5G Media Streaming Player and a Media Session Handler for downlink streaming and that may be accessed through well-defined interfaces/APIs.

5G Media Streaming Client for uplink (5GMSu Client): Originator of 5GMSu service that may be accessed through well-defined interfaces/APIs.

NOTE 1: The 5GMSu receives a Media Streamer Entry to initiate an uplink streaming session.

NOTE 2: The 5GMSu captures the media on the provided input devices. The 5GMUS exposes some basic controls such as capture, pause, and stop to the App.

Dynamic policy: a Dynamic PCC Rule (cf. TS 23.503[x]) for an uplink or downlink application flow during a media session.

Egest Session: a uplink media streaming session from the 5GMSd AS towards the 5GMSd Application provider.

Ingest Session: a session to upload the media content into a Media AS

Policy type identification: an indication of the desired policy, which is used by 5GMSd AF to select the appropriate PCC Rule and API towards 5G System.

media player entry: a document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file.

media streamer entry: a pointer, e.g. in the form of a URL that defines an entry point of a media uplink streaming session.

presentation entry: a document or a pointer to a document that defines an application presentation e.g. an HTML5 document as defined in e.g. TS 26.307 [6].

5GMSd Media Player:  UE function that enables playback and rendering of a media presentation based on a Media Player Entry and exposing some basic controls such as play, pause, seek, stop to the App.

NOTE 1:	It is expected to include a Media Access Client, Media Decoders, Media rendering/presentation, and possibly also DRM Client and Metrics Collection and Reporting. The 5GMS Player's Media Access Client receives a Media Player Entry. The 5GMS player renders the media on the provided output devices, such as a display in case of video.

NOTE 2:	The 5GMSd Player is functionally similar to the combination of a TS 26.247 [7] 3GP-DASH client and a TS 26.234 [8] PSS media decoder and renderer.

service access information: set of parameters and addresses (including Media AF and Media AS addresses), which are needed to activate the reception of a streaming session.

third-party player: A Third-Party Player is a part of an application and this Player uses APIs to exercise selected 5GMS Media Functions to play-back the Media content.

NOTE:	Such APIs are for example defined in TS 26.307 [6] when using the Media Source Extensions for media playback. This type of Player is downloaded by or built-in to an application, or it is downloaded with the Presentation Entry (e.g. as a JavaScript library).

third-party uplink streamer: The third-party uplink streamer is a part of an application and the application uses APIs to exercise selected 5GMSu functions to capture and stream the media content.

NOTE:	This type of Streamer is typically implemented as downloadable software.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5GMS	5G Media Streaming

5GMSd	5G Media Streaming downlink

5GMSu	5G Media Streaming uplink

5GS	5G Systems

AF	Application Function

ABR	Adaptive BitRate

API	Application Programming Interface

App	Application

AS	Application Server

CDN	Content Delivery Network

DASH	Dynamic and Adaptive Streaming over HTTP

DRM	Digital Rights Management

GPU	Graphics Processing Unit

MNO	Mobile Network Operator

MPD	Media Presentation Description

NA	Network Assistance

NR	New Radio

NEF	Network Exposure Function

PCF	Policy and Charging Function

SBA	Service based Architecture

# 4	Media Streaming General Service Architecture

## 4.1	Overall Media Architecture

Streaming in the context of the specification is defined as the delivery of time-continuous media as the predominant media. Streaming points to the fact that the media is predominantly sent only into a single direction and consumed as it is received. Additionally, the media content may be streamed as it is produced.

The overall 5G Media Streaming Architecture is shown in figure 4.1-1.

Figure 4.1-1: 5G Media Streaming within the 5G System

NOTE: The functions indicated by the yellow filled boxes are in scope of stage 3 specifications for 5GMS. The functions indicated by the grey boxes are defined in 5G System specifications. The functions indicated by the blue boxes are neither in scope of 5G Media Streaming nor 5G System specifications.

The 5GMS Application Provider uses 5GMS for streaming services. It provides a 5GMS Aware Application on the UE to make use of 5GMS Client and network functions using interfaces and APIs defined in 5GMS.

The architecture in Figure 4.1-1 represents the specified 5GMS functions within the 5G System (5GS) as defined in TS23.501 [2]. Three main functions are defined:

-	Media AF: An Application Function similar as defined in TS23.501 [2], clause 6.2.10, dedicated to 5G Media Streaming.

-	Media AS: An Application Server dedicated to 5G Media Streaming.

-	5GMS Client: A UE internal function dedicated to 5G Media Streaming.

Media AF and Media AS are Data Network (DN) functions and communicate with the UE via N6 as defined in TS23.501 [2].

Functions in trusted DNs are trusted by the operator’s network as illustrated in Figure 4.2.3-5 of TS 23.501[2]. Therefore, AFs in trusted DNs may directly communicate with all 5G Core functions.

Functions in external DNs may only communicate with 5G Core functions via the NEF using N33.

The present document specifies the according network architectures for 5GS.

NOTE: EPS is ffs.

The 5G Media Services Architecture maps the overall high-level architecture shown in figure 4.1-1 to the architecture shown in figure 4.1-2.

Figure 4.1-2: 5G Media Streaming General Architecture

The remainder of the present document specifies stage 2 aspects of the media streaming functional entities shown in the general architecture of figure 4.1-2.

This architecture specification addresses two main scenarios as concerns each individual media streaming operation:

-	Downlink streaming: The network is the origin of the media and the UE acts as the consumption device

-	Uplink streaming: The UE is the origin of the media and the network acts as the consumption entity

The functional entities and interfaces of the media streaming general architecture need to be elaborated with specificities relating to downlink and uplink streaming. For this purpose corresponding descriptions add the suffix “d” for downlink and “u” for uplink functionality as appropriate in each case.

Clause 4.2 introduces the 5G Unicast Media Downlink Streaming Architecture.

Clause 4.3 introduces the 5G Unicast Media Uplink Streaming Architecture.

## 4.2	5G Unicast Media Downlink Streaming Architecture

### 4.2.1	Standalone – Non-Roaming

The 5GMSd Application Provider uses 5GMSd for downlink streaming services. It provides a 5GMSd Aware Application on the UE to make use of 5GMSd Client and network functions using interfaces and APIs defined in 5GMS.

The architecture in Figure 4.2.1-1 represents the specified 5GMSd functions within the 5G System (5GS) as defined in TS23.501 [2]. Three main functions are defined:

-	5GMSd AF: An Application Function similar as defined in TS23.501 [2], clause 6.2.10, dedicated to 5G Media Downlink Streaming.

-	5GMSd AS: An Application Server dedicated to 5G Media Downlink Streaming.

-	5GMSd Client: A UE internal function dedicated to 5G Media Downlink Streaming.

5GMSd AF and 5GMSd AS are Data Network (DN) functions and communicate with the UE via N6 as defined in TS23.501 [2].

Functions in trusted DNs are trusted by the operator’s network as illustrated in Figure 4.2.3-5 of TS 23.501[2]. Therefore, AFs in trusted DNs may directly communicate with all 5G Core functions.

Functions in external DNs may only communicate with 5G Core functions via the NEF using N33.

NOTE: EPS is ffs.

Figure 4.2.1-1: 5G Media Downlink Streaming within 5G System

NOTE: The functions indicated by the yellow filled boxes are in scope of stage 3 specifications for 5GMS. The functions indicated by the grey boxes are defined in 5G System specifications. The functions indicated by the blue boxes are neither in scope of 5G Media Streaming nor 5G System specifications.

The architecture in Figure 4.2.1-2 represents the media architecture connecting UE internal functions and related network functions.

Figure 4.2.1-2: Media Architecture for unicast media downlink streaming

NOTE 1: The functions indicated by the yellow filled boxes are in scope of stage 3 for 5GMSd. The functions indicated by the grey boxes are defined in 5GS. The interfaces indicated by solid lines are in scope of stage 3 for 5GMSd. The interfaces indicated by dashed lines are defined in 5GS. The interfaces indicated by dotted lines are neither in scope of 5GS nor 5GMSd, but are considered as part of informative call flows.

NOTE 2:	Red ovals indicate API provider functions.

NOTE 3:	The 5GMSd AF may also interact with the NEF for NEF-enabled API access. However, within Release 16, only interactions with the Policy and Charging Function (PCF) are considered in 5GMS specifications.

NOTE 4:	Some information might also be exchanged via the OAM, although the OAM is not explicitly shown in the architecture.

The following functions are defined:

-	5G Media Streaming Client for downlink (5GMSd Client) on UE: Receiver of 5GMS downlink media streaming service that may be accessed through well-defined interfaces/APIs.

-	The 5GMSd Client contains two sub-functions

-	Media Session Handler: A function on the UE that communicates with the 5GMSd AF in order to establish, control and support the delivery of a media session. The Media Session Handler exposes APIs that can be used by the 5GMSd Aware Application.

-	Media Player: A function on the UE that communicates with the 5GMSd AS in order to stream the media content and provides a service to the 5GMSd Aware Application for media playback and the Media Session Handler for media session control.

-	5GMSd Aware Application: The 5GMSd Client is typically controlled by an external media application, e.g. an App, which implements external application or content service provider specific logic and enables establishing a media session. The 5GMSd Aware Application is not defined within 5G Media Streaming specifications, but the function makes use of 5GMSd Client and network functions using interfaces and APIs defined in 5GMSd.

-	5GMSd AS: An Application Server which hosts 5G media functions. Note that there may be different realizations of 5GMSd AS's, for example a Content Delivery Network (CDN).

-	5GMS Application Provider: External application or content specific media functionality, e.g., media creation, encoding and formatting that uses 5GMSd to stream media to 5GMSd Aware applications.

-	5GMSd AF: provides various control functions to the Media Session Handler on the UE and/or to 5GMS Application Provider. It may relay or initiate a request for different Policy or Charging Function (PCF) treatment or interact with other network functions.

NOTE 5:	There may be multiple 5GMSd AFs residing within the Trusted Media Functions entity each exposing one or more APIs.

The following interfaces are defined for 5G Media Downlink Streaming:

-	M1d (5GMSd Provisioning API): External API, exposed by the 5GMSd AF to provision the usage of the 5G Media Streaming System and to obtain feedback.

-	M2d (5GMSd Ingest API): Optional External API exposed by the 5GMSd AS used when the Media AS in the trusted DN is selected to host content for the streaming service.

-	M3d: (Internal and NOT SPECIFIED): Internal API used to exchange information for content hosting on a 5GMSd AS within the trusted DN.

-	M4d (Media Streaming APIs): APIs exposed by a 5GMSd AS to the Media Player to stream media content.

-	M5d (Media Session Handling API): APIs exposed by a Media AF to the Media Session Handler for media session handling, control and assistance that also include appropriate security mechanisms e.g. authorization and authentication.

-	M6d (UE Media Session Handling APIs): APIs exposed by a Media Session Handler to the 5GMSd Media Session Handler for client-internal communication and 5GMSd Aware Application to make use of 5GMS functions.

-	M7d (UE Media Player APIs): APIs exposed by a Media Player to the 5GMSd Aware Application and Media Session Handler to make use of the Media Player.

-	M8d: (EXTERNAL and NOT SPECIFIED): unspecified application interface, which is used for information exchange between the 5GMSd Aware application and the 5GMSd Application Provider, for example to provide service access information to the application.

NOTE 6:	Non-Standalone, Roaming, Non-3GPP Access and EPC-5GC interworking aspects are FFS.

The following subfunctions are identified as a part of a more detailed breakdown of the 5GMSd AS for stage 3 specifications:

-	Adaptive Bit Rate (ABR) Encoder, Encryption and Encapsulator

-	Manifest (e.g. MPD) Generator and Segment (e.g. DASH) Packager

-	Origin Server

-	CDN Server (e.g. Edge Servers)

-	DRM Server (e.g. DRM License Server)

-	Service Directory

-	Content Guide Server

-	Replacement content server (e.g. Ad content server)

-	Manifest Proxy, i.e. MPD modification server

-	App Server

-	Session Management Server

5GMSd UE Media function breakdown is provided in clause 4.2.2.

### 4.2.2	UE 5GMSd Functions

The UE may include many detailed subfunctions that can be used individually or controlled individually by the 5GMSd-Aware application. This clause breaks down several relevant identified sub-functions for which stage-3 specification is available.

The 5GMSd Aware Application itself may include many functions that are not provided by the 5GMSd client or the 5G UE. Examples include content discovery, notifications, social network integration, etc. The 5GMSd Aware application may also include functions that are equivalent to ones provided by the 5GMSd client and may only use a subset of the 5GMSd client functions.

With respect to Media Player functions, Figure 4.2.2-1 shows more detailed functional components of a UE for media player functions to access the 5GMSd Media AS.

Figure 4.2.2-1: UE 5G Media Downlink Streaming Functions (Media Player centric)

The following subfunctions are identified as part of a more detailed breakdown of the Media Player Function:

-	Media Access Client: Accesses Media Content such as DASH-formatted Media Segments.

- 	Media Decapsulation: Extracts the elementary media streams for decoding and provides media system related functions such as time synchronization, capability signalling, accessibility signalling, etc.

-	DRM Client (optional): when present, DRM client might or might not be a part of Media player. It provides a content protection mechanism with its unique key management and key delivery system, authentication/authorization, policy enforcement and entitlement check. DRM client is not defined within 5G Media Streaming specifications.

- 	Media Decryption (optional): when present, media decryiton is responsible to decrype the media samples using the keys provided in the DRM license, and further passing to the media decoder to enable playback of encrypted media. The media decryption and media decoding could be implemnted on general-purpose processor in software or hardware, or for more secured and robust architecture, the decryption, decoding and rednerfing could be impleted on the hardware on secure processors.

-	Media Decoders: Decodes the media like audio or video.

-	Media Rendering / Presentation: Renders the media using an appropriate output device.

-

With respect to the Media Session Handler, Figure 4.2.2-2 shows more detailed functional components of a UE to access the 5GMSd AF.

Figure 4.2.2-2: UE 5G Media Streaming Functions (Control-Centric)

NOTE 1:	SBA refers to principles of 5G Service based Architecture (SBA) realizations of control transactions.

NOTE 2:	The yellow color indicate here that the 3GPP has created specifications for the function.

NOTE 3:	A UE is a logical device which may correspond to the tethering of multiple physical devices or other types of realizations.

The following subfunctions are identified as part of a more detailed breakdown of Media Session Handler:

-	Media Session Handler Core: Realization of a "session" concept for media communications, spanning optionally over multiple stateless sessions. May optionally interact with network-based 5GMSd AFs.

-	Metrics collection and reporting: executes the collection and reporting of metrics.

-	Consumption reporting: reports to a 5GMSd AF about the currently consumed media within the available presentation, about the UE capabilities and about the environment of the media session for potential transport optimizations by the network or consumption report analysis.

-	Associated Delivery Procedures: Functionalities provided by the 5GMSd client to support the application in the delivery of the media presentations, such as location filtering

-	Network Assistance: assisting functions provided to the 5GMSd Client and Media Player

NOTE 4:	Based on such a decomposition, additional interfaces and APIs may exist in inside the UE.

-	Media Control Interface(s) to configure and interact with the different UE media functions.

-	Media Control Interface for media session management.

-	Control interface for metrics collection and reporting.

-	Decoded media samples are handed over to the media renderer.

-	Decrypted, compressed media samples are handed over to a trusted media decoder.

-	In case of encryption, the encrypted, compressed media samples are handed over to the DRM Client.

NOTE 5:	Non-Standalone, Roaming, Non-3GPP Access and EPC-5GC interworking aspects are FFS.

## 4.3	5G Media Uplink Streaming Architecture

### 4.3.1	Media Architecture

The 5GMSu Application Provider uses 5GMSu for uplink streaming services. It provides a 5GMSu Aware Application on the UE to make use of 5GMSu Client and network functions using interfaces and APIs defined in 5GMSu.

The architecture in Figure 4.3.1-1 represents the specified 5GMSu functions within the 5G System (5GS) as defined in TS23.501 [2]. Three main functions are defined:

- 5GMSu AF: An Application Function similar as defined in TS23.501 [2], clause 6.2.10, dedicated to 5G Media Uplink Streaming.

- 5GMSu AS: An Application Server dedicated to 5G Media Uplink Streaming.

- 5GMSu Client: A UE internal function dedicated to 5G Media Uplink Streaming.

5GMSu AF and 5GMSu AS are Data Network (DN) functions and communicate with the UE via N6 as defined in TS23.501 [2].

Functions in trusted DNs are trusted by the operator’s network as illustrated in Figure 4.2.3-5 of TS 23.501[2]. Therefore, AFs in trusted DNs may directly communicate with all 5G Core functions.

Functions in external DNs may only communicate with 5G Core functions via the NEF using N33.

Figure 4.3.1-1: Media Architecture for unicast media uplink streaming

NOTE 1: The functions indicated by the yellow filled boxes are in scope of stage 3 specifications for 5GMS. The functions indicated by the grey boxes are defined in 5G System specifications. The functions indicated by the blue boxes are neither in scope of 5G Media Streaming nor 5G System specifications.

The architecture in Figure 4.3.1-2 represents the media architecture connecting UE internal functions and related network functions for 5G Media Uplink Streaming.

Figure 4.3.1-2: Media Architecture for unicast media uplink streaming

NOTE 1: The functions indicated by the yellow filled boxes are in scope of stage 3 for 5GMSu. The functions indicated by the grey boxes are defined in 5GS. The interfaces indicated by solid lines are in scope of stage 3 for 5GMSu. The interfaces indicated by dashed lines are defined in 5GS. The interfaces indicated by dotted lines are neither in scope of 5GS nor 5GMSu , but are considered as part of informative call flows.

NOTE 2:	Red ovals indicate API provider functions.

NOTE 3:	The 5GMSu AF may also interact with the NEF for NEF-enabled API access. However, within Release 16, only interactions with the Policy and Charging Function (PCF) are considered in 5GMSu specifications.

NOTE 4:	Some information might also be exchanged via the OAM, although the OAM is not explicitly shown in the architecture.

The following functions are defined:

-	5G Media Streaming Client for uplink (5GMSu Client) on UE: Originator of 5GMSu service that may be accessed through well-defined interfaces/APIs.

The 5GMSu Client contains two sub-functions

-	Media Session Handler: A function on the UE that communicates with the 5GMSuAF in order to establish, control and support the delivery of a media session. The Media Session Handler exposes APIs that can be used by the 5GMSu Aware Application.

-	Media Streamer: A function on the UE that communicates with the 5GMSu AS in order to stream the media content and provides a service to the 5GMSu Aware Application for media capturing and streaming and the Media Session Handler for media session control.

-	5GMSu Aware Application: The 5GMSu Client is typically controlled by an external media application, e.g. an App, which implements external application or content service provider specific logic and enables establishing a media session. The 5GMSu Aware Application is not defined within 5G Media Streaming specifications, but the function makes use of 5GMSu Client and network functions using interfaces and APIs defined in 5GMSu.

-	5GMSu AS: An Application Server which hosts 5G media functions. Note that there may be different realizations of 5GMSu AS's, for example a Content Delivery Network (CDN).

-	5GMSu Application Provider: External application or content specific media functionality, e.g., media storage, consumption, transcoding and redistribution that uses 5GMSu to stream media from 5GMSu Aware applications.

-	5GMSu AF: provides various control functions to the Media Session Handler on the UE and/or to 5GMSu Application Provider. It may relay or initiate a request for different Policy or Charging Function (PCF) treatment or interact with other network functions.

NOTE 4:	There may be multiple 5GMSu AFs residing within the Trusted Media Functions entity each exposing one or more APIs.

The following interfaces are defined for 5G Media Uplink Streaming:

-	M1u (5GMSu Provisioning API): External API, exposed by the 5GMSu AF to provision the usage of the 5G Media Streaming Uplink Streaming system and to obtain feedback.

-	M2u (5GMSu Publish API): Optional External API exposed by the 5GMSu AS used when the 5GMSu AS in the trusted DN is selected to receive the content for the streaming service.

-	M3u: (Internal and NOT SPECIFIED): Internal API used to exchange information for content hosting on a 5GMSu AS within the trusted DN.

-	M4u (Media Upling Streaming APIs): APIs exposed by a 5GMSu AS to the Media Streamer to stream media content.

-	M5u (Media Session Handling API): APIs exposed by a 5GMSu AF to the Media Session Handler for media session handling, control and assistance that also include appropriate security mechanisms e.g. authorization and authentication.

-	M6u (UE Media Session Handling APIs): APIs exposed by a Media Session Handler to the 5GMSu Aware Application to make use of 5GMSu functions.

-	M7u (UE Media Streamer APIs): APIs exposed by a Media Streamer to the 5GMSu Aware Application and Media Session Handler to make use of the Media Streamer.

-	M8u: (EXTERNAL and NOT SPECIFIED): unspecified application interface, which is used for information exchange between the 5GMSu Aware application and the 5GMSu Application Provider, for example to provide service access information to the application.

NOTE 5:	Non-Standalone, Roaming, Non-3GPP Access and EPC-5GC interworking aspects are FFS.

### 4.3.2	UE Media Functions

Editor's Note: This still needs updates

Figure 4.3.2-1: UE 5G Media Uplink Streaming Functions

NOTE 1: A UE is a logical device which may correspond to the tethering of multiple physical devices or other types of realizations.

The following subfunctions are identified as part of a more detailed breakdown of the UE 5G Media Uplink Streaming Functions:

-	Operator App. or Third-party App.: application which is out of scope of the present specification and uses the UE 5G Media Uplink Streaming Functions and APIs.

-	Media Capturing(s): Devices such as video camera or microphone that transform an analog media signal into digital media data.

-	Media Encoder(s): Compress the media data.

-	Media Upstream Client: encapsulates encoded media data and pushes it upstream.

NOTE 2:	This maps to the FLUS media function in the FLUS Source specified in TS 26.238 Uplink Streaming stage 3 [5].

-	Assistance Handler: requests and/or subscribes to receive assistance information from one or more 5GMSu AF in support of the streaming process.

NOTE 3:	This is expected to map to the FLUS Assistance function specified in TS 26.238 Uplink Streaming stage 3 [5].

-	Media Remote Control Handler: receives control commands from a 5GMSu AF.

NOTE 4:	This is expected to map to the FLUS Remote control function specified in TS 26.238 Uplink Streaming stage 3 [5].

-	Media Configuration: configures the 5GMSu AS for uplink streaming reception.

NOTE 5:	This maps to the FLUS control function in the FLUS source specified in TS 26.238 Uplink Streaming stage 3 [5].

Here are the role of the different APIs of the UE 5G Media Uplink Streaming Functions:

-	M7u: Control Plane API used to configure, activate and stop the Media Capturing(s), Media Encoding(s) and Media Upstream Client functions.

-	M8u: Control Plane API used to control Media Configuration, Assistance Handler, Media Remote Control Handler functions.

-	MF1: User Plane API between Media Capturing(s) and Media Encoder(s) functions that carries Media data to be encoded. Can also be used between a Third-Party Uplink Streamer in an App. and the Media Encoder(s) function.

-	MF2: User Plane API between Media Encoder(s) and Media Upstream Client functions to carry encoded Media data. Can also be used between a Third-Party Uplink Streamer in an App. and the Media Upstream Client function.

## 4.4	Network Slicing

The 5GMS architecture offers the option to create an Ingest and Distribution configuration to external 5GMSd AS(s) through an ingest interface as described in clause 5.4. An Ingest and Distribution configuration is a 5GMS optimized configuration that is tailored to a 3rd party content or service provider to leverage the capabilities of the 5GS to achieve optimized delivery.

Upon a successful setup of an Ingest and Distribution configuration, the 5GMSd AF uses the NSaaS to creates or uses an existing corresponding network slice and assigns the Ingest and Distribution configuration domain as the network slice selection assistance information rule. This allows any traffic to and from the Ingest and Distribution configuration domain to be mapped to the correct network slice instance. The concept of Network Slice as a Service (NSaaS) is defined in [TS28.530]. NSaaS can be offered by an MNO to 3rd party providers in the form of a service. This service allows the providers to use the network slice instance as the end user and to manage the network slice instance via a management interface exposed by the MNO.

In turn, these providers offer their own services, e.g. OTT service, on top of the network slice instance obtained from the MNO. The NSaaS offered by the MNO can be characterized by certain properties (capabilities to satisfy service level requirements), e.g.

- 	radio access technology,

-	 bandwidth,

- 	end-to-end latency,

-	 reliability,

- 	guaranteed / non-guaranteed QoS,

- 	security level, etc.

The interface that is used for the creation and management of network slices is defined in [TS28.531] and the information elements are defined in [TS28.541].

The network (HPLMN) may provision the UE with Network Slice selection policy (NSSP) as part of the UE Route Selection Policy rules as described in TS 23.503 [4]. This ensures that the UE will request a particular network slice when establishing the PDU. Note that this mapping can also be established through associating an Application Identifier with the Ingest and Distribution configuration and the network slice, which in turn will be associated with a Packet Flow Id that includes the Ingest and Distribution configuration domain as a matching domain.

Once the PDU is established using the network slice instance that corresponds to the Ingest and Distribution configuration, the media distribution to the UE may start. The network slice instance may be provisioned to support processing and edge computing in addition to the appropriate QoS allocation. The Media AF is responsible to ensure appropriate traffic routing, e.g. request the routing of traffic to a local access to a Data Network (identified by a DNAI) that hosts the media processing compute instances.

# 5	Procedures for Downlink Streaming

## 5.1	General

The present clause describes the procedures for downlink streaming.

The downlink streaming procedures follow a general high-level workflow, starting from provisioning and ingest session preparation to the actual content streaming sessions. The ingest session refers to the time during which media content is uploaded into the 5GMSd AS. The provisioning session refers to the time, during which the 5GMS Client can access the media content. Interactions between the 5GMSd AF and the 5GMS Application Provider may occur at any time, while the Provisioning Session is active.

The 5GMS provisioning API allows selection of media session handling (M5d) and media streaming (M4d) options, including whether the media content is hosted on trusted 5GMSd ASs. The 5GMSd AF selects the M5d interface according to the provisioning option. The Media Session Handling interface can be used for consumption, QoE reporting, requesting different policy and charging treatments or other network assistance services.

When the content should be hosted by trusted Media ASs, then the 5GMSd AF selects the 5GMSd AS. Interactions between a 5GMSd AF and a 5GMSd AS (M3d interactions) take place for 5GMS Ingest (M2d) and Media Streaming (M4d) resource reservations. The 5GMSd AS allocates M2d and M4d resources and communicates resource identifiers back to the 5GMSd AF. The 5GMSd AF provides information about the provisioned resources (in form of resource identifiers) for Media Session Handling, 5GMS Ingest and Media Streaming to the 5GMS Application Provider. The resource identifiers for Media Session Handling and Media Streaming are needed by the 5GMSd Client to access the selected features.

When content hosting is provided by a 5GMSd AS in the external DN, then the M3d interface is not used and the 5GMSd AF does not provide 5GMS Ingest (M2d) and Media Streaming (M4d) resource reservations. M3d procedures are not standardized.

5GMS Clients can (in principle) start the media streaming as soon as the content is ingested by activating its unicast downlink streaming session. However, it may take some time until the media content is available for Media Streaming (via the Media Streaming API) or the distribution availability might be based on a provisioned schedule. The unicast downlink streaming session for a given UE (or "for each UE") is active from the time, at which the 5GMS Aware Application activates the reception of a streaming service until its termination.

The 5GMS Aware Application received application metadata from the 5GMS Application Provider before receiving the downlink streaming media. The application metadata contains service access information, which act as entry point for the 5GMS Client to start the downlink streaming session. The 5GMS Client may either receive a reference to that service access information or the full service access information from the 5GMS Application Provider.

Figure 5.1-1: High Level Procedure for downlink streaming

Steps

1.	The 5GMS Application Provider creates a Provisioning Session and starts provisioning the usage of the 5G Media Streaming System. During the establishment phase, the used features are negotiated and detailed configurations are exchanged. The 5GMS Application Provider receives service access information for M5d (Media Session Handling) and, when media content hosting is negotiated, service access information for M2d (Ingestion) and M4d (Media Streaming). This information is needed by the 5GMS Client to access the service. Depending on the provisioning, only a reference to the service access information may be used.

2.	When media hosting is provided, then there may be interactions between the 5GMSd AF and 5GMSd AS, e.g. to allocate 5GMS ingest and Media Distribution resources. The 5GMSd AS provides resource identifiers for the allocated resources to the 5GMSd AF, which then provides the information to the 5GMS Application Provider. The M3d procedures between Media AF and Media AS are not specified.

3.	The 5GMS Application Provider starts the Ingest Session by ingesting content. In case of live services, the content is continuously ingested. In case of on-demand streaming services, the content may be uploaded once and then updated later on. Note, a 5GMSd AS in the external Data Network may provide the content hosting.

4.	The 5GMS Application Provider provides the service announcement information to the 5GMS Aware Application. The service announcement includes either the whole service access information (i.e. details for Media Session Handling (M5d) and for Media Streaming access (M4d)) or a reference to the service access information. When a reference is included, the 5GMS Client fetches (in a later step) the services access information when needed.

5.	When the 5GMS Aware Application decides to activate the streaming service reception, the service access information (all or a reference) is provided to the 5GMS Client. Thep 5GMS Client activates the unicast downlink streaming session.

6.	When the 5GMS Client got a reference to the service access information, then it acquires the service access information. Note, pre-caching of service access information may also be supported by the 5GMS Client to speed up the activation of the service.

7.	The 5GMS Client uses the Media Session Handling API towards the 5GMSd AF. The Media Session Handling API is used for consumption, QoE reporting, requesting different policy and charging treatments or other.  network assistance services. The actual time of API usage depends on the feature and interactions that may be used during the media content reception.

8.	The 5GMS Client activates reception of the Media Content.

## 5.2	Baseline procedure for Unicast Media Downlink Streaming Session establishment

### 5.2.1	General

This clause describes the baseline procedure for establishment of a unicast media downlink streaming session. This baseline procedure assumes that the 5GMSd AF and the 5GMSd AS resides in the external DN and that there are no interactions between the 5GMSd AF and the 5G System.

### 5.2.2	Progressive Download of On-Demand Content

This baseline procedure describes the establishment of a unicast downlink streaming session. A streaming session may use 3GP File Format (Progressive Download), 3GP Time Text or other (potentially non-3GPP defined) formats.

Figure 5.2-1: High Level Procedure for progressive download for on-demand media

Prerequisite:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMSd Aware Application has received the service announcement from the 5GMS Application Provider.

Steps:

1: The 5GMSd Aware Application triggers the Service Announcement and Content Discovery procedure. The service announcement includes either the whole service access information (i.e. details for Media Session Handling (M5d) and for Media Streaming access (M4d)) or a reference to the service access information.

The Content Discovery procedure only involves the 5GMSd Aware Application and the 5GMSd Application Provider and hence is out of scope of the present document.

2: A Media Player Entry is selected.

3: The 5GMSd Aware Application triggers the Media Session handler to start the playback. The media player entry is provided to the Media Session handler.

4: When the 5GMS Aware Application has received a reference to the service access information (see step 1), the Media Session Handler interacts with the Media AF to acquire the whole service access information.

5: The Media Session Handler triggers the 5GMS Player to start the session.

6: The Media Player establishes the transport session.

7: The Media Player sends the request for the progressive download content.

8: The Media Player receives the initialization information of the progressive download content. The initialization information contains configuration parameters for reception of the media, and optionally also DRM information.

9: The Media Player configures the rendering pipeline for media playback.

10: The Media Player notifies the media session handler, providing the transport session information and some media content related information.

11: Optional: Media Player acquires a DRM License from an External AS.

12: The Media Player receives media content and puts it into the rendering pipeline.

13: The Media Player continuously receives and plays back the media content.

### 5.2.3	DASH Streaming

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.2-2: High Level Procedure for DASH content

Prerequisite:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMSd Aware Application has received the service announcement from the 5GMS Application Provider.

Steps:

1: The 5GMSd Aware Application triggers the Service Announcement and Content Discovery procedure. The Content Discovery procedure only involves the App and the external Application Server. The service announcement includes either the whole service access information (i.e. details for Media Session Handling (M5d) and for Media Streaming access (M4d)) or a reference to the service access information.

2: A media content item is selected.

3: The 5GMSd Aware Application triggers the Media Session handler to start media playback. The Media Player Entry is provided to the Media Session handler.

4: When the 5GMS Aware Application has received a reference to the service access information (See Step 1), the Media Session Handler interacts with the Media AF to acquire the whole service access information.

5: The Media Session Handler triggers the Media Player to start the session.

6: The Media Player establishes the transport session for acquiring the manifest referenced by the Media Player Entry.

7: The Media Player requests the manifest

8: The media access client receives the manifest

9: The Media Player processes the manifest. It determines for example the number of needed transport sessions for media acquisition. The 5GMS Player should be able to use the manifest information to initialize the media pipelines for each media stream. The manifest should also contain information to start the DRM client initialization, when DRM is used.

10: The Media Player notifies the media session handler about the manifest. The notification may contain parameters from the manifest

11: Optional: the Media Player acquires the necessary DRM information, for example a DRM License.

12: The Media Player configures the media rendering pipeline.

13: The Media Player establishes the necessary transport sessions for the content. For example, the media access client may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations.

14: The media session handler is notified about the completion of the rendering pipeline, providing, providing the transport session information and some media content related information.

15: The Media Player requests initialization information. The 5GMSP repeats this step for each required initialization segment.

16: The Media Player receives the initialization information.

17: The Media Player requests media segments according to the manifest.

18: the Media Player receives media segments and puts the information into the according media rendering pipeline.

19: Previous steps are repeated according to the manifest information.

## 5.3	Provisioning Session for Media Streaming

The present clause describes the procedure to provision the features using the 5GMS System. The 5GMS Application provider.

Figure 5.3-1: High Level Procedure for provisioning the 5GMS System for downlink streaming sessions

Steps

1.	The 5GMSd Application Provider discovers the address (URL) of the 5GMSd AF for Session Provisioning. The address maybe be the result of a manual SLA negotiation procedure or a self-on-boarding procedure.

2.	The 5GMSd Application Provider authenticates itself with the system.

3.	The 5GMSd Application Provider creates a provisioning session for a Downlink Streaming Session, providing its 5GMSd Application Provider ID as input. 5GMSd Application Provider queries the capabilities and authorized features.

4. 	The 5GMSd Application Provider provisions the unicast downlink streaming session. A set of authorized features are activated, such as QoE Reporting, Consumption Reporting, Dynamic Policy, Network Assistance and Content Hosting. When Content Hosting is provided by the 5GMS System, the 5GMS Application provider selects the ingest format.

5.	When content hosting is desired, the 5GMSd AF interacts with the 5GMSd AS to allocate resources and receive the address for 5GMSd AS (M2d address). The 5GMSd AF selects the desired ingest format.

6.	The 5GMSd AF compiles the service access information. The Service Access Information contain access details and options such as M5d (Media Session Handling) Addresses for Consumption Reporting, QoE Reporting, Dynamic policy, Network Assistance, etc. When content hosting is provisioned, then also M4d (Media Streaming) information such as the DASH MPD is included.

7.	The 5GMSd AF provides the results to the 5GMSd Application Provider.

- When the 5GMSd Application provider selected the full service access information, then the results are provided in form or addresses and configurations for M2d (Ingest), M5d (Media Session Handling) and M4d (Media Streaming)

- When the 5GMSd Application Provider delegated the service access information handling to the 5GMS System, then a reference to the service access information (e.g. an URL) is provided. The Media Session Handler fetches the full service access information later from the 5GMSd AF.

8.	The 5GMSd Application provider can start ingesting content, using the M2d API address.

9.	The 5GMSd Application Provider executes Service Announcement and updates the UEs.

Optional

10.	The 5GMSd Application Provider may update the provisioning session.

According Provisioning

11.	The 5GMSd AF may send event related or periodic notifications to the 5GMS Application Provider.

According to Schedule or upon request

12. 	The 5GMSd Application Provider may manually terminate the Provisioning Session (at any time). All associated resources are released. Content may be removed from the 5GMSd AS. The 5GMSd Provider may configure a schedule for Provisioning Session termination.

13.	The 5GMSd AF sends a notification upon Provisioning Session terminations.

## 5.4	Media Ingest for Downlink Streaming

The 5G Media Streaming architecture defines a common interface for content ingest for downlink media streaming over 5G. The control part of the ingest interface may be performed through the NEF. After the ingest, the content is accessible from the 5GMSd AS through a new location identifier.

The interface supports the ingest of the following types of content:

-	Live streaming content

-	On demand streaming content

-	Static files such as images, scene description files, etc.

The interface provides an API that allows a 5GMSd AS to create/update/delete an Ingest and Distribution configuration. An Ingest and Distribution configuration contains all the parameters and configurations to a particular content ingest and distribution setup.

NOTE 1:	In the current version of the present document, the ingest interface only supports Unicast downlink streaming.

The media ingest procedure is as follows:

Figure 5.4-1: Media Ingest procedure

The steps are explained as follows:

1:	Initialization: the external content provider discovers the entry point, gets authorization and authentication. The procedures should leverage existing stage 3 definitions for northbound APIs e.g. xMB and CAPIF.

2:	Create Ingest and Distribution configuration: the external 5GMSd AS creates a new Ingest and Distribution configuration for its content through the MNO's 5G system. It associates it with a domain name, supplies the certificate for HTTPS access to the content, sets the caching rules per media type, the distribution area, distribution protocol, logging information, register notifications, etc. Upon successful configuration, the 5GMSd AF will respond with the Ingest and Distribution configuration ID, and the location of the 5GMSd AS to which to send the content (if using the push mode).

3:	Provision 5GMSd AS(s): The 5GMSd AF configures the related 5GMSd AS(s) to prepare for media ingest for that particular Ingest and Distribution configuration. This step may involve instructing the 5GMSd AS(s) to set the appropriate caching rules.

4:	Update configuration information: the 5GMSd AF communicates the Ingest and Distribution configuration of the 5GMSd AS(s) to the External Media Application Servers for further Media push or pull.

5:	Media data push or pull: The 5GMSd AS(s) may start pulling or receiving (if using push mode).

NOTE 2:	Pull of Media content from the external 5GMSd AS(s) may be triggered by a request from the 5MGS Client.

## 5.5	Metrics collection and reporting

Metrics collection and reporting can be done in different ways, depending on the relation between the Service Provider and the Network Operator. The following figures shows simplified signalling examples for two different use-cases.

In the first use-case, shown in figure 5.5-1 below, the MNO is controlling the metric reporting, using the out-of-band configuration and reporting option. Note that in this scenario the App is not actively involved in collection or reporting of the metrics, so metrics can be collected for any App which uses the trusted media functions in the UE, even when the media server is not under control of the MNO.

Figure 5.5-1: Metrics collection using the out-of-band 5G control plane option

The different steps are explained below:

1: Overall metrics configuration is done on the network level, for instance defining which geographical areas that shall have metrics collection active.

2: The metric configuration(s) is sent from the OAM to the RAN, which does not forward that information to the UE at this stage.

3: Time passes, and it assumes that the UE moves around during that period.

4: The UE enters an area (cell, location area, etc.,) which is inside the geographical constraint. This is discovered by the RAN, and it now needs to activate metric reporting for the UE.

5: The actual metric configuration is sent from the RAN to the metrics collection reporting function in the UE, via the 5G control plane.

6: Additional time passes, and it is assumed that the UE has a metrics configuration, but no streaming session has started.

7: A streaming session is started.

8: The session setup is done in conjunction with signalling transactions (not shown here). No metrics configuration is done during this setup.

9: Media is delivered and rendered, and...

10: ...more media is delivered...

11: Metrics are collected during the session according to the configuration, and reported via the 5G control plane with the frequency defined in the configuration.

12: The session continues...

13: more media is delivered...

14: One or more additional metrics collection and reporting transaction(s) are performed, until the session ends.

15: Time passes during which no streaming session is active.

16 to 23: Another session is started, and as the UE is still inside the collection area, this session will also be collecting metrics in the same way as in steps 7-14.

24: Time passes, the UE moves around.

25: The UE leaves the geographical area specified by the metrics configuration.

26: RAN sends metrics (de)configuration to the UE, to stop future metrics collection.

The second use-case, shown in figure 5.5-2 below, illustrates a scenario where the metrics collection is configured in-band. Thus together with the media there are metadata which describe how metrics shall be collected, and how they shall be reported. The Media OAM and Media /AS functions can both be either trusted or untrusted (but not mixed).

Figure 5.5-2: Metrics collection using the in-band metadata option

The different steps are explained below:

1: The content servers are set up, including any metrics configuration.

2: If the Media OAM and 5GMSd AS are located in a trusted domain, metadata regarding the metrics collection and reporting are sent to the AS. The procedure for configuring the 5GMSd AS, when the Media OAM and 5GMSd AS reside in an untrusted domain, is unspecified.

3: Time passes during which no streaming session is active...

4: A streaming session starts.

5: The App and the session handler initializes a new session.

6: The media access client requests any metadata, from the 5GMSd AS, which is needed for downloading the media content.

7: Metadata is returned to the access client and also forwarded to the App.

8: The metrics parts of the metadata is sent by the App to the metrics collection and reporting function.

9: The streaming session is running.

10: Media is delivered and rendered, and...

11: more media is delivered...

12: Metrics are collected during the session according to the configuration, and reported with the frequency defined in the configuration. The reporting method is defined in the configuration, for instance HTTP upload.

13: The session continues...

14: more media is delivered...

15: Occurrence(s) of additional metrics reporting, until the session ends.

## 5.6	Consumption reporting

### 5.6.1	Consumption reporting procedures

Figure 5.6-1: Consumption reporting

Steps:

The first phase is the initialisation phase:

1: The App is started.

2: A media content item is selected.

3: The App triggers the Media Session Handler to start playback. The media player entry is provided.

4: The 5GMSd AF initializes the parameters for the consumption reporting configuration (e.g. frequency).

5: The Media Session Handler triggers the consumption reporting.

6: The Media Session Handler starts the 5GMS Player with the Entry URL.

The second phase is media playback:

The user preferences may be changed:

7: The App selects/changes the user preferences.

8: The 5GMS Player transmits user preferences to the consumption reporting.

When Media is playing, the consumption reporting parameters may be updated.

9:The 5GMSd AF updates the parameters for the consumption reporting.

10: The Media Session Handler triggers an update to the Consumption reporting.

When Media is playing

11: The 5GMS player regularly accesses to the media content.

12: In case of changes to the consumed media properties, the 5GMS Player transmits the changes to the consumption reporting function.

13: The consumption reporting regularly sends report(s) to the 5GMSd AF.

The last phase is to stop the media:

14: The App triggers the Media Session Handler to stop the video.

15: The Media Session Handler stops the Consumption reporting.

16: The consumption reporting may send the last report(s) to the 5GMSd AF.

17: The Media Session Handler stops the 5GMS player.

### 5.6.2	Consumption reporting parameters

Table 5.6.2-1 describes the parameters used in step 4 of clause 5.6.1.

Table 5.6.2-1: Parameters for consumption reporting configuration

| Parameters | Description |
| --- | --- |
| Report interval | Identify the interval when the consumption report messages are sent by the 5GMS Media Session Handler. |
| Server address | A list of addresses where the consumption reporting messages are sent by the 5GMS Media Session Handler. |
| Location reporting | Identify whether the 5GMS Media Session Handler provides the location data to the Media AF (in case of MNO or trusted third parties) |

Table 5.6.2-2 describes the parameters used in steps 12 or 15 of clause 5.6.1.

Table 5.6.2-2: Consumption reporting parameters

| Parameters | Description |
| --- | --- |
| Media player entry pointer | Identify the Media player entry. In case of DASH, the media player entry pointer can be a URL of the MPD. |
| Consumption reporting client ID | Identify the identifier of the UE that consumes the data. [The value of this identifier should be global unique ID such as MSISDN] |
| Location | Identify the UE location. This parameter is only used when the location reporting is enabled for the UE or for the Downlink Streaming session with a condition that the UE allows to share its location within operator’s trust domain. [The value of this value should be Cell ID] |
| Media consumed | Identify the media consumed. In case of DASH, the AdaptationSet@id may be used. |
| Start time | Define the time when the consumption starts. |
| Duration | Define the duration of the consumption of the media consumed. |

## 5.7	Establish a Unicast Media Downlink Streaming Session with 5GMSd AF interactions for dynamic policy updates

### 5.7.1	General

This clause describes the establishment of a unicast media downlink streaming session with 5GMSdAF interactions for dynamic policy update, as described in Steps 4 to 8 of Clause 5.1. The dynamic policy API allows separate handling of application flows within the same PDU Session. The Media Session Handler provides a Policy type indication together with a Flow Description, when requesting a dynamic policy. An example Policy type indication value is “background_data”.

The Media Session Handler provides needed information to the 5GMSd AF, so that the 5GMSd AF can trigger a dynamic PCC rule update using the NEF. The 5GMSd AF may trigger for example the AF session with required QoS procedure (See TS 23.502 [3], Clause 4.15.6.6) for updating QoS related dynamic PCC rules. For changing charging related rules, the 5GMSd AF may trigger the Change the chargeable party during the session procedure (See TS 23.502 [3], Clause 4.15.6.5).

### 5.7.2	Progressive Download of On-Demand Content

This procedure describes the establishment of a unicast media downlink streaming session with 5GMSd AF interactions to dynamic policy updates. A streaming session may use 3GP File Format (Progressive Download), 3GP Time Text or other (potentially non-3GPP defined) formats.

Figure 5.7-1: High Level Procedure for progressive download for on-demand media

Prerequisite:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMSd Aware Application has received the service announcement from the 5GMSd Application Provider

- 	The service announcement information contains detailed information for the policy related interactions. Specifically, the information includes an identifier of the 5GMS Application Provider and a list of authorized Policy Type indications for that specific application.

-	The 5GMSd Application Provider has agreed an SLA with the operator defining the possible QoS levels and their charging rates (See TS 23.503, Clause 6.1.3.22). The PCF is configured with the corresponding QoS parameters and charging information.

-	The service announcement information includes URLs for the 5GMSd AF and information around the possible Policy type indication.

Steps:

1: The 5GMSd Aware Application triggers the Service Announcement and Content Discovery procedure. The service announcement includes either the whole service access information (i.e. details for Media Session Handling (M5d) and for Media Streaming access (M4d)) or a reference to the service access information.

The Content Discovery procedure only involves the App and the external Application Server and hence is out of scope of the present document.

2: A Media Player Entry is selected.

3: The application triggers the Media Player and provides the entry point for the progressive download content

4: The Media Player establishes the transport session, e.g. the TCP connection.

5: The Media Player retrieves the header for the progressive download content. The information is used to determine the characteristics of the content as well as any required DRM licenses.

7: The Media Player notifies the Media Session Handler about the starting media session. It provides information about the characteristics of the content to be used for the network assistance.

8: The media session handler establishes a media session with the 5GMSd AF. The Media Session Handler creates an application instances for the media session. The Media Session Handler provides the 5GMS Application Provider Id (and optionally additional authentication information) to relate the application instance to authorized features.

9:	The Media Session Handler requests to apply a dynamic policy to the media session. The request includes at least the Flow description(s) and the Policy Type indication, which should be applied to the described transport session. The 5GMSd AF uses the Policy Type indication to derive the related procedure and to identify the related network function.

10: This step applies when the 5GMSd AF resides in the trusted data network. Depending on the Policy type indication, the step is executed:

- When the Policy type indication relates to QoS, the 5GMSd AF may either directly interact with the PCF or may use a NEF service

- when directly interacting with the PCF, the 5GMSd AF uses the Npcf_PolicyAuthorization Service as defined in TS 23.502, Clause 5.2.5.3).

- when interacting via the NEF with the PCF, continue with step 11.

- When the Policy type indication relates to a different charging scheme, the 5GMSd AF may either directly interact with the PCF or may use a NEF service

- when directly interacting with the PCF, the 5GMSd AF uses the Npcf_PolicyAuthorization Service as defined in TS 23.502, Clause 5.2.5.3).

- when interacting via the NEF with the PCF, continue at step 11.

11: This step applies when the 5GMSd AF resides in the external data network. Depending on the Policy type indication, the step is executed:

- When the Policy type indication relates to QoS, the 5GMSd AF may use the Nnef_AFsessionWithQoS service as defined in TS 23.502, Clause 5.2.6.9. The complete call flow is described in TS 23.502, Clause 4.15.6.6.

- When the Policy type indication relates to a different charging scheme, the 5GMSd AF may use the Nnef_ChargeableParty service as defined in TS 23.502, Clause 5.2.6.8. The complete call flow is described in TS 23.502, Clause 4.15.6.4 and Clause 4.15.6.5. provides the 5GMSA Provider Id (AF Identifier), Flow description (as provided by the Media Session Handler in Step 9), Sponsor Information (values, based on SLA negotiation), Background Data Transfer Reference ID.

11: The Media Player sends the request for the progressive download content.

12: The Media Player receives the initialization information of the progressive download content. The initialization information contains configuration parameters for reception of the media, and optionally also DRM information.

13: The Media Player configures the rendering pipeline for media playback.

14: The Media Player notifies the media session handler, providing the transport session information and some media content related information.

15: Optional: Media Player acquires a DRM License from an External AS.

16: The Media Player receives media content and puts it into the rendering pipeline.

17: The Media Player continuously receives and plays back the media content.

### 5.7.3	DASH Streaming

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.7-2: High Level Procedure for DASH content

Prerequisite:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMSd Aware Application has received the service announcement from the 5GMSd Application Provider.

-	The service announcement information contains detailed information for the policy related interactions. Specifically, the information includes an identifier of the 5GMSd Application Provider and a list of authorized Policy Type indications for that specific application.

-	The 5GMSd Application Provider has agreed an SLA with the operator defining the possible QoS levels and their charging rates (See TS 23.503, Clause 6.1.3.22). The PCF is configured with the corresponding QoS parameters and charging information.

-	The service announcement information includes URLs for the 5GMSd AF and information around the possible Policy type indication.

Steps:

1: The 5GMSd Aware Application triggers the Service Announcement and Content Discovery procedure. The Content Discovery procedure only involves the App and the external Application Server. The service announcement includes either the whole service access information (i.e. details for Media Session Handling (M5d) and for Media Streaming access (M4d)) or a reference to the service access information.

2: A media content item is selected.

3: The 5GMSd Aware Application triggers the playback by launching the Media Player.

4: The Media Player

notifies the Media Session Handler about the starting media session and provides the entry point and other information to the Media Session Handler.

5: The Media Session Handler interacts with the 5GMSd AF to acquire additionalservice access information.

6: The Media Player establishes the transport session for acquiring the manifest referenced by the Media Player Entry.

7: The MediaPlayer requests the manifest

8: The media access client receives the manifest

9: The MediaPlayer processes the manifest. It determines for example the number of needed transport sessions for media acquisition. The 5GMS Player should be able to use the manifest information to initialize the media pipelines for each media stream. The manifest should also contain information to start the DRM client initialization, when DRM is used.

10: The MediaPlayer notifies the media session handler about the manifest. The notification may contain parameters from the manifest

11:The Media Session handler establishes session with the 5GMSd AF. The Media Session Handler creates an application instances for the media session. The Media Session Handler provides the 5GMS Application Id or other filtering criteria such as the FQDN (and optionally additional authentication information) to relate the application instance to authorized features.

12: Optional: the Media Player acquires the necessary DRM information, for example a DRM License.

13: The Media Player configures the media rendering pipeline.

14: The Media Player establishes the necessary transport sessions for the content. For example, the media access client may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations.

15: The media session handler is notified about the established transport sessions for the content by providing the description of the application flows e.g. 5-Tuples.

16:	The Media Session Handler requests to apply a dynamic policy to the media session. The request includes at least the Flow description(s)  and the Policy Type indication, which should be applied to the described transport session. The 5GMSd AF uses the Policy Type indication to derive the related procedure and to identify the related network function.

17: This step applies when the 5GMSd AF resides in the trusted data network. Depending on the Policy type indication, the step is executed:

- When the Policy type indication relates to QoS, the 5GMSd AF may either directly interact with the PCF or may use a NEF service

- when directly interacting with the PCF, the 5GMSd AF uses the Npcf_PolicyAuthorization Service as defined in TS 23.502, Clause 5.2.5.3).

- when interacting via the NEF with the PCF, continue at step 18.

- When the Policy type indication relates to a different charging scheme, the 5GMSd AF may either directly interact with the PCF or may use a NEF service

- when directly interacting with the PCF, the 5GMSd AF uses the Npcf_PolicyAuthorization Service as defined in TS 23.502, Clause 5.2.5.3).

- when interacting via the NEF with the PCF, continue at step 18..

18: This step applies when the 5GMSd AF resides in the external data network. Depending on the Policy type indication, the step is executed:

- When the Policy type indication relates to QoS, the 5GMSd AF may use the Nnef_AFsessionWithQoS service as defined in TS 23.502, Clause 5.2.6.9. The complete call flow is described in TS 23.502, Clause 4.15.6.6.

- When the Policy type indication relates to a different charging scheme, the 5GMSd AF may use the Nnef_ChargeableParty service as defined in TS 23.502, Clause 5.2.6.8. The complete call flow is described in TS 23.502, Clause 4.15.6.4 and Clause 4.15.6.5. provides the 5GMSA Provider Id (AF Identifier), Flow description (as provided by the Media Session Handler in Step 9), Sponsor Information (values, based on SLA negotiation), Background Data Transfer Reference ID.

19: The Media Player requests initialization information. The 5GMSP repeats this step for each required initialization segment.

20: The Media Player receives the initialization information.

21: The Media Player requests media segments according to the manifest.

22: the Media Player receives media segments and puts the information into the according media rendering pipeline.

23: Previous steps are repeated according to the manifest information.

## 5.8	Dynamic Policy based on Network Slicing for Downlink Streaming

### 5.8.1	Flow Chart

The service provider requests the assignment of one or more network slices for the distribution of the service. The service provider indicates the desired network slice features that correspond to the service information. Upon successful assignment of the network slices for the service, the Media AF will respond with the list of allowed S-NSSAIs to the service provider.

Figure 5.8.1-1 is the flowchart diagram for this procedure.

Figure 5.8.1-1 Dynamic Policy based on Network Slicing for Downlink Streaming

The steps are as follows:

- The initial provision step consists of the following sub-steps, with steps b, c, d, and g as exemplary and out of scope of this specification:

  - The external 5GMSA Application provider requests the creation of a new Ingest session. The 5GMSA Application provider indicates the anticipated operation points for the service. An operation point consists of the bandwidth and latency requirements, as well as any other parameters that may influence the policy for the sessions of this application (e.g. the charging profile, coverage area, route selection information,…).

  - The NSMF is checked for suitable network slice instances for this provisioning session. If suitable network slice instance(s) are available, the Media AF selects the one(s) to use. Otherwise, the media AF uses the interfaces defined in [7] to request the creation of new network slice instance(s)  and provisions them for the new distribution configuration..

  - The newly created network slice instances are provisioned.

  - The NS Management function confirms the creation of the new network slice instance(s) to the Media AF and provides the list of S-NSSAI(s) with their corresponding parameters.

  - The Ingest and Distribution Configuration is updated with the information about the network slices.

  - The Media AF confirms successful setup of the Ingest and Distribution Configuration to the Application provider as described in section 5.4.

  - The PCF updates the URSP rules on the target set of UE(s), e.g. based on geographical service area of the offered application as described in 23.502 section 4.2.4.3, 23.503 section 6.6.2.2, and 24.501 Annex D.2.1.

- The UE starts a streaming session by fetching the entry point to the service

- The Media Player notifies the Media Session Hanlder about the upcoming Media Session and may optionally request specific network assistance for that session.

- The UE may retrieve information from the Media AF to assist with the route selection for the target operation point of the session. The Media Session Handler may get information about the target operation point from the media player as described in step 3.

- The 5GMSA Player and Session Handler in the UE perform the route selection procedure. The UE, such as the traffic descriptors, the domain descriptors, or the Application descriptors. The UE will use the matching filter to retrieve the matching Route Selection descriptor, which provides the DNN, and the S-NSSAI(s) as defined in 23.503 [4] section 6.6.2 and 23.501 [2] section 5.15.5.3 .

- The UE reuses an existing PDU session with the identified S-NSSAI (as defined in [3] section 4.3.2 and 4.3.3), or requests the establishment or modification of a PDU session with the identified parameters, if one doesn’t exist already. Upon successful PDU session establishment, the AMF notifies the UE of the assigned QoS profile ([2] section 5.7.1.2) for the PDU session

- The streaming of the media content at the target operation point starts.

### 5.8.2	Support for Multiple Operation Points

The service provider may define a set of operation points for the service that it offers. The Media AF maps each potential operation point into a QoS profile that matches the required resources for receiving the service at that operation point. The AF may influence the QoS policy for the media session, potentially after perfoming session binding, using the Npcf_PolicyAuthorization  operation as described in [23.502] section 5.2.5.3. The AF provides the S-NSSAI together with the QoS parameters to the PCF as part of the request.

The selection/establishment of the correct PDU session is performed based on information from the Player about the  the Media AF, the Session Handler, and the 5GMSd Player.

# 6	Procedures for Uplink Streaming

Editor's Note: This still needs updates

## 6.1	General

The procedures for uplink streaming allow a system user to create, modify, establish and delete sessions. Media sessions exists between a UE and a UL 5GMSu AS. The term "Sink Configuration" refers to the provisioned parameters of an UL 5GMSu AS. The term "Source Session" refers to the provisioned parameters on the UE.

The uplink streaming procedures follow a general high-level workflow, starting from provisioning to the actual uplink streaming sessions. The egset session refers to the time during which media content is upstreamed into the 5GMSu AS. The provisioning session refers to the time, during which the 5GMSu client can upstream the media content. Interactions between the 5GMSu AF and the 5GMSu Application Provider may occur at any time, while the Provisioning Session is active.

The 5GMSu Provisioning API allows selection of Media Session Handling (M5u) and Uplink Streaming (M4u) options, including whether the media content is published to a trusted 5GMSu ASs. The 5GMSu AF selects the M5u interface according to the provisioning option. The Media Session Handling interface can be used for remote control, QoE reporting, requesting different policy and charging treatments or other network assistance services.

When the 5GMSu AF and AS are in the same DN, then the 5GMSu AF selects the 5GMSu AS. Interactions between a 5GMSu AF and a 5GMSu AS (M3u interactions) take place for 5GMS Egest (M2u) and Uplink Streaming (M4u) resource reservations. The 5GMSu AS allocates M2u and M4u resources and communicates resource identifiers back to the 5GMSu AF. The 5GMSu AF provides information about the provisioned resources (in form of resource identifiers) for Media Session Handling, 5GMS Egest and Uplink Streaming to the 5GMSu Application Provider. The resource identifiers for Media Session Handling and Uplink Streaming are needed by the 5GMSu Client to access the selected features.

When publishing function is provided by a 5GMSu AS in the external DN, then the M3u interface is not used and the 5GMSu AF does not provide 5GMS Egest (M2u) and Uplink Streaming (M4u) resource reservations. M3u procedures are not standardized.

5GMSu Client can (in principle) start the uplink streaming by activating its uplink streaming session. The uplink streaming session for a given UE (or "for each UE") is active from the time, at which the 5GMSu Aware Application activates the transmission of an uplink streaming service until its termination.

The 5GMSu Aware Application received application metadata from the 5GMSu Application Provider before transmitting the uplink streaming media. The application metadata contains service access information, which act as entry point for the 5GMSu Client to start the uplink streaming session. The 5GMSu Client may either receive the full service access information from the 5GMSu Application Provider or instructions for a remote control session. When remote control is activated, then the 5GMSu Client is remote configured and controlled by a 5GMSu AF.

Figure 5.1-1: High Level Procedure for uplink streaming

Steps

1.	The 5GMSu Application Provider creates a Provisioning Session and starts provisioning the usage of the 5G Media Streaming System. During the establishment phase, the used features are negotiated and detailed configurations are exchanged. The 5GMSu Application Provider receives service access information for M5u (Media Session Handling) and, when media content reception is negotiated, service access information for M2u (Egest) and M4d (Uplink Streaming). This information is needed by the 5GMSu Client to access the service. Depending on the provisioning, only a reference to the service access information may be used.

2.	When 5GMSu AF and 5GMSu AS are in the same Data Network, then there may be interactions between the 5GMSu AF and 5GMSu AS, e.g. to allocate 5GMSu egest and uplink streaming resources. The 5GMSu AS provides resource identifiers for the allocated resources to the 5GMSu AF, which then provides the information to the 5GMSu Application Provider. The M3u procedures between 5GMSu AF and 5GMSu AS are not specified.

3.	The 5GMSu Application Provider provides the service announcement information to the 5GMSu Aware Application. The service announcement include either the whole service access information (i.e. details for Media Session Handling (M5u) and for Media Streaming access (M4u)) or a remote configuration and control address (URL). When a remote configuration and control is selected, the 5GMSu Client gets the services access information in a later step.

4.	The 5GMSu Aware Application configures and starts the 5GMSu Client.

5.	When the 5GMSu Aware Application decides to activate the streaming service transmission, the service access information is provided to the 5GMSu Client. When remote configuration and control is activated, then the the 5GMSu AF configures and controls the 5GMSu Client remotely.

6.	Depending on the configurations, the 5GMSu Client uses the Media Session Handling API towards the 5GMSu AF. The Media Session Handling API is used for requesting different policy and charging treatments or other network assistance services.

7.	The 5GMSu Client starts the Publishing Session by activating the uplink streaming session.

8.	The 5GMSu AS Publishes the content towards the 5GMSu Application Provider.

## 6.2	Preparing for Uplink Streaming

### 6.2.1	Introduction

Before streaming media data in uplink direction, a set of parameters are provisioned at the 5GMSu AF / AS and at the UE. On the UE side, the M8u API is used for local provisioning and the M5u API for remote control. On the 5GMSu AF / AS, the M1u API is used for session provisioning.

### 6.2.2	Sink Configuration at the 5GMSu AF / AS

The procedure allows the configuration of an Uplink 5GMSu AS for media reception. A Sink Session is created and updated. One or more media sessions are associated with a sink session.

Figure 6.2.2-1: Sink Configuration states

The states describe the Sink Configuration and the relation to the media session:

-	Sink Configuration created, no media session(s) active: The Sink Configuration entry is created at the UL 5GMSu AF and may be updated / modified before any media session(s) start.

-	Sink Configuration Provisioned, media session(s) active: The UL 5GMSu AS is receiving UL media data. Some properties of the Sink Configuration may be updated while receiving media data.

-	Sink Configuration Provisioned, media session(s) paused: The UL 5GMSu AS is temporarily not receiving UL media data. Some properties of the Sink Configuration may be updated.

-	Sink Configuration Provisioned, media session(s) terminated: The UL 5GMSu AS is not receiving UL media data anymore.

### 6.2.3	Source Configuration at the UE

The procedure allows the (remote) configuration of UE for media transmission. When remote control is used, then the application authorizes remote control e.g. by providing the remote control server address. It is assumed, that a UE can have only a single Source Configuration at a time, which can be remotely configured. One or more media sessions are associated with the Source Configuration.

Figure 6.2.3-1: Source Configuration states

The states describe the Source Configuration and the relation to the media session.

-	Source Configuration created, no media session(s) active: The Source Configuration entry is created and may be updated / modified before any media session(s) start. When remote control is enabled, then the connection to the remote control function is established.

-	Source Configuration Provisioned, media session(s) active: The Media Upstream Client is sending UL media data. Some properties of the Source Configuration may be updated while sending media data.

-	Source Configuration Provisioned, media session(s) paused: The Media Upstream Client is temporarily not sending UL media data. Some properties of the Source Configuration may be updated.

-	Source Configuration Provisioned, media session(s) terminated: The Media Upstream Client is not receiving UL media data anymore.

## 6.3	Establishment of an Uplink Streaming Session

The procedure allows an 5G Media Uplink Streamer to establish a uplink streaming session with a 5GMSu AS.

Figure 6.3-1: Uplink Streaming Session Establishment

Steps:

1:	During provisioning, the 5G Media Uplink Streamer, the 5GMSu AS is provisioned with basic information, such as the 5GMSu AF and 5GMSu AS addresses. For some devices, only the remote control information is provisioned and all additional parameters are retrieved from Remote Controller.

2:	The App starts the uplink media streaming.

3:	The App instructs the 5GMSu to start providing a Media Streaming Entry.

When remote control is provisioning in step 1:

4:	The transport session is established for the remote control.

5:	The 5GMSu starts waiting for incoming remote control commands.

6:	The 5GMSu receives a remote control command to start upstreaming (either now or with a timestamp). The Remote control command may contain details around the 5GMSu AS, etc.

7:	The 5GMSu establishes the transport session.

8:	The 5GMSu establishes the media streaming session.

When client assistance is provisioned:

9: The 5GMSu establishes the assistance channel to the provisioned 5GMSu AF(s).

When server assistance is desired (e.g. for QoS or charging).

10: The 5GMSu AS establishes a assistance session with the 5GMSu AF.

## 6.4	Termination of an Uplink Streaming Session

The procedure defines the termination of an uplink streaming session.

Figure 6.4-1: Uplink Streaming Session Teardown

Steps:

1:	An Uplink streaming session is active and should be terminated

When the App terminates the session, e.g. triggered by user input:

2:	The App sends a Stop commands to the 5G Media Uplink Streamer

When remote control is used and the remote control session is established:

3:	The 5G Media Uplink Streamer receives a stop command.

4: The 5G Media Uplink Streamer stops the capturing process

5: The uplink media streaming process is terminated

6: The transport session is released

When client assistance was established:

7: The 5GMSu AF is notified that the media uplink streaming session is terminated

When server assistance was established:

8: The 5GMSu AS is notified the 5GMSu AF that the media uplink streaming session is terminated

## 6.5	Providing Network Assistance

The procedure defines the network assistance from a 5GMSu AF.

Figure 6.5-1: Providing Network Assistance

Steps:

1:	An Uplink streaming session is active. The Assistance Session is established with the 5GMSu AF.

2:	The 5GMSu AF determines the need to send an assistance message.

3:	The assistance message is sent.

4: The 5G Media Uplink Streamer takes an appropriate action to the message.

## 6.6	Providing Remote Control

The procedure defines the remote control from a 5GMSu AF. Note that the 5GMSu AF for remote control may be deployed as a network based function or a UE based function.

Figure 6.6-1: Uplink Streaming Session Establishment

Steps:

1:	An Uplink streaming session is active. The Remote Control Session is established with the 5GMSu AF.

2:	The 5GMSu AF for Remote Control determines the need to send a remote control message.

3:	The remote control message is sent.

4: The 5G Media Uplink Streamer takes the defined action to the message.

## 6.7	RAN Signaling based Support for Uplink Network Assistance

The diagram in figure 6.7-1 depicts an uplink streaming architecture where RAN signaling is employed to support uplink network assistance functionality.

The RAN in the architecture contains control plane and user plane entities that interact with peer control and user plane entities in the UE, in the request/response for boost of the uplink streaming data rate, and subsequent media transport. The user plane functionality as shown in figure 6.7-1 (solid line going from UE to RAN to 5GMSu AS) illustrates the scheduling/pass-through functionality associated with user plane communications, governed by the RAN, as result of the uplink assistance messaging over the control plane (shown by dotted line between the UE and RAN). It is assumed in this case that application-level network assistance, i.e., as represented by the "NA AF sub-function of the "UL 5GMSu AF" is not utilized.

It should be noted that although the NA AF is not utilized when RAN signaling based network assistance is performed, it is assumed that there is a higher level network entity which coordinates and tracks network assistance performed using RAN signaling vs. application signaling for individual UEs.

Figure 6.7-1: RAN Signaling based Uplink Network Assistance

# 7	5GMS Network Media Processing

## 7.1	General

A service or content provider may request media processing to be performed on the media data. This can be instantiated as part of the Uplink or Downlink streaming. For uplink, FLUS TS 26.238 [5] provides measures to describe the required media processing as part of the FLUS session creation request. For downlink media distribution, the content provider may wish to execute some media processing functions within the MNO network infrastructure.

The media processing is performed by a set of 5GMSu AS(s), which may be combined together to build complex media processing workflows. A 5GMSu AF coordinates the media processing and ensures that the appropriate QoS and traffic handling for the session is provided.

## 7.2	Media Processing Procedures for Downlink

As part of setting up a Ingest and Distribution configuration for downlink media streaming, an External 5GMSu application server may request custom processing to be performed.

The following processing operations may be available:

-	Adaptive Bit Rate (ABR) Encoding, Encryption and Encapsulation

-	Manifest (e.g. MPD) Generator and Segment (e.g. DASH) Packager

-	Content Replacement (e.g. Ad insertion, blackouts, regional content):

-	Manifest (e.g. MPD) modification

-	App Server: other content enrichment functions such as Closed Caption insertion, object detection, content filtering, etc.

The procedures are as follows:

Figure 7.2-1: Media Processing Procedures for Downlink

The steps are as follows:

1.	Upon setting up an Ingest and Distribution configuration, the external Media application server requests media processing to be setup. The external Media application server provides a description of the type and placement of the processing. The MNO may only allow a shortlisted set of media processing functions to be used by an external Media application server.

2.	The 5GMSu AF provisions a selected set of 5GMSu AS(s) to fulfil the requested media processing in the appropriate placement. Depending on the configuration one or multiple AS(s) may be involved.

3.	The 5GMSu AS(s) confirm successful provisioning to the 5GMSu AF.

4.	The 5GMSu AF confirms the successful creation of the Ingest and Distribution configuration with the requested media processing to the external Media application server.

5.	A 5GMS Client sends a request for media content using the provisioned Ingest and Distribution configuration as explained in the media ingest procedures.

6.	The 5GMSu AS(s) fetch(es) the media from the external Media application server.

7.	The 5GMSu AS(s) process(es) it based on the provisioned media processing workflow.

8.	The 5GMSu AS(s) respond(s) to the UE with the requested media after successful media processing.

Different variants of these procedures may be possible, depending on the type of processing, the placement of the processing, and the characteristics of the Ingest and Distribution configuration.

## 7.3	Media Processing Procedures for Uplink

FLUS TS 26.238 [5] allows for the provisioning of media processing by the FLUS Source to the FLUS Sink. In such a scenario, the FLUS Sink will consist of a FLUS 5GMSu AF and a FLUS 5GMSu AS. The 5GMSu AF instructs the 5GMSu AS to perform processing of the media according to the provided media processing document. Alternatively, it can share the media processing load among multiple 5GMSu AS(s).

The procedure is defined as follows:

Figure 7.3-1: Media Processing Procedures for Uplink

The steps are as follows:

1:	Setup of uplink streaming configuration: The external 5GMSu application server sends a request to start a FLUS session to the 5GMSu AF. The request contains a description of the media processing that is to be performed by the 5GMSu AS. Depending on the configuration one or multiple 5GMSu AS(s) may be involved.

2:	Provision 5GMSu AS(s): The 5GMSu AF parses the media processing description and provisions the 5GMSu AS(s) that will perform the requested processing. If the requested processing is not accepted, the session creation fails.

3:	5GMSu AS(s) ready: The 5GMSu AS(s) confirm(s) correct configuration and inform(s) the 5GMSu AF that it is ready to receive and process media as requested.

4:	Confirm uplink streaming configuration: The 5GMSu AF confirms the successful creation of the uplink streaming configuration to the external Media application server.

5:	Uplink streaming session start: the session is triggered in the Media upstream client.

6:	Uplink media streaming: Media content is streamed from the upstream client to the 5GMSu AS(s).

7:	The 5GMSu AS(s) process(es) the received media based on the provisioned media processing.

## 7.4	Edge Computing

During session provisioning, the Application Provider may request edge computing resources for the sessions that it offers. The Network Slice Manamgement Function will assign an LADN DNN to be used for these sessions. The PCF updates the URSP rules to indicate the LADN DNN for the traffic that matches the filtering criteria for sessions of this application provider. The UE establishes or modifies an existing PDU session with the appropriate S-NSSAI and DNN to route its traffic through the LADN for edge processing.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 03-2019 | SA#83 | SP-190037 |  |  |  | Presented to TSG SA#83 (for information) | 1.0.0 |
| 06-2019 | SA#84 | SP-190333 |  |  |  | Presented to TSG SA#84 (for approval) | 2.0.0 |
| 06-2019 | SA#84 |  |  |  |  | Approved at TSG SA#84 | 16.0.0 |
| 09-2019 | SA#85 | SP-190654 | 0001 | 2 | F | Clarification of configuration updates when not streaming media data | 16.1.0 |
| 09-2019 | SA#85 | SP-190654 | 0002 | 3 | F | Correction of Architecture, Unicast Streaming Procedure, QoE metrics reporting, Consumption reporting and Session Handling for 5GMS | 16.1.0 |
| 10-2019 | SA4#106 | S4-191311 |  |  |  | Agreed draft CRs added to this draft CR S4-191236, S4-191123, S4-191156, S4-191242, S4-191243, S4-191241, S4-191165, S4-191250 S4-191313 |  |