3GPP TSG-SA4 Ad hoc meeting on 5GMS3	S4-AHI896

# The Hague, The Netherlands, 12-14 November, 2019					 revision of S4-AHI890

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR |  | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture Diagram and Text Revisions on RAN Signaling based Uplink Assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2019-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The existing Figure 6.7-1 contains a mistake in the depiction of RAN signaling based uplink network assistance. The communicatiion between the UE and the RAN represents Layer 2 functionality and is outside the scope of 5GMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification of Figure 6.7-1 on uplink streaming architecture with RAN signaling based uplink assistance. In addition, revision of existing description text in clause 6.7. Addition of normative reference to the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrection illustration of in-scope vs. out-of-scope aspects of the 5GMS uplink streaming architecture in the context of RAN signaling based uplink streaming. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 26.238: "Uplink streaming".

[6]	3GPP TS 26.307: "Presentation layer for 3GPP services".

[7]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[8]	3GPP TS 26.234: "Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs".

[X]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

END OF FIRST CHANGE

START OF SECOND CHANGE

## 6.7	RAN Signaling based Support for Uplink Network Assistance

The diagram in Figure 6.7-1 depicts an uplink streaming architecture where RAN signaling is employed to support uplink network assistance functionality.

The RAN in the architecture contains control plane and user plane entities that interact with peer control and user plane entities in the UE. The control plane interactions involve the sending by the RAN of uplink bit recommendations to the UE, which may occur either independently or in response to a boost request from the UE, via the exchange of the bit rate recommendation message and bit rate recommendation query message between the RAN and UE as described in TS 38.321 [X]. This control interface is outside the scope of 5GMSA. The user plane interaction pertains to uplink streaming media transport, and Figure 6.7-1 (solid line going from UE to RAN to 5GMSu AS) illustrates the scheduling/pass-through functionality associated with user plane communications, governed by the RAN and which may occur as result the bitrate recommendation messaging over the control plane (shown by dotted line between the UE and RAN). It is assumed in this case that application-level network assistance, i.e., as represented by the "NA AF sub-function of the "UL 5GMSu AF" is not utilized.

It is assumed in this deployment scenario that application-level network assistance, i.e., as represented by the "NA AF sub-function of the "5GMSu AF" is not utilized. However, it should also be assumed that when RAN signaling based uplink assistance is performed, there is a higher level network entity which coordinates and tracks network assistance performed using RAN signaling vs. application signaling for individual UEs.

Figure 6.7-1: RAN Signaling based Uplink Network Assistance

.

END OF CHANGE