Source:		KPN

Title:	VR Telepresence Use Case for FLUS

Agenda Item:	4

Document for:	Discussion and Agreement

Proposal

As VR is becoming more and more relevant for industry and consumers we believe that people will also more and more communicate in immersive environments (i.e. AR/VR). In order to bring other people into the situation they are currently experiencing they will need to transfer extended data in terms of 180-360 degree video and spatial audio. As this can be a highly mobile and in-situ experience it is envisioned that this will mostly happen over 3GPP networks. Therefor we propose to add a VR conferencing use case to FLUS as presented in [1]. This can also help to better link the VR feasibility study [1] to the efforts of FLUS as well as linking to the envisioned FLUS data rates presented in [2]. Thus, we propose to add the following use case (VR Telepresence) to the FLUS permanent document [3].

*** Start change 1 ***

## Use-Case: VR Telepresence

  - Description

The general concept of a VR Telepresence system it to broadcast your own environment (e.g. as 360 degree video) and allow another party to enter your environment, while being able to interact with you. The VR feasibility study [1] presents 2 scenarios for such a VR telepresence, (1) a spherical video based call and (2) video conferencing with 360-degree video:

“In a Spherical Video based call (1), two parties communicate with each other. The parties experience the feeling of being immersed in the far-end party’s environment. For example, one call participant may be calling home from a beach. The call participant at home is able to have a spherical video and audio experience of the beach location.”[1]

As an example of (2), “Anne is holding a video conference call with her team from her home office. The team is located in a meeting room around a 360 camera and a microphone array is located on top of the camera to capture spatial sound. The conferencing application on her tablet shows a section of the 360°-view of the conferencing room. On voice activity, the camera shows the person currently talking. Anne can swipe the image to look into any direction in the conference room.”[1]

In both scenarios one side captures a 360-degree spherical video (e.g. accomplished through the use of an omnidirectional camera) and 3D spatial audio (e.g. captured with a microphone arrays). This captured image can be shown on a traditional display or within an HMD (for full immersion).

  - Realization Considerations

- Capture the user’s environment in 360 degree video with 3D spatial audio (alternatively only a part of the environment can be captured, e.g. 180-degree)

- Further the user might capture the environment with multiple cameras that change their recording behavior over time

- Render the video on either an HMD or add sufficient UI capabilities to allow a user to navigate the 360-degree video on a traditional screen or tablet

- Render spatial audio based on the current viewport of the user in the 360 degree space

  - Potential Requirements

- Sufficient SDP-based mechanisms for the negotiation of streaming and VR capabilities across senders and receivers during both call setup and mid-call

- Spatial audio has to be transmitted in sync and via multiple channels (to support 3D and spatial characteristics)

- The network should support different modes to follow the requirements of real-time communication:

  - Low latency mode, with a camera to display delay of 200 – 300 ms

  - High bandwidth mode, with minimum of 22Mbps (2k+ video resolution)

  - Ultra High bandwidth mode, with up to 1Gbps (16k+ video resolution in 120+ fps)

*** End change 1 ***

References

[1] TR 26.918 - Virtual Reality (VR) media services over 3GPP (v 2.0.0)

[2] S4-170628 - FLUS Work Planning and QoS Requirements

[3] S4-AHM363 - Framework for Live Uplink Streaming Permanent Document v0.3