Source:	HEAD acoustics GmbH

Title:	VoLTE Clock Drift Measurements

## Document for:	Discussion

## Agenda Item:	X.X

# Introduction

This contribution provides an overview of different types of clock drift measurements using different types of signals and different types of improved slope estimation.

# Measurements

## Configuration setup

The setup shown in Fig. 1

Figure 1: Test setup of VoLTE acoustic testing.

The connections are as follows:

- ACQUA measurement system to

- MFE VI.1 via USB to

- MFE VIII.1 via AES to

- CMW 500 to

- DUT to HMS with HHP to MFE VI.1

The evaluation was performed with 3+2 VoLTE DUTs in receiving direction.

The delay vs. time curves are calculated as described in the draft CR to 26.132 in Annex D.

## P.501 test signals with step size 32 s

The British English P.501 sentences (8 sentences each centered in 4 seconds grid) are repeated 30 times leading to a sound file of 960 s = 16 min. To calculate the delay vs. time curve, the whole measurement is used with a step size of 32 s leading to 30 delay values.

The estimated clock drift for DUT A is ∆delay∆time⋅106= 136.9ms-141.4ms938.2s-21.8s⋅106 = -4.9 ppm, meaning that the clock of the DUT is a little bit faster than the clock of the MFE VIII.1.

The estimated clock drift for DUT B is ∆delay∆time⋅106= 156.7ms-159.4ms+145.4ms-150.5ms306.2s-21.8s+938.2s-337.8s⋅106 = -8.8 ppm, calculated in the two “jump-free” segments.

The estimated clock drift for DUT C is ∆delay∆time⋅106= 187.8ms-210.4ms938.2s-21.8s⋅106 = -24.8 ppm.

To simulate a DUT with higher clock drift, the clock of the MFE VIII.1 is put out of tune by -50ppm and used with DUT A. This is called DUT A+ in the following.

The estimated clock drift for DUT A+ is ∆delay∆time⋅106= 128.6ms-139.3ms+118.6ms-145.1ms+128.0ms-137.1ms243.0s-53.4s+748.6s-306.2s+938.2s-780.2s⋅106 = -58.6 ppm.

To simulate another DUT with even higher clock drift, the clock of the MFE VIII.1 is put out of tune by -100ppm and used with DUT B. This is called DUT B+ in the following.

In this measurement a delay jump of about 20 ms occurs about every 3 minutes, i.e., every 5 to 6 steps. A clean trend however, can only be seen in the first and the last two segments. In the two segments between, the neighboring time steps are severely affected by the delay jumps leaving to few time steps for a clean trend.

The estimated clock drift for DUT B+, based on the last two segments, is ∆delay∆time⋅106= 148.4ms-162.4ms+147.0ms-160.8ms717.0s-590.6s+906.6s-780.2s⋅106 = -110.2 ppm. The clock drift based on the first segment is ∆delay∆time⋅106= 134.6ms-146.2ms148.2s-21.8s⋅106 = -91.6 ppm and thus strongly deviates from the last two segments.

## P.501 test signals with step size 4 s

For this evaluation, the first 160 s = 2 min 40 s of the same measurement as before are used to calculate the delay vs. time curve, this time with a step size of 4 s, i.e., one step per sentence, leading to 40 delay values.

The estimated clock drift for DUT A is ∆delay∆time⋅106= 140.4ms-141.4ms159.8s-2.7s⋅106 = -6.2 ppm.

The estimated clock drift for DUT B is ∆delay∆time⋅106= 159.6ms-158.1ms159.8s-2.7s⋅106 = -9.4 ppm.

The estimated clock drift for DUT C is ∆delay∆time⋅106= 206.8ms-210.5ms159.8s-2.7s⋅106 = -24.0 ppm.

The estimated clock drift for DUT A+ is ∆delay∆time⋅106= 131.3ms-138.8ms159.8s-18.8s⋅106 = -53.2 ppm.

The estimated clock drift for DUT B+ is ∆delay∆time⋅106= 130.1ms-145.6ms159.8s-14.8s⋅106 = -107.3 ppm.

## CSS test signals

As an alternative test signal, a CSS burst is repeated to fill a sound file of 160 s length.

Note, that the step size must be smaller than the repetition period of the CSS burst to avoid random delay jumps by the length of the CSS burst.

For the following evaluation, a step size of ⅓ s is chosen leading to 479 delay values.

The estimated clock drift for DUT A is ∆delay∆time⋅106= 127.4ms-128.2ms159.8s-0.5s⋅106 = -5.0 ppm.

The estimated clock drift for DUT B is ∆delay∆time⋅106= 483.6ms-485.1ms159.5s-0.5s⋅106 = -9.4 ppm.

The estimated clock drift for DUT C is ∆delay∆time⋅106= 149.5ms-153.4ms156.8s-0.2s⋅106 = -24.7 ppm.

The estimated clock drift for DUT A+ is ∆delay∆time⋅106= 468.3ms-476.8ms159.5s-2.8s⋅106 = -54.8 ppm.

The estimated clock drift for DUT B+ is ∆delay∆time⋅106 = -109.3 ppm, calculated in the 4 clean segments.

## MSMP test signals

As a last test signal, the MSMP signal from P.501 of length 150 s is used.

For the following evaluation, a step size of 4 s is chosen.

The estimated clock drift for DUT A is ∆delay∆time⋅106= 125.4ms-126.1ms150.7s-2.7s⋅106 = -4.8 ppm.

The estimated clock drift for DUT B is ∆delay∆time⋅106= 155.1ms-155.4ms+144.6ms-145.6ms34.7s-2.7s+150.7s-38.7s⋅106 = -9.3 ppm.

The estimated clock drift for DUT C is ∆delay∆time⋅106= 157.8ms-161.3ms150.7s-2.7s⋅106 = -24.1 ppm.

The estimated clock drift for DUT A+ is ∆delay∆time⋅106= 125.2ms-133.3ms150.7s-2.7s⋅106 = -54.9 ppm.

Due to organisational restrictions, DUT B+ was not measured with the MSMP test signal. However, a similar behaviour as for DUT A+ is expected.

# Discussion

Using the P.501 test signals, the benefit of a step size of 32 s is a very smooth delay vs. time curve which eases automatic clock drift estimation for low clock drifts. On the downside, each measurement cycle takes 16 minutes. Furthermore, the maximum measureable clock drift is limited by the too frequent occurrence of delay jumps of 10 ms or 20 ms, which are caused by jitter buffer over-/underruns.

With step size 4 s and P.501 test sentences, a measurement cycle takes less than 3 minutes but has a high drift variance.

A fast measurement in less than 3 minutes with high precision can, however, be accomplished using a composite source signal. Using the CSS test signal, large segments with clean slopes, i.e., very low drift variance, can be found in every measurement. In theory, the CSS test signal of 160 s length can under ideal circumstances yield a clock drift precision of less than 0.15ppm (which is given by resolution of the cross correlation due delay calculation).

The MSMP signal has a comparably low drift variance, which is in between the CSS bursts and the P.501 sentences and yields good results in less than 3 minutes.

The following table summarizes the clock drifts which are extracted by hand from the measurements above. The clock drifts hand derived from the CSS signal are assumed to be the ground truth, since

- the CSS signal yields very clean slopes with many consistent data points and

- one of the hand derived slopes from the P.501 signal is the same as the CSS results (within 0.1 ppm) for each DUT A, B, and C.

|  | P.501 – 32 s | P.501 – 4 s | CSS – ⅓ s | MSMP – 4 s | Assumed ground truth |
| --- | --- | --- | --- | --- | --- |
| DUT A | -4.9 ppm | -6.2 ppm | -5.0 ppm | -4.8 ppm | -5.0 ppm |
| DUT B | -8.8 ppm | -9.4 ppm | -9.4 ppm | -9.3 ppm | -9.4 ppm |
| DUT C | -24.8 ppm | -24.0 ppm | -24.7 ppm | -24.1 ppm | -24.7 ppm |
| DUT A+ | -58.6 ppm | -53.2 ppm | -54.8 ppm | -54.9 ppm | -54.8 ppm |
| DUT B+ | -110.2 ppm | -107.3 ppm | -109.3 ppm | – | -109.3 ppm |

# Improved slope estimation algorithm

In this section, two improved algorithms to analyze the delay vs. time graph to estimate the clock drift in parts per million (PPM) are presented.

The main problem of clock drift estimation are the delay jumps which are caused by packet insertions or losses due to jitter buffer over-/underruns. Therefore, the general idea is cut out the delay jumps and stitch together the remaining parts of the curve without gap. In the exemplary curve above, the first step of the orange part is stitched on the last step of the green part, the first step of the yellow part is stitched on the last step of the orange part, and so on. This way, a “continuous” delay vs. time curve is generated whose slope is calculated.

A delay jump caused by a jitter buffer over-/underrun results in a high delay difference between two time steps of a few milliseconds. Therefore, delay jumps can easily be detected after differentiation of the measured delay vs. time curve.

Silent signal segments, e.g. at the start or the end, should also be cutted out.

In detail, the cutting and stitching algorithm consists of the following steps:

- Apply a median filter of length 3 to the delay vs. time series Y(k) yielding the time series Y(k). This smoothes away small spikes without soften the delay jumps.

- Calculate the delay difference per time step ΔYk=Yk+1-Yk.

- Exclude all time steps whose absolute delay differences is larger than a threshold ΔYmax.

  - With larger step sizes, the normal clock drift results in higher differences ΔYk and therefore requires a larger threshold ΔYmax. On the one hand, a minimum threshold of 1 ms is given by the “quantization” of the delay vs. time curve. On the other hand, the threshold should not be too large to reliably detect packet losses and insertions:
ΔYmax= minmaxmaximum detectable clock drift in PPM106⋅step size, 1 ms, 4 ms
where the maximum detectable clock drift is chosen to 500 PPM.

  - Since the delay jump might also effect the surrounding time steps, the steps before and after the jump are also excluded.

- Exclude the first two and the last two time steps to avoid transient effects.

- Generate the “continuous” delay vs. time series Y(k) as the cumulative sum of all remaining, i.e., not excluded delay differences ΔYk. Note, that the length of the “continuous” series Y(k) is smaller than the length of the series Yk.

The clock drift can be estimated in two ways based on the stitched, “continuous” delay vs. time series Yk:

- The clock drift is estimated using the methods of linear regression. The clock drift is calculated from the regression coefficient β, which minimizes the squared error:
D= βstep size⋅106 PPM.

- The clock drift estimate is calculated as the overall slope, i.e., the ratio of the overall delay change of the time series Yk and the overall duration:
D= Ykmax- Y1kmax-1⋅step size⋅106 PPM.

# Evaluation

## Linear regression method

|  | P.501 – 32 s | diff. to hand r. | P.501 – 4 s | diff. to hand r. | CSS – ⅓ s | diff. to hand r. | MSMP – 2 s | diff. to hand r. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DUT A | -4.9 ppm | (0.0) | -4.3 ppm | (+1.9) | -4.9 ppm | (+0.1) | -4.9 ppm | (-0.1) |
| DUT B | -8.3 ppm | (+0.5) | -9.4 ppm | (0.0) | -9.4 ppm | (0.0) | -9.6 ppm | (-0.3) |
| DUT C | -25.2 ppm | (-0.4) | -24.6 ppm | (-0.6) | -24.5 ppm | (+0.2) | -24.3 ppm | (-0.2) |
| DUT A+ | -60.7 ppm | (-2.1) | -57.4 ppm | (-4.2) | -55.2 ppm | (-0.4) | -55.3 ppm | (-0.4) |
| DUT B+ | -111.4 ppm | (-1.2) | -110.4 ppm | (-3.1) | -109.4 ppm | (-0.1) | – |  |

## Overall slope method

|  | P.501 – 32 s | diff. to hand r. | P.501 – 4 s | diff. to hand r. | CSS – ⅓ s | diff. to hand r. | MSMP – 2 s | diff. to hand r. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DUT A | -4.9 ppm | (0.0) | -1.8 ppm | (+4.4) | -5.0 ppm | (0.0) | -4.7 ppm | (+0.1) |
| DUT B | -8.6 ppm | (+0.2) | -7.4 ppm | (+2.0) | -9.3 ppm | (+0.1) | -9.4 ppm | (-0.1) |
| DUT C | -25.2 ppm | (-0.4) | -30.9 ppm | (-6.9) | -24.8 ppm | (-0.1) | -24.4 ppm | (-0.3) |
| DUT A+ | -59.4 ppm | (-0.8) | -54.2 ppm | (-1.0) | -55.3 ppm | (-0.5) | -55.8 ppm | (-0.9) |
| DUT B+ | -110.8 ppm | (-0.6) | -108.0 ppm | (-0.7) | -109.4 ppm | (-0.1) | – |  |

## Old method

|  | P.501 – 32 s | diff. to hand r. | P.501 – 4 s | diff. to hand r. | CSS – ⅓ s | diff. to hand r. | MSMP – 2 s | diff. to hand r. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DUT A | -4.9 ppm | (0.0) | -17.1 ppm | (-10.9) | 0.0 ppm | (+5.0) | -5.0 ppm | (-0.2) |
| DUT B | -9.5 ppm | (-0.7) | -18.5 ppm | (-9.1) | 0.0 ppm | (+9.4) | -10.7 ppm | (-1.4) |
| DUT C | -25.0 ppm | (-0.2) | -23.8 ppm | (+0.2) | -20.5 ppm | (+4.2) | -23.4 ppm | (-0.7) |
| DUT A+ | -55.8 ppm | (+2.8) | -55.8 ppm | (-2.6) | -62.4 ppm | (-7.6) | -56.4 ppm | (-1.5) |
| DUT B+ | -56.9 ppm | (+53.3) | -112.0 ppm | (-4.7) | -118.2 ppm | (-8.9) | – |  |

In general, the proposed improved slope estimation algorithms are both capable of estimating the clock drift well with a precision

- of 0.4 ppm for the CSS signal with ⅓ s step size for all DUTs,

- of 0.4 ppm for the MSMP signal with 4 s step size for all DUTs but DUT A+ with the overall slope method and

- of 0.5 ppm for the P.501 sentences with 32 s step size for all DUTs but DUT A+ and B+. For this test signal, the precision decreases with increasing the clock drift, especially for DUT A+ and B+. If the clock drift is too high, the estimation might fail because the distance between the delay jumps becomes too small.

For the P.501 sentences with 4 s step size, the high variance of the slope of the delay curve results in high estimation errors of the overall slope method. The linear regression method is less affected and has a precision of 0.6 ppm for all DUTs but DUT A+ and B+.

In general, the linear regression method is superior to the overall slope method.

# References

- ITU-T Recommendation P.501:Test Signals for Use in Telephonometry, 01/2012

- 3GPP Tdoc S4-140715: Draft CR to 26.132 on LTE UE delay test methods”, TSG SA4-79 Meeting