3GPP TS 26.260 V1.0.0 (2018-08)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and Systems Aspects;

Objective Test Methodologies for the Evaluation of Immersive Audio Systems;

(Release 15	)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword, …]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Objective Test Methodologies for Immersive Audio Systems	6

4.1	Objective Test Methodologies for Assessment of Immersive Audio Systems in the Sending Direction	6

4.1.1	Diffuse-field Send Frequency Response for Scene-based Audio	6

4.1.1.0	Introduction	6

4.1.1.1	Definition	6

4.1.1.2	Test Conditions	7

4.1.1.3	Measurement	7

4.2	Objective Test Methodologies for Assessment of Immersive Audio Systems in the Receiving Direction	8

4.2.1	Headset Binaural Diffuse-field Receive frequency response for Scene-based audio	8

4.2.1.0	Introduction	8

4.2.1.1	Definition	8

4.2.1.2	 Test Conditions	9

4.2.1.2	 Measurement	9

4.2.3.1	Introduction	9

4.2.3.2	Definition	9

4.2.3.3	 Test Conditions	9

4.2.3.4	 Measurement	10

Annex A (informative): Change history	11

Annex B (normative): Order-dependent directions	11

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

Audio is a key component of an immersive multimedia experience and 3GPP systems are expected to deliver immersive audio with a high Quality of Experience. However, industry agreed methods to assess the Quality of Experience for immersive audio are relatively few and this Technical Specification seeks to address this gap by providing objective test methods for the assessment of immersive audio.

# 1	Scope

The present document specifies objective test methodologies for 3GPP immersive audio systems including channel based, object based, scene-based and hybrids of these formats. The subjective evaluation methods described in the present document are applicable to audio capture, coding, transmission and rendering as indicated in their corresponding clauses.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	J. Fliege und U. Maier - A two-stage approach for computing cubature formulae for the sphere,“ Dortmund University, 1999

.

[3]	ISO 3745 Annex A – General procedures for qualification of anechoic and hemi-anechoic rooms.

[4]	ISO 1996 Acoustics – Description, measurement and assessment of environmental noise

[5]	ANSI S1.4 – American National Standard for Sound Level Meters

[6]	ISO 3 – Preferred numbers – Series of preferred numbers

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

LAeq 	the sound level in decibels equivalent to the total A-weighted sound energy measured over a stated period of time.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

# 4	Objective Test Methodologies for Immersive Audio Systems

## 4.1	Objective Test Methodologies for Assessment of Immersive Audio Systems in the Sending Direction

### 4.1.1	Diffuse-field Send Frequency Response for Scene-based Audio

#### 4.1.1.0	Introduction

This test is applicable to UEs capturing scene-based audio (e.g. First and Higher Order Ambisonics).

#### 4.1.1.1	Definition

The Diffuse-field Send Frequency Response for Scene-based Audio is defined as the transfer function, G(f)Gf, between:

- Pf, the estimated sound pressure magnitude spectrum obtained from a diffuse-field scene-based audio capture and reference synthesis at the geometric center of a free-field volume; and

- P(f)Pf, the sound pressure magnitude spectrum obtained from a diffuse-field microphone recording the same diffuse field at the origin of a spherical coordinate system.

Figure 1 describes a typical block diagram for the scene-based audio sending direction with measurement points.

Figure 1 - Scene-based audio capture block diagram for sending direction measurements

Definition of Equivalent Spatial Domain

The equivalent spatial domain representation, w(t), of a NthNth order Ambisonics soundfield representation c(t) ct is obtained by rendering ct ct  c(t) to KK virtual loudspeaker signals wjt1≤j≤Kwj (t), 1  j  K1≤j≤K, with K=(N+1)2K = (N+1)2. The respective virtual loudspeaker positions are expressed by means of a spherical coordinate system, where each position lies on the unit sphere, i.e., a radius of 1. Hence, the positions can be equivalently expressed by order-dependent directions j(N)=(j(N),  j(N)), 1  j  KΩj(N)=(θj(N),ϕj(N)),  1≤j≤K, where θj(N)j(N) and ϕj(N) j(N) θj(N)ϕj(N)denote the inclinations and azimuths, respectively. These directions are defined according to [2] Fliege und U. Maier - A two-stage approach for computing cubature formulae for the sphere,“ Dortmund University, 1999 and reproduced in Annex B for convenience.

The rendering of ct into the equivalent spatial domain can be formulated as a matrix multiplication,

wt=Ψ(N,N)-1⋅ct,

w(t) = ((N,N))-1 c(t),

where ⋅-1()-1 denotes the inversion.

The matrixΨ(N,N) (N,N) of order N with respect to the order-dependent directions Ωj(N)j(N) is defined by:

ΨN,N:=S1NS2N…SKN

(N,N) := [S1(N)    S2(N)    …    SK(N)],

with

Sj(N):=S00Ωj(N)S-1-1Ωj(N)S-10Ωj(N)S-11Ωj(N)S-2-2Ωj(N) …SNNΩj(N)T

Snm∙ represents the Sj(N) := [S00(j(N))    S-1-1(j(N))    S-10(j(N))    S-11(j(N))    S-11(j(N))    …    SNN(j(N))]T ,

where Snm() represents the real valued spherical harmonics of the order n and degree m.

The matrix Ψ(N,N)(N,N) is invertible so that the HOA representation ctc(t) can be converted back from the equivalent spatial domain by:

ct=Ψ(N,N) ⋅wt⋅

c(t) = (N,N)w(t)

#### 4.1.1.2	Test Conditions

Free-field propagation conditions

- The test environment shall contain a free-field volume, wherein free-field sound propagation conditions shall be observed.

- The free-field sound propagation conditions shall be observed down to a frequency of 200 Hz or less.

- Qualification of the free-field volume shall be performed using the method and limits for deviation from ideal free-field conditions described in [3].

Test environment noise floor

Within the free-field volume, the equivalent continuous sound level of the test environment in each 1/3rd octave band, Leq(f)Leq(f), shall be less than the limits of the NR10 curve, following the noise rating determination procedures in [4].

Periphonic loudspeaker array for diffuse sound field generation

- A periphonic loudspeaker array shall be placed within the free-field volume with the geometric center of the periphonic loudspeaker array coinciding with the geometric center of the free-field volume.

- The periphonic loudspeaker array shall have a radius greater or equal than 1 meter.

- The periphonic loudspeaker array shall be composed of (N+1)2 coaxial loudspeaker elements

- . Each of the (N+1)2 coaxial loudspeaker elements shall be equalized (if necessary) and level compensated to conform with the operational room response curve limits given in [5] Section 8.3.4.1. N should be equal or greater than the maximum ambisonics order supported by the device under test (DUT), e.g. N>=4 for a DUT supporting 4th order Ambisonics capture.

- The (N+1)2 coaxial loudspeaker elements shall be positioned according to the azimuth and elevation coordinates given in Annex B.

- All coaxial loudspeaker elements shall be oriented such that their acoustic axis intersects at the geometric center of the free field volume.

- The radius of each coaxial loudspeaker element shall be such that, at the geometric center of the free-field volume, the far field approximation for the coaxial loudspeaker axial pressure amplitude decay holds true.

#### 4.1.1.3	Measurement

Reference Spectrum measurement

- A diffuse-field / random incidence, or multi-field microphone is mounted in the free-field volume such that the tip of the microphone corresponds to the geometric center of the free-field volume and the geometric center of the periphonic loudspeaker array.

NOTE:  Diffuse-field / random incidence microphones, are described in [5]

- (N+1)2 decorrelated pink noise signals are played simultaneously over each of the (N+1)2 coaxial loudspeakers of the periphonic loudspeaker array.

- The playback level is adjusted such that the LAeq, measured over a 30s time window at the geometric center of the periphonic loudspeaker array, is equal to 78dBSPL(A)  0.5dB.

- The reference sound pressure at the geometric center of the free-field volume, p(t)pt, is captured with the diffuse-field or multi-field microphone.

- The magnitude spectrum of the reference sound pressure, P(f)Pf, is calculated for the 1/12th octave intervals as given by the R40 series of preferred numbers in [6].

Estimated Spectrum measurement

- The scene-based audio capture device under test is mounted in the free-field volume such that its geometric center coincides with the geometric center of free-field volume and the geometric center of the periphonic loudspeaker array.

- (N+1)2 decorrelated pink noise signals are played simultaneously over each of the (N+1)2 coaxial loudspeakers of the periphonic loudspeaker array. The pink noise signals shall be identical to the signals used for the reference spectrum measurement.

- The B-format scene-based audio format representation (compressed or uncompressed, depending on the use case being tested) is stored for offline analysis.

- The B-format scene-based audio format representation is uncompressed (if necessary) and converted to an equivalent spatial domain representation of order NDUT (B-Format to ESD conversion in Figure 1), where NDUT corresponds to the Ambisonics order of the device under test.

- pt, the estimate of the sound field at the geometric center of the free-field volume and periphonic loudspeaker array, is synthesized using the equivalent spatial domain representation of order NDUT.

- The magnitude spectrum of the estimated sound pressure, Pf, is calculated for the 1/12th octave intervals as given by the R40 series of preferred numbers in [6].

Calculation of send frequency response for scene-based audio

The send frequency response for scene-based audio, G(f)Gf, is calculated as:

Gf=PfPf

## 4.2	Objective Test Methodologies for Assessment of Immersive Audio Systems in the Receiving Direction

### 4.2.1	Headset Binaural Diffuse-field Receive frequency response for Scene-based audio

#### 4.2.1.0	Introduction

This test is applicable to UEs rendering scene-based audio (e.g. First and Higher Order Ambisonics) over a binaural headset.

#### 4.2.1.1	Definition

The Headset Binaural Diffuse-field Receive Frequency Response for Scene-based Audio (for left and right ears) is defined as the transfer function, GL,R (f)Gf, between:

- PL,R(f), the binaurally recorded sound pressure magnitude spectra, obtained when a diffuse field signal in the equivalent spatial domain representation, w(t), is played on the DUT and,

- Pref L,R(f)Pf, the reference sound pressure magnitude spectra, obtained by direct convolution of the diffuse field signal in the equivalent spatial domain representation, w(t) with its corresponding set of HRTFs.

#### 4.2.1.2		Test Conditions

Test environment noise floor

The equivalent continuous sound level of the test environment in each 1/3rd octave band, Leq(f)Leq(f), shall be less than the limits of the NR10 curve, following the noise rating determination procedures in [4].

#### 4.2.1.2		Measurement

Reference sound pressure magnitude spectra

The reference sound pressure magnitude spectra are derived offline. The reference sound pressure magnitude spectra for the left and right ears, Pref L,R (f) is the frequency domain representation of the convolution between the set of equivalent spatial domain signals, w(t), with its corresponding set head related transfer functions h L,R(t), for each direction j in a equivalent spatial domain of order NDUT , i.e.:

The signals wj(t), for 1  j  (NDUT +1)2, are uncorrelated pink noise signals of 30s length.

Binaurally recorded sound pressure magnitude spectra

The binaurally recorded sound pressure magnitude spectra is obtained as follows:

- The binaural headset is placed on a HATS.

- The DUT shall be configured such that the set of HRTFs used for binaural rendering correspond to the HATS used for testing.

- The DUT volume control (if any) is adjusted for its nominal setting.

- The binaural time-domain signals are recorded with HATS.

- The binaurally recorded sound pressure magnitude spectra, PL,R(f) is obtained by taking the fourier transform of the binaurally recorded time-domain signals.

Calculation of headset binaural diffuse-field receive frequency response for scene-based audio

The headset binaural diffuse-field frequency response for scene-based audio, G(f)Gf, is calculated for each supported Ambisonics order NDUT as:

### 4.2.3	Nominal System Sensitivity in Receive Direction for Channel-based audio

#### 4.2.3.1	Introduction

This test is applicable to UEs rendering channel-based audio (e.g. 7.1.4) over a binaural headset.

#### 4.2.3.2	Definition

The nominal system sensitivity in receive direction for channel-based audio is defined as the ratio between the sound pressure level (in dBSPL(A)) produced by the DUT on HATS and the root mean square of the digital test signal (in dBFS).

#### 4.2.3.3		Test Conditions

Test environment noise floor

The equivalent continuous sound level of the test environment in each 1/3rd octave band, Leq(f)Leq(f), shall be less than the limits of the NR10 curve, following the noise rating determination procedures in [4].

#### 4.2.3.4		Measurement

For each audio channel supported by the DUT, a pink noise signal with -18dBFS RMS level is played, with the signals played only one channel at a time.

The LAeq (in dBSPL(A)) is measured continuously for a period of 30s for each of the left and right ears.

The sensitivity Gi L,R is expressed as the ratio of the recorded sound pressure levels at the left and right ears and the root mean square digital level of the pink noise test signal, i.e. -18dBFS.

Gi L,R= LAeq – 18 [dBSPL(A)/FS]

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 03/10/17 | SA4#95 | S4-170909 |  |  |  | Initial Skeleton Draft Input to SA4#95 | 0.0.1 |
| 12/10/17 | SA4#95 | S4-171042 |  |  |  | Output from SA4#95 | 0.0.2 |
| 15/11/17 | SA4#96 | S4-171345 |  |  |  | Output from SA4#96 | 0.0.3 |
| 08/02/18 | SA4#97 | S4-180268 |  |  |  | Output from SA4#97 | 0.0.4 |

Annex B (

normative): Order-dependent directions

The following tables order-dependent directions Ωj(N)=(θj(N),ϕj(N)),  1≤j≤K, where θj(N) and ϕj(N) denote the inclinations and azimuths in radians, respectively.

| Index j | θj(N=1) | ϕj(N=1) |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 1.910633 | 0 |
| 3 | 1.910633 | 2.094395 |
| 4 | 1.910633 | -2.0944 |

| Index j | θj(N=2) | ϕj(N=2) |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 2.361073 | 0 |
| 3 | 1.207589 | -1.95668 |
| 4 | 1.207589 | 1.956682 |
| 5 | 2.415178 | -1.95668 |
| 6 | 1.561039 | -3.14159 |
| 7 | 2.415178 | 1.956681 |
| 8 | 1.325668 | 0.687124 |
| 9 | 1.325667 | -0.68712 |

| Index j | θj(N=3) | ϕj(N=3) |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 0.854098 | 0 |
| 3 | 2.031969 | 1.119907 |
| 4 | 2.605106 | -0.25283 |
| 5 | 1.078622 | 1.155586 |
| 6 | 1.736608 | 2.040481 |
| 7 | 2.031968 | -1.38118 |
| 8 | 1.736609 | 0.270692 |
| 9 | 1.56888 | -2.20417 |
| 10 | 0.917087 | 2.297267 |
| 11 | 0.917087 | -2.80293 |
| 12 | 1.763476 | 3.010956 |
| 13 | 2.649852 | 2.154919 |
| 14 | 1.568881 | -0.63529 |
| 15 | 0.953962 | -1.41973 |
| 16 | 2.458122 | -2.46809 |

| Index j | θj(N=4) | ϕj(N=4) |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 0.823218 | 0 |
| 3 | 1.73912 | -2.00759 |
| 4 | 0.724297 | 1.927637 |
| 5 | 1.336281 | -1.41208 |
| 6 | 0.871631 | -2.10001 |
| 7 | 1.263705 | 2.512927 |
| 8 | 1.440147 | 1.667633 |
| 9 | 2.248313 | 1.442383 |
| 10 | 1.433953 | -0.60062 |
| 11 | 2.888065 | 0.329968 |
| 12 | 2.003914 | -1.18621 |
| 13 | 1.80266 | 2.983332 |
| 14 | 1.396554 | -2.69222 |
| 15 | 2.170781 | 0.507602 |
| 16 | 1.952805 | 2.208977 |
| 17 | 1.58019 | 0.952319 |
| 18 | 2.584609 | -1.71565 |
| 19 | 0.874597 | 0.934402 |
| 20 | 2.172935 | -0.38654 |
| 21 | 2.612717 | 2.675958 |
| 22 | 2.193907 | -2.62842 |
| 23 | 1.51674 | 0.165012 |
| 24 | 0.715307 | -1.02504 |
| 25 | 0.762553 | -3.13121 |

| Index j | θj(N=5) | ϕj(N=5) |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 2.024896 | 0 |
| 3 | 1.247057 | -1.19666 |
| 4 | 2.746177 | 0.184066 |
| 5 | 0.623575 | 0.124282 |
| 6 | 1.764494 | -2.84022 |
| 7 | 1.070515 | -1.84701 |
| 8 | 2.234325 | 0.698758 |
| 9 | 2.184128 | 2.280239 |
| 10 | 2.158839 | -2.28482 |
| 11 | 0.624151 | -2.37569 |
| 12 | 1.237485 | 2.883411 |
| 13 | 0.603422 | -1.18504 |
| 14 | 1.133942 | -2.76846 |
| 15 | 1.060655 | 0.763488 |
| 16 | 1.634607 | -0.46491 |
| 17 | 1.52253 | -2.27504 |
| 18 | 1.719188 | 1.762138 |
| 19 | 0.625061 | 2.804486 |
| 20 | 1.696573 | -1.69175 |
| 21 | 1.812314 | -1.0321 |
| 22 | 1.63462 | 0.509415 |
| 23 | 2.811188 | -1.95737 |
| 24 | 1.028624 | -0.567 |
| 25 | 1.527149 | 2.319619 |
| 26 | 1.861841 | 2.853233 |
| 27 | 2.411897 | -3.07101 |
| 28 | 2.784687 | 2.113132 |
| 29 | 2.277422 | -1.50877 |
| 30 | 2.345421 | -0.65404 |
| 31 | 2.278241 | 1.464227 |
| 32 | 0.579954 | 1.373127 |
| 33 | 1.69346 | 1.112751 |
| 34 | 0.972182 | 2.113949 |
| 35 | 1.264106 | 0.057137 |
| 36 | 1.188862 | 1.457925 |

| Index j | θj(N=6) | ϕj(N=6) |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 0.850652 | 0 |
| 3 | 1.879161 | 3.024454 |
| 4 | 1.502365 | 2.080642 |
| 5 | 2.066473 | -2.21373 |
| 6 | 1.589575 | -2.03598 |
| 7 | 1.144753 | 1.678014 |
| 8 | 1.830538 | 0.964363 |
| 9 | 1.391476 | -3.03552 |
| 10 | 1.820414 | -2.70206 |
| 11 | 0.496613 | 0.581055 |
| 12 | 2.351968 | -2.80103 |
| 13 | 1.112947 | 0.550136 |
| 14 | 1.046845 | -1.98436 |
| 15 | 1.577042 | -0.51212 |
| 16 | 2.359303 | -1.1411 |
| 17 | 1.342615 | -2.48765 |
| 18 | 1.988906 | -1.62282 |
| 19 | 2.083484 | -0.57506 |
| 20 | 0.998656 | 2.286204 |
| 21 | 2.438372 | -0.08741 |
| 22 | 2.195595 | 0.547028 |
| 23 | 2.017483 | 1.878965 |
| 24 | 2.360463 | 2.746717 |
| 25 | 0.523033 | -0.76025 |
| 26 | 1.323604 | -1.01978 |
| 27 | 0.850653 | 1.162107 |
| 28 | 1.652615 | 1.507148 |
| 29 | 1.344756 | 1.062706 |
| 30 | 0.861708 | -2.68135 |
| 31 | 1.819892 | -1.08377 |
| 32 | 0.996837 | 2.91352 |
| 33 | 0.501675 | 2.939099 |
| 34 | 1.435415 | -1.53966 |
| 35 | 1.6283 | 0.473238 |
| 36 | 2.546165 | -1.95522 |
| 37 | 2.236832 | 1.294994 |
| 38 | 2.717718 | 0.887936 |
| 39 | 1.927866 | 2.427548 |
| 40 | 1.370154 | -0.01608 |
| 41 | 2.53588 | 1.97199 |
| 42 | 0.88913 | -1.35341 |
| 43 | 1.458362 | 2.651183 |
| 44 | 1.042321 | -0.57647 |
| 45 | 0.567169 | 1.857517 |
| 46 | 2.84677 | -0.77916 |
| 47 | 0.519694 | -2.01121 |
| 48 | 2.885875 | 3.087768 |
| 49 | 1.89749 | -0.00446 |