Source:	HEAD acoustics GmbH, Intel

Title:	Preliminary Results of Round Robin Tests for FS_ANTeM

Document for:	Discussion

# Introduction

For device testing in handset mode, the reproduction accuracy of the system according to ETSI ES 202 396-1 [1] was evaluated across labs with a round-robin test [2]. Also, several variants of the noise field simulation systems according to ETSI TS 103 224 [3] were taken into account: the eight-channel microphone array as well as binaural equalization are considered here, too.

This contribution presents first preliminary results of the round robin test.

# Status of RR-Test

The conducted measurements followed the test plan of the round robin test as specified in [2], no update or modification was necessary. Up to five noise field simulations were tested per bandwidth, device and room.

So far, more than 5100 measurements were collected, each with a duration of at least 80 seconds. This corresponds to more than 113h of recording time.

The current data still shows several outliers, which are currently analysed. This ongoing work will be concluded until SA4#106.

# Noise field simulations

An overview about the investigated noise field simulations is provided in Table 1. The short names in the first column are used in the following clauses to differentiate between the systems. More detailed descriptions are given in clause 5.3.2 of TR 26.921 [4]. Eight noise types according to Tables 1 and 2 of the test plan [2] were used for each noise field simulation to generate the recordings.

Table 1: Overview of noise field simulations

| Noise field simulation (short name) | Equalization Points | Noise source from |
| --- | --- | --- |
| ES202 | Ear Mics (2) | ES 202 396-1 (binaural) |
| TS103-4.0 | Ear Mics (2) | ES 202 396-1 (binaural) |
| TS103-4.1 | Ear Mics (2) | ES 202 396-1 (binaural) |
| TS103-8.0 | Ear Mics (2) | ES 202 396-1 (binaural) |
| TS103-HS | Mics of array (8) | TS 103 224 (8 Channels) |

# Devices

The devices according to Table 2 were available for testing. In addition to the test plan, one more device was tested in some of the labs.

Table 2: Devices for round robin test

| Device ID | Manufacturer | Year | VoLTE possible | Test in NB | Test in WB | Test in SWB |
| --- | --- | --- | --- | --- | --- | --- |
| DUT 02 | A | 2018 | Yes (only for EVS- SWB) | Yes | Yes | Yes |
| DUT 03 | B | 2018 | Yes | Yes | Yes | Yes |
| DUT 08 | C | 2015 | No | Yes | Yes | No |
| DUT 09 | D | 2012 | No | Yes | Yes | No |
| DUT 10 | E | 2014 | No | Yes | Yes | No |
| DUT 05 | F | 2018 | Yes | Yes | Yes | Yes |
| DUT 04 | F | 2017 | Yes | Yes | Yes | Yes |
| DUT 07 | G | 2018 | Yes | Yes | Yes | Yes |
| DUT 06 | H | 2018 | Yes | Yes | Yes | Yes |
| DUT 39 | I | 2017 | Yes (not for EVS- SWB) | Yes | Yes | No |

Four out of these ten devices could not be evaluated in SWB mode due to several reasons (at least in conjunction with test equipment):

- DUT 09 and 10 do not provide LTE functionality (no VoLTE possible).

- DUT 08 offers VoLTE calls in the settings, but none of the codecs worked.

- DUT 39 supports VoLTE calls, but EVS-SWB codec seems not available at all.

# Rooms

A detailed description of the participating labs/rooms can be found in S4-190954 [5].

Due to internal restructuring and time constraints, room #6 could only evaluate two devices (DUT 01 & 11).

# Additional measurements

After a submission of a liaison statement [6], four devices of the round robin test were kindly provided by CTIA CPWG, sub-working group “Audio”. In return, they asked to provide some basic measurements according to TS 26.132 [7] in NB, WB and SWB mode. These additional measurements were addressed described in the latest test plan [2] and were collected in advance to the ambient noise testing.

Sending Direction:

- DUT Delay (compensated for test equipment delay)

- Frequency Response

- Loudness Rating

- Predicted speech quality acc. to ITU-T P.863 V2.4

Receiving Direction:

- DUT Delay (compensated for test equipment delay)

- Frequency Response

- Loudness Rating (at nominal level)

- Predicted speech quality acc. to ITU-T P.863 V2.4 (at nominal level)

The results of these additional measurements will be reported after finalization of the whole round robin test. A dedicated contribution is expected for SA4#106.

# Results for noise field simulation ES 202 396-1

Table 3 to Table 5 show the results of S-MOS and N-MOS of the noise field simulation according to ES 202 396-1 [1] (labelled as ES202 in the following) for NB, WB and SWB mode for all devices and all measurements rooms. Each number is calculated as the average across the eight noise types as described in Table 1 of the test plan (which equals the noise types of TS 26.132 [7]). In addition, the average across rooms is provided in an additional column.

Table 3: RR-Test results for ES 202 396-1 noise field simulation (NB)

|  | S-MOS |  |  |  |  |  |  | N-MOS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Room | R#1 | R#2 | R#3 | R#4 | R#5 | R#6 | Avg. | R#1 | R#2 | R#3 | R#4 | R#5 | R#6 | Avg. |
| DUT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DUT 02 | 4.19 | 4.18 | 4.08 | 4.21 | 4.27 | 3.47 | 4.07 | 3.57 | 3.73 | 3.30 | 3.48 | 3.71 | 4.09 | 3.65 |
| DUT 03 | 4.09 | 4.12 | 3.94 | 3.95 | 4.21 | 4.04 | 4.06 | 4.32 | 4.48 | 4.31 | 4.30 | 4.42 | 4.54 | 4.40 |
| DUT 04 | 3.75 | 3.88 | 3.80 | 3.74 | 4.04 |  | 3.84 | 4.48 | 4.45 | 4.36 | 4.40 | 4.52 |  | 4.44 |
| DUT 05 | 3.99 | 3.89 | 3.97 | 3.95 | 4.16 |  | 3.99 | 4.10 | 4.11 | 3.93 | 3.91 | 4.08 |  | 4.03 |
| DUT 06 | 3.66 | 3.75 | 3.90 | 3.93 | 4.06 |  | 3.86 | 4.25 | 4.39 | 4.32 | 4.43 | 4.50 |  | 4.38 |
| DUT 07 | 3.96 | 4.00 | 3.95 | 4.02 | 4.12 |  | 4.01 | 4.48 | 4.53 | 4.41 | 4.51 | 4.52 |  | 4.49 |
| DUT 08 | 4.06 | 4.04 | 4.04 | 4.06 | 4.22 |  | 4.08 | 3.47 | 3.69 | 3.43 | 3.47 | 3.83 |  | 3.58 |
| DUT 09 | 4.11 | 4.19 | 4.03 | 4.07 | 4.20 |  | 4.12 | 3.07 | 3.51 | 3.11 | 3.12 | 3.59 |  | 3.28 |
| DUT 10 | 3.97 | 4.01 | 3.88 | 3.94 | 3.99 |  | 3.96 | 3.71 | 3.95 | 3.69 | 3.79 | 3.99 |  | 3.83 |
| DUT 39 | 4.08 | 4.22 | 4.16 | 4.15 |  |  | 4.15 | 4.08 | 4.17 | 3.98 | 4.03 |  |  | 4.07 |

Table 4: RR-Test results for ES 202 396-1 noise field simulation (WB)

|  | S-MOS |  |  |  |  |  |  | N-MOS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Room | R#1 | R#2 | R#3 | R#4 | R#5 | R#6 | Avg. | R#1 | R#2 | R#3 | R#4 | R#5 | R#6 | Avg. |
| DUT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DUT 02 | 3.95 | 4.01 | 3.90 | 3.98 | 4.06 | 4.08 | 4.00 | 3.94 | 4.06 | 3.70 | 3.78 | 4.05 | 4.37 | 3.98 |
| DUT 03 | 3.87 | 3.96 | 3.90 | 3.91 | 4.04 | 3.96 | 3.94 | 4.44 | 4.41 | 4.34 | 4.32 | 4.43 | 4.41 | 4.39 |
| DUT 04 | 3.84 | 3.97 | 3.87 | 3.88 | 4.02 |  | 3.92 | 4.18 | 4.26 | 4.04 | 4.04 | 4.33 |  | 4.17 |
| DUT 05 | 3.79 | 3.95 | 3.83 | 3.91 | 4.01 |  | 3.90 | 4.14 | 4.29 | 3.89 | 4.06 | 4.13 |  | 4.10 |
| DUT 06 | 3.94 | 3.96 | 3.80 | 3.84 | 3.94 |  | 3.90 | 4.32 | 4.49 | 4.35 | 4.39 | 4.51 |  | 4.41 |
| DUT 07 | 4.00 | 4.02 | 3.95 | 4.04 | 4.09 |  | 4.02 | 4.16 | 4.24 | 4.06 | 4.11 | 4.26 |  | 4.17 |
| DUT 08 | 3.77 | 3.91 | 3.77 | 3.84 | 4.04 |  | 3.87 | 3.81 | 4.00 | 3.76 | 3.70 | 4.10 |  | 3.88 |
| DUT 09 | 3.95 | 4.06 | 3.89 | 3.94 | 4.08 |  | 3.98 | 3.26 | 3.57 | 3.31 | 3.36 | 3.63 |  | 3.43 |
| DUT 10 | 3.85 | 3.94 | 3.78 | 4.00 |  |  | 3.89 | 4.24 | 4.41 | 4.21 | 3.75 |  |  | 4.15 |
| DUT 39 | 3.85 | 3.97 | 3.93 | 3.94 |  |  | 3.92 | 4.26 | 4.32 | 4.18 | 4.24 |  |  | 4.25 |

Table 5: RR-Test results for ES 202 396-1 noise field simulation (SWB)

|  | S-MOS |  |  |  |  |  |  | N-MOS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Room | R#1 | R#2 | R#3 | R#4 | R#5 | R#6 | Avg. | R#1 | R#2 | R#3 | R#4 | R#5 | R#6 | Avg. |
| DUT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DUT 02 | 3.72 | 3.84 | 3.72 | 3.69 | 3.91 | 3.81 | 3.78 | 4.18 | 4.32 | 4.05 | 4.27 | 4.37 | 4.28 | 4.24 |
| DUT 03 | 3.76 | 3.85 | 3.76 | 3.74 | 4.03 | 3.88 | 3.84 | 4.23 | 4.28 | 4.11 | 4.17 | 4.34 | 4.33 | 4.24 |
| DUT 04 | 3.92 | 4.01 | 3.82 | 3.76 | 3.99 |  | 3.90 | 3.99 | 4.05 | 3.69 | 3.87 | 4.10 |  | 3.94 |
| DUT 05 | 3.95 | 3.95 | 3.89 | 3.87 | 4.06 |  | 3.94 | 3.74 | 3.91 | 3.49 | 3.78 | 3.87 |  | 3.76 |
| DUT 06 | 3.52 |  | 3.53 | 3.48 | 3.69 |  | 3.56 | 4.48 |  | 4.47 | 4.55 | 4.57 |  | 4.52 |
| DUT 07 | 3.86 | 3.97 | 3.75 | 3.66 | 4.05 |  | 3.86 | 4.08 | 4.16 | 3.94 | 4.03 | 4.21 |  | 4.08 |

# Results for noise field simulation TS 103 224 (binaural)

As described in the test plan, also the flexible equalization methods according to ETSI TS 103 224 [3] with different loudspeaker setups were investigated in the round robin test (labelled as TS103-4.0, TS103-4.1 and TS103-8.0 in the following). Since the noise sources used for these equalizations are identical to ES202, similar results are expected.

## Narrowband Mode

### Results for all noise types, rooms and devices

Figure 1 to Figure 3 provide the results of the three binaural equalization methods compared to the data of ES202 (cf. Table 3) for NB mode. Each combination of device, room and background noise represents one dot in these scatter plots.

Figure 1: Results S-/N-MOS for TS103-4.0 vs. ES202 (NB)

Figure 2: Results S-/N-MOS for TS103-4.1 vs. ES202 (NB)

Figure 3: Results S-/N-MOS for TS103-8.0 vs. ES202 (NB)

### Results for all rooms and devices (average across noises)

Figure 4 to Figure 6 provide the results averaged across the eight noise types for NB mode. Each combination of device and room represents one dot in these scatter plots.

Figure 4: Average results S-/N-MOS for TS103-4.0 vs. ES202 (NB)

Figure 5: Average results S-/N-MOS for TS103-4.1 vs. ES202 (NB)

Figure 6: Average results S-/N-MOS for TS103-8.0 vs. ES202 (NB)

## Wideband Mode

### Results for all noise types, rooms and devices

Figure 7 to Figure 9 provide the results of the three binaural equalization methods compared to the data of ES202 (cf. Table 4) for WB mode. Each combination of device, room and background noise represents one dot in these scatter plots.

Figure 7: Results S-/N-MOS for TS103-4.0 vs. ES202 (WB)

Figure 8: Results S-/N-MOS for TS103-4.1 vs. ES202 (WB)

Figure 9: Results S-/N-MOS for TS103-8.0 vs. ES202 (WB)

### Results for all rooms and devices (average across noises)

Figure 10 to Figure 12 provide the results averaged across the eight noise types for WB mode. Each combination of device and room represents one dot in these scatter plots.

Figure 10: Average results S-/N-MOS for TS103-4.0 vs. ES202 (WB)

Figure 11: Average results S-/N-MOS for TS103-4.1 vs. ES202 (WB)

Figure 12: Average results S-/N-MOS for TS103-8.0 vs. ES202 (WB)

## Super-wideband Mode

### Results for all noise types, rooms and devices

Figure 13 to Figure 15 provide the results of the three binaural equalization methods compared to the data of ES202 (cf. Table 5) for SWB mode. Each combination of device, room and background noise represents one dot in these scatter plots.

Figure 13: Results S-/N-MOS for TS103-4.0 vs. ES202 (SWB)

Figure 14: Results S-/N-MOS for TS103-4.1 vs. ES202 (SWB)

Figure 15: Results S-/N-MOS for TS103-8.0 vs. ES202 (SWB)

### Results for all rooms and devices (average across noises)

Figure 16 to Figure 18 provide the results averaged across the eight noise types for SWB mode. Each combination of device and room represents one dot in these scatter plots.

Figure 16: Average results S-/N-MOS for TS103-4.0 vs. ES202 (SWB)

Figure 17: Average results S-/N-MOS for TS103-4.1 vs. ES202 (SWB)

Figure 18: Average results S-/N-MOS for TS103-8.0 vs. ES202 (SWB)

# Results for noise field simulation TS 103 224 (8-channel)

As described in the test plan, also the equalization method based on eight-channel microphone array and eight loudspeakers according to ETSI TS 103 224 [3] were investigated in the round robin test (labelled as TS103-HS in the following). Since the noise sources used for this system are different from ES202, results averaged across noises might have a bias or offset.

Figure 19: Average results S-/N-MOS for TS103-HS vs. ES202 (NB)

Figure 20: Average results S-/N-MOS for TS103-HS vs. ES202 (WB)

Figure 21: Average results S-/N-MOS for TS103-HS vs. ES202 (SWB)

# Conclusions

- The binaural equalizations TS103-4.0 and TS103-4.1 provide almost identical results as ES202.

- TS103-8.0 seems to provide too optimistic results compared with ES202, TS103-4.0 and TS103-4.1.

- TS103-HS:

- Rank order for S-MOS maintained compared to ES202 (NB, WB, SWB)

  - NB mode:

    - Significant bias/shift for S-MOS in NB

    - Slight bias for N-MOS

  - WB mode:

    - Slight bias for S-MOS

    - Almost free of bias for N-MOS

  - SWB:

    - Almost identical measure, only very slight bias in S- and N-MOS

- Variance for each method and outliers have to be analysed more in detail until SA4#106.

# References

| [1] | ETSI ES 202 396-1 v1.7.1, „Speech Processing, Transmission and Quality Aspects (STQ); Speech quality performance,“ 10/2017. |
| --- | --- |
| [2] | 3GPP S4-190683, „Test plan for a Round-Robin-Test - comparison of noise field simulations for handset mode v0.2,“ HEAD acoustics GmbH, Intel, Cork, Ireland, 05 July 2019. |
| [3] | ETSI TS 103 224 v1.3.1, „Speech and multimedia Transmission Quality (STQ);. A sound field reproduction method for terminal testing including a background noise database,“ 07/2017. |
| [4] | 3GPP TR 26.921, „Investigations on ambient noise reproduction systems for acoustic testing of terminals,“ in Release-16. |
| [5] | 3GPP S4-190954, „Improved description of labs participating in RR-Test,“ HEAD acoustics GmbH, Intel, Ljubljana, Slovenia, 16 August 2019. |
| [6] | 3GPP S4-190554, „LS on Round-robin test plan for FS_AnTEM (To: CTIA),“ TSG SA WG4, Newport Beach, USA, 17 April 2019. |
| [7] | 3GPP TS 26.132, „Speech and video telephony terminal acoustic test specification,“ Release 16.0.0. |
| [8] | 3GPP S4-181500, „New SID on ambient noise test methodology for evaluation of acoustic UE performance (FS_ANTeM),“ HEAD acoustics GmbH, Qualcomm Incorporated, Deutsche Telekom AG, Intel, HuaWei Technologies Co., Ltd, Busan, South Korea, 25 November 2018. |
| [9] | 3GPP TS 26.131, „Terminal acoustic characteristics for telephony; Requirements,“ Release 16.0.0. |
| [10] | ETSI TS 103 106 v1.3.1, „Speech quality performance in the presence of background noise: Background noise transmission for mobile terminals-objective test methods,“ 04/2014. |
| [11] | ETSI TS 103 281 v1.2.1, „Speech quality in the presence of background noise: Objective test methods for super-wideband and fullband terminals,“ 01/2018. |