Source:	Alcatel

Title:	Introduction of AMR WB in section 5 of 3GPP 28.062

Agenda Item:	5 (TFO Release 5: Extension of TFO to AMR-WB)

Document for:	Discussion and Approval

This document presents the modifications on section 5 of TS 28.062 necessary to introduce AMR WB.

Two TFO frame formats are introduced : AMR_WB_TFO_16k and AMR_WB_TFO_32k.

The first one is intended to be used when at least one side does not support codec modes higher than 14.25 kbit/s (i.e. does not support 32 kbit/s sub-multiplexing scheme). It is proposed to use the 32kbit/s sub-multiplexing scheme in all other cases.

The structure of the TFO frames is directly derived from the structure of the TRAU frames.

The frame type (C1-C4) for UMTS_AMR_WB is currently undefined.

# 5	TFO Frame Structure

## 5.1	General

TFO Frame formats are defined for the following Speech Codec Types:

- GSM Full Rate (GSM_FR);

- GSM Half Rate (GSM_HR);

- GSM Enhanced Full Rate (GSM_EFR);

- Adaptive Multi Rate Family (FR_AMR, HR_AMR,UMTS_AMR, UMTS_AMR_2, FR_AMR_WB, UMTS_AMR_WB).

TFO Frame formats for 8 kbit/s, 16 kbit/s and 32 kbit/s sub-multiplexing are defined in the following clauses.

## 5.2	TFO Frames for 16 kbit/s sub-multiplexing

### 5.2.1	TFO Frames for GSM Full Rate and GSM Enhanced Full Rate

The TFO Frames for GSM_FR and GSM_EFR are derived from the uplink TRAU Frames as defined in the 3GPP TS 48.060. Table 5.2.1-1 below defines the coding of the Control Bits for these TFO Frames.

- Table 5.2.1-1: Control Bits in TFO Frames for GSM_FR and GSM_EFR

| Control Bit | Description | Comment |
| --- | --- | --- |
| C1 - C4 0.0.0.1 1.1.0.1 | Frame Type GSM_FR GSM_EFR | copied from uplink TRAU Frames All other code words are reserved. |
| C5 | EMBED | Indicates the presence of an embedded TFO Message |
| C6 - C11 | Spare | (is Time Alignment in TRAU Frame) set to Spare by TRAU |
| C12 | BFI | Copied from the uplink TRAU Frame |
| C13 - C14 | SID | Copied from the uplink TRAU Frame |
| C15 | TAF | Copied from the uplink TRAU Frame |
| C16 | Spare | set to Spare by TRAU |
| C17 | DTXd | Copied from the uplink TRAU Frame |
| C18 - C21 | Spare | set to Spare by TRAU |

Any spare control bit shall be coded as binary "1". They are reserved for future use and may change.

The Synchronisation Pattern is similar to the Synchronisation Pattern in the 3GPP TS 48.060, with some exceptions depending on the value of the EMBED Bit:

EMBED equal "0": the Synchronisation Pattern is exactly as described in the 3GPP TS 48.060;
EMBED equal "1": the Synchronisation Pattern contains an embedded TFO Message.

For the coding of the Data Bits see 3GPP TS 48.060.

For the coding of the Time Alignment Bits (T_Bits, T1.. T4) see 3GPP TS 48.060. The T_Bits normally correspond to the T_Bits received in the up-link TRAU Frame.

### 5.2.2	TFO Frames for the Adaptive Multi Rate Family

The TFO Frames for any narrow band AMR Codec Type use always 16 kbit/s sub-multiplexing on the A-Interface, regardless which sub-multiplexing is used on the Abis-Interfaces. Two different AMR_TFO Frame formats exist. One, called AMR_TFO_16k, is based on the TRAU Frame format for 16 kBit/s sub-multiplexing, as described in 3GPP TS 48.060. The other one, called AMR_TFO_8+8k, is based on the TRAU Frame format for 8 kbit/s sub-multiplexing, as described in 3GPP TS 48.061, with an added synchronisation pattern, to improve transmission and synchronisation quality on the A-Interface.

Optionally the TRAU frame format AMR_TRAU_8+8k may be used on the Abis-Interface for 16 kBit/s sub-multiplexing, when a TFO connection with HR_AMR on the distant side is established.

Additionnaly, a frame format using 16 kbit/s sub-multiplexing is defined for wide band AMR codec types, called AMR_WB_TFO_16k. It is based on the TRAU frame format for 16 kBit/s sub-multiplexing, as described in 3GPP TS 48.060

#### 5.2.2.1	TFO Frame Format AMR_TFO_16k

TFO Frames with format AMR_TFO_16k are derived from the TRAU Frames for Adaptive Multi Rate as defined in the 3GPP TS 48.060. The AMR_TFO_16k Frame structure is illustrated in Figure 5.2.2.1-1 below, using the same notations as in 3GPP TS 48.060. Table 5.2.2-1 defines the coding of the Control Bits for AMR TFO Frames. Note that additional TFO Configuration Parameters may be carried by the Data Bits of the TFO Frames, as defined in Annex C.

- Figure 5.2.2.1-1: Stucture of AMR_TFO_16k Frames

|  |  |  | Bit number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet no. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 1 | C1 | C2 | C3 | C4 | C5 | C6 | C7 |
| 3 | C8 | C9 | C10 | C11 | C12 | C13 | C14 | C15 |
| 4 | 1 | C16 | C17 | C18 | C19 | C20 | C21 | C22 |
| 5 | C23 | C24 | C25 | D1 | D2 | D3 | D4 | D5 |
| 6 | 1 | D6 | D7 | D8 | D9 | D10 | D11 | D12 |
| 7 | D13 | D14 | D15 | D16 | D17 | D18 | D19 | D20 |
| 8..36 |  |  |  |  |  |  |  |  |
| 37 | D238 | D239 | D240 | D241 | D242 | D243 | D244 | D245 |
| 38 | 1 | D246 | D247 | D248 | D249 | D250 | D251 | D252 |
| 39 | D253 | D254 | D255 | D256 | T1 | T2 | T3 | T4 |

- Table 5.2.2.1-2: Coding of the Control Bits for AMR_TFO_16k Frames

| Control Bits | Description |  | Comment |  |
| --- | --- | --- | --- | --- |
|  | FR_AMR, HR_AMR, UMTS_AMR_2 | UMTS_AMR | FR_AMR, HR_AMR | UMTS_AMR, UMTS_AMR_2 |
| C1 - C4 (0.0.0.1) 0.0.1.1 0.1.0.0 0.1.0.1 0.1.1.0 (1.1.0.1) | Frame_Type / Codec Type (GSM_FR) FR_AMR HR_AMR UMTS_AMR UMTS_AMR_2 (GSM_EFR) |  | The coding is different from the coding in TFO Messages. It is also not identical to the coding on Abis/Ater. The TRAU shall translate the coding between TRAU and TFO Frames |  |
| C5 0 1 | EMBED No TFO Message embedded A TFO Message is embedded |  | Indicates the presence of an embedded TFO Message. Set by the TRAU. |  |
| C6 – C8 | Set to "1.1.1"(see note) | Codec Mode Request (CMR)) | In GSM TRAU Frames, these bits carry part of the Time Alignment. They are set to 1.1.1 by the TRAU. | Coding as defined in 3GPP TS 48.060 |
| C9 - C11 0.0.0 0.0.1 0.1.0 0.1.1 1.0.0 1.0.1 1.1.0 1.1.1 | TFO andHandover_Notifications TFO_On TFO_Soon TFO_Off Handover_Soon Handover_Complete undefined undefined undefined |  | In GSM TRAU Frames these bits are part of the Time Alignment field. These bits are copied from TRAU frames to TFO Frames and vice versa. TFO_On is the default value in TFO Frames. |  |
| C12 | RIF (Request or Indication Flag) | set to 0 | Copied from the uplink TRAU Frame in GSM Generated by the Transcoder in 3G systems for FR_AMR and HR_AMR: The changes of the uplink Codec Mode, as received via the Iu Frames, are monitored. Whenever the Codec Mode changes, the RIF bit is set to "0". The next frames are then alternatingly marked with RIF = "1", "0", "1" and so on. |  |
| C14 – C16 | Config_Prot |  | Coding defined in Annex C. |  |
| C17 C18 | Mess No |  | Coding defined in Annex C. |  |
| C19 | DTXd (see note) |  | Copied from uplink TRAU Frame in GSM |  |
| C20 0 1 | TFOE TFO Disable TFO Enable |  | Copied from the uplink TRAU Frame in GSM Generated by the Transcoder in 3G systems with the same coding as in the 3GPP TS 48.060 |  |
| C21 – C22 1 1 1 0 0 1 0 0 | Frame_Classification "Speech_Good" "Speech_Degraded" "Speech_Bad" "No_Speech" |  | Copied from the uplink TRAU Frame in GSM Derived from the Frame Quality Indicator and Frame Type for 3G systems (see Table 5.2.2-2 below) |  |
| C23 – C25 | (see 3GPP TS 48.060) CMI (if RIF == 0) or CMR (if RIF == 1) or 0.0.0 (if Frame_Classification == 0.0) | Codec Mode Indication (CMI); (RIF ==0 is always the case in UMTS_AMR) | Carry CMI or CMR depending of the value of RIF, if the Frame Classification bits are different from "0 0" (No_Speech), and set to "000" otherwise. Copied from the uplink TRAU Frame in GSM Derived from the Frame Quality Indicator and Frame Type for 3G systems (see Table 5.2.2-2 below) | Coding as defined in 3GPP TS 48.060 |
| T1 - T4 | Time Alignment Bits |  | In GSM copied from the uplink TRAU Frame In 3G, generated by the TC (UMTS) based on Iu Frame arrival time(s) |  |

NOTE:	Any spare control bits shall be coded as binary "1". They are reserved for future use and may change.

- The CRC1 covering also the control bits C1..C25 shall be recomputed in the transcoders.

- The coding of the Data Bits is described in 3GPP TS 48.060.

In 3G systems, the Frame_Classification Bits must be derived from the Frame Quality Indicator (FQI) and Frame Type Index as defined in the 3GPP TS 26.101. Table 5.2.2.1-2 provides the conversion rules between the generic AMR Frames (as defined in 3GPP TS 26.101) and TFO Frames. In this table, the arrows in the fourth column indicate the direction for which the conversion applies.

NOTE 1:	A one-to-one relationship between Generic AMR Frames and TFO Frames does not always exist, but the conversion is always possible.

NOTE 2:	In the generic AMR Frames (3GPP TS 26.101), the differentiation between SID_FIRST and SID_UPDATE is done in the Data bits (SID Type Indicator). The Codec Mode Indication (CMI) is carried in 3G systems within the SID payload.

For 2G and 3G systems using the FR_AMR or HR_AMR Speech Codec Types, bits C23 - C25 shall carry either the Codec Mode Request (CMR) or the Codec Mode Indication (CMI), depending on the value of RIF, if the Frame_Classification bits are different from "0.0". If the Frame_Classification bits are equal to "0.0" (SID_First and SID_Update Frames), C23 - C25 are set to 0.0.0, and the CMI and CMR are carried in the data bits D35 - D40.

For 3G systems using the UMTS_AMR_2 or FR_AMR Speech Codec Types, the TC shall monitor the changes of the uplink Codec Mode, as received in the Iu Frames. Every time the Codec Mode changes in the Iu Frames the TC shall set RIF = "0" in the corresponding TFO Frame. The next TFO Frames are alternatively marked with RIF = "1", "0", "1" and so on.

NOTE 3:	Per definition for UMTS_AMR_2 or FR_AMR the UE shall select the phase of potential Codec Mode changes in uplink once at call set-up and shall not alter this later on. At call set-up TFO is not active and the TC has enough time to find the phase of the RIF by the proposed implicit method, before the first TFO Frame has to be sent.

Table 5.2.2.1-3: Conversion between Generic AMR Frames and AMR_TFO_16k Frames

| Generic AMR Frame |  |  |  | AMR_TFO_16k Frame |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Quality Indicator | Frame Type Index | TX_TYPE or RX_TYPE (see 3GPP TS 26.101) |  | Frame_ Classification C21 - C22 | CMI or CMR C23 - C25 | Data bits in No_Speech frames D32 .. D34 | Equivalent Frame Type in 3GPP TS 48.060) |
| 1 | 0-7 | SPEECH_GOOD | < > | 1 1 | 0-7 | - | Speech_Good |
| 1 | 0-7 | SPEECH_GOOD | < | 1 0 | 0-7 | - | Speech_Degraded |
| 0 | 0-7 | SPEECH_BAD | < > | 0 1 | 0-7 | - | Speech_Bad |
| 1 | 8 | SID_FIRST | < > | 0 0 | 0 0 0 | SID_First | No_Speech |
| 1 | 15 | NO_DATA | < | 0 0 | 0 0 0 | Onset | No_Speech |
| 1 | 8 | SID_UPDATE | < > | 0 0 | 0 0 0 | SID_Update | No_Speech |
| 0 | 8 | SID_BAD | < > | 0 0 | 0 0 0 | SID_Bad | No_Speech |
| 1 | 15 | NO_DATA | < > | 0 0 | 0 0 0 | No_Data | No_Speech |

The Synchronisation Pattern is similar to the Synchronisation Pattern in 3GPP TS 48.060, with some exceptions related to the value of the EMBED Bit:

EMBED equal "0": the Synchronisation Pattern is exactly as described in the 3GPP TS 48.060;
EMBED equal "1": the Synchronisation Pattern contains an embedded TFO Message.

For the coding of the Data Bits see 3GPP TS 48.060 and Annex C for the bits reserved for TFO Configuration Parameters.

For the coding of the Time Alignment Bits (T_Bits, T1 .. T4) see 3GPP TS 48.060 and Annex C. When the TFO Frame is generated by a GSM Network, the T_Bits normally correspond to the T_Bits received in the up-link TRAU Frame.

#### 5.2.2.2	TFO Frame Format AMR_TFO_8+8k

The AMR_TFO_8+8k Frame formats are derived from the GSM Adaptive Multi-Rate 8 kbit/s TRAU Frame formats defined in 3GPP TS 48.061. AMR Codec Modes with rates up to 7,40 kbit/s can be used with these AMR_TFO_8+8k Frame formats. The AMR_TFO_8+8k is described in an 8 kbit/s frame structure for the second LSB of the PCM samples and an 8 kbit/s synchronisation pattern for the LSB. The TFO Frame structures for the second LSB are illustrated in Figures 5.2.2.2-1 to 5.2.2.2-3 below, using the same notations as in 3GPP TS 48.061. Figure 5.2.2.2-4 defines the additional Synchronisation pattern for the LSB. Both frames shall be exactly synchronised on the A-Interface. This additional Synchronisation Pattern is sometimes modified by embedding of TFO Messages, indicated by the value of the EMBED bit:

EMBED equal "0": 	the Synchronisation Pattern is exactly as described in Figure 5.2.2.2-4;
EMBED equal "1": the Synchronisation Pattern contains an embedded TFO Message.

Figure 5.2.2.2-1: AMR_TFO_8+8k Frame Structure, second LSB:
NO_SPEECH frames and SPEECH frames for Codec Modes 4.75, 5.15 and 5.90 kbit/s

|  | Bit number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet no | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 1 | D1 | D2 | D3 | D4 | D5 | D6 | D7 |
| 3 | 1 | C1 | C2 | C3 | C4 | C5 | D8 | D9 |
| 4 | 0 | 1 | D10 | D11 | D12 | D13 | D14 | D15 |
| 5...19 | 1 |  |  |  |  |  |  |  |
| 20 | 1 | D121 | D122 | D123 | D124 | D125 | D126 | T |

- Figure 5.2.2.2-2: AMR_TFO_8+8k Frame Structure, second LSB:
 Speech frame for Codec Mode 6.70 kbit/s

|  | Bit number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet no | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 1 | D1 | D2 | D3 | D4 | D5 | D5 | D7 |
| 3 | 1 | C1 | C2 | C3 | D8 | D9 | D10 | D11 |
| 4...19 |  |  |  |  |  |  |  |  |
| 20 | D130 | D | D | D | D | D | D | D137 |

- Figure 5.2.2.2-3: AMR_TFO_8+8k Frame Structure, second LSB:
Speech frame for Codec Mode 7.40 kbit/s

|  | Bit number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet no | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | 0 | 0 | 1 | D1 | D2 | D3 | D4 | D5 |
| 2 | 0 | D6 | D7 | D8 | D9 | D10 | D11 | D12 |
| 3 | 1 | C1 | C2 | C3 | D13 | D14 | D15 | D16 |
| 4 | 0 | D17 | D18 | D19 | D20 | D21 | D22 | D23 |
| 5 | D24 | D | D | D | D | D | D | D31 |
| 6 … 19 |  |  |  |  |  |  |  |  |
| 20 | D144 | D145 | D146 | D147 | D148 | D149 | D150 | D151 |

- Figure 5.2.2.2-4: AMR_TFO_8+8k Frame Structure, LSB:
Additional Synchronisation Pattern

|  | Bit number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet no | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 1 | EMBED | EXTEND |  |  |  |  |  |
| 3…6 | 1 |  |  |  |  |  |  | 1 |
| 7 | 0 | 1 |  |  |  |  |  |  |
| 8 … 19 | 1 |  |  |  |  |  |  |  |
| 20 | 1 |  |  |  |  |  | 1 | 1 |

EXTEND equal "0":	The bits not defined in the Synchronisation Pattern described in Figure 
5.2.2.2-4 are "spare" (equal 1). In AMR_TFO_8+8k frames these undefined bit positions shall leave the original bits of the PCM coded speech unaltered.
In TRAU_8+8k frames these undefined bits shall be set to "1" (spare).

EXTEND equal "1":	The bits not defined in the Synchronisation Pattern described in Figure 
5.2.2.2-4 transport other parameters (tbd).

Table 5.2.2.2-1 defines the coding of the Control Bits for AMR TFO Frames. Note that additional TFO Configuration Parameters may be carried by the Data Bits of the TFO Frames, as defined in Annex C.

Table 5.2.2.2-1: The coding of the Control Bits (C1 .. C5) for AMR_TFO_8+8k Frames

| Control Bit | Description | No_Speech frames and Speech frames for 4.75, 5.15 and 5.9 kbit/s Codec Modes | 6.7 + 7.4 kbit/s Codec Mode |
| --- | --- | --- | --- |
| C1 – C3 | see 3GPP TS 48.061 | - For the low rates frame types, these bits jointly define the CMI, CMR and RIF. - For the No_Speech frame type, they define the RIF. - Copied from the uplink TRAU Frame in GSM. - Derived from the Frame Quality Indicator and Frame Type for 3G systems (see Table 5.2.2.2-2 below) | - For the 6.7 and 7.4 kbit/s speech frame, these bits jointly provide the CMR, RIF, and the Frame Classification. - Copied from the uplink TRAU Frame in GSM. - Derived from the Frame Quality Indicator and Frame Type for 3G systems (see Table 5.2.2.2-2 below) |
| C4 - C5 1 1 1 0 0 1 0 0 | Frame_Classification (No_Speech and low rates modes only) "Speech_Good" "Speech_Degraded" "Speech_Bad" "No_Speech" | - Copied from the uplink TRAU Frame in GSM - Derived from the Frame Quality Indicator and Frame Type for 3G systems (see Table 5.3.2-2 below) | The Frame_Classification is defined by bits C1-C3 in 6.70 and 7.40 kbit/s TFO Frames C4..C5 are not existent for this codec modes |

The CRC1 covering also the control bits shall be recomputed in the transcoders.

- The coding of the Data Bits is described in 3GPP TS 48.061 [Error: Reference source not found].

- For 3G systems, Table 5.2.2.2-2 provides the conversion rules between the generic AMR Frames as defined in 3GPP TS 26.101 and the AMR_TFO_8+8k Frames. In this table, the arrows in the fourth column indicate the direction for which the conversion applies. The Transcoder shall autonomously and internally generate a RIF alternating between the binary "0" and "1" values (see Annex D).

Table 5.2.2.2-2: Conversion between Generic AMR Frames and AMR_TFO_8+8k Frames

| Generic AMR Frame |  |  |  |  | TFO Frame for 8 kbit/s submultiplexing |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Quality Indicator | Frame Type Index | TX_TYPE or RX_TYPE (see 3GPP TS 26.101) |  |  | Bits C1 .. C3 | Bits C4 – C5 | Data bits in No_Speech frames D8 .. D10 | Equivalent Frame Type in 3GPP TS 48.061 | Frame Type |
| 1 | 0-2 | SPEECH_GOOD | < > |  | as 3GPP TS 48.061 | 1 1 | - | Speech_Good | 4,75 kbit/s, 5,15 kbit/s, 5,90 kbit/s Modes |
| 1 | 0-2 | SPEECH_GOOD | < |  | as 3GPP TS 48.061 | 1 0 | - | Speech_Degraded |  |
| 0 | 0-2 | SPEECH_BAD | < > |  | as 3GPP TS 48.061 | 0 1 | - | Speech_Bad |  |
| 1 | 3-4 | SPEECH_GOOD | < > |  | as 3GPP TS 48.061 | Speech bits | - | Speech_Good | 6,70 kbit/s, 7,40 kbit/s Modes |
| 1 | 3-4 | SPEECH_GOOD | < |  | as 3GPP TS 48.061 | Speech bits | - | Speech_Degraded |  |
| 0 | 3-4 | SPEECH_BAD | < > |  | as 3GPP TS 48.061 | Speech bits | - | Speech_Bad |  |
| 1 | 8 | SID_FIRST | < > |  | as 3GPP TS 48.061 | 0 0 | SID_First | No_Speech | No Speech |
| 1 | 15 | NO_DATA | < |  | as 3GPP TS 48.061 | 0 0 | Onset | No_Speech |  |
| 1 | 8 | SID_UPDATE | < > |  | as 3GPP TS 48.061 | 0 0 | SID_Update | No_Speech |  |
| 0 | 8 | SID_BAD | < > |  | as 3GPP TS 48.061 | 0 0 | SID_Bad | No_Speech |  |
| 1 | 15 | NO_DATA | < > |  | as 3GPP TS 48.061 | 0 0 | No_Data | No_Speech |  |

The Synchronisation Pattern in the second LSB of the PCM samples is identical to the Synchronisation Pattern in 3GPP TS 48.061. Embedding of TFO Messages has no influence on this synchronisation pattern.

For the coding of the Time Alignment Bit (T Bit) for all modes below 5.9 kbit/s and the No_Seech Frame, see 3GPP TS 48.061.The T-Bit in a TFO Frame normally corresponds to the T_Bit received in the up-link TRAU Frame.

#### 5.2.2.3	TFO Frame Format FR_AMR_WB_TFO_16k

TFO Frames with format AMR_WB_TFO_16k are derived from the TRAU Frames for Wide Band Adaptive Multi Rate as defined in the 3GPP TS 48.060. The AMR_WB_TFO_16k Frame structure is illustrated in Figures 5.2.2.3-1 and 5.2.2.3-2 below, using the same notations as in 3GPP TS 48.060. Table 5.2.2.3-3 defines the coding of the Control Bits for AMR WB TFO Frames. Note that additional TFO Configuration Parameters may be carried by the Data Bits of the TFO Frames, as defined in Annex C.

- Figure 5.2.2.3-1: Stucture of AMR_WB_TFO_16k Frames for Codec Mode 14.25 kbit/s

|  |  |  | Bit number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet no. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 1 | C1 | C2 | C3 | C4 | C5 | C6 | C7 |
| 3 | C8 | C9 | C10 | C11 | C12 | C13 | D1 | D2 |
| 4 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 |
| 5 | D11 | D12 | D13 | D14 | D15 | D16 | D17 | D18 |
| 6 | D19 | D20 | D21 | D22 | D23 | D24 | D25 | D26 |
| 7 | D27 | D28 | D29 | D30 | D31 | D32 | D33 | D34 |
| 8..36 |  |  |  |  |  |  |  |  |
| 37 | D267 | D268 | D269 | D270 | D271 | D272 | D273 | D274 |
| 38 | D275 | D276 | D277 | D278 | D279 | D280 | D281 | D282 |
| 39 | D283 | D284 | D285 | D286 | D287 | D288 | T1 | T2 |

- Figure 5.2.2.3-2: Stucture of AMR_WB_TFO_16k Frames for No_Speech frames, Codec Modes 12.65, 8.85 and 6.60 kbit/s

|  |  |  | Bit number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet no. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 1 | C1 | C2 | C3 | C4 | C5 | C6 | C7 |
| 3 | C8 | C9 | C10 | C11 | C12 | C13 | D1 | D2 |
| 4 | 1 | D3 | D | D5 | D6 | D7 | D8 | D9 |
| 5 | D10 | D11 | D12 | D13 | D14 | D15 | D16 | D17 |
| 6 | 1 | D18 | D19 | D20 | D21 | D22 | D23 | D24 |
| 7 | D25 | D26 | D27 | D28 | D29 | D30 | D31 | D32 |
| 8..36 |  |  |  |  |  |  |  |  |
| 37 | D250 | D251 | D252 | D253 | D254 | D255 | D256 | D257 |
| 38 | 1 | D258 | D259 | D260 | D261 | D262 | D263 | D264 |
| 39 | D265 | D266 | D267 | D268 | D269 | D270 | T1 | T2 |

- Table 5.2.2.3-3: Coding of the Control Bits for AMR_WB_TFO_16k Frames

| Control Bits | Description |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
|  | FR_AMR_WB, UMTS_AMR_WB |  |  | FR_AMR_WB, UMTS_AMR_WB |  |
| C1 – C4 1.0.0.1 1.0.1.0 | Frame_Type / Codec Type FR_AMR_WB UMTS_AMR_WB |  |  | The coding is different from the coding in TFO messages. It is also not identical to the coding on Abis/Ater. The TRAU shall translate the coding between TRAU and TFO frames. |  |
| C5 0 1 | EMBED No TFO message embedded A TFO message embedded |  |  | Indicates the presence of an embedded TFO message. Set by the TRAU. |  |
| C6 | RIF (Request or Indication flag) |  |  | Copied from the uplink TRAU frame in GSM. Generated by the Transocder in 3G systems for FR_AMR and HR_AMR. The changes of the uplink Codec Mode, as received via the Iu Frames, are monitored. Whenever the Codec Mode changes, the RIF bit is set to “0”. The next frames are then alternatingly marked with RIF = “1”, “0”, “1” and so on. |  |
| C7 | set to 1 |  |  | Copied from the uplink TRAU Frame in GSM. Generated by the TC in UMTS. |  |
| C8 | DTXd |  |  | Copied from uplink TRAU frame in GSM. |  |
| C9 0 1 | TFOE TFO Disabled TFO Enable |  |  | Copied from the uplink TRAU frame in GSM. Generated by the Transcoder in 3G systems with the same coding as in the 3GPP TS 48.060. |  |
| C10-C11 1 1 1 0 0 1 0 0 | Frame_Classification “Speech_Good” “Speech_Degraded” “Speech_Bad” “No_Speech” |  |  | Copied from the uplink TRAU frame in GSM. Derived from the Frame Quality Indicator and Frame Type for 3G systems (see Table 5.2.2.3-4 below). |  |
| C12-C13 | (see 3GPP TS 48.060) CMI (if RIF == 0) or CMR (if RIF == 1) or 0.0.0 (if Frame Classification == 0.0) |  |  | Carry CMI or CMR depening of the value of RIF, if the Frame Classification bits are different from “0 0” (No_Speech), and set to “000” otherwise. Copied from the uplink TRAU Frame in GSM. Derived from the Frame Quality Indicator and Frame Type for 3G systems (see Table 5.2.2.3-4). |  |
| T1-T2 | Time alignment bits |  |  | In GSM copied from the uplink TRAU frame. In 3G, generated by the TC (UMTS) based on Iu Frame arrival time. |  |

NOTE:	Any spare control bits shall be coded as binary "1". They are reserved for future use and may change.

- The CRC1 covering also the control bits C1..C13 shall be recomputed in the transcoders.

- The coding of the Data Bits is described in 3GPP TS 48.060.

In 3G systems, the Frame_Classification Bits must be derived from the Frame Quality Indicator (FQI) and Frame Type Index as defined in the 3GPP TS 26.101. Table 5.2.2.3-4 provides the conversion rules between the generic WB AMR Frames (as defined in 3GPP TS 26.101) and TFO Frames. In this table, the arrows in the fourth column indicate the direction for which the conversion applies.

NOTE 1:	A one-to-one relationship between Generic WB AMR Frames and TFO Frames does not always exist, but the conversion is always possible.

NOTE 2:	In the generic WB AMR Frames (3GPP TS 26.101), the differentiation between SID_FIRST and SID_UPDATE is done in the Data bits (SID Type Indicator). The Codec Mode Indication (CMI) is carried in 3G systems within the SID payload.

For 2G using the FR_AMR-WB or UMTS_AMR-WB and 3G systems using UMTS_AMR-WB, bits C12 – C13 shall carry either the Codec Mode Request (CMR) or the Codec Mode Indication (CMI), depending on the value of RIF, if the Frame_Classification bits are different from "0.0". If the Frame_Classification bits are equal to "0.0" (NO_SPEECH   Frames), C12 – C13 are set to 0.0, and the CMI and CMR are carried in the data bits D35 - D40.

3G systems using the UMTS_AMR-WB Speech Codec Type, the TC shall monitor the changes of the uplink Codec Mode, as received in the Iu Frames. Every time the Codec Mode changes in the Iu Frames the TC shall set RIF = "0" in the corresponding TFO Frame. The next TFO Frames are alternatively marked with RIF = "1", "0", "1" and so on.

NOTE 3:	Per definition for FR_WB-AMR the UE selects the phase of potential Codec Mode changes in uplink once at call set-up and does not alter this later on. At call set-up TFO is not active and the TC has enough time to find the phase of the RIF by the proposed implicit method, before the first TFO Frame has to be sent.

Table 5.2.2.3-4: Conversion between Generic AMR Frames and FR_WB_AMR_TFO_16k Frames

| Generic AMR Frame |  |  |  | AMR_WB_TFO_16k Frame |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Quality Indicator | Frame Type Index | TX_TYPE or RX_TYPE (see 3GPP TS 26.101) |  | Frame_ Classification C10 – C11 | CMI or CMR C12 – C13 | Data bits in No_Speech frames D32 .. D34 | Equivalent Frame Type in 3GPP TS 48.060) |
| 1 | 0-7 | SPEECH_GOOD | < > | 1 1 | 0-7 | - | Speech_Good |
| 1 | 0-7 | SPEECH_GOOD | < | 1 0 | 0-7 | - | Speech_Degraded |
| 0 | 0-7 | SPEECH_BAD | < > | 0 1 | 0-7 | - | Speech_Bad |
| 1 | 8 | SID_FIRST | < > | 0 0 | 0 0 0 | SID_First | No_Speech |
| 1 | 15 | NO_DATA | < | 0 0 | 0 0 0 | Onset | No_Speech |
| 1 | 8 | SID_UPDATE | < > | 0 0 | 0 0 0 | SID_Update | No_Speech |
| 0 | 8 | SID_BAD | < > | 0 0 | 0 0 0 | SID_Bad | No_Speech |
| 1 | 15 | NO_DATA | < > | 0 0 | 0 0 0 | No_Data | No_Speech |

The Synchronisation Pattern is similar to the Synchronisation Pattern in 3GPP TS 48.060, with some exceptions related to the value of the EMBED Bit:

EMBED equal "0": the Synchronisation Pattern is exactly as described in the 3GPP TS 48.060;
EMBED equal "1": the Synchronisation Pattern contains an embedded TFO Message.

For the coding of the Data Bits see 3GPP TS 48.060 and Annex C for the bits reserved for TFO Configuration Parameters.

For the coding of the Time Alignment Bits (T_Bits, T1 .. T2) see 3GPP TS 48.060 and Annex C. When the TFO Frame is generated by a GSM Network, the T_Bits normally correspond to the T_Bits received in the up-link TRAU Frame.

### 5.2.3	Transmission of the bits of 16 kbit/s TFO Frames

For the purpose of this description the 320 bits of one TFO Frame are arranged in 40 rows (0..39), with 8 bit each (1..8: one octet) as in 3GPP TS 48.060.

- The bits of 16 kbit/s TFO Frames are transmitted in the following order:

- Bit m of octet n, shall be transmitted in the Least Significant Bit of the

PCM sample k = n*4 + (m+1)/2	for m = (1, 3, 5, 7) and n = (0..39).

- Bit m of octet n shall be transmitted in the second Least Significant Bit of the

PCM sample k = n*4 +  m/2	for  m = (2, 4, 6, 8) and n = (0..39).

PCM sample (k=1) is the first PCM sample of the TFO Frame, which follows the received uplink TRAU frame with a small delay (Tultfo), as described in clause 8, see figure 8.1.2-1.

### 5.2.4	Transmission of the bits of AMR_TFO_8+8k Frames

For the purpose of this description the 160+160 bits of one AMR_TFO_8+8k frame are arranged in 20 rows (1..20), with 8 bit each (1..8: one octet) as shown in Figures 5.2.2.2-1 to 5.2.2.2-4.

The bits of AMR_TFO_8+8k frames are transmitted in the following order:

- Bit m of octet n of the additional synchronisation pattern described in Figure 5.2.2.2-4 shall be transmitted in the Least Significant Bit of the

PCM sample k = (n-1)*8+m;	with  m = (1..8) and n = (1..20).

- Bit m of octet n of the No_Speech and Speech frames as described in Figures 5.2.2.2-1 to 5.2.2.2-3 shall be transmitted in the Second Least Significant Bit of the

PCM sample k = (n-1)*8+m;	with  m = (1..8) and n = (1..20).

PCM sample (k=1) is the first PCM sample of the TFO Frame, which follows the received uplink TRAU frame with a small delay (Tultfo), as described in clause 8, see figure 8.1.2-1.

### 5.2.5	Optional AMR_TRAU_8+8k Frames

For TFO Connections with FR_AMR on the local side and HR_AMR on the distant side the local side may use the AMR_TRAU_8+8k frame format after TFO has been established. The AMR_TRAU_8+8k Frame is based on the TRAU Frame formats for the AMR for 8 kBit/s sub-multiplexing as defined in 3GPP TS 28.061 (TRAU_8k), with the additional Synchronisation pattern as defined in Figure 5.2.2.2-4. The differences to AMR_TFO_8+8k frames are:

- the additional synchronisation pattern shall be transmitted in the Second LSBs of the 16 kbit/s sub-multiplexed channel, while the TRAU_8k frames shall be transmitted in the LSBs;

- no embedded TFO Messages shall exist in TRAU_8+8k frames;

- the EMBED bit shall be set to "0";

- the EXTEND bit shall be set to "0";

- undefined bits in Figure 5.2.2.2-4 shall be set to "1" (spare) in TRAU_8+8k frames.

The potential transition from regular TRAU_16k frames to AMR_TRAU_8+8k frames shall be triggered by the FR_TRAU with TFO_Soon and Dis_Req (including the distant Codec Type: HR_AMR) in downlink direction.

If the BTS applies the optional AMR_TRAU_8+8k format, then the BTS shall respond with the acknowledging  TFO_Soon in the first AMR_TRAU_8+8k frame in uplink. This will result in a small additional delay for the decoded PCM samples, which the TRAU shall handle by proper concealment techniques. The delay for TFO Messages and TFO Frames is, however, not increased: since no format conversion is necessary in the TRAU the delay for AMR_TFO_8+8k frames is minimised. After TFO has been established the TRAU shall change from TRAU_16k to AMR_TRAU_8+8k in downlink with the reception of the first AMR_TFO_8+8k frame.

If the BTS does not apply the AMR_TRAU_8+8k frame format in uplink, the TRAU shall also not use this in downlink. The TRAU shall perform format conversion in uplink from TRAU_16k format to AMR_TFO_8+8k format and in downlink from AMR_TFO_8+8k format to TRAU_16k format. This will cause an additional delay of TFO Messages and TFO Frames, which shall be handled by inserting the necessary number of T_Bits. This format conversion causes also an additional delay in downlink, which the BTS shall handle by proper buffering technique.

## 5.3	TFO Frames for 8 kbit/s sub-multiplexing

### 5.3.1	TFO Frame for the GSM Half Rate

The GSM Half Rate (GSM_HR) TFO Frames are always based on the uplink GSM Half Rate TRAU Frames for 8 kbit/s submultiplexing scheme, as defined in the 3GPP TS 48.061.

If GSM_HR TRAU Frames with 16 kbit/s submultiplexing are used on the Abis/Ater interface, then the Control and Extended Control Bits for the 8 kbit/s TFO Frame need to be generated on basis of the received Control Bits from the TRAU Frame.

The coding of the Control Bits (C1 .. C9) is defined by the following Table 5.3.1-1:

- Table 5.3.1-1: Coding of the Control Bits (C1 .. C9) for the GSM_HR

| Control Bit | Description | Comment |
| --- | --- | --- |
| C1 - C4 0.0.0.1 | Frame Type GSM_HR | All other code words are reserved. |
| C5 | EMBED | Indicates the presence of an embedded TFO Message |
| C7 - C8 | spare |  |
| C9 | DTXd | Copied from the uplink TRAU Frame |

Any spare control bits shall be coded as binary "1". They are reserved for future use and may change.

The Synchronisation Pattern is similar to the Synchronisation Pattern in the 3GPP TS 48.061, with some exceptions depending on the value of the EMBED Bit:

EMBED equal "0": the Synchronisation Pattern is exactly as described in the 3GPP TS 48.061;
EMBED equal "1": the Synchronisation Pattern contains an embedded TFO Message.

- Coding of the Extended Control Bits (XC1 .. XC6):

XC1 is copied from the uplink TRAU Frame.
XC2 .. XC6: These bits are normally copied from the 8 kbit/s TRAU Frame.
All other codes are reserved.

For the coding of the Data Bits see 3GPP TS 48.061.

For the coding of the Time Alignment Bits see 3GPP TS 48.061. The T_Bits normally correspond to the T_Bits received in the up-link TRAU Frame.

### 5.3.2	Transmission of the bits of 8 kbit/s TFO frames

For the purpose of this description the 160 bits of one frame are arranged in 20 rows (1..20), with 8 bit each (1..8: one octet) as in 3GPP TS 48.061.

The bits of 8 kbit/s TFO Frames are transmitted in the following order:

- Bit m of octet n shall be transmitted in the Least Significant Bit of the

PCM sample k = (n-1)*8+m;	with  m = (1..8) and n = (1..20).

PCM sample (k=1) is the first PCM sample of the TFO frame which follows the received uplink TRAU frame with a small delay (Tultfo), as described in clause 8, see figure 8.1.2-1.

## 5.3a	TFO Frames for 32 kbit/s sub-multiplexing

#### 5.3a.1	TFO Frame Format AMR_WB_TFO_32k

TFO Frames with format AMR_WB_TFO_32k are derived from the TRAU Frames for Wide Band Adaptive Multi Rate as defined in the 3GPP TS 48.060. The AMR_WB_TFO_32k Frame structure is illustrated in Figures 5.3a.1-1 and 5.3a.1-2 below, using the same notations as in 3GPP TS 48.060. Table 5.3a.1-3 defines the coding of the Control Bits for AMR WB TFO Frames. Note that additional TFO Configuration Parameters may be carried by the Data Bits of the TFO Frames, as defined in Annex C.

In the following, the control bits C1 to C25 refer to both sub-channels, the control bits C1a to C25a refer to the sub-channel a and the control bits C1b to C25b refer to the sub-channel b.

- Figure 5.3a.1-1: Stucture of AMR_WB_TFO_32k Frames, first channel (channel a)

|  |  |  | Bit number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet no. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 1 | C1a | C2a | C3a | C4a | C5a | C6a | C7a |
| 3 | C8a | C9a | C10a | C11a | C12a | C13a | C14a | C15a |
| 4 | 1 | C16a | C17a | C18a | C19a | C20a | C21a | C22a |
| 5 | C23a | C24a | C25a | D1a | D2a | D3a | D4a | D5a |
| 6 | 1 | D6a | D7a | D8a | D9a | D10a | D11a | D12a |
| 7 | D13a | D14a | D15a | D16a | D17a | D18a | D19a | D20a |
| 8..36 |  |  |  |  |  |  |  |  |
| 37 | D238a | D239a | D240a | D241a | D242a | D243a | D244a | D245a |
| 38 | 1 | D246a | D247a | D248a | D249a | D250a | D251a | D252a |
| 39 | D253a | D254a | D255a | D256a | T1 | T2 | T3 | T4 |

- Figure 5.3a.1-2: Stucture of AMR_WB_TFO_32k Frames, second channel (channel b)

|  |  |  | Bit number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet no. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 1 | C1b | C2b | C3b | C4b | C5b | C6b | C7b |
| 3 | C8b | C9b | C10b | C11b | C12b | C13b | C14b | C15b |
| 4 | 1 | C16b | C17b | C18b | C19b | C20b | C21b | C22b |
| 5 | C23b | C24b | C25b | D1b | D2b | D3b | D4b | D5b |
| 6 | 1 | D6b | D7b | D8b | D9b | D10b | D11b | D12b |
| 7 | D13b | D14b | D15b | D16b | D17b | D18b | D19b | D20b |
| 8..36 |  |  |  |  |  |  |  |  |
| 37 | D238b | D239b | D240b | D241b | D242b | D243b | D244b | D245bb |
| 38 | 1 | D246b | D247b | D248b | D249b | D250b | D251b | D252b |
| 39 | D253b | D254b | D255b | D256b | T1 | T2 | T3 | T4 |

- Table 5.3a.1-3: Coding of the Control Bits for AMR_WB_TFO_32k Frames

| Control Bits | Description |  | Comment |  |
| --- | --- | --- | --- | --- |
|  | FR_AMR-WB, UMTS_AMR-WB |  | FR_AMR-WB, UMTS_AMR-WB |  |
| C1 - C4 1.0.1.1 xxxx | Frame_Type / Codec Type FR_AMR_WB UMTS_AMR_WB |  | The coding is different from the coding in TFO Messages. It is also not identical to the coding on Abis/Ater. The TRAU shall translate the coding between TRAU and TFO Frames |  |
| C5a 0 1 | EMBED No TFO Message embedded A TFO Message is embedded |  | Indicates the presence of an embedded TFO Message. Set by the TRAU. |  |
| C5b | Set to 1 (spare) |  |  |  |
| C6 – C8 | Set to "1.1.1"(see note) |  | In GSM TRAU Frames, these bits carry part of the Time Alignment. They are set to 1.1.1 by the TRAU. |  |
| C9 - C11 0.0.0 0.0.1 0.1.0 0.1.1 1.0.0 1.0.1 1.1.0 1.1.1 | TFO and Handover_Notifications TFO_On TFO_Soon TFO_Off Handover_Soon Handover_Complete undefined undefined undefined |  | In GSM TRAU Frames these bits are part of the Time Alignment field. These bits are copied from TRAU frames to TFO Frames and vice versa. TFO_On is the default value in TFO Frames. |  |
| C12 | RIF (Request or Indication Flag) |  | Copied from the uplink TRAU Frame in GSM Generated by the Transcoder in 3G systems for FR_AMR and HR_AMR: The changes of the uplink Codec Mode, as received via the Iu Frames, are monitored. Whenever the Codec Mode changes, the RIF bit is set to "0". The next frames are then alternatingly marked with RIF = "1", "0", "1" and so on. |  |
| C13 | set to 1 |  | Copied from the uplink TRAU Frame in GSM. Generated by the TC in UMTS. |  |
| C14 – C16 | Config_Prot |  | Coding defined in Annex C. |  |
| C17 C18 | Mess No |  | Coding defined in Annex C. |  |
| C19 | DTXd (see note) |  | Copied from uplink TRAU Frame in GSM |  |
| C20 0 1 | TFOE TFO Disable TFO Enable |  | Copied from the uplink TRAU Frame in GSM Generated by the Transcoder in 3G systems with the same coding as in the 3GPP TS 48.060 |  |
| C21 – C22 1 1 1 0 0 1 0 0 | Frame_Classification "Speech_Good" "Speech_Degraded" "Speech_Bad" "No_Speech" |  | Copied from the uplink TRAU Frame in GSM Derived from the Frame Quality Indicator and Frame Type for 3G systems (see Table 5.3a.1-4 below) |  |
| C23a – C25a and C23b-C25b | (see 3GPP TS 48.060) CMI (if RIF == 0) or CMR (if RIF == 1) or 0.0.0 (if Frame_Classification == 0.0) |  | Carry CMI or CMR depending of the value of RIF, if the Frame Classification bits are different from "0 0" (No_Speech), and set to "000" otherwise. Copied from the uplink TRAU Frame in GSM Derived from the Frame Quality Indicator and Frame Type for 3G systems (see Table 5.3a.1-4 below) |  |
| T1 - T4 | Time Alignment Bits |  | In GSM copied from the uplink TRAU Frame In 3G, generated by the TC (UMTS) based on Iu Frame arrival time(s) |  |

NOTE:	Any spare control bits shall be coded as binary "1". They are reserved for future use and may change.

- The CRC1 covering also the control bits C1…C25 shall be recomputed in the transcoders.

- The coding of the Data Bits is described in 3GPP TS 48.060.

In 3G systems, the Frame_Classification Bits must be derived from the Frame Quality Indicator (FQI) and Frame Type Index as defined in the 3GPP TS 26.101. Table 5.3a.1-4 provides the conversion rules between the generic WB AMR Frames (as defined in 3GPP TS 26.101) and TFO Frames. In this table, the arrows in the fourth column indicate the direction for which the conversion applies.

NOTE 1:	A one-to-one relationship between Generic WB AMR Frames and TFO Frames does not always exist, but the conversion is always possible.

NOTE 2:	In the generic WB AMR Frames (3GPP TS 26.101), the differentiation between SID_FIRST and SID_UPDATE is done in the Data bits (SID Type Indicator). The Codec Mode Indication (CMI) is carried in 3G systems within the SID payload.

For 2G systems using the FR_AMR-WB or UMTS_AMR-WB and 3G systems using UMTS_AMR-WB, bits C23a – C25a and C23b-C25b shall carry either the Codec Mode Request (CMR) or the Codec Mode Indication (CMI), depending on the value of RIF, if the Frame_Classification bits are different from "0.0". If the Frame_Classification bits are equal to "0.0" (NO_SPEECH Frames), C23a – C25a and C23b – C25b are set to 0.0, and the CMI and CMR are carried in the data bits D35 - D40.

3G systems using the UMTS_AMR-WB Speech Codec Type, the TC shall monitor the changes of the uplink Codec Mode, as received in the Iu Frames. Every time the Codec Mode changes in the Iu Frames the TC shall set RIF = "0" in the corresponding TFO Frame. The next TFO Frames are alternatively marked with RIF = "1", "0", "1" and so on.

NOTE 3:	Per definition for FR_AMR-WB the UE selects the phase of potential Codec Mode changes in uplink once at call set-up and does not alter this later on. At call set-up TFO is not active and the TC has enough time to find the phase of the RIF by the proposed implicit method, before the first TFO Frame has to be sent.

Table 5.3a.1-4: Conversion between Generic AMR Frames and FR_WB_AMR_TFO_32k Frames

| Generic AMR Frame |  |  |  | AMR_WB_TFO_32k Frame |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Quality Indicator | Frame Type Index | TX_TYPE or RX_TYPE (see 3GPP TS 26.101) |  | Frame_ Classification C21 – C22 | CMI or CMR C23b – C25b C23a – C25a | Data bits in No_Speech frames D32 ... D34 | Equivalent Frame Type in 3GPP TS 48.060) |
| 1 | 0-9 | SPEECH_GOOD | < > | 1 1 | 0-4 0-7 | - | Speech_Good |
| 1 | 0-9 | SPEECH_GOOD | < | 1 0 | 0-4 0-7 | - | Speech_Degraded |
| 0 | 0-9 | SPEECH_BAD | < > | 0 1 | 0-4 0-7 | - | Speech_Bad |
| 1 | 8 | SID_FIRST | < > | 0 0 | 0 0 0 0 0 0 | SID_First | No_Speech |
| 1 | 15 | NO_DATA | < | 0 0 | 0 0 0 0 0 0 | Onset | No_Speech |
| 1 | 8 | SID_UPDATE | < > | 0 0 | 0 0 0 0 0 0 | SID_Update | No_Speech |
| 0 | 8 | SID_BAD | < > | 0 0 | 0 0 0 0 0 0 | SID_Bad | No_Speech |
| 1 | 15 | NO_DATA | < > | 0 0 | 0 0 0 0 0 0 | No_Data | No_Speech |

The Synchronisation Pattern is similar to the Synchronisation Pattern in 3GPP TS 48.060, with some exceptions related to the value of the EMBED Bit:

EMBED equal "0": the Synchronisation Pattern is exactly as described in the 3GPP TS 48.060;
EMBED equal "1": the Synchronisation Pattern contains an embedded TFO Message.

For the coding of the Data Bits see 3GPP TS 48.060 and Annex C for the bits reserved for TFO Configuration Parameters.

For the coding of the Time Alignment Bits (T_Bits, T1... T4) see 3GPP TS 48.060 and Annex C. When the TFO Frame is generated by a GSM Network, the T_Bits normally correspond to the T_Bits received in the up-link TRAU Frame.

### 5.3a.2	Transmission of the bits of 32 kbit/s TFO Frames

For the purpose of this description the 640 bits of one TFO Frame are arranged in 2 x 40 rows (0..39a, 0…39b), with 8 bit each (1..8: one octet) as in 3GPP TS 48.060.

- The bits of 32 kbit/s TFO Frames are transmitted in the following order:

- Bit m of octet n, shall be transmitted in the Least Significant Bit of the

PCM sample k = n*4 + (m+1)/2	for m = (1, 3, 5, 7) and n = (0a...39a).

- Bit m of octet n shall be transmitted in the second Least Significant Bit of the

PCM sample k = n*4 +  m/2	for  m = (2, 4, 6, 8) and n = (0a..39a).

- Bit m of octet n, shall be transmitted in the third Least Significant Bit of the

PCM sample k = n*4 + (m+1)/2	for m = (1, 3, 5, 7) and n = (0b...39b).

- Bit m of octet n shall be transmitted in the forth Least Significant Bit of the

PCM sample k = n*4 +  m/2	for  m = (2, 4, 6, 8) and n = (0b..39b).

PCM sample (k=1) is the first PCM sample of the TFO Frame, which follows the received uplink TRAU frame with a small delay (Tultfo), as described in clause 8, see figure 8.1.2-1.

## 5.4	Determination of the TFO Frame format

The TFO Frame format is depending on the Codec Types at both ends of the TFO connection.

For the GSM FR and GSM EFR Speech Codec Types, the TFO Frame format shall be 16 kbit/s (see clause 5.2.1).

For the GSM HR Speech Codec Type, the TFO Frame format shall be 8 kbit/s (see clause 5.3.1).

For any TFO connection with at least one side using the HR_AMR (HR_AMR–HR_AMR, HR_AMR–FR_AMR, HR_AMR-UMTS_AMR_2) the TFO frame format shall be AMR_TFO_8+8k (see clause 5.2.2.2).

For  the following AMR TFO connections : UMTS_AMR-UMTS_AMR, UMTS_AMR_2-UMTS_AMR_2 and UMTS_AMR_2-FR_AMR-FR_AMR, the TFO Frame format shall be AMR_TFO_16k (see clause 5.2.2.1).

For any AMR-WB TFO connection not supporting codec modes higher than 14,25 kbit/s, the TFO frame format shall be AMR_WB_TFO_16k (see 5.2.2.3).

For all other AMR WB TFO connections, the TFO frame format shall be AMR_WB_TFO_32k (see 5.3a).