3GPP SA4 Video SWG Telco on 5G Video #3	S4-AHVIC250

12th May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | <CR#> | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Comments on 4K TV scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 2020-04-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 4K is not included yet |  |  |  |  |  |  |  |  |
| Summary of change: |  | Focus on 4K-TV without temporal sublayering |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This assumes that AHVIC-246 is accepted |  |  |  |  |  |  |  |  |

===== CHANGE 1 =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[3]	3GPP TS 26.116: "Television (TV) over 3GPP services; Video profiles".

[4]	3GPP TS 26.118: "3GPP Virtual reality profiles for streaming applications".

[5]	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[6]	3GPP TR 26.928: "Extended Reality over 5G".

[7]	Recommendation ITU-T H.264 (06/2017): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2014/Amd.3 2016: "Information technology - Coding of audio-visual objects - Part 10: Advanced Video Coding".

[8]	Recommendation ITU-T H.265 (11/2019): "High efficiency video coding" | ISO/IEC 23008-2:2020: "High Efficiency Coding and Media Delivery in Heterogeneous Environments - Part 2: High Efficiency Video Coding".

[9]	2020 Mobile Internet Phenomena Report, accessible here: https://www.sandvine.com/download-report-mobile-internet-phenomena-report-2020-sandvine, February 2020.

[10]	2019 Ericsson Mobility Report, accessible here: https://www.ericsson.com/4acd7e/assets/local/mobility-report/documents/2019/emr-november-2019.pdf, November 2019.

[11]	T. Fautier, "New Codecs for 5G", DASH-IF Workshop on "Media Streaming meets 5G", December 2019, accessible here: https://dashif.org/docs/workshop-2019/04-thierry%20fautier%20-%20Harmonic%20Codec%20Comparison%205G%20Media%20Workshop_Final%20v3.pdf

[12]	Bitmovin Video Developer Report, accessible here: https://go.bitmovin.com/video-developer-report-2019, September 2019.

[13]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[14]	Recommendation ITU-R BT.709-6 (06/2015): "Parameter values for the HDTV standards for production and international programme exchange".

[15]	Recommendation ITU-R BT.2020-2 (10/2015): "Parameter values for ultra-high definition television systems for production and international programme exchange".

[16]	Recommendation ITU-R BT.2100-1 (06/2017): "Image parameter values for high dynamic range television for use in production and international programme exchange".

[17]	“You can watch Netflix on any screen you want, but you’re probably watching it on a TV”, https://www.vox.com/2018/3/7/17094610/netflix-70-percent-tv-viewing-statistics

[18]	YouTube Revenue and Usage Statistics (2020), https://www.businessofapps.com/data/youtube-statistics/

[19]	Ultra HD Forum service tracker B2C, https://ultrahdforum.org/uhd-service-tracker/“

[20]	HIS Market “4K-TV and UHD: the whole picture”, https://cdn.ihs.com/www/pdf/4ktv-uhd-ebook.pdf

[21]	R. Jullian, Y. Chen, F. Galpin, E. François, M. Kerdranvat (InterDigital), “Extra results to JVET-N605 “Comparative study of video coding solutions VVC, AV1 and EVC versus HEVC””, document JVET-O0898, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[22]	R. Jullian, Y. Chen, F. Galpin, E. François, M. Kerdranvat (InterDigital), “Comparative study of video coding solutions VVC, AV1 and EVC versus HEVC”, document JVET-N0605, 14th Meeting: Geneva, CH, 19–27 March 2019.

[23]	S. Iwamura, S. Nemoto, A. Ichigaya, “[AHG13] Compression performance analysis for 4K and 8K HLG test sequences”, document JVET-N0828, 14th Meeting: Geneva, CH, 19–27 March 2019.

[24]	A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video,” document JVET-P2011, 16th JVET meeting: Geneva, CH, 1–11 Oct. 2019.

===== End of CHANGE 1 =====

===== CHANGE 2 =====

## 6.3	Scenario 2: 4K-TV

### 6.3.1	Motivation

Streaming towards mobile devices is undoubtly the first natural use-case expected for 5G-media streaming. However, consumption of video services on fixed receivers (e.g. TV sets) remains a preferable way of experiencing the high-quality content, whether it is for on-demand (e.g., blockbuster movies) or live services (e.g., sport events). Recent reports from largest VOD platforms such as Netflix confirm that assumption and show that the primary way of watching content remains fixed TV screens, covering 70% of devices 6 months after subscription [17]. In the same way, YouTube indicates that service usage on fixed TV set remains an inevitable way of accessing the content, with 250M of hours viewed per day on TV screens [18]. As 5G media streaming targets a wide range of connected devices and should be able to deliver video streams to many compatible high-resolution receivers, (e.g. 5G-HDMI-sticks, 5G-StB/5G-MediaGateway or even 5G-TV sets) the inclusion of 4K TV Scenarios for 5G Video codec evaluation is important.

First, the 4K-TV is currently the most established way of accessing premium quality services using latest technology improvements for video content, including High-Dynamic-Range (HDR) and Wide-Colour-Gamut (WCG). Latest statistics from the Ultra-HD forum indicates that 148 UHD services are currently on-air, 74% being linear, 45% of those using HDR [19]. In addition, a large number of SVOD operators propose 4K access in their subscription packages (e.g. Netflix™ and Amazon Prime™). All these services may eventually take advantage of 5G-network capabilities to increase the device reach and enlarge audiences. This scenario is also endorsed by strong shipment forecasts, as indicated in the latest IHS 4K-TV UHD bluebook [20].

### 6.3.2	Description of the Anticipated Application

In the context of 3GPP services, 5G Media Streaming [13] as well as the TV Video Profiles [3] are specifications addressing this 4K-TV scenario. Both, 5G Media Streaming [13] and TV Video Profiles [3] builds on CMAF-based Segment formats and DASH distribution. From TS 26.116, the following operation points may be considered in scope of the Full HD Streaming Scenario (pending availability of appropriate test content):

-	H.265/HEVC UHD HDR, see TS26.116 [3] clause 4.5.6.

-	H.265/HEVC UHD, see TS26.116 [3] clause 4.5.4.

-	H.265/HEVC UHD HDR HLG, see TS26.116 [3] clause 4.5.8.

For this scenario targets CMAF (including LL-DASH and HLS-LL) distribution of linear UHD-TV video services over 5G networks to 5G/non-5G capable devices. This includes 5G-equipped devices (e.g. smartphone, tablets, …) but also other devices gateway (e.g. TV sets, HDMI-Stick…) accessing services through a “5G-gateway” which can be a mobile phone or a home gateway. As multiple linear services will be delivered in parallel (news, sport, talk show…) in a similar manner as traditional TV services in a multiplex (potentially using multicast/broadcast delivery over 5G). In certain environments, High Frame Rate (HFR) beyond 60 fps is considered, e.g. in DVB and ATSC broadcast specifications. 3GPP does not yet have any HFR TV video profiles.

Important aspects that are expected to be considered when evaluating a codec in the context of this 4K-TV context are:

-	Quality and Coding Efficiency:

-	High and uninterrupted visual quality, taking into account the service constraints.

-	Any savings can provide significant benefits due to the expected large volume of the traffic either in quality or network utilization.

- 	Adaptive Bitrate streaming:

-	Multiple bit rates are provided, typically with a ladder of 30–50% to permit bandwidth adaptation. The use of constant bit rate (CBR) encoding maximises reuse of a common ladder of encoded representations across multiple distribution networks. The use of capped variable bit rate (VBR) encoding allows the bit rate to be varied according to the difficulty of the source material while maintaining the ability to distribute the encoded representations through distribution networks with fixed capacity. This also maximises reuse of a common ladder across multiple distribution networks.

-	CMAF Fragments of size typically in the range of 1–6s to permit seamless switching for bit rate adaptation.

-	Regular Random Access, typically every 1–2 seconds according to TS 26.116 [3]. To achieve clean switching in both sound and picture when moving between different encoded representations in the ladder, 3.84 seconds enables video segment boundaries to be aligned with an integer number of audio Access Units, if a 50fps video signal and 48kHz audio signal is used.

-	Encoding in this scenario is typically done as

-	Live and On-Demand distribution and encoding

- 	Server and Cloud-based Encoding

-	Capable of encoding multiple services at variable bitrate, inside a fixed dedicated resource (statistical multiplexing).

-	No specific encoding latency constraints

### 6.3.3	Source Format Properties

Table 6.3-1 provides an overview of the different source signal properties for 4K-TV. As TS26.116 [3] does not cover all the operating points, the table introduces new formats (HFR-related). This information is used to select proper test sequences.

Table 6.3-1 4K-TV source format properties

| Source format properties | 4K-TV |
| --- | --- |
| Spatial resolution | 3840 x 2160 (Permitted distribution formats: 2560 × 1440, 1920 × 1080, 1600 × 900, 1280 × 720) |
| Chroma format | Y’CbCr |
| Chroma subsampling | 4:2:0 |
| Picture aspec ratio | 16:9 |
| Frame rates | 24; 50; 60; 24/1.001; 60/1.001[, 100 120] Hz |
| Bit depth | 10 |
| Colour space formats | BT.2020 [15] |
| Transfer characteristics | BT.2020 [15], BT.2100 [16] (PQ & HLG) |

### 6.3.4	Encoding and Decoding Constraints

Table 6.3-2 provides an overview of encoding and decoding constraints for 4K-TV category using legacy codec HEVC. This will support the definition of detailed test conditions. It is noted that no relevant profiles exist in TS26.116 and TS26.511 for HFR 4K-TV content.

Table 6.3-2 Encoding and Decoding Configurations for 4K-TV with legacy HEVC codec

| Encoding and Decoding Constraints | H.265/HEVC 4K-TV | H.265/HEVC 4K-TV HFR |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. | H.265/HEVC Main-10 Profile Level 5.1 [8] | No relevant 3GPP profiles, should be aligned with H.265/HEVC Main-10 Profile Level 5.2 [8] |
| Segment Duration | 3.84 seconds | 3.84 seconds |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | B = {10,15,20,25} Mbps CBR and capped-VBR | B = {10,15,20,25} Mbps CBR and capped-VBR |
| Latency requirements and specific encoding settings | No latency requirements beyond RAP so picture reordering allowed | No latency requirements beyond RAP so picture reordering allowed |
| Encoding complexity context | real-time encoding, cloud-based encoding, offline encoding, etc. | real-time encoding, cloud-based encoding, offline encoding, etc. |
| Required decoding capabilities | H.265/HEVC Main-10 Profile Level 5.1 [8] | H.265/HEVC Main-10 Profile Level 5.2 [8] |

### 6.3.5	Performance Metrics

The following performance metrics are considered for this scenario:

tbd

### 6.3.6	Interoperability Considerations

In order to use a codec in the context of 5G Media Streaming services in TS 26.511 and for TV Video profiles in TS 26.116, the following list provides a set of potentially relevant interoperability aspects for 4K-TV Streaming:

- The receiver requirements on elementary stream level, in particular the profile/level and additional considerations.

- The encapsulation of an elementary stream into an ISO Base Media File Format track

- The definition of a CMAF media profile.

- The static mapping of parameters to a DASH MPD, in particular to the MPD parameters, such as @mimeType, @codecs, etc

- The dynamic mapping of parameters to a DASH MPD from a CMAF Master Header, in particular to the MPD parameters, such as @width, @height, etc.

- The signaling of HDR and WCG source signal formats

- All MPD-level signalling for the codec to support capability discovery

- Encryption requirements and recommendations.

- Capability discovery options, for example mapping to HTML-5, MSE and media capability APIs.

- Source Buffer Initialization Requirements.

- Playback Requirements, for example by referencing CTA WAVE Specifications

- Relation to other specifications, such as in DVB, ATSC, MPEG, ETSI, etc.

For additional details, please refer to TS 26.116 and TS 26.511.

### 6.3.7	Test Sequences

### tbd

### 6.3.8	Detailed Test Conditions

#### 6.3.8.1	Overview

First, the legacy codec HEVC will be tested to assess the relevance of what’s already in the 3GPP specification for thisparticular 4K-TV scenario.

#### 6.2.8.2	Reference Software HEVC 1: HM16.20

As reference software for HEVC, the following was used

-	https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.20/

Example setting: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.20/cfg/encoder_randomaccess_main10.cfg with following proposed changes

- 	IntraPeriod: Intra Period such that 1 second is achieved

- 	DecodingRefreshType: 1 (CRA)  2 (IDR)

- 	GOPSize: adjusted to Intra

-	QP: [25, 28, 31, 34]

Tbd

### 6.3.9	External Performance Data

Tbd (with [21] [22] [23] as references ).

===== End of CHANGE 2 =====