3GPP TSG-<TSG/WG> Meeting # <MTG_SEQ><MTG_TITLE>	AVHIC-283

# <Location>, <Country>,  <Start_Date> - <End_Date>

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR |  | rev |  | Current version: | 0.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Screen content versus Online Gaming scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 21st July 2020 |
| Category: |  |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Screen content and online gaming are two different scenarios. It seems approprietate to have two separate sections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Split seciton 6.4 in two. Develop the gaming scenario. Include sections for codec tools relevant for each scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sections remains uncomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First Change

## 6.4 Scenario 3: Screen Content Scenario

### 6.4.1 Motivation

This scenario mostly motivates cases for which content goes beyond videographic content, an in particular includes computer generated imagery (CGI). Several application spaces rely on screen content types such as videoconferencing and webinars with presentation slides displayed, or remote desktop applications for demos or remote assistance. These services include massive online videoconferencing systems and telepresence systems both being particularly popular with remote collaboration.

For telepresence with screen-sharing applications, some information related to video is collected in the following:

- MS Teams™ [26] as of end of 2019.

  - There are several formats supported for video. Two key properties of a video format are its frame size and color format. Supported frame sizes include 640x360 ("360p"), 1280x720 ("720p"), and 1920x1080 ("1080p"). Supported color formats include NV12 (12 bits per pixel) and RGB24 (24 bits per pixel).

  - A "720p" video frame contains 921,600 pixels (1280 times 720). In the RGB24 color format, each pixel is represented as 3 bytes (24-bits) comprised of one byte each of red, green, and blue color components. Therefore, a single 720p RGB24 video frame requires 2,764,800 bytes of data (921,600 pixels times 3 bytes/pixel). At a frame rate of 30fps, sending 720p RGB24 video frames means processing approximately 80 MB/s of content (which is substantially compressed by the H.264 video codec before network transmission).

- Other tools are for further study.

### 6.4.2	Description of the Anticipated Application

3GPP until now has very restricted set of services but based on the considerations in clause 6.4.1, the following encoding benchmark capabilities are considered for decoding:

-	H.264 (AVCHD) YUV 4:2:0, YUV 4:4:4, 8 bit, Max Resolution 1920x1080

-	H.265 (HEVC) YUV 4:2:0, YUV 4:4:4, 10 bit, Max Resolution 4096 x 2048

The considered scenario is low-latency streaming or conversational. Important aspects that are expected to be considered when evaluating a codec in the context of this:

-	Quality and Coding Efficiency:

-	The ability to compress computer-generated content. Typically, it means the ability to have non perceptible intra refreshes and the ability to maintain stability on low frequency areas (such as uniform backgrounds) as well as maintaining details on high frequencies (particularly for text)

-	The ability compress YUV 4:2:0 and 4:4:4 content.

- 	Considered settings for encoding:

-	Low-latency settings

-	No specific error resilience mechanisms

-	Encoding in this scenario is typically done as

-	Real-time encoding

### 6.4.3 Source Format Properties

Table 6.4-1 provides an overview of the different source signal properties for Screen Content Sharing. This information is used to select proper test sequences.

Table 6.4-1 Screen Content Sharing source properties

| Source format properties | Screen Content |
| --- | --- |
| Spatial resolution | 1920 x 1080 |
| Chroma format | Y’CbCr, RGB |
| Chroma subsampling | 4:2:0, 4:4:4 |
| Picture aspec ratio | 16:9 |
| Frame rates | 25, 30, 50, 60 Hz |
| Bit depth | 8 |
| Colour space formats | BT.709 |
| Transfer characteristics | BT.709 |

### 6.4.4	Screen content codec tools

| Screen content tools | AVC | HEVC |
| --- | --- | --- |
| main profile | NA | Transform skip |
| range extension profile | NA | Residual Differential Pulse Code Modulation (RDPCM) (implicit intra/explicit inter), |
| screen content profile | NA | Intra Block Copy (full frame or less), Palette, Adaptive Color Transform |

Table 6.4-2 provides an overview of the different codec tools per profile that may be suitable for coding screen content sequences.

Table 6.4-2 Screen Content Tools per Profile

| Screen content tools | AVC | HEVC |
| --- | --- | --- |
| main profile | NA | Transform skip |
| range extension profile | NA | Residual Differential Pulse Code Modulation (RDPCM) (implicit intra/explicit inter), |
| screen content profile | NA | Intra Block Copy (full frame or less), Palette, Adaptive Color Transform |

Note 1:  HEVC specifies nested profiles. The screen content profile contains all the screen content tools from the main and range extension profiles.

Note 2:  Tools specific results could be gathered from JCT-VC.

### 6.4.5	Encoding and Decoding Constraints

Table 6.4-3 provides an overview of encoding and decoding constraints for Screen Content scenario using AVC and HEVC codecs operating points. This will support the definition of detailed test conditions.

Table 6.4-3 Encoding and Decoding Configurations for Screen Content

| Encoding and Decoding Constraints | AVC | HEVC |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. | H.264/AVC Main Profile Level 4.0 [X] | H.265/HEVC Main-10 Profile H.265/HEVC Screen Content profile Level 4.1, Level 5.1, Level 6.1 |
| RAP period | Gradual refresh: 1 second, no fix interval intra | Gradual refresh: 1 second, no fix interval intra |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | Fixed QP | Fixed QP |
| Latency requirements and specific encoding settings | Low-latency requirements, Low-delay configuration | Low-latency requirements, Low-delay B, low-delay-P |
| Encoding complexity context | real-time encoding. | real-time encoding. |
| Required decoding capabilities | H.264/AVC Main Profile Level 4.0 [X] | H.265/HEVC Main-10 Profile H.265/HEVC Screen Content profile Level 4.1, 5.1, 6.1 [8] |

### 6.4.6 Performance Metrics

tbd

### 6.4.7	Interoperability Considerations

tbd

### 6.4.8	Test Sequences

Tests sequences illustrating the screen content scenario should ideally contain either snyhtetic content from a presentation such as a slide deck with text and graphics or natural content presenting participants to a video conference call, or a mix of the two]

### 6.4.9	Detailed Test Conditions

For screen content, would like to at least test HEVC, 720p and 1080p, Main and Screen Extended Main profiles]

### 6.4.10	External Performance Data

tbd

### 6.4.11	Additional Information

tbd

End of First Change

Start of Second Change

# Section 6.X scenario X: Gaming

### 6.x.1 Motivation

According to the 2020 Mobile Internet Phenomena Report from Sandvine [9] gaming is continuing to grow on mobile network. The improved performance of 4G and the coming promise of 5G will continue to drive at least casual gamers to mobile networks.

Online gaming was discussed and introduced in detail in TR 26.928 [X]. At least the following use cases are in context of Online gaming:

-	Use Case 5: Untethered Immersive Online Gaming

-	Use Case 6: Immersive Game Spectator Mode

For raster-based split rendering, according to TR 26.928, clause 4.4, rasterized 3D scenes available in frame buffers are provided by the XR engine and need to be encoded, distributed, and decoded. According to clause 4.2.1, relevant formats for frame buffers are 2k by 2k per eye, potentially even higher. Frame rates are expected to be at least 60fps, potentially higher up to 90 fps. The formats of frame buffers are regular texture video signals that are then directly rendered. As the processing is graphics centric, formats beyond commonly used 4:2:0 signals and YUV signals may be considered. It is known from experiments that with H.264/AVC the bitrates are in the order of 50 Mbps per eye buffer. It is expected that this can be reduced to lower bitrates with improved compression tools as for example available for H.265/HEVC. For use case 5 from above and split rendering, encoding is required to be done in low-latency based on the considerations in TR 26.928. For the spectator mode, higher latency may be acceptable.

As an example, a comprehensive set of API including high-performance tools, samples and documentation for hardware accelerated video encode and decode on Windows and Linux for NVIDIA™ Video Codec SDK is available [N]. For example, in a game recording and streaming scenario like streaming to Twitch.tv using Open Broadcaster Software (OBS), encoding being completely offloaded to NVENC makes the graphics engine bandwidth fully available for game rendering. As of May 2020, the following formats are supported for hardware-based encoding as documented on the high-end Turing encoding:

-	H.264 (AVCHD) YUV 4:2:0, YUV 4:4:4, and Lossless, all 8 bit, Max Resolution 4096 x 4096;

-	H.265 (HEVC) YUV 4:2:0, YUV 4:4:4, and Lossless, all 10 bit, Max Resolution 8192 x 8192;

In typical cloud gaming environments, the game server produces rasterized frames at a fixed resolution, framerate and colour bit depth which are negotiated with the player client. Negotiation takes into account game capabilities, player choices and eventually bandwidth constraints.

Typical characteristics of rasterized frames produced by the game engine are:

- Resolution of 720p, 1080p or 4K

- Framerate of 30fps, 60fps or 120 fps

- Typical color bit depth of 8bits (RGB frames) but higher bit depth may be offered for HDR compatible games

Rasterized frames are directly passed to a video encoder (typically H.264 but H.265 may be used in a few environments) and content is live encoded to fit target quality. As an example, the following quality categorization may be done:

- High Quality: 4k at 60/120fps with an average throughput of 60/100 Mbps

- Main Quality: 1080p at 60/120fps with an average throughput of 30/40 Mbps

- Low Quality: 720p/1080p at 30fps with an average throughput of 10/12 Mbps

### 6.X.2	Description of the Anticipated Application

3GPP until now has very restricted set of services, but based on the considerations in clause 6.Y.1, the following encoding benchmark capabilities are considered for decoding:

-	H.264 (AVCHD) YUV 4:2:0, YUV 4:4:4, 8 bit, Max Resolution 1920x1080 and 4096 x 2048

-	H.265 (HEVC) YUV 4:2:0, YUV 4:4:4, 10 bit, Max Resolutions 4096 x 2048, 8192 x 4096

The considered scenario is low-latency streaming, possibly using UDP/IP based distribution. Important aspects that are expected to be considered when evaluating a codec in the context of this:

-	Quality and Coding Efficiency:

-	The ability to compress computer-generated content.

-	The ability compress YUV 4:2:0 and 4:4:4 content

- 	Considered settings for encoding:

-	Low-latency settings

-	No specific error resilience mechanisms

-	Encoding in this scenario is typically done as

-	Real-time encoding

- 	Cloud-based encoding

### 6.X.4 Source Format Properties

Table 6.X-1 provides an overview of the different source signal properties for Online Gaming. This information is used to select proper test sequences.

Table 6.X-1 Online Gaming source properties

| Source format properties | Ultra low latency game (first person) | Low latency game (Role Play Game) | Moderate latency game (conversational) | Non-critical game (Strategy) |
| --- | --- | --- | --- | --- |
| Total Delay (ms) | < 50 | < 100 | < 200 | > 200 |
| Spatial resolution | 1920x1080, 3840x2160 | 1920x1080, 3840x2160 | 1920x1080, 3840x2160 | 1920x1080, 3840x2160 |
| Chroma format | Y’CbC | Y’CbC | Y’CbC | Y’CbCr |
| Chroma subsampling | 4:2:0, 4:4:4 | 4:2:0, 4:4:4 | 4:2:0, 4:4:4 | 4:2:0, 4:4:4 |
| Picture aspec ratio | 16:9 | 16:9 | 16:9 | 16:9 |
| Frame rates | 30, 50, 60, 90, 120 Hz | 30, 50, 60, 90, 120 Hz | 30, 50, 60, 90, 120 Hz | 30, 50, 60, 90, 120 Hz |
| Bit depth | 8, 10 | 8, 10 | 8, 10 | 8, 10 |
| Colour space formats | BT.709, BT.2020 | BT.709, BT.2020 | BT.709, BT.2020 | BT.709, BT.2020 |
| Transfer characteristics | BT.2100 (HDR) | BT.2100 (HDR) | BT.2100 (HDR) | BT.2100 (HDR) |

*Note: The total delay could vary in a game over time.

### 6.x.5	Gaming codec tools

Table 6.X-2 provides an overview of the different coding tools per profile that may be suitable for coding different types of gaming sequences.

Table 6.X-2 Gaming Codec Tools per Profile

| gaming tools | AVC | HEVC |
| --- | --- | --- |
| main profile | NA | TBD |
| range extension profile | NA | TBD |
| screen content profile | NA | Palette, TBD |

Note: Palette being heavily used in the design of games, this tool should be particularly tested for game content.

For AAA games, the quality of the graphic is coming close to realistic natural video sequences. Some ”screen content” tools may not provide much benefits for this type of game content and would need to be tested.

### 6.X.6Encoding and Decoding Constraints

Table 6.X-2 provides an overview of encoding and decoding constraints Online Gaming scenario using AVC and HEVC codecs as operation points. This will support the definition of detailed test conditions.

Table 6.X-3 Encoding and Decoding Configurations for Online Gaming

| Encoding and Decoding Constraints | AVC | HEVC |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. | H.264/AVC Main Profile Level 4.0 [X] | H.265/HEVC Main-10 Profile Level 4.1, Level 5.1, Level 6.1 |
| RAP period | 1 second | 1 second |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | Fixed QP | Fixed QP |
| Latency requirements and specific encoding settings | Low-latency requirements, -Low-delay configuration -With and without use of future reference frames | Low-latency requirements, -Low-delay-P -Low-delay-B -With (moderate latency to non-critical latency games) and without (ultra low and low latency game) use of future reference frames. |
| Encoding complexity context | real-time encoding | real-time encoding. |
| Required decoding capabilities | H.264/AVC Main Profile Level 4.0 [X] | H.265/HEVC Main-10 Profile Level 4.1, 5.1, 6.1 [8] |

### 6.X.	_Performance Metrics

tbd

### 6.X.	Interoperability Considerations

tbd

### 6.X.10	Test Sequences

[Test sequences illustrating the Online Gaming scenario should ideally contain synthetic content with detailed textures and realistic movements. This should include FPS, RPG and Strategy games.

### 6.X.11	Detailed Test Conditions

[For online gaming, would like to test at least AVC and HEVC in 480p/720p/1080p with bitrates ranging from 5-20Mbps for AVC and 2 to 15Mbps for HEVC, with AVC Main Profile and HEVC Main10 profile, both codecs in low delay mode

### 6.X.11	External Performance Data

tbd

### 6.X.12	Additional Information

tbd

End of Second Change