Source:	Editor

Title:	IVAS Performance Requirements (IVAS-3)

Version:	0.3.0

Agenda Item:	14.2

- Scope

This document presents the Performance Requirements of the EVS Codec Extension for Immersive Voice and Audio Services (IVAS). Additional information on the codec development project can be found in the other IVAS permanent documents, for which the latest versions can be found at: https://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/IVAS_Permanent_Documents.

- Introduction

The overall objective of the IVAS_Codec work item is to develop a single general-purpose audio codec for immersive 4G and 5G services and applications. Further details on the objectives can be found in the work item description (SP-220608).

It is to be noted that it may not be possible to test all requirements of this Pdoc during IVAS selection. The choice of requirements to be tested is made in the IVAS selection test plan IVAS-8a.

- Nomenclature

In this document, the following conventions apply:

- Notation “Req1 OR Req2” (e.g. NWT 4xEVS @24.4 OR BT 4xEVS @16.4) means that either requirement “Req1” or requirement “Req2” shall be met.

- IVAS Performance Requirements

## 4.1 Stereo operation

IVAS stereo operation shall provide a benefit over the case when it is not included, i.e. mono operation.

High-level definition of stereo requirements

- For certain correlated stereo content (e.g. speech in conferencing use case, mixed/music, binaural signals), IVAS stereo operation shall be not worse than EVS dual-mono operation at (at least) next higher bit rate or better than EVS dual-mono operation at the same rate. Table 1 describes the details of these requirements.

- No other content shall be degraded with respect to dual mono operation

- The IVAS stereo operation with mono compatible decoding shall be not worse than the stereo to mono downmix of dual-mono at the same bit rate.

Table 1: Stereo requirements.

Dual mono is abbreviated with a “2x”. It is assumed that the IVAS codec and the EVS reference codec are running in the bandwidth mode as outlined in the BW column, if not otherwise specified.

Note: for all bit rates / audio bandwidth with no requirement, the following applies: If provided it should be characterized; characterization results shall be used for the decision of inclusion

| BW | Bitrate (kbit/s) | FER | DTX* | Stereo Requirements | Mono compatible requirements |
| --- | --- | --- | --- | --- | --- |
| WB | 13.2 | All | On/Off | NWT EVS @ 2x 8 kbps OR BT EVS-WB @ 2x 7.2 kbps |  |
|  | 16.4 |  |  | NWT EVS @ 2x 9.6 kbps OR BT EVS @ 2x 8 kbps | NWT stereo downmix of EVS @ 2x 8 kbps |
|  | 24.4 |  |  | NWT EVS @ 2x 16.4 kbps OR BT EVS @ 2x 13.2 kbps | NWT stereo downmix of EVS @ 2x 13.2 kbps |
|  | 32 |  |  | NWT EVS @ 2x 24.4 kbps OR BT EVS @ 2x 16.4 kbps | NWT stereo downmix of EVS @ 2x 16.4 kbps |
|  | 48 |  |  | NWT EVS @ 2x 32 kbps OR BT EVS @ 2x 24.4 kbps | NWT stereo downmix of EVS @ 2x 24.4 kbps |
|  | 64 |  |  | NWT EVS @ 2x 48 kbps OR BT EVS @ 2x 32 kbps | NWT stereo downmix of EVS @ 2x 32 kbps |
|  | 96 |  |  | NWT EVS @ 2x 64 kbps OR BT EVS @ 2x 48 kbps | NWT stereo downmix of EVS @ 2x 48 kbps |
| SWB FB | 13.2 | All | On/Off | NWT EVS-WB @ 2x 8 kbps OR BT EVS-WB @ 2x 7.2 kbps |  |
|  | 16.4 |  |  | NWT EVS @ 2x 9.6 kbps OR BT EVS-WB @ 2x 8 kbps | NWT stereo downmix of EVS @ 2x 8 kbps |
|  | 24.4 |  |  | NWT EVS @ 2x 16.4 kbps OR BT EVS @ 2x 13.2 kbps | NWT stereo downmix of EVS @ 2x 13.2 kbps |
|  | 32 |  |  | NWT EVS @ 2x 24.4 kbps OR BT EVS @ 2x 16.4 kbps | NWT stereo downmix of EVS @ 2x 16.4 kbps |
|  | 48 |  |  | NWT EVS @ 2x 32 kbps OR BT EVS @ 2x 24.4 kbps | NWT stereo downmix of EVS @ 2x 24.4 kbps |
|  | 64 |  |  | NWT EVS @ 2x 48 kbps OR BT EVS @ 2x 32 kbps | NWT stereo downmix of EVS @ 2x 32 kbps |
|  | 96 |  |  | NWT EVS @ 2x 64 kbps OR BT EVS @ 2x 48 kbps | NWT stereo downmix of EVS @ 2x 48 kbps |
|  | 128 |  |  | NWT EVS @ 2x 96 kbps OR BT EVS @ 2x 64 kbps | NWT stereo downmix of EVS @ 2x 64 kbps |

* DTX will be tested for stereo if provided.

## 4.2 Performance requirements for operation with Scene-Based Audio Content (first-order and higher-order Ambisonics)

The following scene-based audio requirements apply predominantly to speech, ambient sound, and music content. Both natural and synthetic contents is relevant.

High-level definition of scene-based audio requirements

The general requirement is that IVAS operated at rate X shall either

- be better than the EVS multi-mono system encoding the FOA component channels (while truncating the higher-order channels), where each EVS instance is operated at the closest bit rate to X/4.

- or be no worse than the EVS multi-mono system encoding the FOA component channels (while truncating the higher-order channels), where each EVS instance is operated at the next higher available EVS bit rate than the EVS bitrate closest to X/4.

Detailed scene-based audio requirements

The following table illustrates corresponding detailed performance requirements for scene-based audio content (***:

| BW | Bitrate (kbit/s) | FER | DTX(* | Requirements(** | Objectives |
| --- | --- | --- | --- | --- | --- |
| SWB FB | 13.2 | All | On/Off |  | NWT EVS @ 3x 7.2 kbps (WB) (planar FOA) |
|  | 16.4 |  |  | NWT EVS @ 3x 7.2 kbps (WB) (planar FOA) | NWT EVS @ 4x 7.2 kbps (WB) |
|  | 24.4 |  |  | NWT EVS @ 4x 7.2 kbps (WB) |  |
|  | 32 |  |  | NWT EVS @ 4x 9.6 kbps (SWB) OR BT EVS @ 4x 8 kbps (WB) |  |
|  | 48 |  |  | NWT EVS @ 4x 16.4 kbps (SWB/FB) OR BT EVS @ 4x 13.2 kbps (SWB) |  |
|  | 64 |  |  | NWT EVS @ 4x 24.4 kbps (SWB/FB) OR BT EVS @ 4x 16.4 kbps (SWB/FB) |  |
|  | 80 |  |  | NWT EVS @ 4x 24.4 kbps (SWB/FB) OR BT EVS @ 4x 16.4 kbps (SWB/FB) |  |
|  | 96 |  | Off | NWT EVS @ 4x 32 kbps (SWB/FB) OR BT EVS @ 4x 24.4 kbps (SWB/FB) |  |
|  | 128 |  |  | NWT EVS @ 4x 48 kbps (SWB/FB) OR BT EVS @ 4x 32 kbps (SWB/FB) |  |
|  | 160 |  |  | NWT EVS @ 4x 48 kbps (SWB/FB) OR BT EVS @ 4x 32 kbps (SWB/FB) |  |
|  | 192 |  |  | NWT EVS @ 4x 64 kbps (SWB/FB) OR BT EVS @ 4x 48 kbps (SWB/FB) |  |
|  | 256 |  |  | NWT EVS @ 4x 96 kbps (SWB/FB) OR BT EVS @ 4x 64 kbps (SWB/FB) |  |
|  | 384 |  |  | NWT EVS @ 4x 128 kbps (SWB/FB) OR BT EVS @ 4x 96 kbps (SWB/FB) |  |
|  | 512 |  |  | NWT EVS @ 4x 128 kbps (SWB/FB) |  |

(* DTX will be tested for rates up to 80 kbit/s where mandatory DTX operation is available for the multi-mono EVS reference.

(** The multi-mono EVS reference shall be produced by individual EVS coding of the 4 FOA B-format component signals in ACN/SN3D format (while truncating the higher-order component signals).

Further notes:

Note: No performance requirements are defined for WB content.

The FOA requirement will be tested with predominant voice content in naïve listener tests. The HOA3 requirement will be tested with general audio content in experienced listener tests. The HOA2 requirement will not be tested during selection.

The requirements will be tested with binaural rendering over headphones using a suitable reference renderer that will be specified in the selection processing plan IVAS-7a. The requirements may also be tested with rendering over a room loudspeaker system in experienced listener tests with general audio content.

Editor’s note: The SWB requirement will be tested with predominant voice content in naïve listener tests. The FB requirement will be tested with general audio content in experienced listener tests.

## 4.3 Performance requirements for operation with MASA Content

The following MASA audio requirements apply predominantly to speech, ambient sound, and music content.

4.3.1	For Stereo-MASA:

| BW | Bitrate (kbit/s) | FER | DTX(* | Requirements(** |
| --- | --- | --- | --- | --- |
| SWB FB | 13.2 | All | On/Off | NWT EVS @ 2x 7.2 kbps (WB) + UQ metadataOR NWT EVS @ 3x 7.2 kbps (WB) (planar FOA) |
|  | 16.4 |  |  | NWT EVS @ 2x 7.2 kbps (WB) + UQ metadata OR NWT EVS @ 4x 7.2 kbps (WB) |
|  | 24.4 |  |  | NWT EVS @ 2x 8 kbps (WB) + UQ metadata OR NWT EVS @ 4x 8.0 kbps (WB) |
|  | 32 |  |  | NWT EVS @ 2x 9.6 kbps (SWB) + UQ metadata OR NWT EVS @ 4x 9.6 kbps (SWB) |
|  | 48 |  |  | NWT EVS @ 2x 16.4 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 16.4 kbps (SWB/FB) |
|  | 64 |  |  | NWT EVS @ 2x 24.4 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 24.4 kbps (SWB/FB) |
|  | 80 |  |  | NWT EVS @ 2x 24.4 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 24.4 kbps (SWB/FB) |
|  | 96 |  |  | NWT EVS @ 2x 32 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 32 kbps (SWB/FB) |
|  | 128 |  |  | NWT EVS @ 2x 48 kbps (WB/SWB/FB) + UQ metadata OR NWT EVS @ 4x 48 kbps (SWB/FB) |
|  | 160 |  |  | NWT EVS @ 2x 48 kbps (WB/SWB/FB) + UQ metadata OR NWT EVS @ 4x 48 kbps (SWB/FB) |
|  | 192 |  |  | NWT EVS @ 2x 64 kbps (WB/SWB/FB) + UQ metadata OR NWT EVS @ 4x 64 kbps (SWB/FB) |
|  | 256 |  |  | NWT EVS @ 2x 96 kbps (WB/SWB/FB) + UQ metadata OR NWT EVS @ 4x 96 kbps (SWB/FB) |

(* DTX will be tested for the rates where DTX must be supported. DTX operation applies also for the (multi-mono) EVS references.

(** The multi-mono 2x EVS + UQ metadata reference shall be produced by individual EVS coding of the stereo MASA format transport channels and passthrough of the original MASA metadata. The multi-mono 3x EVS reference shall be produced by individual EVS coding of the 3 “planar FOA” component signals in ACN/SN3D format. The multi-mono 4x EVS reference shall be produced by individual EVS coding of the 4 FOA component signals in ACN/SN3D format.

6.3.2	For Mono-MASA:

| BW | Bitrate (kbit/s) | FER | DTX(* | Requirements(** |
| --- | --- | --- | --- | --- |
| SWB FB | 13.2 | All | On/Off | NWT EVS @ 8 kbps (WB) + UQ metadataOR NWT EVS @ 3x 7.2 kbps (WB) (planar FOA) |
|  | 16.4 |  |  | NWT EVS @ 9.6 kbps (SWB) + UQ metadata OR NWT EVS @ 4x 7.2 kbps (WB) |
|  | 24.4 |  |  | NWT EVS @ 13.2 kbps (SWB) + UQ metadata OR NWT EVS @ 4x 8 kbps (WB) |
|  | 32 |  |  | NWT EVS @ 16.4 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 9.6 kbps (SWB) |
|  | 48 |  |  | NWT EVS @ 32 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 16.4 kbps (SWB/FB) |
|  | 64 |  |  | NWT EVS @ 48 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 24.4 kbps (SWB/FB) |
|  | 80 |  |  | NWT EVS @ 48 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 24.4 kbps (SWB/FB) |
|  | 96 |  |  | NWT EVS @ 64 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 32 kbps (SWB/FB) |
|  | 128 |  |  | NWT EVS @ 64 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 48 kbps (SWB/FB) |
|  | 160 |  |  | NWT EVS @ 96 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 48 kbps (SWB/FB) |
|  | 192 |  |  | NWT EVS @ 128 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 64 kbps (SWB/FB) |
|  | 256 |  |  | NWT EVS @ 128 kbps (SWB/FB) + UQ metadata OR NWT EVS @ 4x 96 kbps (SWB/FB) |

(* DTX will be tested for the rates where DTX must be supported. DTX operation applies also for the (multi-mono) EVS references.

(** The EVS + UQ metadata reference shall be produced by EVS coding of the mono MASA format transport channel and passthrough of the original MASA metadata. The multi-mono 3x EVS reference shall be produced by individual EVS coding of the 3 “planar FOA” component signals in ACN/SN3D format. The multi-mono 4x EVS reference shall be produced by individual EVS coding of the 4 FOA component signals in ACN/SN3D format.

## 6.4 Performance requirements for operation with Multi-channel Audio Content

The following multi-channel audio requirements apply predominantly to cinematic, mixed, and music content.

6.4.1 Multi-channel 5.1:

High-level definition of Multi-Channel 5.1 requirements

The general requirement is that IVAS operated at rate X shall either

- be better than the EVS multi-mono system, where each EVS instance is operated at the closest bit rate to X/5.

- or be no worse than the EVS multi-mono system, where each EVS instance is operated at the next higher available EVS bit rate than the EVS bitrate closest to X/5.

- For the EVS multi-mono system, the LFE is considered to be encoded with EVS 9.6 kbit/s NB.

Detailed Multi-Channel 5.1 requirements

The following table illustrates corresponding detailed performance requirements for Multi-Channel 5.1 audio content:

| BW | Bitrate (kbit/s) | FER | Requirements (* | Objectives (* |
| --- | --- | --- | --- | --- |
| SWB FB | 13.2 | All |  | NWT EVS @ 5x 7.2 kbps (WB) |
|  | 16.4 |  |  | NWT EVS @ 5x 7.2 kbps (WB) |
|  | 24.4 |  | NWT EVS @ 5x 7.2 kbps (WB) |  |
|  | 32 |  | NWT EVS @ 5x 8.0 kbps (WB) OR BT EVS @ 5x 7.2 kbps (WB) |  |
|  | 48 |  | NWT EVS @ 5x 13.2 kbps (SWB) OR BT EVS @ 5x 9.6 kbps (SWB) |  |
|  | 64 |  | NWT EVS @ 5x 16.4 kbps (SWB) OR BT EVS @ 5x 13.2 kbps (SWB) |  |
|  | 80 |  | NWT EVS @ 5x 24.4 kbps (SWB/FB) OR BT EVS @ 5x 16.4 kbps (SWB) |  |
|  | 96 |  | NWT EVS @ 5x 24.4 kbps (SWB/FB) OR BT EVS @ 5x 16.4 kbps (SWB) |  |
|  | 128 |  | NWT EVS @ 5x 32 kbps (SWB/FB) OR BT EVS @ 5x 24.4 kbps (SWB/FB) |  |
|  | 160 |  | NWT EVS @ 5x 48 kbps (SWB/FB) OR BT EVS @ 5x 32 kbps (SWB/FB) |  |
|  | 192 |  | NWT EVS @ 5x 48 kbps (SWB/FB) OR BT EVS @ 5x 32 kbps (SWB/FB) |  |
|  | 256 |  | NWT EVS @ 5x 64 kbps (SWB/FB) OR BT EVS @ 5x 48 kbps (SWB/FB) |  |
|  | 384 |  | NWT EVS @ 5x 96 kbps (SWB/FB) OR BT EVS @ 5x 64 kbps (SWB/FB) |  |
|  | 512 |  | NWT EVS @ 5x 128 kbps (SWB/FB) OR BT EVS @ 5x 96 kbps (SWB/FB) |  |

(* The multi-mono EVS reference shall be produced by individual EVS coding of the 5 channels, and the LFE channel shall be encoded with EVS 9.6 kbit/s NB.

Note: No performance requirements are defined for WB content.

6.4.2 Multi-channel 7.1:

High-level definition of Multi-Channel 7.1 requirements

The general requirement is that IVAS operated at rate X shall either

- be better than the EVS multi-mono system, where each EVS instance is operated at the closest bit rate to X/7.

- or be no worse than the EVS multi-mono system, where each EVS instance is operated at the next higher available EVS bit rate than the EVS bitrate closest to X/7.

- For the EVS multi-mono system, the LFE is considered to be encoded with EVS 9.6 kbit/s NB.

Detailed Multi-Channel 7.1 requirements

The following table illustrates corresponding detailed performance requirements for Multi-channel 7.1 audio content:

| BW | Bitrate (kbit/s) | FER | Requirements (* | Objectives (* |
| --- | --- | --- | --- | --- |
| SWB FB | 13.2 | All |  | NWT EVS @ 7x 7.2 kbps (WB) |
|  | 16.4 |  |  | NWT EVS @ 7x 7.2 kbps (WB) |
|  | 24.4 |  |  | NWT EVS @ 7x 7.2 kbps (WB) |
|  | 32 |  | NWT EVS @ 7x 7.2 kbps (WB) |  |
|  | 48 |  | NWT EVS @ 7x 8.0 kbps (WB) OR BT EVS @ 7x 7.2 kbps (WB) |  |
|  | 64 |  | NWT EVS @ 7x 13.2 kbps (SWB) OR BT EVS @ 7x 9.6 kbps (SWB) |  |
|  | 80 |  | NWT EVS @ 7x 16.4 kbps (SWB) OR BT EVS @ 7x 13.2 kbps (SWB) |  |
|  | 96 |  | NWT EVS @ 7x 16.4 kbps (SWB) OR BT EVS @ 7x 13.2 kbps (SWB) |  |
|  | 128 |  | NWT EVS @ 7x 24.4 kbps (SWB/FB) OR BT EVS @ 7x 16.4 kbps (SWB) |  |
|  | 160 |  | NWT EVS @ 7x 32 kbps (SWB/FB) OR BT EVS @ 7x 24.4 kbps (SWB/FB) |  |
|  | 192 |  | NWT EVS @ 7x 32 kbps (SWB/FB) OR BT EVS @ 7x 24.4 kbps (SWB/FB) |  |
|  | 256 |  | NWT EVS @ 7x 48 kbps (SWB/FB) OR BT EVS @ 7x 32 kbps (SWB/FB) |  |
|  | 384 |  | NWT EVS @ 7x 64 kbps (SWB/FB) OR BT EVS @ 7x 48 kbps (SWB/FB) |  |
|  | 512 |  | NWT EVS @ 7x 96 kbps (SWB/FB) OR BT EVS @ 7x 64 kbps (SWB/FB) |  |

(* The multi-mono EVS reference shall be produced by individual EVS coding of the 7 channels, and the LFE channel shall be encoded with EVS 9.6 kbit/s NB.

Note: No performance requirements are defined for WB content.

6.4.3 Multi-channel 5.1+4:

High-level definition of Multi-Channel 5.1+4 requirements

The general requirement is that IVAS operated at rate X shall either

- be better than the EVS multi-mono system, where each EVS instance is operated at the closest bit rate to X/9.

- or be no worse than the EVS multi-mono system, where each EVS instance is operated at the next higher available EVS bit rate than the EVS bitrate closest to X/9.

- For the EVS multi-mono system, the LFE is considered to be encoded with EVS 9.6 kbit/s NB.

Detailed Multi-Channel 5.1+4 requirements

The following table illustrates corresponding detailed performance requirements for Multi-Channel 5.1+4 audio content:

| BW | Bitrate (kbit/s) | FER | Requirements (* | Objectives (* |
| --- | --- | --- | --- | --- |
| SWB FB | 13.2 | All |  | NWT EVS @ 9x 7.2 kbps (WB) |
|  | 16.4 |  |  | NWT EVS @ 9x 7.2 kbps (WB) |
|  | 24.4 |  |  | NWT EVS @ 9x 7.2 kbps (WB) |
|  | 32 |  |  | NWT EVS @ 9x 7.2 kbps (WB) |
|  | 48 |  | NWT EVS @ 9x 7.2 kbps (WB) |  |
|  | 64 |  | NWT EVS @ 9x 8.0 kbps (WB) OR BT EVS @ 9x 7.2 kbps (WB) |  |
|  | 80 |  | NWT EVS @ 9x 9.6 kbps (SWB) OR BT EVS @ 9x 8.0 kbps (WB) |  |
|  | 96 |  | NWT EVS @ 9x 13.2 kbps (SWB) OR BT EVS @ 9x 9.6 kbps (SWB) |  |
|  | 128 |  | NWT EVS @ 9x 16.4 kbps (SWB) OR BT EVS @ 9x 13.2 kbps (SWB) |  |
|  | 160 |  | NWT EVS @ 9x 24.4 kbps (SWB/FB) OR BT EVS @ 9x 16.4 kbps (SWB) |  |
|  | 192 |  | NWT EVS @ 9x 32 kbps (SWB/FB) OR BT EVS @ 9x 24.4 kbps (SWB/FB) |  |
|  | 256 |  | NWT EVS @ 9x 48 kbps (SWB/FB) OR BT EVS @ 9x 32 kbps (SWB/FB) |  |
|  | 384 |  | NWT EVS @ 9x 64 kbps (SWB/FB) OR BT EVS @ 9x 48 kbps (SWB/FB) |  |
|  | 512 |  | NWT EVS @ 9x 96 kbps (SWB/FB) OR BT EVS @ 9x 64 kbps (SWB/FB) |  |

(* The multi-mono EVS reference shall be produced by individual EVS coding of the 5+4 channels, and the LFE channel shall be encoded with EVS 9.6 kbit/s NB.

Note: No performance requirements are defined for WB content.

6.4.4 Multi-channel 7.1+4:

High-level definition of Multi-Channel 7.1+4 requirements

The general requirement is that IVAS operated at rate X shall either

- be better than the EVS multi-mono system, where each EVS instance is operated at the closest bit rate to X/11.

- or be no worse than the EVS multi-mono system, where each EVS instance is operated at the next higher available EVS bit rate than the EVS bitrate closest to X/11.

- For the EVS multi-mono system, the LFE is considered to be encoded with EVS 9.6 kbit/s NB.

Detailed Multi-Channel 7.1+4 requirements

The following table illustrates corresponding detailed performance requirements for Multi-Channel 7.1+4  audio content:

| BW | Bitrate (kbit/s) | FER | Requirements (* | Objectives (* |
| --- | --- | --- | --- | --- |
| SWB FB | 13.2 | All |  | NWT EVS @ 11x 7.2 kbps (WB) |
|  | 16.4 |  |  | NWT EVS @ 11x 7.2 kbps (WB) |
|  | 24.4 |  |  | NWT EVS @ 11x 7.2 kbps (WB) |
|  | 32 |  |  | NWT EVS @ 11x 7.2 kbps (WB) |
|  | 48 |  |  | NWT EVS @ 11x 7.2 kbps (WB) |
|  | 64 |  | NWT EVS @ 11x 7.2 kbps (WB) |  |
|  | 80 |  | NWT EVS @ 11x 8.0 kbps (WB) OR BT EVS @ 11x 7.2 kbps (WB) |  |
|  | 96 |  | NWT EVS @ 11x 9.6 kbps (SWB) OR BT EVS @ 11x 8.0 kbps (WB) |  |
|  | 128 |  | NWT EVS @ 11x 13.2 kbps (SWB) OR BT EVS @ 11x 9.6 kbps (SWB) |  |
|  | 160 |  | NWT EVS @ 11x 16.4 kbps (SWB) OR BT EVS @ 11x 13.2 kbps (SWB) |  |
|  | 192 |  | NWT EVS @ 11x 24.4 kbps (SWB/FB) OR BT EVS @ 11x 16.4 kbps (SWB) |  |
|  | 256 |  | NWT EVS @ 11x 32 kbps (SWB/FB) OR BT EVS @ 11x 24.4 kbps (SWB/FB) |  |
|  | 384 |  | NWT EVS @ 11x 48 kbps (SWB/FB) OR BT EVS @ 11x 32 kbps (SWB/FB) |  |
|  | 512 |  | NWT EVS @ 11x 64 kbps (SWB/FB) OR BT EVS @ 11x 48 kbps (SWB/FB) |  |

(* The multi-mono EVS reference shall be produced by individual EVS coding of the 7+4 channels, and the LFE channel shall be encoded with EVS 9.6 kbit/s NB.

Note: No performance requirements are defined for WB content.

# X. Revision history

| Date | Meeting | Subject/Comment | Old | New |
| --- | --- | --- | --- | --- |
| 2019-08-16 | SA4#105 | Agreement of Initial Skeleton of Performance Requirements (IVAS-3) including draft stereo requirements | N/A | 0.0.1… 0.0.4 |
| 2021-11-16 | SA4#116-e | Inclusion of draft requirements for FOA | 0.0.4 | 0.1.0 |
| 2022-08-22 | SA4#120-e | Updates of Scope and Introduction Inclusion of draft requirements for MASA | 0.1.0 | 0.2.0 |
| 2022-11-16 | SA4#121 | General updates and updates to/inclusion of requirements for stereo, scene-based, MASA and multi-channel audio formats (output of Audio SWG editing session) | 0.2.0 | 0.3.0 |

Stefan Bruhn, Dolby Laboratories Inc.