Source:	ORANGE

Title:	On IVAS input and output formats

## Document for:	Discussion

## Agenda Item:	8.5

## Introduction

This contribution is a continuation of S4-180823. The proposal is partly repeated with some further details on the support of binaural audio and direct headphone presentation.

## Proposal

| Input audio formats | The IVAS codec shall support the following input formats: - Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] - Binaural audio Editor's note: 2.0 audio might be seen as comprising binaural audio. However, it seems better to distinguish the two input types, to avoid applying binaural rendering or head-tracking to a captured binaural signal. - Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. Note: ACN component ordering and SN3D normalization. - [Spatial audio, [N] channels and spatial metadata defined by [TBD].] [Editor’s Note FFS: Spatial metadata definition for the spatial audio format will require further input.] - Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. [In addition, the IVAS codec shall support combinations of the above, totalling to no more than [TBD] audio streams. Note: It will be necessary to specify how capture/presentations could be achieved in mobile communications.] |
| --- | --- |
| Output audio formats | The IVAS codec (including the renderer) shall support: - a "pass-trough mode" in which the output format is identical to the input format Editor's note1: This mode may be limited to a subset of inputs. Editor's note2: In channel-based (2.0) and binaural cases, this mode allows presentation directly to the headphone, for cases when material was authored directly for headphones. - a "rendering mode" which implements rendering over loudspeakers (e.g. using the same configurations as channel-based audio listed in the 'Audio input formats' or allowing more flexible configurations) and binaural rendering |
| Input/output format combinations | The following input/ouput combinations shall be supported: - channel to channel (identical configuration, i.e. pass-though mode, or rendering mode) - object to object (pass-though mode) / channel (rendering mode) - scene to scene (pass-trough mode) / channel (rendering mode) |
| Interface for binaural rendering | The IVAS decoder/renderer shall provide an interface to provide [HRTF/BRIR] data for binaural rendering. The interface is [tbd]. [Note: There was some support for this interface to follow the SOFA SimpleFreeFieldHRIR convention - See AES69-2015]. The IVAS decoder/renderer shall provide an API to provide [TBD scene displacement data]. The IVAS decoder/renderer shall support direct headphone presentation (with signalling indicating if material is for monaural or diotic listening). |