3GPP TSG-SA4 Meeting #101	S4-181340

# Busan, Korea,  19.11. – 23.11.2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.819 | CR | <CR#> | rev | <Re#> | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoE Asessment of Simulator Sickness in VR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom AG |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_QoE_VR |  |  |  |  |  | Date: |  |  | 15.9.2017 |
| Category: | B |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During SA4#100 it was agreed to consider the proposed content in S4-181113 to be added as new section TR26.819. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The new proposed section compares subjective different quality test methods for VR360 video. In addition, the influence of simulator sickness is studied. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new section would provide relevant input on subjective test method for VR360 video, which would be missing if not approved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2 , 7.4 adding new clause, Annex C inserted |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### **** First Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Savino, Peter J.; Danesh-Meyer, Helen V. (1 May 2012). Color Atlas and Synopsis of Clinical Ophthalmology -- Wills Eye Institute -- Neuro-Ophthalmology. Lippincott Williams & Wilkins. p. 12.

[3]	Dagnelie, Gislin (21 February 2011). Visual Prosthetics: Physiology, Bioengineering, Rehabilitation. Springer Science & Business Media. p. 398.

[4]	T.E. Boult and G. Wolberg, "Correcting chromatic aberrations using image warping", Computer Vision and Pattern Recognition 1992. Proceedings CVPR '92. 1992 IEEE Computer Society Conference on, pp. 684-687, 1992, ISSN 1063-6919.

[5]	Levoy, Marc, and Pat Hanrahan. "Light field rendering". Proceedings of the 23rd annual conference on Computer graphics and interactive techniques. ACM, 1996.

[6]	J. P. Snyder, "Flattening the Earth: Two Thousand Years of Map Projections," University of Chicago Press, 1993.

[7]	Skupin, R., Sanchez, Y., Hellge, C., & Schierl, T., "Tile Based HEVC Video for Head Mounted Displays", IEEE International Symposium on Multimedia (ISM), December 2016.

[8]	Michael A. Gerzon, Periphony: With-Height Sound Reproduction. Journal of the Audio Engineering Society, 1973, 21(1):2-10.

[9]	Recommendation ITU-R BS.2266-2 (04/2014): "Framework of future audio broadcasting systems".

[10]	Recommendation ITU-R BS.2051-0 (02/2014): "Advanced sound system for programme production".

[11]	Lee et al., "Scalable Multiband Binaural Renderer for MPEG-H 3D Audio", in IEEE Journal of Selected Topics in Signal Processing, vol. 9, no. 5, pp. 907-920, Aug. 2015.

[12]	Breebaart, J., Nater, F., and Kohlrausch, A., "Parametric binaural synthesis: Background, applications and standards," NAG/DAGA International Conference on Acoustics, 2009.

[13]	Brungart, D. S., Kordik, A.K., and Simpson, B.D - "Effects of headtracker latency in virtual audio displays", J. Audio Eng. Soc., Vol. 54, No. 1/2, 2006 January/February.

[14]	Lindau, A. - "The Perception of System Latency in Dynamic Binaural Synthesis", In Fortschritte der Akustik: Tagungsband der 35, NAG/DAGA pp. 1063-1066 - Rotterdam - 2009.

[15]	Lindau, A., et al., "A Spatial Audio Quality Inventory (SAQI)", Acta Acustica united with Acustica, 100(5), 2014.

[16]	Silzle, A., "3D Audio Quality Evaluation: Theory and Practice", ICSA, 2014.

[17]	C. Schissler, A. Nicholls & R. Mehra, "Efficient HRTF-based Spatial Audio for Area and Volumetric Sources", IEEE Trans on Visualization and Computer Graphics, 2016, Vol 22, No 4, pp 1356 - 1366.

[18]	K. E. Bystrom, W. Barfield and C. Hendrix, "A Conceptual Model of the Sense of Presence in Virtual Environments," in Presence, vol. 8, no. 2, pp. 241-244, April 1999.

[19]	Sheridan, Thomas B. "Musings on telepresence and virtual presence". Presence: Teleoperators & Virtual Environments 1, no. 1 (1992): 120-126.

[20]	Sheridan, T., Zeltzer, D., & Slater, M. (1995). "Presence and performance within virtual environments", In W. Barfield and T. Furness III (Eds.), Virtual Environments and Advanced Interface Design. (pp. 473-513). Oxford University Press.

[21]	Lindau, A., Weinzierl, S., (2011). "Assessing the plausibility of virtual acoustic environments", Forum Acusticum 2011, 27 June -1 July, Aalborg., European Acoustic Association, pp. 1187-1192.

[22]	Recommendation ITU-R BS.1534-3 (10/2015): "Method for the subjective assessment of intermediate quality level of audio systems".

[23]	Bertet, Stéphanie, et al. "Influence of Microphone and Loudspeaker Setup on Perceived Higher Order Ambisonics Sound Field", in Ambisonics Symposium 2009, June 25-27, Graz.

[24]	Durlach N.I, Mavor A.S, "Virtual reality Scientific and Technological Challenges" National Academy Press, 1995.

[25]	Draper M.H, "The adaptive effects of virtual interfaces: Vestibulo-ocular reflex and simulator sickness", PhD Thesis, University Washington, Sponsored by US Airforce Department, 1995.

[26]	Di Girolamo S, Picciotti P, Sergi B, Di Nardo W, Paludetti G, Ottaviani F "Vestibulo-ocular reflex modification after virtual environment exposure.", Acta Oto-Laryngologica. 2001 Jan; Vol 121 Issue 2 pp 211-215.

[27]	Jombı'k P, Bahy'l V, "Short latency disconjugate vestibulo-ocular responses to transient stimuli in the audio frequency range." Journal of Neurology, Neurosurgery, and Psychiatry; Vol 76 No 10, Oct 2005 pp 1398-1402.

[28]	Amin M.S, "Vestibuloocular Reflex Testing" Medscape Article Number 1836134, Feb 10 2016.

[29]	Jerald J, "Scene-Motion- and Latency-Perception Thresholds for Head-Mounted Displays" University of North Carolina PhD Dissertation, 2010.

[30]	Brungart et al., "Effects of Headtracker Latency in Virtual Audio Displays", Proceedings of International Conference on Auditory Display; ICAD-05, July 2005.

[31]	http://www.york.ac.uk/inst/mustech/3d_audio/higher_order_ambisonics.pdf.

[32]	Recommendation ITU-R BT.1359-1 (1998): "Relative Timing of Sound and Vision for Broadcasting".

[33]	EBU Technical Recommendation R37 (2007): "The relative timing of the sound and vision components of a television signal".

[34]	3GPP TS 26.116: "Television (TV) over 3GPP services; Video profiles".

[35]	ISO/IEC AWI 23090-2: "Coded representation of immersive media -- Part 2: Application Format for Omnidirectional Media" - OMAF.

[36]	MPEG-H 3D Audio Verification Test Report, Geneva, 2017. http://mpeg.chiariglione.org/standards/mpeg-h/3d-audio/mpeg-h-3d-audio-verification-test-report.

[37]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[38]	3GPP TS 26.223: "Telepresence using the IP Multimedia Subsystem (IMS); Media handling and interaction".

[39]	3GPP TS 26.247: "Transparent end-to-end Packet-switched, Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[40]	Jerome Daniel, Spatial Sound Encoding Including Near Field Effect: Introducing Distance Coding Filters and a Viable, New Ambisonic Format, 23rd AES Conference, Copenhagen 2003, p. 13.

[41]	Recommendation ITU-R BS.2076-0 (06/2015): "Audio Definition Model".

[42]	ETSI TS 103 223 (V1.1.1): "MDA; Object-Based Audio Immersive Sound Metadata and Bitstream".

[43]	C. Nachbar, F. Zotter, E. Deleflie & A. Sontacchi, "AmbiX - A Suggested Ambisonics Format" Procedings of Ambisonics Symposium 2011, June 2-3, 2011, Lexington, USA.

[44]	ETSI TS 103 190-1 (V1.2.1): "Digital Audio Compression (AC-4) Standard; Part 1: Channel based coding".

[45] 	ETSI TS 103 190-2 (V1.1.1): "Digital Audio Compression (AC-4) Standard; Part 2: Immersive and personalized audio".

[46]	ISO/IEC 23008-3:2015: "Information technology -- High efficiency coding and media delivery in heterogeneous environments -- Part 3: 3D audio".

[47]	ISO/IEC 23008-3:2015/Amd 3:2017; "MPEG-H 3D Audio Phase 2".

[48]	Wenzel, E.M. - "Analysis of the Role of Update rate and System Latency in Interactive Virtual Acoustic Environments", 103rd AES Convention, New York, 1997.

[49]	Sandvad, J. - Dynamic Aspects of Auditory Virtual Environments, 100th AES Convention, Copenhagen, 1996 May 11-14.

[50]	Michael Chapman et al., A standard for interchange of Ambisonic signal sets, Ambisonics Symposium, Graz 2009.

[51]	Stitt, P., Hendrickx, E., Messonier, J.C., Katz, B. FG - "Effect of Head Tracking Latency on Binaural Rendering in Simple and Complex Sound Scenes", 140th AES Convention, 2016 June 4-7, Paris, France.

[52]	Satoshi Yairi, Yukio Iwaya and Yoiti Suzuki - "Investigation of system latency detection threshold of virtual auditory display", Proceedings of the 12th International Conference on Auditory Display, London, UK June 20 - 23, 2006.

[53]	Wenzel, E.M., "Effect of increasing system latency on localization of virtual sounds with short and long duration", Proceedings of the 7th International Conference on Auditory Display, pp.185-190, 2001.

[54]	Recommendation ITU-R BT.2021-1 (02/2015): "Subjective methods for the assessment of stereoscopic 3DTV systems".

[55]	Recommendation ITU-T P.915 (03/2016): "Subjective assessment methods for 3D video".

[56]	Brungart, D. S., Kordik, A.K., and Simpson, B.D - "The Detectability of Headtracker Latency in Virtual Auditory Displays" In: Proc. Of ICAD 2005 - 11th Meeting of the International Conference on Auditory Display, Limerick, Ireland, July 6-9, 2005.

[57]	Levitt,H. - "Transformed up-down methods in psycho-acoustics." J.Acoust Soc America,49(2), 467-477, 1970.

[58]	Mackensen, P. - "Auditive Localization, Head Movements, an additional cue in Localization." PhD thesis, TU-Berlin, 2004.

[59]	Klein, S. - "Measuring, estimating, and understanding the psychometric function: A commentary.", Perception & Psychophysics, 63(8) pp.1421-1455, 2001.

[60]	Recommendation ITU-R BT.1788 (01/2007): "Methodology for the subjective assessment of video quality in multimedia applications".

[61]	Recommendation ITU-R BT.500-13 (01/2012): "Methodology for the subjective assessment of the quality of television pictures".

[62]	Recommendation ITU-T P.910 (04/2008): "Subjective video quality assessment methods for multimedia applications".

[63]	E. Upenik, M. Rerabek and T. Ebrahimi. A testbed for subjective evaluation of omnidirectonal visual content, 32nd Picture Coding Symposium (PCS), 4-7 December 2016, Nuremberg, Germany.

[64]	Purnhagen H., Hirvonen T., Villemoes L., Samuelsson J., Klejsa J., "Immersive Audio Delivery Using Joint Object Coding" AES 140th Convention 2016 June 4-7, Paris, France.

[65]		ITU-T COM 12-C97-E, October 2009, “New subjective test method for evaluation of overall and spatial sound quality”.

[66]		Recommendation ITU-T P.806 (02/2014): “A subjective quality test methodology using multiple rating scales”.

[67]	3GPP TS 26.401 V14.0.0 (2017-03) – General audio codec audio processing functions; Enhanced aacPlus general audio codec; General description (Release 14).

[68]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General overview".

[69]	3GPP TR 26.952: “Codec for Enhanced Voice Services (EVS); Performance characterization”.

[70]	E. Asbun, Y. He, Y. He, “AHG8: InterDigital Test Sequences for Virtual Reality Video Coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0039, 4th Meeting, Oct. 2016.

[71]	Adeel Abbas, Balu Adsumilli, “AHG8: New GoPro Test Sequences for Virtual Reality Video Coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0026, 4th Meeting, Oct. 2016.

[72]	A. Singla, S. Fremerey, W. Robitza, A. Raake, “Measuring and comparing QoE and simulator sickness of omnidirectional videos in different head mounted displays”, in 9th International Conference on Quality of Multimedia Experience (QoMEX), May 2017.

[73]	R. S. Kennedy, N. E. Lane, K. S. Berbaum, and M. G. Lilienthal, “Simulator sickness questionnaire: An enhanced method for quantifying simulator sickness,” The International Journal of Aviation Psychology, 1993.

[74]	A. Singla, S. Fremerey, W. Robitza, P. Lebreton A. Raake, “Comparison of Subjective quality Evaluation for HEVC Encoded Omnidirectional Videos at Different Bit-rates for UHD and FHD”, in 25th ACM Multimedia (ACM MM) Thematic Workshops, October 2017.

[75]	Tominaga, Toshiko, et al, “Performance comparisons of subjective quality assessment methods for mobile video”, in second IEEE international workshop on Quality of multimedia experience (QoMEX), 2010.

### **** Second Change, adding section 7.4  ****

## 7.4    QoE Asessment of Simulator Sickness in VR

### 7.4.1    Introduction

There are different subjective evaluation methodologies standardized in ITU-T Rec. P.910 and ITU-R Rec. BT.500-13 for evaluating the perceived quality of 2D videos on 2D displays. However, respective methodologies do not exist for the evaluation of 360-degree videos. In this contribution, we are comparing three different subjective evaluation methodologies: Absolute Category Rating (ACR), Modified – ACR (M – ACR), and Double Stimulus Impairment Scale (DSIS) with the aim of clarifying their applicability in the assessment of 360-degree videos.

We compare the performance of these three test methods based on the following criteria:

- Statistical reliability

- Assessment time

- Simulator Sickness

#### 7.4.2    Experimental Setup

#### 7.4.2.1    Preparation of Test Stimuli

Five different video sequences (without audio) were used from the standard 360-degree videos proposed by Joint Video Exploration Team (JVET) [70,71]. All the test sequences are in YUV 420 color space, 30 fps, and 8–bits per channel. The downloaded video sequences are in Equirectangular Projection (ERP) format with a duration of 10 s. Table 7.4 shows the resolutions and bit-rates used in the experiment. The video sequences name and identifier are given in Table 7.5.

Table 7.4. Target bit-rates in MBit/s for different resolutions.

| Resolution | Bit-rate |  |  |  |
| --- | --- | --- | --- | --- |
| UHD/4K | 0.5 | 1 | 4 | 8 |
| 6K | 2 | 4 | 10 | 15 |
| 8K | 6 | 10 | 18 | 25 |

Table 7.5. Source contents and IDs [70,71]

| Content No. | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Content Name | Gaslamp | Harbor | KiteFlite | Trolley | SkateboardInLot |

#### 7.4.2.2    Test Setup

The HTC Vive Pro was used in the experiment for assessing the quality of 360-videos. The resolution of the device is 2560×1600 and the field of view is 110°. We used Whirligig player (version 4.2) for playing the videos in an HMD. The HTC Vive Pro was connected to a desktop PC equipped with an NVIDIA GTX980 graphics card and an Intel Core i7 processor.

Two subjects were performing the test one after the other. Each subject had to rate 60 Processed Video Sequences (PVS) (5 video sequences, 3 resolutions, and 4 bit-rates). The rating scale was displayed on the Vive Pro screen and the user was asked to rate either the quality of the video or compression artefacts depending on the test method. The test subject had to say a number aloud so that the experimenter could note down the ratings.

#### 7.4.2.3    Test Methods

Table 7.6 show time for assessing one stimulus for different test methods.

Table 7.6. Assessment time for different test methods

| Test Method | ACR | M – ACR | DSIS |
| --- | --- | --- | --- |
| Assessment Time | 16 s | 29 s | 29 s |

ACR Test Method

The ACR was proposed in [61,62] for evaluating the quality of the videos. In this test method, the test stimuli are shown only once and the test videos are rated independently. Subjects were asked to provide their rating after the experimenter asked “How would you rate the quality of this video?” during the voting time. The Presentation of one stimulus is shown in Fig. 7.26.

Figure 7.26:  Presentation of one stimulus in ACR method [61,62].

Modified-ACR Test Method

The M-ACR was proposed in [74] for evaluating the quality of the 360° videos in an HMD based displays. In this test method, the test stimuli are shown twice and the test videos are rated independently. Subjects were asked to provide their rating after the experimenter asked “How would you rate the quality of this video?” during the voting time. The Presentation of one stimulus is shown in Fig. 7.27.

Figure 7.27:  Presentation of one stimulus in M-ACR method [74].

Double Stimulus Impairment Scale Test Method

The DSIS test method is also known as degradation category rating (DCR) method [61,62]. In this method, stimuli are always presented in pairs. The first stimulus is always the reference/unimpaired one and the second stimulus is the same reference sequence, but obtained after processing by the system under test. We encoded the reference video with the highest quality or highest bit-rate (35 Mbit/s) with HEVC encoder so that there should not be any perceivable/noticeable degradations present in the reference video. This is done because Whirligig player cannot be used for playing the raw video sequences in raw YUV format. The presence of one stimulus is shown in Fig. 7.28.

Figure 7.28: Presentation of one stimulus in DSIS method [61,62].

#### 7.4.3   Results

In order to check the reliability of the users, outlier detection was performed base on the Pearson correlation coefficient. We selected the threshold value 0.75 for considering the reliability of the users.

In ACR study, 29 subjects (18 males and 11 females) participated in the study with an average age of 26.06 years.

In M – ACR study, 30 subjects (16 males and 14 females) participated in the study with an average age of 25.6 years.

In DSIS study, 28 subjects (19 males and 9 females) participated in the study with an average age of 24.96 years.

Figure 7.29. Average MOS for different resolutions for ACR test method

#### 7.4.3.1   Comparisons of MOS between Test Methods

Mean Opinion Scores (MOS) and the corresponding 95% Confidence Interval (CI) were calculated for each of the test stimuli in all the test methods (ACR, M-ACR and DSIS) as shown in Figures 7.29, 7.30 and 7.31. DSIS evaluates the compression artefacts or impairments present in the video, while ACR and M – ACR evaluate the absolute quality.

Figure 7.30: Average MOS for different resolutions for M – ACR test method.

One of our goals was to study whether subjects would be able to notice the difference in subjective video quality provided by the 8K, 6K and 4K. The subjective test results show that users are able to find the difference between the 4K and 6K irrespective of the test methods. But the advantage of using 8K cannot be clearly seen from the results due to resolution limitation of the HTC Vive Pro HMD.

Furthermore, from the results, it can be seen that the perceived quality at 25 Mbit/s at 8K resolution is slightly higher than 15 Mbit/s for 6K resolution irrespective of the test methodologies. Therefore, almost 40% of the bandwidth can be saved by transmitting the 360-degree videos at 15 Mbit/s at 6K resolution at a marginal loss of quality.

Figure 7.31: Average MOS for different resolutions for DSIS test method.

Figure 7.32: MOS vs CI for different test methods.

Figure 7.32 shows the plot between MOS and CI for DSIS, M – ACR, and ACR test methods. From the figure it is clear that DSIS provides higher subjective quality as compared to the other two test methods (ACR and M – ACR), thus provides higher subjective resolution power. It is interesting to note that DSIS has a slightly higher CI values as compared M – ACR.

In order to find out which evaluation methodologies is more reliable, statistical reliability was computed based on [75]. MCInorm was calculated based on the equation shown below for ACR, M-ACR, and DSIS.

MCInorm=MCIMOS Range

MCI:  Mean Confidence Interval
MOS Range: Absolute difference between the highest and lowest MOS for each test method.

Table 7.7 shows the MOS range, MCI, and MCInorm for DSIS, M – ACR and ACR test methods. Results indicate that DSIS is statistically more reliable as compared to other two test methods (M – ACR and ACR).

Table 7.7. MCI, MOS Range and MCInorm for ACR, M – ACR and DSIS test methods

|  | ACR | M – ACR | DSIS |
| --- | --- | --- | --- |
| MCI | 0.1382 | 0.1392 | 0.1405 |
| MOS Range | 2.824 | 3.088 | 3.16 |
| MCInorm | 0.0489 | 0.0450 | 0.0444 |

### 7.4.3.2    Assessment of simulator sickness in HMD

While watching the 360° videos in HMD, users may experience symptoms of simulator sickness. Therefore, assessing the simulator sickness is a relevant additional step. Simulator sickness is a sub-category of motion sickness and symptoms comprise fatigue, sweating, vertigo, nausea, etc. [72,73]. The most popular Questionnaire for assessing the simulator sickness is Simulator Sickness Questionnaire (SSQ) published in 1993 [73], which we used for our experiment (shown in Annex C).

SSQ was derived from the motion sickness questionnaire (MSQ) [73], selecting 16 out of the original 28 symptoms for analysis. These symptoms are further classified into three sub-categories: nausea (N), oculomotor (O), and disorientation (D). Not all 16 symptoms are used for calculating N, O, D, and unit weights are assigned in each category. For obtaining the scores, a 4-point scale (0, 1, 2, 3) is used and weighted values are added to get the scores for each category. N, O, D, and Total Score (TS) are then calculated using the method shown in [73] and Annex A.

Simulator sickness scores were computed for each of the test methods. Subjects were instructed to fill in simulator sickness questionnaire before and after each of the test session. We also collected SSQ scores just after the pre–screening, but before the training session.

In this work, we wanted to find out how does the SSQ scores changes with time. Also, we wanted to study the impact of breaks on reducing the simulator sickness scores. From the figures 7.33, 7.34, and 7.35, it is clear that SSQ scores increase with time irrespective of the test methodology. Also, breaks help in reducing the simulator sickness among subjects which underlines the significance of breaks during the tests. It is interesting to note that users felt less prone to simulator sickness while evaluating the 360-degree videos using DSIS methodology as compared to ACR and M – ACR methodologies. Although the duration of one test session in ACR (7 mins) is much lower as compared to the one of DSIS (11 mins) test method.

Figure 7.33. Simulator Sickness Scores for all the test sessions for ACR test method.

Figure 7.34: Simulator Sickness Scores for all the test sessions for M – ACR test method.

#### 7.4.4    Conclusion

In this contribution, we have compared the three evaluation methodologies ACR, M – ACR and DSIS on the subjective testing of the 360-degree videos using HMD. We found out that DSIS provided higher resolution power as compared to the other two test methodologies. Also, we evaluated the simulator sickness in each of the test methodologies. Results show that simulator sickness increases with time and breaks help in reducing the simulator sickness which highlights the significance of breaks during the test.

Figure 7.35: Simulator Sickness Scores for all the test sessions for DSIS test method.

### **** Third Change, inserting an additional Annex ****

Annex C:
Simulator Sickness Questionnarie (SSQ)

1. General Discomfort

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

2. Fatigue

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

3. Headache

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

4. Eye Strain

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

5. Difficulty Focusing

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

6. Salivation Increasing

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

7. Sweating

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

8. Nausea

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

9. Difficulty Concentrating

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

10. Fullness of the Head

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

11. Blurred Vision

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

12. Dizziness with Eye Open

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

13. Dizziness with Eye Closed

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

14. Vertigo

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

15. Stomach Awareness

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

16. Burping

| None | Slight | Moderate | Severe |
| --- | --- | --- | --- |

Nausea ([1]): 1, 6, 7, 8, 9, 15, 16

Occulomotor ([2]): 1, 2, 3, 4, 5, 9, 11

Disorientation ([3]): 5, 8, 10, 11, 12, 13, 14

Score:

N = [1] * 9.54

O = [2] * 7.58

D = [3] * 13.92

TotalScore = ([1] + [2] + [3]) * 3.74

Annex D:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 06-2017 | SA#76 | SP-170331 |  |  |  | Presented to TSG SA#76 for information | 1.0.0 |
| 06-2017 | SA4#94 | S4-170651 |  |  |  | Completion of the TR with the inclusion of scope, media formats description, audio and video quality subjective tests, motion to sound analysis, use cases completion, QoE metrics analysis and a set of conclusions | 1.1.0 |
| 06-2017 | SA4#94 | S4-170752 |  |  |  | Editorial modifications | 1.2.0 |
| 09-2017 | SA#77 | SP-170619 |  |  |  | Presented to TSG SA#77 for approval | 2.0.0 |
| 09-2017 |  |  |  |  |  | Version 15.0.0 | 15.0.0 |
| 12-2017 | SA#78 | SP-170832 | 0003 | - | B | Findings and Conclusions from study on 3GPP codecs for VR audio | 15.1.0 |
| 03-2018 | SA#79 | SP-180030 | 0004 | 1 | F | Corrections to subjective listening tests | 15.2.0 |