Source:	Nokia Corporation

Title:	Mobile capture of immersive audio for IVAS: Use cases

Document for:	Discussion & Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

The IVAS codec will introduce a significant change in the overall audio capability over the previous conversational 3GPP codecs, such as the EVS codec which IVAS will be an extension of. Adding support for stereo and immersive audio will enable many new use cases as well as improvements over ones that were relevant already for mono operation. With such major change it can easily appear that the codec requirements start to get overly complicated, and this can trigger the desire to limit the design/project complexity. On the other hand, it should be made sure that the resulting standard will meet foreseen market needs and no critical capability is lost. It is the opinion of the source that these aspects still require further discussion and progress to help develop a successful standard.

In this input document, the source presents two use cases that are relevant particularly for the mobile device (smartphone) market. Considering the IVAS WID [1], a strong focus is currently seen in use cases relating to Virtual Reality (VR) with the IVAS Design Constraints currently reflecting this. For successful IVAS deployment, it is the view of the source that more focus on immersive communications, teleconferencing, user-generated content (UGC) and potentially also augmented reality (AR) (that could in many cases be audio-only) is necessary. The current input document discusses some of these topics particularly from the viewpoint of the smartphone implementations.

# 2. Mobile capture of immersive audio

The smartphone, with yearly global shipments high above the one billion mark, currently dominates the digital user equipment space. This trend can be expected to continue in the coming years, although new device categories and form factors may also grow in significance affecting among other things the way people record and experience audio, whether alone or in connection with visual content. One related trend is the increase in headphone use especially among younger people but in also other groups such as the professional office environment.

Smartphone spatial audio capture based on a three-microphone configuration was first introduced on the market in early 2014. This enabled, e.g., a 5.1 audio capture by parametric capture analysis and audio synthesis means. Since then several manufacturers have released devices capable of immersive audio capture in various forms, although the services for users to fully enjoy this enhanced capability have been lacking. By the end of 2018, at least a stereo capture capability can finally be considered the baseline quality level for audio recording in the smartphone market with interest in immersive audio capture steadily on the rise.

As demonstrated in [2] and [3], a parametric representation of the immersive audio best guarantees the high quality of reproduction both in binaural headphone and loudspeaker listening of the mobile-captured content. The quality difference of an example parametric representation over a first-order ambisonics (FOA) representation of the same capture was shown to be significant both for expert and naïve listeners. Worth noting is also the inherent efficiency advantage of the example parametric approach over the corresponding FOA signal, where only 2 channels and additional spatial metadata (in the order of 25 kbit/s in the listening test experiments) would need to be encoded instead of the 4 channels for FOA.

In order to enable the IVAS codec to optimally deal with this important class of devices and their immersive audio capture capability, it has previously been proposed by the source to agree on a metadata-assisted spatial audio (MASA) format. The definition of the format has been developed in SA4 over five meetings with some progress. However, it has not been possible to reach agreement on even the principle of supporting a parametric format for multi-microphone smartphone capture of immersive audio. Specifically, IVAS-4 currently covers this important aspect with only text in square brackets under the title Audio Formats [4]:

- [Spatial audio, [N] channels and spatial metadata defined by [TBD].]

[Editor’s Note FFS: Spatial metadata definition for the spatial audio format will require further input.]

# 3. Use cases for IVAS

Two use cases relating to immersive audio capture using a mobile device (smartphone) are presented. The first use case relates to the immediate sharing of experiences that is currently popular, e.g., in many social media applications. The smartphone is generally the preferred tool for this for the modern consumer, in part because it is easy to bring to many new locations and activities in addition to allowing for the connectivity required for digital content sharing. On the other hand, capture and transmission of immersive audio (using IVAS) now make this possible in the context of voice calls allowing for a more personal experience sharing. This use case is described in detail under clause 3.1 “Immersive audio call with experience sharing”.

The second use case relates to teleconferencing, where significant improvement in user experience can be achieved by spatial capture and presentation of the talkers. High-end teleconferencing systems can in many cases provide a real alternative, e.g., for business travel or at least reduce its need. The availability of such systems can however be limited, and a great deal of planning and attention to schedules is often required to utilize them efficiently through long calls. Therefore, teleconference participation often happens with mobile devices, sometimes even with several participants in the same space captured by a single device. Currently, this can limit the experience for all participants. Improvements are therefore welcome, and one significant source of improvement is the use of spatial audio capture on mobile devices. This capability improves the overall listening experience for the other end particularly when there are several users captured in the same space. The source believes this helps to make adhoc teleconferencing more enjoyable and efficient thus providing not only increased quality of experience but also boosting overall productivity in many aspects of both work and personal life. This second use case is described in detail under clause 3.2 “Adhoc spatial audio telco”.

The two use cases are described using the template proposed in [5].

## 3.1 Immersive audio call with experience sharing

The use case is summarized as follows:

Use Case Name

Immersive audio call with experience sharing

Description

Use case short description: - Audio call is established between two participants - During the call, participant with immersive audio capture capability wishes to share their experience (its atmosphere/ambience) with the other party - A spatial sound scene is transmitted and rendered to the other party User story: Alice is at home sitting on the living room couch. Alice calls her son Bob wishing to hear what Bob and Alice’s grandchildren are up to. Bob is at the race track, where his children are racing go-karts. Bob is standing in the middle of a hairpin corner with go-karts driving around him. Bob answers the call, greets Alice, and wishes to share the race track atmosphere with her. Bob extends his hand holding his smartphone towards the track corner. Alice now clearly hears the go-karts driving around her, with Bob excitedly cheering his children to go faster, over her headphones. This brings a smile on Alice’s face.

Categorization

Type: Immersive Degrees of Freedom: 0DoF/3DoF Delivery: Conversational Media Components: Audio-only (Audio-Visual possible) Device: UE (Smartphone), Headphones

Preconditions

- Alice is wearing headphones (potentially with head-tracking) - Bob’s smartphone implements multi-microphone immersive audio capture

Requirements and QoS/QoE Considerations

- QoS: Codec bit rate (16.4 or) 24.4 kbps and higher for high-quality encoding of immersive audio captured by smartphones - QoE: Immersive audio rendering/binauralization quality

Feasibility

Multi-microphone capture of immersive audio on smartphones is getting more common. The smartphone currently dominates the entertainment and communications device categories. Use of headphones for entertainment and communications purposes is increasingly popular. This includes mobile use. Head-tracking technologies are currently not common in consumer devices (such as headphones). There is however growing interest in this capability. On the other hand, while head-tracking is beneficial for the current use case, it is not strictly required.

Potential Standardization Status and Needs

Required: - Support for high-quality encoding of immersive audio captured by smartphones - Binaural rendering of immersive audio to headphones Potentially required: - Head-tracking information interface for decoder/renderer - Mode switching between capture/input formats

Figure 1 provides an illustration of the use case. A first user (Bob) captures the audio using a smartphone with multi-microphone audio capture and analysis. A second user (Alice) hears the binaural rendering of the immersive scene over headphones.

In addition to the audio-only use case described here, a somewhat similar audio-visual use case could be realized by inclusion of visual content. The visual content could be any of 2D video, 3D video or VR video. Furthermore, similar use case descriptions (in audio-only or audio-visual form) could be realized for user-generated content (UCG) creation/distribution, e.g., for streaming or social media service upload. The underlying audio capture capability is the same.

Figure 1. Example of immersive audio capture with binaural rendering for experience sharing.

## 3.2 Adhoc spatial audio telco

The use case is summarized as follows:

Use Case Name

Adhoc spatial audio telco

Description

Use case short description: - Audio call is established between two participating sites with several participants each - One site utilizes a mobile device for audio capture and rendering, and the other site features a meeting room with a spatial audio system - A spatial sound scene is rendered in the meeting room, and a mono audio downmix of the spatial audio is rendered for the mobile device participants User story: Bob and his colleagues have scheduled a teleconference with their clients. They are returning from a business trip, and their flight has been delayed. Bob and his colleagues will therefore not make it to the office in time. Instead of cancelling the call, Bob finds a meeting room at the airport and uses his smartphone to join the teleconference. Bob greets the clients who are seated in a meeting room with spatial audio capture and loudspeaker presentation and introduces the participants joining with him. Bob’s smartphone captures the spatial audio allowing for the voices of Bob and his colleagues to be spatially rendered in the meeting room. The meeting room audio is played back in mono on the smartphone loudspeaker. The meeting is a great success.

Categorization

Type: Immersive Degrees of Freedom: 0DoF Delivery: Conversational Media Components: Audio-only Device: UE (Smartphone), Loudspeakers (meeting room spatial audio capture and presentation system)

Preconditions

- Bob’s smartphone implements multi-microphone immersive audio capture

Requirements and QoS/QoE Considerations

- QoS: Codec bit rate (16.4 or) 24.4 kbps and higher for high-quality encoding of immersive audio captured by smartphones - QoE: Immersive audio loudspeaker rendering quality, accuracy of talker separation in spatial audio capture

Feasibility

Multi-microphone capture of immersive audio on smartphones is getting more common. The smartphone currently dominates the (entertainment and) communications device category(ies). Spatial audio capture in a meeting room can be achieved using various microphone array configurations or discrete channels can be obtained from several microphones (worn by or otherwise allocated to talkers).

Potential Standardization Status and Needs

Required: - Support for high-quality encoding of immersive audio captured by smartphones - Spatial audio signal rendering to loudspeakers Potentially required: - Rendering scalability, i.e., a spatial signal is rendered at lower spatial dimension (e.g., as mono or stereo only)

Figure 2 provides an illustration of the mobile audio capture for the use case. A smartphone implementing a multi-microphone capture and analysis has been placed on a table. User voices around the table are captured into an immersive audio format, encoded, and transmitted using the smartphone. The audio presentation from the smartphone according to the use case description is playback of a mono downmix. Alternatively, this could also be a stereo presentation according to device capability.

Figure 2. Audio capture example for a spatial audio telco use case. A smartphone with a multi-microphone capture creates a spatial representation of the sound field.

# 4. Conclusion and Proposal

In this document two immersive audio use cases based on a smartphone capture are discussed.

The source proposes to document the two use cases in the “new Pdoc on scenarios”, agreed to be created in the Busan meeting [6]. Considering the skeleton provided in Annex A of [5], the first proposed use case (“Immersive audio call with experience sharing”) could be considered under 3.1.1 Stereo and Immersive Telephony. The second proposed use case (“Adhoc spatial audio telco”) fits under 3.2.1 Spatial conferencing. It is the source’s view that at least these use case sections are clearly relevant for IVAS.

The source believes that further work on the IVAS use cases (or scenarios) continues to be useful particularly to develop the IVAS Design Constraints but also for reasons discussed in [5]. Advanced use cases, e.g., related to user interaction have so far not been thoroughly discussed in the project. Use cases (or scenarios) particularly for immersive communications, teleconferencing, user-generated content (UGC) and augmented reality (AR) (potentially with 6DoF support) seem relevant.

# References

[1]	Tdoc SP-170611: New WID on EVS Codec Extension for Immersive Voice and Audio Services.

[2]	Tdoc S4-180087: On IVAS audio formats for mobile capture devices. Source: Nokia Corporation.

[3]	Tdoc S4-180462: On spatial metadata for IVAS spatial audio input format. Source: Nokia Corporation.

[4]	Tdoc S4-181218: IVAS Design Constraints (IVAS-4), v0.0.6.

[5]	Tdoc S4-181301: Collection of IVAS Use Cases. Source: Dolby Laboratories, Inc.

[6]	Tdoc S4-181456: Draft report from SA4#101 EVS SWG. Source: EVS SWG Secretary (Orange).