# Liaison Statement

| Liaison Statement Title: | LS to SA4 on Codec Mode-sets |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| RILTE #68 | 2018-06-21 | London, United Kingdom |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| RILTE #68 Doc 104 | 2018-11-26 | Wayne Cutler, GSMA |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| NG RILTE | RILTE#68(see below) | GSMALiaisons@gsma.com |
| Security Classification | Non-confidential |  |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | None |
| External Recipients: | 3GPP WG SA4 |

Introduction

This LS concerns the AMR-WB mode-set parameter. In particular, it concerns the SDP negotiation related to the mode-set parameter for inter-IMS network sessions where each of the peer networks has a different network preference, and configures its UEs to reflect that network preference (e.g. some networks have a preferred mode-set of 0-7, whilst others have a preference for 0-2).

When an IMS session is established between networks with differing N/W preferences, there is no current guidance in 3GPP specifications as to how mode-set selection should be handled during the SDP negotiation. A more fundamental point is whether the chosen mode set for an inter-IMS call should be determined by:

- the originating N/W Preference taking precedence,

- the terminating N/W Preference taking precedence,

- configuration data at a given NNI.

  - As the number of IMS NNIs increases, this approach has scalability concerns as a given N/W needs to reach a SLA with each of its interconnected peer N/Ws.

- the SDP negotiation whereby the “optimum” mode-set is agreed (i.e. the least constrained mode-set acceptable to both networks).

Finally, RILTE also notes that issues raised in LS are not limited to AMR-WB but are generic issues that applicable to any multi-mode codec.

Analysis of Current SDP Negotiation Rules

NG RILTE notes that:

- Section 6.2.2.2 of 3GPP TS 26.114 and GSMA PRD IR.92 state that   a compliant originating UE is required to propose in the SDP Offer at least one AMR-WB payload type without an associated mode-set parameter.

  - A compliant UE is not precluded from also including one or more additional AMR-WB payloads with associated mode-set parameters.

- 3GPP TS 26.114 table 6.3 states that if the terminating UE receives and selects an SDP offer for a payload type in which the mode-set parameter is present, it must indicate the same value (set of modes) in the SDP answer.

- Modification of an SDP Offer by any intervening N/W is not precluded.

  - Any N/W can remove any payload type or add a mode-set parameter to a payload type without an associated mode-set parameter prior to forwarding the SDP offer to a subsequent N/W or terminating UE.

    - In the former case, removal of a payload could result in AMR-WB not being included in the offer that reaches the terminating UE.

    - In the latter case, a N/W can enforce its preference by adding a mode-set parameter to a payload which did not previously have an associated mode-set parameter,

  - A N/W on receipt of an SDP Offer including only payloads with associated mode-set parameters can only modify a received mode-set (to a sub-set of the received value) if that mode-set was inserted by a previous N/W. However, there is no current mechanism to denote whether a mode-set parameter was initially included by the UE or a previous N/W.

The key point in the above bullets is that a given network can only alter a mode-set parameter (e.g. 0-7 to 0-2) if that mode-set parameter was inserted by a preceding network and not by the originating UE.  However, there is no current mechanism to indicate whether the mode-set was included by the UE or the network.  To illustrate this point, consider the following example flows between Network1 (preference of mode-set 0-7) and Network2 (preference of mode-set 0-2).

- In Flow 1:

  - the originating UE generates an Offer with a single AMR-WB payload with no mode-set

  - Network1 enforces its preference and adds a mode-set (0-7) to the single AMR-WB payload and forwards the Offer to Network2,

  - Network2 receives the Offer with a single AMR-WB payload with mode-set 0-7 and overwrites the mode-set to 0-2 in line with its network preference prior to forwarding Offer to the UE.

  - The terminating UE responds with SDP Answer containing the mode-set 0-2 which is transited back to the originating UE.

- In Flow 2:

  - the originating UE generates an Offer with a 2 AMR-WB payloads, one with no mode-set and one with mode-set 0-7.

  - Network1 notes that one payload already reflects its N/W preference and removes the AMR-WB payload without a mode-set and forwards the Offer to Network2.

  - Network2 receives the Offer with a single AMR-WB payload with mode-set 0-7 and overwrites the mode-set to 0-2 in line with its network preference prior to forwarding Offer to the UE.

  - The terminating UE responds with SDP Answer containing the mode-set 0-2 which is transited back to the originating UE.

- In Flow 3:

  - the originating UE generates an Offer with a 2 AMR-WB payloads, one with no mode-set and one with mode-set 0-7.

  - Network1 removes the AMR-WB payload with a mode-set and also enforces its preference by adding mode-set 0-7 to the other payload prior to forwarding the Offer to Network2.

  - Network2 receives the Offer with AMR-WB single payload with mode-set 0-7 and overwrites the mode-set to 0-2 in line with its network preference prior to forwarding Offer to the UE.

  - The terminating UE responds with SDP Answer containing the mode-set 0-2 which is transited back to the originating UE.

In all flows, Network2 receives an identical SDP Offer containing a single AMR-WB payload with mode-set 0-7 which is modified to mode-set 0-2 prior to being forwarded to the terminating UE and reflected in the SDP Answer.

However, it is observed that:

- In Flow 1 and Flow 3:

  - The mode-set parameter was inserted by Network1,

  - Network2 can thus overwrite this mode-set with its own preference of 0-2 as this is a sub-set of the received value of 0-7.

  - The final mode-set is the “optimum” value and the call is successful.

- In Flow 2:

  - The mode-set was inserted by the UE,

  - Network2 cannot overwrite this mode-set,

  - If Network2 altered this mode-set to 0-2, the call would fail.

As stated previously, there is no current mechanism defined to enable Network2 to determine whether a received mode-set was inserted by the originating UE or by Network1. In the absence of such a mechanism, RILTE notes that further guidelines   could be considered such that Network1 would only send mode-sets to Network2 that have not been inserted by the UE and thus can be modified (i.e. Network1 behaves as for Flow 1 / Flow 3 above and not as Flow 2).

As a related point, when reducing a mode-set, are there any constraints as to how a mode-set can be reduced, apart from the reduced mode-set being a sub-set of the received mode-set?

Action to SA4

NG RILTE requests SA4 to:

- Consider whether further guidance can be added to 3GPP specifications to address recommended N/W behaviour for selecting a mutually acceptable mode-set where there is a miss-alignment between the AMR-WB mode-set preferences in an originating and terminating N/W.

- Consider whether an additional indicator is required to be added to the mode-set parameter to denote that it is “Provided by the Network”.  Such an indicator would be added by any N/W Element modifying the SDP Offer and would only be checked by any N/W Element wishing to modify the SDP Offer.

  - Furthermore, if such an indicator is added, then it is recommended be removed by the terminating network prior to forwarding the SDP Offer to the terminating UE.

- Confirm that there are no constraints when reducing a mode-set, i.e  a received mode-set of “j-k”  can be changed to “n-m” as long as :-

  - n>=j, and

  - m <= k, and

  - n <= m

Next Meetings:

RILTE#69 – 4th April 2019, Split, Croatia

RILTE#70 – 20th June 2019, London, UK