Source: 	Fraunhofer IIS

Title: 	Pass-through Operation and External Rendering Interface

Document for: 	Discussion and Agreement

Agenda Item: 	7.5 – IVAS_Codec

1. Introduction

This contribution addresses the open points with respect to the proposed pass-through operation of IVAS and the external rendering interface. It is proposed to incorporate the proposed changes into the IVAS Design Constraints permanent document [1].

2. Discussion

Pass-Through Operation

To preserve maximum audio quality, artistic intend and to facilitate testing, the IVAS codec shall support pass-through of channel-based audio, binaural audio, scene-based audio and object-based audio. Support for pass-through operation for “Spatial audio, [N] channels and spatial metadata defined by [TBD]” should be studied once such a format is clearly specified. It is understood that this mode might not work at all audio bitrates, especially not at very low bitrates. Thus, a minimum bitrate per format for the pass-through operation shall be set in the IVAS performance requirements [2].

In previous discussions the question was raised what exact information needs to be preserved in pass-through mode. The IVAS codec is not seen as lossless, so it is understood that quantization takes place which of course also impacts the pass-through mode. However, the principles outlined above (preserve maximum audio quality, artistic intend and to facilitate testing) should be followed, so the following clarification should be added:

- Channel-based audio: For pass-through operation, the output channel configuration shall be the same as the input channel configuration (e.g. 7.1 input shall result in 7.1 output).

- Binaural audio: For pass-through operation, binaural input shall result in binaural output.

- Scene-based audio: For pass-through operation, scene-based audio input shall result in scene-based audio output of the same order as the input.

- Object-based audio: For pass-through operation, audio objects input shall result in audio object output of the exact same number of audio objects. Quantization might be applied to accompanying metadata, so not all metadata attributes might be preserved.

- For mixed formats, the same principle applies accordingly for the individual components.

External Rendering Interface

In addition to a built-in renderer, there is a strong industry demand for providing an additional interface, which could be used to build own rendering solutions to both, headphones and loudspeakers. It is proposed that this interface has direct access to

- Channels

- Scene-based audio

- Objects

This allows maximum flexibility and connects directly to the pass-through operation as defined above and thus preserves maximum audio quality.

3. Proposed changes to IVAS-4

The following changes are proposed to the IVAS Codec Design Constraints

In section 3, apply the following change:

*** Start change 1 ***

| Pass-Through Operation | The IVAS codec shall support pass-through operation for the following audio formats: - Channel-based audio, including mono (1.0), stereo (2.0 - Binaural audio - Scene-based audio, first-order (FOA) and up to [N]-order Ambisonics. - Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. For pass-through operation, the decoder output format is identical to the encoder input format according to the following definition: - Channel-based audio: For pass-through operation, the output channel configuration shall be the same as the input channel configuration (e.g. 7.1 input shall result in 7.1 output). - Binaural audio: For pass-through operation, binaural input shall result in binaural output. - Scene-based audio: For pass-through operation, scene-based audio input shall result in scene-based audio output of the same order as the input. - Object-based audio: For pass-through operation, audio objects input shall result in audio object output of the exact same number of audio objects. Quantization might be applied to accompanying metadata, so not all metadata attributes might be preserved. - For mixed formats, the same principle applies accordingly for the individual components. The exact bitrates for which pass-through operation is to be supported is specified in IVAS Performance Requirements (IVAS-3). |
| --- | --- |
| External Rendering Interface | The IVAS codec shall support the following interface formats for optional external rendering solutions: - Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] - Scene-based audio, first-order (FOA) and up to [N]-order Ambisonics. - Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. |

*** End change 1 ***

4. Conclusion

This contribution addresses the aspects of pass-through operation and external rendering interface for the IVAS codec. It is proposed to agree on the inclusion of the changes outlined above into IVAS-4.

5. Reference

[1] 3GPP S4-190248 – IVAS Design Constraints (IVAS-4), Version 0.0.7.

[2] 3GPP S4-181219, IVAS Performance Requirements (IVAS-3)