3GPP TSG-SA4 Meeting #103 	S4-190470

# Newport Beach, CA, US, Apr 8 - 12, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | - | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Metrics collection and reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing descriptions for metric collection and reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a metrics control interface, as well as procedure examples. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how metrics are handled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 5 (new), 5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

================================= Start of first change =================================

### 4.2.2	UE Media Functions

Figure 4.2.2-1 shows the functional components of a UE to access network-based media functions.

Figure 4.2.2-1: UE Media Functions

NOTE 1: 	SBA refers to principles of 5G Service based Architecture (SBA) realizations of control transactions.

NOTE 2: 	The white color indicate here that the 3GPP has created specifications for the function.

NOTE 3: 	A UE is a logical device, allowing for tethering or other type of realizations.

The following subfunctions are identified as more detailed breakdown of the UE based Trusted Media Functions:

-	Operator App or Third party App: Uses APIs to access media services

-	Media Access Client: Accesses e.g. DASH Formatted Media

-	DRM Client (Optional): When present, the DRM client realizes the policies.

-	Media Decoders: Decodes the media like audio or video

-	Media Rendering / Presentation: Renders the media using an appropriate output device.

-	Media Session Handler: Realizes a session concept, spanning optionally over multiple stateless sessions. May optionally interact with network based Media AFs.

-	Metrics collection and reporting: executes metrics collection and reporting.

Interfaces:

NOTE 4: 	The following device internal APIs may exist inside the UE.

- MC1: Media Control Interface(s) to configure and interact with the different UE media functions.

- MC2: Media Control Interface for media session management.

- MC3: Control interface for metrics collection and reporting

- MU1: Decoded media samples are handed over to the media renderer.

- MU2: Decrypted, compressed media samples are handed over to a trusted media decoder.

- MU3: In case of encryption, the encrypted, compressed media samples are handed over to the DRM Client.

Editor's Note: Roaming, Non-3GPP Access and EPC-5GC interworking aspects are TBD

[

Editor's Note: The text in square brackets is not agreed.

================================= Start of next change =================================

# 5	Procedures for Downlink Streaming

## 5.x	Metrics collection and reporting

Metrics collection and reporting can be done in different ways, depending on the relation between the Service Provider and the Network Operator. The following figures shows simplified signalling examples for two different use-cases.

In the first use-case, shown in figure 5.x.1 below, the MNO is controlling the metric reporting, using the out-of-band configuration and reporting option. Note that in this scenario the App is not actively involved in collection or reporting of the metrics, so metrics can be collected for any App which uses the trusted media functions in the UE, even when the media server is not under control of the MNO.

Figure 5.x.1: Metrics collection using the out-of-band 5G control plane option

The different steps are explained below:

1: Overall metrics configuration is done on the network level, for instance defining which geographical areas that shall have metrics collection active.

2: The metric configuration(s) is sent to the RAN, but not forwarded to the UE at this stage.

3: Time passes, the UE moves around.

4: The UE enters an area (cell, location area, etc.,) which is inside the geographical constraint. This is discovered by the RAN, and it now need to activate metric reporting for the UE.

5: The actual metric configuration is sent from the RAN to the metrics collection reporting function in the UE, via the 5G control plane.

6: Time passes, the UE has a metrics configuration, but no streaming session has started.

7: A streaming session is started.

8: The session setup is done, including a lot of signalling (not shown here). No metrics configuration is done during this setup.

9: Media is delivered and rendered, and...

10: ...more media is delivered...

11: Metrics are collected during the session according to the configuration, and reported via the 5G control plane with the frequency defined in the configuration.

12: The session continues...

13: ...with more media...

14: ...and more metrics reports, until the session ends.

15: Time passes, no streaming session is active.

16 to 23: Another session is started, and as the UE is still inside the collection area, this session will also be collecting metrics in the same way as steps 7-14.

24: Time passes, the UE moves around.

25: The UE leaves the wanted geographical area.

26: RAN sends metrics (de)configuration to the UE, to stop future metrics collection.

The second use-case, shown in figure 5.x.2 below, illustrates a scenario where the metrics collection is configured in-band. Thus together with the media there are metadata which describe how metrics shall be collected, and how they shall be reported. The AF/AS functions can both be either trusted or untrusted (but not mixed).

Figure 5.x.2: Metrics collection using the in-band metadata option

The different steps are explained below:

1: The content servers are setup, including any metrics configuration.

2: If the AF and AS are in the trusted domain, metadata regarding the metrics collection and reporting are sent to the AS. If they are in the untrusted domain, the procedure how to configure the AS is unspecified.

3: Time passes, no streaming session is active...

4: A streaming session starts.

5: The App and the session handler initializes a new session.

6: The media access client requests any metadata which is needed to be able to download the content

7: Metadata is returned to the access client and also forwarded to the App.

8: The metrics parts of the metadata is sent by the App to the metrics collection and reporting function.

9: The streaming session is running.

10: Media is delivered and rendered, and...

11: ...more media is delivered...

12: Metrics are collected during the session according to the configuration, and reported with the frequency defined in the configuration. The reporting method is defined in the configuration, for instance HTTP upload.

13: The session continues...

14: ...with more media...

15: ...and more metrics reports, until the session ends.