TSG SA4#104 meeting	Tdoc S4-190572

1st – 5th July 2019, Cork, Republic of Ireland	Agenda Item: 5.2

3GPP TSG RAN WG1 Meeting #97  												R1-1907911

Reno, USA, May 13-17, 2019

Title:	Reply LS on physical layer data rate of RAN-assisted codec adaptation

Response to:	S4-190556

Release: 	Rel-14, 16

Work Item: 	LTE_VoLTE_ViLTE_enh-S4, E2E_DELAY

Source:	RAN WG1

To:	SA4, RAN2

Cc:	GSMA 5GJA

Contact person:

Name:			Yubo Yang

Email Address:	yangyubo1@huawei.com

# Overall Description

RAN1 would like to thank SA WG4 for their LS in R1-1905922(S4-190556). RAN1 discussed the issue and would like to provide following answers to the following question:

| WG | Question |
| --- | --- |
| RAN1 (ANBR) | - In clauses 16.2.1.1 of TS 38.300 and 5.18.10 of TS 38.321, and clauses 23.15.1 of TS 36.300 and 5.18 of TS 36.321, the recommended bit-rate is defined to be in kbps at the physical layer at the time when the decision is made. How is the bit-rate at the physical layer defined? |

Answer from RAN1:

The uplink/downlink bit-rate at the physical layer is rUL/DL=kLkT, where Lkis the bit-length of the k-th successfully transmitted/received TB by the UE within the window T

Please note that in 3GPP TS 36.321 and 38.321, a window length of 2000ms is applied.

# Action:

To SA4: RAN1 respectfully asks SA WG4 to kindly consider the provided answer from RAN1 point of view.

To RAN2: RAN1 respectfully asks RAN2 to kindly consider the above answer from RAN1 and confirm whether it is proper from RAN2 point of view.

# Date of next RAN1 meetings:

TSG-RAN1 meeting #98		August 26-30 2019			Prague, Czech Republic

TSG-RAN1 meeting #98bis		October 14-18 2019		China