Source:	HEAD acoustics GmbH, Intel

Title:	Test Results of Noise Field Simulations

Document for:	Discussion and agreement

Agenda Item:	9.7

# 1	Introduction

The study item FS_ANTeM [1] was recently introduced in order to investigate ambient noise testing according to the terminal measurement specifications TS 26.131 & 132 [2], [3]. Topics like inter-lab variability of background noise measurements or usage of different noise field simulation techniques are addressed to propose improvements of the specifications.

This contribution looks at the reproducibility of the background noise configurations among various lab by assessing the difference in the noise spectra between measurement at the HATS ear microphones and the reference noise.

# 2	Noise Field Simulations

## 2.1	System according to ES 202 396-1 (binaural noise sources)

The noise field simulation system according to ETSI ES 202 396-1 [4] is currently used for handset UE testing in TS 26.132 [3]. It utilizes a 4.1 loudspeaker setup (four satellites and one sub-woofer) for reproduction and is equalized in order to play back binaurally recorded signals. The equalization procedure is conducted in several automated steps, but also manual adjustments for cross-talk cancellation are necessary.

However, recent investigations (see clause 4.2.3.5.2 of TR 26.921) indicated drawbacks regarding the reproducibility across different labs, presumably due to manual steps in the equalization process.

The setup is denoted as ES202 in the following sections.

## 2.2	System according to TS 103 224 (8-channel sources)

The noise field simulation according to ETSI TS 103 224 [4] provides an improved reproduction accuracy by:

- An eight-channel microphone array (instead of two ear microphones) as shown in Figure 1, developed in particular for testing of handset and handheld hands-free devices,

- Eight loudspeakers (instead of four loudspeakers and a subwoofer),

- An automated procedure for room equalization and cross-talk cancellation (instead of manual adjustment).

Figure 1: Microphone array for handset testing according to ETSI TS 103 224

Due to the different simulation system, the type of noise sources are different as well – instead of binaural noise recordings, eight-channel signals recorded with the same microphone array have to be used. For this purpose, ETSI TS 103 224 provides a sound file database including similar scenarios as in ETSI ES 202 396-1.

The setup is denoted as TS103-HS in the following sections.

## 2.3	System according to TS 103 224 (binaural noise sources)

Beside the default handset and hands-free setups, the recently updated version of ETSI TS 103 224 [4] provides an extension for so-called “flexible setups”. Here an arbitrary number of N microphones and M loudspeakers may be used for the equalization procedure, i.e. the automated equalization procedure is generalized to other configurations (with the restriction of N ≥ 2 and M ≥ N). As microphone inputs, the two ears of the HATS were used (N=2). This allows the usage of the same noise types as for ES202.

Figure 2 illustrates the principle of the different two-point equalizations more in detail. In general, all investigated measurement rooms are equipped with eight loudspeakers and a sub-woofer in order to realize the setups required for ES202 and TS103-HS. Depending on the selected configuration, either four (labelled as 1-4 in Figure 2) or eight (labelled as 1-8 in Figure 2) channels are used for equalization and playback. The subwoofer may also be used as a fifth output channel.

Figure 2: Possible flexible equalization configurations

For the present study, the following setups with M ≥ 4 loudspeakers were investigated:

- 4.1 (four loudspeaker, one sub-woofer – as specified in ETSI ES 202 396-1)

- 4.0 (without sub-woofer)

- 8.0 (same loudspeaker setup as specified in default setup of ETSI 103 224)

The flexible methodology requires the usage of a so-called “reference sound”, which should be a typical example of the microphone configuration. It seems trivial to simply use an arbitrary binaural recording for this purpose; however, several pitfalls were discovered during the equalization across several measurement rooms:

- Since the equalization procedure optimizes transfer functions and filter coefficients based on this sound, it is desirable to select a binaural noise which is not included in the later evaluation,

- The reference sound must provide sufficient “binaural content” in order to ensure adequate cross-talk cancellation between left and right ear,

- The reference sound must provide sufficient energy across all frequency bands in order to accurately determine a transfer function between source and recorded channel.

Based on these observations, the binaural noise TrainStation_bin was selected, which is provided in TS 103 224 as the binaural version of the eight-channel recording (see also Table 3).

The setups are denoted as TS103-4.0, TS103-4.1 and TS103-8.0 in the following sections.

# 3	Noise Types

According to the descriptions of clauses 7.12.1 (NB), 8.12.1 (WB) and 9.12.1 (SWB/FB), the binaural noise types as shown in Table 1 are used for the evaluation of noise field simulations ES202, TS103-4.0, TS103-4.1 and TS103-8.0. For the eight-channel system TS103-HS, these identical noise scenarios are not available in the background noise database of ETSI TS 103 224.

However, the noise database here provides similar / related scenarios, which may be used instead. Table 2 shows the eight noise types which are proposed and used for the present study. The database also provides binaural versions of the eight-channel recordings as shown in Table 3.

NOTE 1:	Since the noise recordings are in 8-channel-format, only level information for channel 1 (close to left ear) and 7 (close to right ear) are provided. More information on the specific recordings can be found in clause 8 of ETSI TS 103 224 [4].

NOTE 2:	In the noise database of TS 103 224, there is no equivalent noise condition for “mensa”. The noise “sales counter” is used instead.

Table 1: Noise types acc. to ETSI ES 202 396-1 (binaural)

| Filename | Duration | Level (L & R) |
| --- | --- | --- |
| Pub_Noise_binaural_V2 | 30 s | L: 75,0 dB(A) R: 73,0 dB(A) |
| Outside_Traffic_Road_binaural | 30 s | L: 74,9 dB(A) R: 73,9 dB(A) |
| Outside_Traffic_Crossroads_binaural | 20 s | L: 69,1 dB(A) R: 69,6 dB(A) |
| Train_Station_binaural | 30 s | L: 68,2 dB(A) R: 69,8 dB(A) |
| Fullsize_Car1_130Kmh_binaural | 30 s | L: 69,1 dB(A) R: 68,1 dB(A) |
| Cafeteria_Noise_binaural | 30 s | L: 68,4 dB(A) R: 67,3 dB(A) |
| Mensa_binaural | 22 s | L: 63,4 dB(A) R: 61,9 dB(A) |
| Work_Noise_Office_Callcenter_binaural | 30 s | L: 56,6 dB(A) R: 57,8 dB(A) |

Table 2: Noise types acc. to ETSI TS 103 224 (8-channel versions)

| Filename | Duration | Level Channel 1 & 7 |
| --- | --- | --- |
| Pub_handset | 30 s | 1: 77,2 dB(A) 7: 76,0 dB(A) |
| Roadnoise_handset | 30 s | 1: 72,8 dB(A) 7: 73,0 dB(A) |
| Crossroadnoise_handset | 30 s | 1: 70,6 dB(A) 7: 71,2 dB(A) |
| TrainStation_handset | 30 s | 1: 78,9 dB(A) 7: 78,8 dB(A) |
| FullSizeCar_130_handset | 30 s | 1: 68,5 dB(A) 7: 70,8 dB(A) |
| SalesCounter_handset | 30 s | 1: 66,6 dB(A) 7: 66,6 dB(A) |
| Cafeteria_handset | 30 s | 1: 70,0 dB(A) 7: 70,6 dB(A) |
| Callcenter2_handset | 30 s | 1: 60,2 dB(A) 7: 60,2 dB(A) |

Table 3: Noise types acc. to ETSI TS 103 224 (binaural versions of Table 2)

| Filename | Duration | Level (L & R) |
| --- | --- | --- |
| Pub_bin | 30 s | L: 77,7 dB(A) R: 76,2 dB(A) |
| Roadnoise_bin | 30 s | L: 74,0 dB(A) R: 74,1 dB(A) |
| Crossroadnoise_bin | 30 s | L: 70,8 dB(A) R: 71,6 dB(A) |
| TrainStation_bin | 30 s | L: 78,1 dB(A) R: 78,8 dB(A) |
| FullSizeCar_130_bin | 30 s | L: 68,7 dB(A) R: 70,7 dB(A) |
| SalesCounter_bin | 30 s | L: 66,7 dB(A) R: 66,6 dB(A) |
| Cafeteria_bin | 30 s | L: 69,8 dB(A) R: 70,3 dB(A) |
| Callcenter2_bin | 30 s | L: 60,2 dB(A) R: 60,0 dB(A) |

# 4	Measurement rooms

The parameters of the measurement rooms evaluated in this study are described in Table 4. All rooms are equipped with sub-woofer setup for the playback according to ES 202 396-1 [5] and with eight satellite loudspeakers in overall for the playback according to TS 103 224 [4].

Table 4: Measurement rooms

| Company | Name | Length [m] | Width [m] | Height [m] | Comment |
| --- | --- | --- | --- | --- | --- |
| HEAD acoustics GmbH | Room#1 | 2.40 | 3.40 | 2.05 | Same chamber type and design as Room 2 |
|  | Room#2 | 2.40 | 3.40 | 2.05 | Same chamber type and design as Room 1 |
|  | Room#3 | 2.90 | 3.10 | 2.05 | Different manufacturer than Room 1,2,4 |
|  | Room#4 | 1.80 | 2.40 | 2.05 | Smallest chamber |
| Intel | Room#5 | 5.57 | 4.67 | 2.93 | Trapezoidal room |
|  | Room#6 | 3.60 | 3.00 | 2.05 |  |

The equalization procedures of the five noise field simulations as described in clause 2 were successfully applied for all rooms. The binaural as well as the eight-channel transfer functions passed the tolerance of +/- 3 dB across the frequency range 50 Hz to 10 kHz, which is specified in ES 202 396-1 as well as in TS 103 224.

Figure 3 to Figure 7 show the validation results of the equalization procedure for all investigated simulation systems and measurement rooms. Note that Figure 3 to Figure 6 show the left and right ear equalization curves, while Figure 7 shows the eight microphone equalization curves of the spatial array.

It can be seen that binaural equalization using TS 103 224-4.0, TS103-4.1 and TS103-8.0 system yield smoother equalization compared to ES202.

NOTE:	Due to time constraints, Room#6 could not perform equalization and measurements with TS103-4.0.

Figure 3: Validation results of equalization (ES202)

|  | Room #6 - N/A |
| --- | --- |

Figure 4: Validation results of equalization (TS103-4.0)

Figure 5: Validation results of equalization (TS103-4.1)

Figure 6: Validation results of equalization (TS103-8.0)

Figure 7: Validation results of equalization (TS103-HS)

# 5	Test Methodology

## 5.1	Spectral Analysis

In order to investigate and quantify the reproduction accuracy of the noise field simulations, for each system, lab and noise type the background noise is recorded with the HATS ears. For each single measurement, the 1/3rd octave spectrum for left and right ear microphone is calculated from the recordings and is then referenced to the corresponding spectrum of the original noise source. If the noise field reproduction was perfect, the difference between the measured spectrum and the original reference spectrum should be zero.

For TS103-HS, the binaural versions of the eight-channel recordings are used as the reference spectrum. Table 4 summarizes the various configurations tested in the current investigation.

Table 5: Configurations of sources and references

| Noise field simulation | Equalization Points | Noise source from | Recording point | Reference noise |
| --- | --- | --- | --- | --- |
| ES202 | Ear Mics (2) | ES 202 396-1 (binaural, Table 1) | Ear Mics | ES 202 396-1 (binaural, Table 1) |
| TS103-4.0 | Ear Mics (2) | ES 202 396-1 (binaural, Table 1) | Ear Mics | ES 202 396-1 (binaural, Table 1) |
| TS103-4.1 | Ear Mics (2) | ES 202 396-1 (binaural, Table 1) | Ear Mics | ES 202 396-1 (binaural, Table 1) |
| TS103-8.0 | Ear Mics (2) | ES 202 396-1 (binaural, Table 1) | Ear Mics | ES 202 396-1 (binaural, Table 1) |
| TS103-HS | Mics of array (8) | TS 103 224 (8 Channels, Table 2) | Ear Mics | TS 103 224 (binaural, Table 3) |

The following definitions apply for the determined spectra and differences:

OX(fi)			Original 1/3rd octave spectrum for left (X=L) or right (X=R) ear at frequency index i

MX(fi)			Measured 1/3rd octave spectrum for left (X=L) or right (X=R) ear at frequency index i.

HXfi=MX(fi)OX(fi)	1/3rd octave spectral distance for left (X=L) or right (X=R) ear at frequency index i.

## 5.2	Error Metrics

In order to quantify the accuracy of each system several error metrics are calculated on the difference between the measured and reference spectrum. Based on the definitions above, the following metrics are specified for left (X=L) or right (X=R) ear (all on dB scale):

Standard deviation:

StdX=1Nifi=fmin fi=fmax20⋅log10HXfi-AvgX2

Absolute maximum deviation:

AbsMaxX=maxfi∈fmin,fmax20⋅log10HXfi

With:

AvgX=1Nifi=fmin fi=fmax20⋅log10HXfi	Average of spectral difference in dB

fmin					Minimum frequency used for metric calculation (50 Hz)

fmax					Maximum frequency used for metric calculation (10 kHz)

Ni=argifmax-argifmin+1		Number of frequencies included in range

Figure 8 depicts two example results of two measured different test cases. The graph display the two spectrum difference for the left and right ear. The graph on the left provides a “good” example, where the transfer function for the noise “Train Station” is almost flat, and results for left and right ear are consistent, whereas the right one shows a “bad” example, where the transfer function of the noise “Full-size car 130 km/h” shows quite substantial deviations. In addition, there are noticeable differences between left and right ear. Table 6 provides the corresponding metrics for left and right ears.

Figure 8: Example for “good” (left) and “bad” (right) binaural transfer function

Table 6: Deviation metrics for two examples

|  | AbsMax [dB] |  | Std [dB] |  |
| --- | --- | --- | --- | --- |
| Example | L | R | L | R |
| “bad” | 3.60 | 6.62 | 1.28 | 2.30 |
| “good” | 0.63 | 0.63 | 0.29 | 0.27 |

# 6	Measurement Results

## 6.1	Overall Results

Table 7 shows the overall results for all the rooms, noise and both ears combined for the five different background noise (BGN) configurations. For both metrics AbsMax and Std, the maximum and average across all included cases. The lower the number the better.

Table 7: Overall results for the five BGN configuration

| System | Max (AbsMax) [dB] | Avg (AbsMax) [dB] | Max (Std) [dB] | Avg (Std) [dB] |
| --- | --- | --- | --- | --- |
| ES202 | 12.03 | 4.38 | 3.72 | 1.57 |
| TS103-4.0 | 6.31 | 2.25 | 1.43 | 0.66 |
| TS103-4.1 | 6.45 | 2.04 | 1.77 | 0.59 |
| TS103-8.0 | 4.05 | 1.70 | 1.48 | 0.46 |
| TS103-HS | 9.67 | 3.74 | 2.21 | 1.06 |

NOTE 1:	For Room#6, there is no data for TS103-4.0 system available.

NOTE 2:	For Room#5, the noise Train_Station_binaural is missing in TS103-4.1.

NOTE 3:	Due to the different equalization points and different noise types used in TS103-HS, this configuration may not be included in all analyses shown in the following clauses.

The ES202 configuration provides the worst metrics, while TS103-8.0 has the best ones (by a factor of about 3 compared to ES202). TS103-4.0 and TS103-4.1 have quite similar results, but still not as accurate as TS103-8.0.

The results of the TS103-HS seem, at first glance, not as good as the binaural configurations of the TS103. But since the ear microphones were not used as equalization points in TS103-HS, higher deviations than for binaural equalizations are expected. Nevertheless, results are still better than for ES202.

## 6.2	Results per Room

Figure 9 and Figure 10 show maximum AbsMax and average Std, respectively, for all noise types and ears as a function of the BGN system and room.

Figure 9: Maximum (top) and average (bottom) AbsMax deviation for all noises and ear depending on the room and BGN system

Figure 10: Maximum (top) and average (bottom) Std for all noises and ear depending on the room and BGN system

It can be seen that the behaviour between the rooms are consistent, with the ES202 giving the worst results in term of both AbsMax and Std. Both metrics also decrease consistently from TS103-4.0 to TS103-8.0, which provides the best results.

Due to the different equalization points, increased error metrics TS103-HS are expected. However, across most rooms, the metrics are better than for ES202.

## 6.3	Results per Background Noise

Figure 11 and Figure 12 show maximum AbsMax and average Std, respectively, for all rooms and ear in function of the BGN systems and noise types.

NOTE:	Since TS103-HS is used with different noises than the binaural systems, it is excluded in this analysis.

Figure 11: Maximum (top) and average (bottom) AbsMax for all rooms and ear depending on the noise type and BGN system

Figure 12: Maximum (top) and average (bottom) Std for all rooms and ear depending on the noise type and BGN system

Similar trends as before can be observed: ES202 provides the worst results in all four metrics across all noise types. In particular, the error metrics here vary a lot across noises; the Pub noise always obtains much worse results here.

On the other hand, the method TS103-8.0 again obtains the best results, TS103-4.0 and TS103-4.1 are slightly worse. Also for these configurations, a variance across noises is visible, but less pronounced.

## 6.4	Impact of Bandwidth

In this chapter the effect of the bandwidth (BW) is examined, which is used to calculate the error metrics. The limit fmax=10 kHz was chosen according to the validation requirement described in [5]. However, most of the energy in the considered noise signals are located in lower frequencies. In order to investigate the dependency of the error metrics on the frequency, all of them are recalculated with fmax=4 kHz.

Figure 13 and Figure 14 show AbsMax and average Std, respectively, for all rooms and ears as a function of the BGN system and BW limit.

Figure 13: Effect of bandwidth on Maximum (top) and average (bottom) AbsMax for all rooms, noise and ear depending on the BGN system

Figure 14: Effect of bandwidth on Maximum (top) and average (bottom) Std for all rooms, noise and ear depending on the BGN system

It can be noticed that the largest differences between the limits 10 kHz and 4 kHz occur for TS103-HS. Results are then get much closer to the binaural equalizations TS103-4.0/4.1/8.0. This indicates that most spectral differences of TS103-HS occur in the frequency range 4-10 kHz, respectively that the system reproduces the noise field quite adequate in the low frequency domain. This could be explained by the difference in the calibration point (ears vs. microphone array), which will be more noticeable in high frequency.

For the systems TS103-4.0/4.1/8.0, error metrics also decrease from 10 kHz to 4 kHz. Since these configurations are based on the same equalization methodology as TS103-HS, also these results seem plausible.

On the other hand, error metrics for ES202 do not improve between the two different limits. Only for Avg(AbsMax), there is a very slight improvement. For Std metrics, results even get slightly worse. This indicates that most of the spectral differences occur in the low frequency domain between 50 Hz and 4 kHz.

# 7	Discussion

The present study investigated six labs/measurement rooms regarding the reproduction accuracy of noise field simulations. Four binaural (ES202, TS103-4.0, TS103-4.1 and TS103-8.0) and one microphone-array-based (TS103-HS) equalization methods were used. Eight noises each were evaluated and analysed based on spectral difference metrics between recording and original sound.

In overall, the configurations TS103-4.0, TS103-4.1 and TS103-8.0 provided consistently better results than ES202. Also an analysis across labs and noise types could confirm this trend.

In several cases, ES202 was even outperformed by TS103-HS, in particular for the lower frequency domain below 4 kHz - even though this method was not equalized at the ear microphones. Since the microphone array used for TS103-HS provides more equalization points on the right side of HATS (used for testing handset devices), error metrics may also be even better when leaving out the left ear.

However, the results of this investigation may only give a note/hint about the variance of different rooms and noise field simulations on acoustic performance evaluation of handset terminals according to TS 26.131/132. Only a subsequent measurement series with multiple devices may confirm the impact on handset terminals.

# 8	Proposal

The sources proposed to include Clause 2 to 7 in the technical report.

# References

| [1] | 3GPP S4-181500, „New SID on ambient noise test methodology for evaluation of acoustic UE performance (FS_ANTeM),“ HEAD acoustics GmbH, Qualcomm Incorporated, Deutsche Telekom AG, Intel, HuaWei Technologies Co., Ltd, Busan, South Korea, 25 November 2018. |
| --- | --- |
| [2] | 3GPP TS 26.131, „Terminal acoustic characteristics for telephony; Requirements,“ Release 15.0.0. |
| [3] | 3GPP TS 26.132, „Speech and video telephony terminal acoustic test specification,“ Release 15.0.0. |
| [4] | ETSI TS 103 224 v1.3.1, „Speech and multimedia Transmission Quality (STQ);. A sound field reproduction method for terminal testing including a background noise database,“ 07/2017. |
| [5] | ETSI ES 202 396-1 v1.7.1, „Speech Processing, Transmission and Quality Aspects (STQ); Speech quality performance,“ 10/2017. |
| [6] | 3GPP S4-171071, „New WID on Speech quality in the presence of ambient noise for super-wideband and fullband modes (SPAN),“ HEAD acoustics GmbH, ORANGE, Intel, Qualcomm Incorporated, Sony Mobile Communications, Samsung Electronics Co., Ltd., Belgrade, Serbia, 09-13 October 2017. |
| [7] | ETSI TS 103 281 v1.1.1, „Speech quality in the presence of background noise: Objective test methods for super-wideband and fullband terminals,“ 04/2017. |
| [8] | 3GPP S4-180326, „Updated results for SPAN work item,“ HEAD acoustics GmbH, Kista, Sweden, 09-13 April 2017. |
| [9] | ETSI TS 103 106 v1.5.1, „Speech Quality performance in the presence of background noise: Background noise transmission for mobile terminals - Objective test methods,“ 04/2018. |