3GPP TSG-SA4 Meeting #104	S4-190641

# Cork, Ireland, 1st Jul 2019 - 5th Jul 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.929 | CR | 0003 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VR QOE metrics correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRQoE |  |  |  |  |  | Date: |  |  | 2019-06-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The device information metric is only logged at the start of each QoE reporting period, but the characteristics may change during the session(for instance if the HMD is rotated from horizaontal to vertical). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The device metric is logged whenever this characteristic change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VR QOE may not reflect the whole users aciton |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================== Start of  change=================================

# 10	VR device impact on QoE

## 10.1	Introduction

Compared with traditional streaming video, the key feature of VR service is to create immersive experience and enable smooth interactivity between user and the environment, in which VR device would play an important role. This contribution proposes device information relevant to user experience of VR service. All the device property information could be collected by OP5 of the reference model described in clause 6.1.

## 10.2	QoE metrics relevant with VR device

### 10.2.1	Field of View

One of the factors that contribute to the uniqueness of 360 video experience is the level of immersion induced by the wider FoV of HMD, which represents the extent of observable environment at any given time. A wider FoV could help provide a more authentic feeling of immersion. Thus, FoV of the HMD is an important parameter that helps evaluate to what extent a VR device could help create immersive experience.

### 10.2.2	Resolution

Resolution here is defined as for per eye. An appropriate screen resolution would provide the best and comfortable experience.

### 10.2.3	Refresh Rate

Refresh rate is the number of times per second the display grabs a new image from the graphic processing unit. Lower refresh rate would contribute to processing latency and lead to VR sickness, i.e. viewing glitches on the screen. While higher refresh rate adds to the sense of presence in virtual worlds.

### 10.2.4	Decoder capability

The support of codec profile and level is an important property that decides the content types it could decode.

### 10.2.5	Detailed QoE metrics

he QoE metrics relevant with VR device as listed in Table 10.2.5.1 is necessary for assessment of device impact on user experience, and suggested to be collected by VR client.  This metric is logged at the start of each QoE reporting period, and whenever the characteristics changes during the session (for instance if the HMD is rotated from horizontal to vertical orientation).