3GPP TSG WG SA4 #104 meeting	S4-190646

01 July – 05 July 2019, Cork, Ireland

Agenda item: 	10.6

Source: 	Fraunhofer HHI

Title: 	Volumetric Video Player Demonstration

Document for	Information

# Introduction

During SA4#99 the New Study Item on “on eXtended Reality (XR) in 5G” FS_XR5G in S4-180973 was agreed and afterwards approved in by SA plenary #81 in SP-180667. A Technical Report is developed in TR 26.928.

One of the goals of the FS_XR5G study item is:

- Analysing the different technologies and equipment in place that provide an Extended Reality experience.

An enabling technology that has recently become the center of attention for various XR applications is volumetric video. Volumetric videos capture the three-dimensional space in a realistic way and have the potential to provide a better immersion compared to 360° videos. They are also naturally suitable for the representation of 6DoF content allowing a viewer to freely move inside the content and observe the volumetric objects from different points of views and distances.

Recently, various technologies have been emerging for capture, processing, compression and transmission of volumetric content. During SA4#103, Fraunhofer HHI presented a mesh-based volumetric video capture and production workflow in S4-190368 [1] used at Volucap – a joint venture of Studio Babelsberg, ARRI, UFA, Interlake and Fraunhofer HHI. Following up on S4-190368, in this contribution, we present a workflow for creation of streamable volumetric videos from meshes and demonstrate a player implementation on an Android smartphone (Samsung Galaxy S9).

# Mesh-based workflow for distribution and playback of volumetric videos

Our workflow addresses similar use cases as the Nokia V-PCC demo presented during SA4#102 meeting in S4-190102 [2] but uses a mesh-based workflow instead of point clouds. While point clouds can be directly rendered, it is also possible to convert them to a connected polygon (mostly triangle) mesh model by adding connectivity between the points of the point cloud which then become the vertices of the mesh faces. Although mesh conversion is computationally expensive, high-density mesh representations tend to provide a highly photorealistic representation of 3D objects. Therefore, in addition to point clouds, meshes are also considered by the volumetric video market for the representation of captured volumetric content.

We would like to note that there is also an ongoing activity in MPEG in the form of an Exploration Experiment (EE) started during the MPEG#126 meeting regarding an architectural extension to Video-based Point Cloud Coding (V-PCC) to support mesh coding [3]. The EE is studying a method to handle animated mesh compression by extending V-PCC which heavily relies on existing video compression schemes and is promising for large adoption by the market. The considered extension proposes to code the coordinates of the mesh vertices and attributes data using V-PCC whereas the vertex connectivity data is coded as auxiliary data. Both coded data are then multiplexed to create the final compressed output bitstream. For a detailed block diagram of the proposed architecture, please refer to [3].

Figure 1. Overview of the volumetric video player demo.

Figure 1 shows an overview of our workflow. In S4-190368 presented during SA4#103, we had already described the multi-camera capture and processing steps including texturing and conversion into animated mesh (yellow blocks in Figure 1). Please see a short summary below and refer to the Permanent Document v0.4.1 [4] for further details.

﻿Recap of S4-190368: Our multi-camera capture setup consists of a number of stereo camera pairs. After the synchronized capture of videos from multiple viewpoints, colour grading and keying (foreground/background segmentation) are applied to the captured views. Then, a depth estimator is applied to each stereo pair to generate pixelwise high accuracy depth information. That is, for each perspective, the 3D information is stored as colour-coded depth values in a two-dimensional image. In the next step, the depth information of all 16 stereo pairs is merged by using data from an initial camera calibration and a related 3D fusion process. The result of this fusion process is a 3D point cloud processed by different post-processing steps to obtain a high-quality mesh representation. For more information please refer to S4-190368.

Our workflow receives the textured animated meshes as input and incorporates mesh, video and audio encoders in order to reduce the size of the obtained volumetric content to levels that are achievable by today’s networks. 3D coordinates/normals, texture coordinates and mesh faces are compressed using a mesh compression algorithm. The texture patches of the volumetric video are arranged into single rectangular texture maps for each time instance with possible additional interpolation between the patches (see Figure 2, left) to alow for better coding efficiency. Then, all texture maps are encoded into a single compressed video. The audio component is also separately compressed. Please note that audio is just included for demo purposes and no considerations were made regarding spatial/immersive audio.

In the next step, the obtained compressed data streams are packaged as different tracks of an MP4 container. Figure 2 visualizes the contents of the texture, mesh and audio tracks as well as the final rendered output. The obtained file is in this case locally streamed to a Unity app, but may also be streamed over a network. The Unity app has a custom plugin including the matching mesh, video and audio decoders that reconstruct the mesh data (3D coordinates/normals, texture coordinates, faces), texture maps and audio. The synchronized reconstructed data is rendered based on the user interaction with the display and played back.

Figure 2. Visualization of different media tracks packaged into an MP4 container.

# Volumetric video player demo

Figure 3 shows a snapshot of the volumetric player demo implemented on the Samsung Galaxy S9 smartphone as a Unity Android application. The demo implementation utilizes ARCore for spatial processing, e.g. to anchor the volumetric objects into the environment.

The demo content consists of two volumetric sequences: “Josh” and “Boxer”. Both are temporally registered mesh sequences, including 20K and 60K triangles, respectively. After placing the volumetric objects into the environment, the users can interact with them by walking around and observing the objects from different viewing angles as well as approaching to or moving away from the objects. The scale of the objects can be changed by zooming in/out.

Figure 3. A snapshot of the Volucap volumetric video player demo.

# Relation to the considered delivery architectures in PD

The presented demo runs locally on a smartphone. The volumetric content packaged into a single MP4 file is streamed to the Unity application where it is decoded into mesh, texture and audio components and played back. As such, currently the whole volumetric asset is locally stored on the device and no network transmission is involved. However, the same workflow can be directly employed in various streaming use cases described in FS_XR5G PD [3], by storing the pre-encoded and packaged volumetric asset on a content server and streaming the compressed volumetric content over a network to the end device, which runs a game engine (in our case a Unity app) including the different media decoders and the volumetric video player. This corresponds to an architecture, where the rendering is locally performed on the end device.

Alternatively, an extension to the present demo can be considered where the compressed textured mesh representations are located on an XR Edge server, as described in PD Clause 5.9.3.2 “Network Rendering Viewport rendering in the edge” (copied below in Fig. 3). In this case, based on the user interaction, the edge server would decode and render a view from the volumetric content, compress the rendered media stream and send it to the end device. This architecture requires low-latency streaming protocols and low-latency encoders e.g. based on hardware/GPU processing.

Figure 4. Viewport rendering in Edge (from PD v0.4.1 [4]).

# Proposal

We invite the SA4 delegates to check out the demo during breaks and take the considerations in Section 4 into account for further discussion on the 3GPP XR architectures.

# References

[1] S4-190368, Fraunhofer HHI, Volumetric Video Capture and Production for XR, 3GPP TSG-SA4 Meeting #103, April 2019, Newport Beach, US.

[2] S4-190102, Nokia Corporation, Video-based Point Cloud Compression (V-PCC) Demonstration, 3GPP TSG-SA4 Meeting #102, February 2019, Bruges, Belgium.

[3] Sungryeul Rhyu, Madhukar Budagavi, Rajan Joshi, “[V-PCC] [New Proposal] V-PCC extension for mesh coding,” ISO/IEC JTC1/SC29 WG11 (MPEG) m47608, Geneva, CH, Mar. 2019.

[4] AHVIC-195, FS_XR5G: Permanent document, v0.4.1, 3GPP TSG-SA4 Video SWG Telco on FS_XR5G May 13.

http://www.volucap.de