Source:	Nokia Corporation

Title:	On IVAS complexity constraint

Document for:	Discussion and Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

IVAS-4 [1] does not presently set any specific design constraints on complexity of the solution. It has been proposed to set the complexity constraints based on the complete IVAS solution that includes the IVAS renderer [2].

We discuss in this input document some aspects to be considered in setting the complexity constraints. It is not presently proposed any exact values for complexity limits; it is first focused on the principle.

# 2. Complexity design constraint

We consider it important for the complexity constraint to reflect the service experience. Therefore, we believe the Orange proposal [2] of including the encoder and decoder/renderer in the complexity figure is a valid starting point. There are some further considerations that should be discussed.

A first consideration relates to the IVAS renderer and external renderer interface. Should it be set one complexity constraint only (based on IVAS renderer) or also a second complexity constraint relating to a decoder output for external rendering? While an external rendering may be mostly expected in case of relaxed complexity constraints, it should be understood whether there are benefits in setting a lower complexity constraint value for decoder output(s) without the rendering step. This discussion includes the so-called pass-through mode that is being considered for the IVAS design constraints.

A second consideration relates to the various input audio formats that the IVAS codec will support. While it is understood that the IVAS codec solution will be a single general-purpose codec implementation, we foresee that certain devices will support only a subset of the IVAS input audio formats. Therefore, it is actually of great practical interest to consider the codec worst-case complexity individually for each of the input audio formats. We note that such a very detailed approach can however make the design constraints fairly complex. It could therefore also be left for codec characterization to test and report all the complexity levels for all the various input formats and input format combinations.

Regardless, it could be considered to at least specify the worst-case complexity individually for each of the broader input audio format categories (e.g., stereo, channel-based multi-channel, etc.) that reflect the device type and capability level. The purpose of such design constraint can be seen to avoid unnecessary complexity of the solution for operation points that are generally less computationally demanding than the expected worst-case operation.

One option is to define an average worst-case complexity figure in addition to the maximum worst-case complexity figure. It is meant here by maximum worst-case complexity the highest complexity of the codec across all operation points. The average worst-case complexity could be defined as a weighted average of the worst-case complexities of the input audio format categories.

Such secondary complexity constraint could furthermore take into account also the case of external rendering.

# 3. Discussion

The present input document does not propose any specific values for the complexity constraint. The practical worst-case complexity is connected to the audio input and output formats (e.g., number of channels to be supported). The audio format design constraints remain open despite lengthy discussions.

As also discussed in [2], it is currently assumed that the IVAS complexity constraints relate to a fixed-point implementation. The set of basic operators is however currently open.

# 4. Summary and Proposal

It was discussed some aspects relating to the complexity constraint for the IVAS codec. In general, the principle of considering the end-to-end complexity including encoder and decoder/renderer could be applied.

The source proposes to specify complexity constraint not only for the worst-case complexity of the solution. In addition, it is proposed to apply the principle of specifying complexity constraint that takes into account the envisioned device types and capability levels. An example of this is to specify a weighted average worst-case complexity figure across the audio input format categories.

# References

[1]	TD S4-190450: IVAS Design Constraints (IVAS-4), v0.0.10.

[2]	AHEVS-472: “On delay and complexity design constraints for IVAS”, Orange.