Source:	Nokia Corporation

Title:	On proposed design constraints and IVAS WID

Document for:	Discussion

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

It has been brought into discussion some concerns on the manageability of the IVAS codec standardization [1, 2]. Particularly, it was considered that many proposed design constraints and features would exceed the scope of the IVAS WID. In addition to such concerns, it is also apparent that the standardization progress has been quite slow with difficulty to reach agreement on certain design constraints. The work item schedule has recently been updated to account for that.

Considering that IVAS is the first conversational immersive standard it is set to truly disrupt the voice and audio experience. It is therefore to be expected that some relevant new features and use cases will be identified only well after the work item has been launched. Update of the work item schedule may also give time for some new proposals. Already now, extending the work item from Rel-16 to Rel-17 timeframe has sparked new industry interest in the work [3]. In order to guarantee an attractive immersive voice and audio standard it is important to fairly evaluate and consider also such new proposals.

It is the source’s view that the currently proposed design constraints are generally well in line with the work item scope. In the hope that potential problems could be identified and solved, it is provided discussion in this input contribution to address the “sore points” listed in [2].

# 2. Discussion on design constraints

According to [2], proposed features that would not necessarily be essential to the work item are:

- certain spatial audio input formats,

- certain bit rates/operation modes,

- the existence and the properties of an external renderer API,

- pass-through operation,

- certain renderer output formats.

The source has previously indicated as their view that the discussion on IVAS design constrains is within the work item scope. According to this view, there does not seem to be such an abundance of out-of-scope proposals for IVAS that it should be started categorizing individual design constraints in terms of the 2017 Rel-16 WID text only. Rather it is the view of the source that an attractive standard meeting the industry needs should be the main goal. This may very well include also features that are not identified in the WID, or even proposed so far in SA4.

According to the project plan, the design constraints are to be completed in January 2020 (SA4#107). This suggests that speedier convergence on some proposed design constraints is desirable, and there may be limited room for new features that have not been proposed so far. In order to take the concerns from Dolby into consideration and to try to speed up the convergence, it is next provided discussion and views of the above listed bullet items in terms of the IVAS WID text.

## Certain spatial audio input formats

The IVAS design constraints, IVAS-4 [4], currently lists as agreed input formats the following:

- Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD]

- Binaural audio

- Scene-based audio, first-order (FOA) and up to [N]-order ambisonics.

- Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters

With the exception of object-based audio, these high-level audio formats are mentioned in the work item objectives [5]. Taking into account the non-agreed parts above and other proposals, it is being discussed at least the following input formats for IVAS:

- Surround audio with height information (at least 5.1+4 and 7.1+4)

- Ambisonics-based scene audio at higher orders (2nd, 3rd, …)

- Metadata-assisted spatial audio (MASA), a scene-based format suitable for some practical form factors

- HOA Transport Format (HTF), a scene-based format suitable, e.g., for reducing number of ambisonics component channels for codec input

In view of the source, all of the above input formats are in the scope of the IVAS work item as such. Specifying the related design constraints should therefore be mainly based on industry needs, identified use cases, and interconnections to other design constraints (e.g., complexity). The WID objectives [4] specifically state that:

-	The solution is expected to handle encoding/decoding/rendering of speech, music and generic sound.

  - It is expected to support encoding of channel-based audio (e.g. mono, stereo or 5.1) and scene-based audio (e.g., higher-order ambisonics) inputs including spatial information about the sound field and sound sources. The solution is expected to provide support for diegetic and non-diegetic input.

While surround audio with height information is not mentioned it falls under the umbrella of channel-based audio. There can be expected to be interest, e.g., for use of 7.1+4 input especially for professionally produced immersive content.

The other three formats relate to scene-based content. The WID uses higher-order ambisonics as an example of scene-based audio. HTF can be seen as one way of achieving this higher-order capability. The MASA format is a scene-based format specifically suitable for practical form factors as used in consumer devices. It also includes “spatial information about the sound field and sound sources”. MASA is of interest particularly for conversational services and UGC.

The input audio formats can in some cases have a significant effect on the codec complexity. To balance the overall codec complexity with the capability for immersive user experience it will remain important to consider, e.g., the number of channels to be supported by the codec. It seems some design constraints, such as ambisonics order and HTF, could be connected in this respect. For example, there may be some audio bus limitations on current and/or upcoming platforms.

## Certain bit rates/operation modes

It currently reads in the IVAS design constraints under “Bit Rates” the following [4]:

When EVS bit-exact operation is used (see Backward interoperability), the IVAS codec shall operate at bit rates of EVS (including all EVS Primary and AMR-WB IO modes)

Note: As implemented in EVS, the bit rates up to 24.4 kb/s are gross bit rates and they are net bit rates above 24.4 kb/s.

In other cases than EVS bit-exact operation: the IVAS codec shall operate at bit rates of [5.9VBR, 7.2, 8, 9.6, 13.2, 16.4], 24.4, 32, 48, 64, 96, 128, 160, 192, 256, [384, 512] kb/s [min and max TBD].

Note: The gross bit rate supported in the DTX/CNG/SID operation is [TBD].

In view of the source, the bit rates currently under discussion are of relevance for an EVS codec extension. At the same time, it is noted that the relevance of some bit rate ranges may depend on use cases and audio input formats. For example, it remains for further study what type of object-based audio IVAS will support, in which configurations, and for what type of use cases.

## Existence and properties of an external renderer API

The external renderer API is currently under discussion. It is being proposed, e.g., to support interface formats for optional external rendering solutions corresponding to the codec input formats.

In view of the source, the provision of external rendering seems an essential feature for the IVAS codec. For example, this will allow for manufacturers to differentiate in the market. This may also potentially prolong the timeframe where the codec solution remains relevant and competitive against future solutions. Market differentiation can be observed, e.g., in terms of balancing the user experience achieved by and the implementation complexity for a certain device or device class.

Use of an external renderer is understood to require a suitable interface. In view of the source, this is connected to the IVAS pass-through operation.

## Pass-through operation

Pass-through operation is currently under discussion. There have so far been presented significantly opposed views on this proposed design constraint.

IVAS WID does not specify a pass-through operation as a separate feature that is expected of the solution. It could however be argued that this functionality is implied. Traditionally, voice and audio codecs (e.g., EVS) have passed through the input format to the output. It has been considered the purpose of the codec as a compression tool to reproduce as faithfully as possible at the output what it sees at the input. For EVS, this operation can further be controlled, e.g., by limiting the audio bandwidth. However, the audio format by default remains unchanged.

What differentiates the IVAS codec here from previous conversational codecs is the added flexibility in rendering. For example, it is included and considered audio input formats that require a rendering to be listened to. Furthermore, the ability to provide many different renderings (at least binaural and loudspeaker) is needed to cover various user device setups and use cases. However, the exact rendering requirements may not always be known on the encoder side. This suggests that to maintain uncompromised quality, it is required the ability to provide the faithful audio format reproduction also in IVAS. This requires pass-through operation.

One key aspect to consider in terms of the pass-through operation is the provision of the external renderer and a suitable codec interface for it. While market differentiation is one important reason for external rendering, it can be useful also when, e.g., a UE is connected to an external resource in a conferencing or VR media content playback scenario. In both cases, uncompromised quality is important. It therefore seems that the pass-through operation should be considered as part of the external renderer interface. It also seems critical to consider the pass-through operation in this context to provide uncompromised quality for the external rendering.

In view of the source, pass-through operation is another potentially very relevant and valuable feature. It should not be considered out of scope of the IVAS work item although this feature as such is not specified in the IVAS WID.

To facilitate agreement on the pass-through operation, it may be useful to come up with a commonly understood definition of this feature. For example, it is currently not clear to the source whether the intention of the proposed design constraints would be to mandate, permit, or exclude an encoder-side signalling of pass-through operation. It could be that a better definition of the operation helps to avoid disagreements from misunderstandings.

## Certain renderer output formats

Discussion relating to renderer output formats is also ongoing. Currently, the IVAS design constraints [4] specifies that:

The IVAS codec shall support the following rendered output formats:

- Mono

- Stereo

- Binaural audio

- Multi-channel [All channel based encoder input formats shall be included]

Editor’s Note: Corresponding output formats for each input formats is TBD

The source notes that in addition to the above text there are documented for discussion several proposals (from Dolby, Ericsson, Fraunhofer) that appear in square brackets.

In view of the source, the WID objectives indicate that all the currently discussed renderer output formats are in the scope of the IVAS work item. Specific renderer output formats as such are not listed in WID. Discussion regarding the rendering capability should consider at least the balancing between user experience and implementation complexity.

# 3. Testing and deliverables

In [2], Dolby proposes a way forward for the IVAS codec standardization. While the source sees no need for the proposed classification process in light of the WID, it may be worth considering certain proposed aspects when it comes to IVAS Selection testing and IVAS Selection deliverables. For example, codec proponents could be invited to provide demo material (“demo tape”) to illustrate candidate codec features that are not otherwise tested, e.g., due to complexity of such tests or needs to limit test sizes. For example, some interactivity features could be demoed instead of tested in the Selection.

# 4. Conclusion

The source appreciates Dolby’s overall aim to speed up the IVAS standardization and to meet the project schedule. It is naturally important to maintain a connection to the work item objectives and to keep in mind the scope of the work. It is invited to continue to take this into account for any new design constraint proposals.

The proposed design constraints currently under discussion however seem to be in the scope of the work, and many seem essential for an attractive conversational immersive voice and audio standard. The source therefore does not see a particular need to start additional classifying processes for the proposed features. Clearly, it should be considered industry needs and relevant use cases to guarantee an attractive and successful standard that meets the needs of the industry and the end users. With the extension of the work item into Rel-17 timeframe, there is also potential that the schedule extension and new interest in the work will result in proposals that have not yet been considered. It should be kept an open mind for such contributions in the interest of not only a speedy but also a successful standardization.

# References

[1]	AHEVS-456: “On the need to keep IVAS standardization focused”, Dolby Laboratories, Inc.

[2]	AHEVS-473: “Keeping IVAS Codec Standardization Manageable”, Dolby Laboratories, Inc.

[3]	AHEVS-469: “Indication of interest”, Philips International BV

[4]	S4-190450: IVAS Design Constraints (IVAS-4), v0.0.10.

[5]	SP-170611: New WID on EVS Codec Extension for Immersive Voice and Audio Services.