Source:		Qualcomm Incorporated

Title:	Considering LiQuImAS Methodologies for IVAS

Agenda Item:	7.5

Document for:	Discussion and Agreement

# 1. Introduction

The SA4 work item LiQuImAS (Test Methodologies for the Evaluation of Perceived Listening Quality in Immersive Audio Systems) resulted in two specifications [1, 2] for Release-15 and was applied during the VRStream characterization tests [3].

With the progression of the IVAS work item, evaluation methodologies need to be discussed and agreed. In this contribution, the source proposes the use of LiQuImAS subjective testing methodologies for IVAS testing.

# 2. Background

With the progression of the IVAS design constraints and ongoing discussions on input/output formats, reference conditions, and baseline codecs, there is a growing need to consider subjective and objective evaluation methodologies for IVAS. While we do not exclude objective assessments methods from this discussion, this contribution addresses only subjective methodologies.

Presuming there are X immersive/multichannel input formats that are coded at Y different bitrates and be rendered over Z immersive/multichannel playback configurations, a fully factorial subjective test reaches X*Y*Z/2 different testing conditions excluding upmixing. The effort to conduct such large number of subjective test conditions does not seem practical. Also, a desire was expressed to keep the IVAS activity within reasonable resource limits for testing [8].

The source believes that this desire can be achieved while maintaining a comprehensive subjective codec assessment by adapting the subjective methodologies already specified in LiQuImAS [1]. One of the fundamental principles LiQuImAS is the specification of separate subjective tests for audio coding and for the immersive audio rendering. By evaluating these two stages separately from each other, the number of test conditions is reduced, under the assumption that the quality performance of the rendering stage does not significantly depend on the quality performance of the audio coding stage.

Under this reasonable assumption, the number of test conditions can be reduced to X*Y codec quality tests and X*Z/2 rendering tests without loss of pertinent information on the IVAS system compared to the fully factorial subjective test.

# 3. Summary of LiQuImAS subjective tests

The following section summarizes the subjective tests specified in LiQuImAS. Since LiQuImAS has been developed as characterization tests for VRStream, these methodologies may need to be refined and extended e.g., to account for multiple codecs under test, such as the proposed EVS baseline codec conditions [7].

Further, as indicated in the IVAS WID, the binaural listening mode is envisioned to be the primary presentation mode. The source believes this importance should also be reflected in IVAS testing. Consequently, head-tracked binaural audio is an important evaluation point.

# 3.1 Codec Quality Tests in LiQuImAS

LiQuImAS specified the Codec Quality Test for loudspeakers [1, clause 5] which is based on the test method defined in [4] (MUSHRA). The Codec Quality Characterization Test assesses the Basic Audio Quality attribute at different bit-rates. The test includes one Hidden Reference and two Anchors. The two Anchors are 3.5kHz and 7kHz low-pass filtered versions of the Reference condition. The reference and hidden reference are rendered to the loudspeakers with the reference renderer of the audio system under test. An additional note states that care will be taken when evaluating the results, because the system’s reference renderer may include degradations to the immersive audio quality. While the note is an attempt to mitigate the issue, it is insufficient in a competitive codec evaluation scenario. Some modifications in reference definition would be necessary to avoid this problem.

LiQuImAS specified the Codec Quality Characterization Test with Binaural Rendering [1, clause 7] which is based on the test method defined in [4] (MUSHRA). It uses the Common Informative Binaural Renderer (CIBR, [6]) for both the reference and degraded conditions. The binauralization shall use either individualized HRTFs or HRTFs based on a head and torso simulator (HATS). The headphones are equalized as specified in [1].

# 3.2 Renderer Quality Tests in LiQuImAS

LiQuImAS specified the Renderer Comparison Test [1, clause 6]. In this test, the assessors compare a Test Condition against Anchor Conditions on the four audio quality attributes Timbre, Artifacts, Spatial Quality, and Basic Audio Quality and indicate any residual loudness differences between the conditions. The presentation of the Test and Anchor Conditions is binaural using head-tracking. For each trial, the Test Condition is compared to one of the Anchor Conditions as an A vs. B comparison.

# 4. Summary of the proposal

- To control the number of test conditions while maintaining comprehensive subjective testing results, the source proposes to develop similar testing paradigms as in LiQuImAS [1], decoupling the codec and renderer evaluation for the immersive/multichannel operation modes in IVAS, and further improving the methodologies as necessary.

- As indicated in the IVAS WID, the binaural listening mode is the essential presentation mode, consequently, head-tracked binaural audio is an important evaluation point.

# References

- 3GPP 26.259 “Subjective test methodologies for the evaluation of immersive audio systems“

- 3GPP 26.260 “Objective test methodologies for the evaluation of immersive audio systems”.

- 3GPP TR 26.818 “Virtual Reality (VR) streaming audio; Characterization test results”.

- ITU-R Recommendation BS.1534-3 "Method for the subjective assessment of intermediate quality level of audio systems".

- ITU-R Recommendation BS.1116-3 "Methods for the subjective assessment of small impairments in audio systems".

- 3GPP TS 26.118 "3GPP Virtual reality profiles for streaming applications".

- AHEVS-471 “IVAS Reference Conditions”, Fraunhofer IIS.

- AHEVS-473 “Keeping IVAS manageable”, Dolby Laboratories, Inc.