Source: 	Nokia Corporation

Title: 	Synthesis of XR 5G Use Cases

Document for	Agreement

Agenda item: 	10.6

# Introduction

This document attempts to synthesize the XR 5G use cases on the basis of the underlying offered functionalities, the nature of the communication, interactivity and real-time requirements. The main idea is to cluster the 20 use cases in Appendix A of [1] into a smaller set of categories, which are more manageable as we move forward into the next steps of the XR 5G standardization phases. Also, a minimum list of standardization needs is given. The synthesized use case categories are discussed in Section 2 and are illustrated with 1) a Figure to show an example architecture, 2) a compact text description of the functional scenario, and 3) a minimum list of combined standardization needs to cover the standardization gaps for the synthesized use cases covered in this document.

It should be clarified that this document is not meant to replace Appendix A of [1] as the use cases contain important details about implementation, feasibility and usage. The intention is to cluster them so that the 3GPP architectural requirements for XR can be easily extracted and streamlined. Table 1 shows the list of the synthesized use cases included in this document and the XR 5G use cases that are covered in [1].

| Category of Synthesized Use Cases | XR 5G Use Cases from [1] |
| --- | --- |
| Offline Sharing of 3D Objects | Use Case 1: 3D Image Messaging Use Case 2: AR Sharing Use Case 10: Online shopping from a catalogue – downloading |
| Real-time XR Sharing | Use Case 7: Real-time 3D Communication Use Case 8: AR guided assistant at remote location (industrial services) Use Case 11: Real-time communication with the shop assistant Use Case 17: AR animated avatar calls |
| XR Multimedia Streaming | Use Case 3: Streaming of Immersive 6DoF Use Case 4: Emotional Streaming Use Case 20: AR Streaming with Localization Registry |
| Online XR Gaming | Use Case 5: Untethered Immersive Online Gaming Use Case 6: Immersive Game Spectator Mode |
| XR Critical Mission | Use Case 9: Police Critical Mission with AR |
| XR Conference | Use Case 12: 360-degree conference meeting Use Case 13: 3D shared experience Use Case 14: 6DOF VR conferencing Use Case 15: XR Meeting Use Case 16: Convention / Poster Session |
| Spatial Audio Multiparty Call | Use Case 18: AR avatar multi-party calls Use Case 19: Front-facing camera video multi-party calls |

Table 1. Synthesized XR 5G use cases

# Synthesized XR 5G Use Cases

The following categories and their accompanying illustrations are drawn for clarity. In actual implementations, elements from more than one sytem may be used together. However, we have attempted to ensure that the functionalities included of any particular use case in Appendix A of [1] are fully covered within a single synthesized system diagram.

The term UE is used to define a 5G-enabled user device that meets the capability requirements of a particular use case. UE may be a mobile handset, AR glasses or a HMD with or without controllers. Note that depending on the actual UE, the usage scenario may differ slightly. Some examples of such differences are given in the accompanying text within each category.

## Offline sharing of 3D objects

Offline sharing is used for sharing 3D models/objects and 3D MR scenes amongst UEs. In Figure 1, UE A shares a 3D static/dynamic object with UE B. The 3D object can be a stored object downloaded by UEA from the cloud, or captured by the device using for example a depth camera. It may include additional information such as colour, texture, size, etc. of the 3D object, which we refer to as effects in the figure. Upon receiving, UE B can render this object (and/or 3D objects it has downloaded from the cloud) in the surrounding reality using a MR rendering engine; it can choose the desired effects for the 3D object. It can then capture the rendered MR scene and send it back to UE A. MMS is used for sharing the 3D object and the captured MR scene between the UEs.

The rendering functionality depends on the type of device. For instance, flat images on a mobile phone vs. 3D rendering on AR glasses.

Figure 1 Offline Sharing 3D Objects and MR Scenes

Minimal standardization needs:

•	Standardized format for 3D objects

•	Standardized format for mixed reality 3D scenes

•	Delivery protocol for 3D objects and scenes (MMS extension and/or other)

•	Decoding, rendering, composition API for 3D objects in an AR scene.

## Real-time XR sharing

UE B is a device capable of XR rendering, such as AR glasses, or a mobile phone that is sending a real-time video stream of the XR experience to UE A, as illustrated in Figure 2. The XR experience is being captured and rendered by the UE B. The experience includes capture of 3D objects in a 2D scene. The rendering experience includes (real-time) 3D (static/dynamic) object placement in a 2D scene, overlay video, avatars, etc., that may be downloaded from the cloud, or sent by UE A to UE B. A bidirectional or unidirectional A/V channel may be open between the devices depending on the use case. The received objects by UE B can be influenced by UE B directly as well, based on direct or indirect input from the user. UE A also sends a real-time stream of motion signals and effects that influence the rendering of the 3D object model on UE B. Examples include: 1) head/body motion or facial expressions resulting in corresponding changes in a dynamic 3D object, e.g. an avatar, and 2) positioning and size of the 3D object within the XR scene, and subsequent changes in these parameters such as moving the object closer to a wall or making it larger. Motion data may be collected using direct input from the user interface of the device or implicitly using data from camera, gyroscopes, accelerometer, etc., including gestures. Other predefined effects for the 3D objects that can be placed on or around it can also be shared from UE A to UE B or downloaded from the cloud.

In a subset of this scenario where XR is not used, a 3D object may be rendered within the received video stream, e.g. a 3D representation of the head of a video call participant.

Figure 2 Real-time sharing of XR content

Minimal standardization needs:

- Support of static/dynamic 3D objects in MTSI (formats and transport)

- Overlaid video rendering in the network or locally

- Synchronization of different capturing devices

- Coded representation of 3D depth signals and transport in MTSI

- Coded representation of XR scenes and transport in MTSI

- MTSI/FLUS uplink of XR video

- Downlink XR video with local/cloud computation and rendering

- Visual coding and transmission of avatars or cut-out heads, alpha channel coding

- Transports and potentially coding of motion data to show attentiveness.

## XR Multimedia Streaming

This category covers live and on-demand streaming of XR multimedia streams, which include 2D/3D streams that are rendered in XR with binaural audio as well as 3DOF and 3DOF+ immersive A/V streams. It is illustrated in Figure 3, where UE is a device capable of receiving and rendering the type of stream in use. It is also capable of controlling the playback of these streams using input from handheld controllers, hand gestures, biometric readings, body and head movements etc., which is communicated to the content server. Control signals include pause, rewind, viewpoint selection or, in case of emotional streaming, adaptive content selection.

In another system instance, the content server can provide Inter-destination Multimedia Synchronization (IDMS) for a group experience. A Spatial Computing Server is used by XR capable devices to register, compute, update and recall the spatial configuration of their surroundings. The service is meant for indoor spaces like a shared room or building. Appropriate surfaces may be selected for display of XR streams and saved in the Spatial Computing Server. The configuration can be shared amongst authorized users to enhance group experience when multiple users are physically sharing the same space.

Figure 3 XR Multimedia Streaming

Minimal standardization needs:

- Coded representation of Audio/Video Formats as well as geometry data for XR (volumetric, 3DoF+)

- Scene composition and description

- Storage and Cloud Access Formats

- Transport protocols to support any media beyond 2D streaming

- Decoding, rendering and sensor APIs

- Biometrics and Emotion Metadata definition and transport

- Seamless splicing and smooth transitions across storylines

- Format for storing and sharing indoor spatial information.

- Inter-destination multimedia synchronization for group/social experience.

## Online XR Gaming

The system as illustrated in Figure 4 consists of a game server capable of serving several online gamers. Each player UE receives a stream of the live game from the game server and sends control signals back to the server that influence the game play. Control signals include handheld controller inputs, biometric readings and 3DOF+ motion as required by the gameplay.

Other users may join the live game in spectator mode and can receive the stream from the perspective of an active player or a spectator view independent of other players; viewpoint changes may be possible. The spectator may enjoy an immersive experience or 2D view depending on the device. Optionally, the spectators may interact with the players through cheering or game reward systems. In an different instance, cloud rendering could be present.

Figure 4 Online XR Gaming

Minimal standardization needs:

- Streaming protocols for online gaming

- Decoding, rendering and sensor APIs

- Architectures for computing support in the network (see gap analysis in TR 22.842, clause 5.3.6)

- Coded Representation of Audio/Video Formats for gaming.

## XR Critical Mission

The system shown in Figure 5 is for critical missions using XR support. In this use case a team in an indoor space is equipped with mission gear that is connected to a centralized control center. The UE in the figure represents a single team member and there may be more than one device included in the misson gear, such as AR glasses, a 360 degree helmet-mounted camera, a microphone, binaural audio headphones and other sensors. 	The control center/conference server performs the role of mission support by providing XR graphics such as maps, text, location pointers of other team members or other objects/people in the surrounding, etc. The mixed audio of the team members as well as audio from control center is also delivered to the UE to aid team communication and coordination. One or more drone-mounted cameras may also be used, which can be controlled by the control center or one of the members of the mission team. The control center is equipped with A/V processing capabilities to extract important information from the multitude of 360 degree video feeds, for instance, for identifying moving objects.

Figure 5 XR Critical Mission

Minimal standardization needs:

- MTSI/FLUS uplink 3D audio

- MTSI/FLUS uplink XR streams

- Downlink XR video with overlaid graphics with local/cloud computation and rendering

- Downlink XR audio with mixed-in 3D audio objects with local/cloud computation and rendering.

## XR Conference

This system caters for an XR conference with multiple physically co-located and remote participants using XR to create telepresence. The shared conference space can be, 1) a physical space that is shared by local participants and sent as an immersive stream to the remote participants, 2) a virtual space that has the same layout as the physical space so that the physically present (local) and remote participants have a similar experience while moving in the space, and 3) a virtual space that is retrieved from an application server (AS). In the first two cases, the remote participants will use a HMD and the local participants will use an XR device, such as AR glasses. In the third case, all participants use a HMD.

Figure 6 illustrates the system. Virtual spaces and avatars are retrieved from the Application Server by the conference server. A spatial computing server is used for storing the layout of the physical space, when used. Remote participants would be seen as avatars within the XR experience located at their relative position in the shared space. Alternatively, they may be represented as themselves using a live video feed and video extraction to isolate the person from the background and using a photo-realistic representation of their face in place of the HMD. The required video processing is located in the conference server in Figure 6.

Remote participants are free to move around when 6DOF motion is used. The conference server processes the audio streams from the participants to create an XR experience. Participants will hear binaural audio of all participants according to their position and a 360-degree surround sound, if needed. If a physical space is used, the conference server would also receive and process input from one or multiple 360-degree A/V capture devices and RGB+depth camera. Note that when 6DOF is supported, all remote participants can move freely within the confines of the designated space, moving from one room to another when there are multiple rooms defined in the space. Using motion signals, relative positioning and location information, it would be possible for participants (local + remote) to form smaller groups for discussion within the XR space as would happen in a real space. The conversation/real-time XR stream shown in the figure is a mix of VR (remote user) or AR (local user) media, room layout (virtual/physical) and mixed binaural audio. The presentation pointer data may be sent from one of the UEs while presenting a shared presentation/poster for highlighting specific parts.

A top-view of the conference space showing its layout and the current positions of the participants can be viewed by participants and is indicated as part of the XR stream label in the figure (but as separate physical stream). The conference server should also provide IDMS.

Figure 6 XR Conference

Minimal standardization needs:

- Position and scaling of people

- Background audio / picture / video

- Shared content (i.e., video background), i.e. multi-device media synchronization

- Network Based Media Processing (e.g., cloud rendering, foreground/background segmentation of the user capture, replace HMD of the user with a photo-realistic representation of their face, etc.)

- Support for RGB+Depth video data.

- 6DOF metadata framework and a 6DOF capable renderer for immersive voice and audio.

- IVAS codec (including directionality of users) and related metadata.

## Spatial Audio Multiparty Call

The system shown in Figure 7 illustrates an AR multiparty call. Each party can see the other parties using 2D video streams captured by the front facing camera of a mobile phone. Alternatively, they can be displayed as avatars, for instance, when a pair of AR glasses are used as UE. Each party hears spatial audio with the audio of the other parties originating from where their avatar/video is placed on the display. Motion such as head turns are tracked to create a realistic impression that the audio is originating from that virtual direction.

In a special case, the avatars and the audio of the other parties on party A’s display is based on their actual geolocation and the relative direction they are with respect to party A. The same would be true for all parties. UEs also have the ability to switch to PTT to suppress surrounding sound if they wish. They may use the “hear what I hear” function to send a 3D audio of their surroundings to the other parties.

Figure 7 Spatial Audio Multiparty Call

Minimal standardization needs:

- Visual coding and transmission of avatars

- Coding of cut-out heads, alpha channel coding

- Audio coding and transmission of mono objects and 3D audio for streams from all participants.

# Overlap with ITT4RT WI

In the analysis, we found Use Case #12 in Appendix A of [1] was very closely related to the use cases discussed in ITT4RT [2].

# Proposal

We propose that the essential content of this contribution (Table 1 and section 2) is moved to the XR_5G Technical Report 26.928, and consider the new synthesized use cases as the new milestone from which further work in the XR for 5G area should be started in the SA4.

Also, we propose that the use case #12 from Appendix A of [1] should be considered for the ITT4RT WI as an extension of the current ITT4RT use case.

# References

[1] 3GPP Technical Report 26.928: “Extended Reality in 5G”.

[2] 3GPP Tdoc S4-190503, “ITT4RT Permanent Document – Requirements, Working Assumptions and Potential Solutions”.

Contact: Igor D.D. Curcio, Saba Ahsan, Nokia Technologies, Tampere, Finland, Email: igor.curcio, saba.ahsan@nokia.com.