3GPP TSG-SA4 Meeting #104	S4-190654

# Cork, Ireland,  June 1 - 5, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | <CR#> | rev | <Rev#> | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New VR metrics clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRQoE |  |  |  |  |  | Date: |  |  | 2019-06-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The work item VRQoE has been approved, aiming at adding relevant VR metrics to this specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for VR metrics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Difficult to measure characteristics related to the VR service performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 9 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ================ First change =================

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3DOF	3 Degrees of freedom

ACN	Ambisonics Channel Number

API	Application Programming Interface

AVC	Advanced Video Coding

BMFF	Base Media File Format

BRIR	Binaural Room Impulse Response

CMP	Cube-Map Projection

CIBR	Common Informative Binaural Renderer

DASH	Dynamic Adaptive Streaming over HTTP

DRC	Dynamic Range Control

EOTF	Electro-Optical Transfer Function

ERP	EquiRectangular Projection

ESD	Equivalent Spatial Domain

FFT	Fast Fourier Transform

FIR	Finite Impulse Response

FOA	First Order Ambisonics

FOV	Field Of View

GPU	Graphics Processing Unit

HDR	High Dynamic Range

HDTV	High Definition TeleVision

HEVC	High Efficiency Video Coding

HMD	Head Mounted Display

HOA	High Order Ambisonics

HRD	Hypothetical Reference Decoder

HRIR	Head-Related Impulse Responses

HRTF	Head-Related Transfer Function

HTTP	HyperText Transfer Protocol

IFFT	Inverse FFT

IRFFT	Inverse RFFT

MAE	MPEG-H Audio Metadata information

MCC	Metrics Collection and Computation

MHAS	MPEG-H Audio Stream

MIME	Multipurpose Internet Mail Extensions

MPD	Media Presentation Description

MPEG	Moving Pictures Experts Group

NAL	Network Abstraction Layer

OMAF	Omnidirectional MediA Format

PCM	Pulse Code Modulation

RAP	Random Access Point

RFFT	Real FFT

RWP	Region-Wise Packing

SDR	Standard Dynamic Range

SEI	Supplemental Enhancement Information

SN3D	Schmidt semi-normalisation

SOFA	Spatially Oriented Format for Acoustics

SPS	Sequence Parameter Set

SRQR	Spherical Region-wise Quality Ranking

VCL	Video Coding Layer

VST	Virtual Studio Technology

VUI	Video Usability Information

VR	Virtual Reality

## ================ Next change =================

# 9	VR Metrics

## 9.1	General

VR metrics is a functionality where the client collects specific quality-related metrics during a session. These collected metrics can then be reported back to a network side node for further analysis. The metric functionality is based on the QoE metrics concept in 3GP-DASH [8], but further extended to also cover VR-specific metrics. A VR client supporting VR metrics shall support all metrics listed in clause 9.3, and shall handle metric configuration and reporting as specified in clauses 9.4 and 9.5.

## 9.2	VR Client Reference Architecture

The client reference architecture for VR metrics, shown below, is based on the client architecture in Figure 4.3-1. It also contains a number of observation points where specific metric-related information can be made available to the Metrics Collection and Computation (MCC) function. The MCC can use and combine information from the different observation points to calculate more complex metrics.

Note that these observation points are only defined conceptually, and might not always directly interface to the MCC. For instance, an implementation might relay information from the actual observation points to the MCC via the VR application. It is also possible that the MCC is not separately implemented, but simply included as an integral part of the VR application.

Also note that in this version of this specification not all of the described observation points are necessarily used to produce VR metrics.

Figure 9.2-1: Client reference architecture for VR metrics

### 9.2.1	Observation Point 1

The access engine fetches the MPD, constructs and issues segment requests for relevant adaptation sets or preselections as ordered by the VR application, and receives segments or parts of segments. It may also adapt between different representations due to changes in available bitrate. The access engine provides a conforming 3GPP VR track to the file decoder.

The interface from the access engine towards MCC is referred to as observation point 1 (OP1) and is defined to monitor:

-	A sequence of transmitted network requests, each defined by its transmission time, contents, and the TCP connection on which it is sent

-	For each network response, the reception time and contents of the response header and the reception time of each byte of the response body

-	The projection/orientation metadata carried in network manifest file if applicable

-	The reception time and intended playout time for each received segment

### 9.2.2	Observation Point 2

The file decoder processes the 3GPP VR Track and typically includes a file parser and a media decoder. The file parser processes the file or segments, extracts elementary streams, and parses the metadata, if present. The processing may be supported by dynamic information provided by the VR application, for example which tracks to choose based on static and dynamic configurations. The media decoder decodes media streams of the selected tracks into the decoded signals. The file decoder outputs the decoded signals and metadata which is used for rendering.

The interface from the file decoder towards MCC is referred to as observation point 2 (OP2) and is defined to monitor:

-	Media resolution

-	Media codec

-	Media frame rate

-	Media projection, such as region wise packing, region wise quality ranking, content coverage

-	Mono vs. stereo 360 video

-	Media decoding time

### 9.2.3	Observation Point 3

The sensor extracts the current pose according to the user's head and/or eye movement and provides it to the renderer for viewport generation. The current pose may also be used by the VR application to control the access engine on which adaptation sets or preselections to fetch.

The interface from the sensor towards MCC is referred to as observation point 3 (OP3) and is defined to monitor:

-	Head pose

-	Gaze direction

-	Pose timestamp

-	Depth

### 9.2.4	Observation Point 4

The VR Renderer uses the decoded signals and rendering metadata, together with the pose and the knowledge of the horizontal/vertical field of view, to determine a viewport and render the appropriate part of the video and audio signals.

The interface from the media presentation towards MCC is referred to as observation point 4 (OP4) and is defined to monitor:

-	The media type

-	The media sample presentation timestamp

-	Wall clock counter

-	Actual presentation viewport

-	Actual presentation time

-	Actual playout frame rate

-	Audio-to-video synchronization

-	Video-to-motion latency

-	Audio-to-motion latency

### 9.2.5	Observation Point 5

The VR application manages the complete device, and controls the access engine, the file decoder and the rendering based on media control information, the dynamic user pose, and the display and device capabilities.

The interface from the VR application towards MCC is referred to as observation point 5 (OP5) and is defined to monitor:

-	Display resolution

-	Max display refresh rate

-	Field of view, horizontal and vertical

-	Eye to screen distance

-	Lens separation distance

-	OS support, e.g. OS type, OS version

## 9.3	Metrics Definitions

### 9.3.1	General

As the VR metrics functionality is based on the DASH QoE metrics [8], all metrics already defined in [8] are valid also for a VR client. Thus the following sub-clauses only define additional VR-related metrics.

### 9.3.2	Metric1

Text for Metric1.

### 9.3.3	Metric2

Text for Metric2.

## 9.4	Metrics Configuration

### 9.4.1	General

Metrics configuration is done according to clauses 10.4 and 10.5 in DASH [8], but can also include any metrics defined in clause 9.3 above.

## 9.5	Metrics Reporting

### 9.5.1	General

Metrics reporting is done according to clause 10.6 in DASH [8], with the additional VR-specific metrics reported according the the XML schema in clause 9.5.2.

### 9.5.2	Reporting Format

<?xml version="1.0"?> <xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:metadata:2019:VR:metrics" xmlns:sv="urn:3gpp:metadata:2016:PSS:schemaVersion" xmlns="urn:3gpp:metadata:2019:VR:metrics" elementFormDefault="qualified"> <xs:element name="VrMetrics" type="VrMetricsType"/> <xs:complexType name="VrMetricsType"> <xs:sequence> <xs:element name="Metric" type="MetricType" minOccurs="1" maxOccurs="unbounded"/> <xs:element ref="sv:delimiter"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="MetricType"> <xs:choice> <xs:element name="Metric1" type="Metric1Type"/> <xs:element name="Metric2" type="Metric2Type"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> </xs:schema>